3GPP TSG-RAN WG2 Meeting #131-bis	R2-2506855

Prague, CZ, 13th –17th October, 2025

Agenda Item:	10.2

Source:	Huawei, HiSilicon

Title:	General considerations for 6G in RAN2

Document for: Discussion and Decision

1	Introduction

The newly approved Study Item (SI) in [1] on 6GR access technology outlines detailed study objectives.

This contribution will discuss some general aspects on 6GR discussion in RAN2, including guideline, deployment scenarios and architecture, energy saving, new services, UE capability and NTN.

2	Discussion

2.1	Principle and Guideline

6GR aims to unlock new business opportunities, reduce Total Cost of Ownership (TCO), and enhance user experience. Its vision is to target a true generational leap—by integrating AI and sensing as native capabilities of the air interface—and by commiting to a green, energy-efficient design.

This design necessitates in-depth analysis and evaluation of both existing and emerging technologies by 3GPP Technical Specification Groups (TSGs)/Working Groups (WGs).

The principle should be considered applying to all candidate technologies proposed for 6GR: 3GPP TSGs/WGs (including RAN2) should adopt an open mindset, while conducting rigorous analysis of the potential benefits and challenges of each solution to fully unleash 6G’s potential.

Proposal 1: RAN2 should maintain an open approach when discussing candidate technologies, supported by thorough evaluation and analysis.

2.2	Deployment Scenarios and Architecture

In RAN#109 meeting, companies showed their views on the requirements for architecture and migration. There is broad consensus that 6G standalone scenario should be treated with highest priority with multi-RAT spectrum sharing (MRSS) and carrier aggregation.

Considering that the existing spectrum utilized by 5G may also be the frequency band for 6G deployment, 5G-6G multi-RAT spectrum sharing (MRSS) can be considered from 6G Day 1.

On the other hand, MRSS only provides efficient spectrum utilization on a single carrier without data aggregation gains. During the migration phase from 5G towards 6G, users are expected to have superior user experience from 6G compared to 5G. Considering 5G-5G CA penetration is increasing in the real-world network, 6G-6G CA needs to be supported as well to achieve the data aggregation benefits since 6G Day 1. However, the existing CA mechanism treats each carrier as an independent serving cell and assumes each carrier is deployed independently. This may introduce unnecessary overhead and latency. Therefore, a more efficient spectrum aggregation over one or more carriers/bands shall be considered[6].

Further, there are different views in the industry on whether to have additional migration option(s). Some companies are proposing 5G-6G NSA aggregation options at RAN-level, some others are proposing dual stack/dual steer aggregation options at CN-level. During RAN#109 meeting, it was fully discussed and the consensus was achieved that RAN plenary shall start this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). Correspondingly, the objective of migration in the SID was updated.

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Therefore, from RAN2 perspective, the discussion on requirements and solutions for 6G standalone scenario including Carrier Aggregation should be prioritized and the discussions regarding requirements and solutions specific to additional migration mechanisms should await the decisions from RAN plenary.

Proposal 2: RAN2 should prioritize discussions on requirements and solutions for the 6G standalone scenario, including enhanced spectrum utilization and aggregation. Discussions regarding requirements and solutions specific to additional migration mechanisms should await decisions from the RAN plenary.

2.3	Energy Saving

Figure 1. Energy saving techniques overview in 5G

The energy saving techniques specified in 5G for network and UE are provided in Figure 1. Although there are various techniques for energy saving, the commercialization for these features is limited. One reason is that most of the features were not 5G Day-1 features, but were gradually introduced in the later releases. Dedicated UE power saving features were started from Rel-16, while major network energy saving features were introduced in Rel-18. These power-saving features usually require support from both UEs and networks. Due to the late introduction of these energy-saving features, there is less support from UEs and networks, thus reducing the power-saving gains. Furthermore, in 5G energy saving for network and UE were motivated and specified independently. Although their cross-side impact was carefully studied during the study/normative phases, there is lack of common motivation among network and UE to support the corresponding techniques simultaneously. Thus, to realize practical benefits and ensure commercialization, 6GR should support native energy savings features via proper designs that consider both networks and UEs. Some preliminary considerations for energy saving technologies are provided in [5].

Observation 3-1: Energy-saving features typically require support from both UEs and networks. The earlier these energy-saving features are supported, the more UEs and networks will be able to adopt them—thus allowing the intended energy-saving gains to be achieved and maximized.

Observation 3-2: The lack of a shared motivation in power-saving design between networks and UEs will reduce the incentives for their commercialization.

Proposal 3: 6GR should support Energy Saving features from Day-1. RAN2 should strive to develop solutions that address both network energy efficiency and UE power saving, to enhance their chance of being commercialized.

2.4	New Services

Regarding the services in the RAN-level study on “6G Scenarios and requirements”, the objective of this SID includes:

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. …..

In this section, we provide the analysis of new services that may have impact on L2 protocol and functions in 6G.

- Immersive communication service

Immersive communication services are supposed to be the mainstream for future wireless communication networks. Recently, the booming AI services have further broadened the scope of immersive applications, from 5G-era services like XR to innovative mobile AI services. The typical use cases of immersive communication services have been specified in SA1 specification TR 22.870 [2].

According to the TR 38.914, the KPI called ‘composite requirement’ will be defined for the immersive communication service. This is because Immersive communication services impose stringent network performance requirements, including high data rate, low latency, and high reliability simultaneously. For example, cloud XR and holographic interactions may require downlink and uplink throughput of 500Mbps and 100 Mbps respectively, with latencies <20 ms while supporting hundreds of users per cell. Furthermore, different immersive communication services require different composite requirements in terms of different combination of service data rate and latency for DL and UL.

Observation 4-1: Immersive communication services have a composite requirement, which means the high data rate, low latency and high reliability need to be satisfied simultaneously for a minimum number of users per TRP.

- New AI services

The AI services powered by large AI/ML models such as large language models (LLM), vision language models (VLM), etc., are utilized widely for main-stream AI services, e.g., AI agent, chatbot, personal assistant and so on.

In these AI services, instead of transmitting raw input data (e.g., text, images), raw data are usually transformed into tokens for efficient transmission between AI server and client and for efficient processing by the AI model. Token is seen as a compact data carrying the context information, and can be generated and processed by AI models as shown in Figure 2. A tokenizer converts the raw data of multiple modalities (e.g., text, image, audio, etc.) to tokens at the transmitter side, and a corresponding de-tokenizer converts the tokens back to the raw data. Token plays a central role in enabling AI agents to understand and respond to natural language instructions, interpret environmental data, make decisions for robotic systems, communicate with other AI agents, etc.

Figure 2: Illustration of 6G token communication service for Agentic AI

The tokens generated by/for AI/ML models have lots of new requirements and special characteristics, which are different from traditional traffics.

- Error tolerance. Tokens are resilient to transmission errors, as they can be predicted based on the co-relationship among themselves via the multi-modal large models.

- Token importance differentiation. The tokens for different modality data and/or different tokens for the same modality data may have different importance depending on their contributions to AI/ML model inference, and they could be treated with different priorities during transmission.

From RAN2 perspective, the above requirements and characteristics of token communications should be considered and utilized in the transmission procedures, to reduce the latency, improve the efficiency, and finally improve the user experience. Potential enhancement under this direction may include enabling flexible reliability requirements or partial (re-)transmissions for token traffic (related to error tolerance) and so on.

Observation 4-2：For the new AI services (e.g., AI agent), transforming data into tokens is becoming a popular approach for efficient transmission and processing compared to transmitting raw input data. Token based communication introduces numerous new requirements and unique characteristics.

Based on the above observations, we propose that:

Proposal 4: RAN2's studies should consider the requirements of new application services emerging in the 6G era, such as immersive communication services and new AI services, by considering their characteristics.

- New AI services served by operators/networks

Considering the rapid progress of AI/ML technologies, forward compatibility to exploit the most powerful AI/ML methods and support the latest AI/ML services is important factor to consider for framework design. According to [3], many AI/ML use cases have been proposed in RAN1 for 6G. Meanwhile, SA1 also achieved agreements on many AI use cases and their service requirements [2]. It is then expected that 6GR should support use cases related to both AI/ML-enabled network and Network for AI (e.g. digital twin etc.).

As a typical AI/ML service, digital twin based on sensing has been discussed in SA1 [2]. To facilitate digital twin construction, sensing data needs to be collected and AI/ML methods can be utilized to facilitate this process. For example, to minimize overhead and energy consumption, each sensing node may only collect sparse samples of the environment, then distributed training and inference can be used to improve the precision and completeness of the environment information. The digital twin can provide environment information which can be used for both new 6G services (e.g., Collaborative robots, Smart Transportation, Smart City and etc.) and optimizing the air interface communication itself.

Figure 3: Distributed AI/ML enabled digital twin construction

Proposal 5: RAN2's studies should consider the requirements of new AI services to be served by networks/operators (e.g. digital twin).

2.5	Device type

According to 6GR SID, one objective is to support diverse device types as follows:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz). - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] …… g) Target scalable and forward compatible design for diverse device types.

It is expected that there are a number of device types supported in 6GR, for example, eMBB devices, IoT devices, and advanced device types supporting advanced services. The design target for each device type will be discussed in RAN plenary and RAN1 WG, such as the maximum bandwidth, coverage, and so on. As per the SID, the aim for RAN2 is to ensure a scalability and future compatible design to support the diverse device types from 6G Day-1.

In our view, RAN2 can wait for the conclusions from RAN plenary and RAN1 on the definition of device types.

Observation 6-1: Device types are being discussed in TSG RAN and RAN1 currently. RAN2 should await and align with the conclusions from TSG RAN and RAN1, and further identify the potential impacts to RAN2.

2.6	UE capability

According to the 6G SID, there are following objectives on UE capability signaling framework.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

From 5G NR experience, the UE AS capability size will keep expanding with every 3GPP releases due to more features being added and more spectrum being introduced. In some regions, the capability size observed in the field for 5G NR is over 20 Kbytes. UE capability in 5G NR is stored in core network to avoid frequent reporting via radio interface when transitioning from RRC_IDLE to RRC_CONENCTED. However, even though the stored UE AS capability is not used by the CN, the required storage overhead and transmission overhead over N2 interface is enormous. From RAN perspective, other than the storage and transmission overhead, the processing load for RRM procedure due to large capability signaling size also brings complexity, especially considering the increasing number of bands/band combinations. In total, a large UE capability size increases the cost for both RAN and CN in terms of storage, transmission and processing resources.

Observation 6-2: Very large UE capability size and number of UEs to handle result in high cost/requirement on storage, transmission and processing resources for the network.

To transfer a capability size of more than 9KB through a radio interface, UL RRC segmentation for UE capability was introduced in NR Rel-16. Maximum of 16 RRC segments is allowed when the segmentation is enabled by the network. However, in commercial deployments, due to the cost from network side as explained above (considering both RAN and CN), the network storage space to store the UE capability information for each UE may be limited. Thus, during NR Rel-19 stage, the maximum number of segmentations from 2 to 16 is further allowed to be indicated from network to the UE explicitly. For example, if the supported number of RRC segments for UE capability segmentation is 2 from network side while the UE capability size needs 3 RRC segments from the UE, the UE will not be able to report its full capability and has to limit the capability reported to the network. One example is the UE limits the reported capability from 3-band combinations to 2-band combinations. Such limit may result in degradation of user experience and network performance.

In other word, a large capability size will not only bring additional cost to network, but also restricts the performance on high-end UEs and networks.

Observation 6-3: Due to large UE capability signaling size and the limit to the message size by the network, the UE may not be able to report its full capabilities to the network, which may result in degradation on user experience and network performance.

Based on above, it is important for 6GR to study the enhancement and simplification on the structure of UE capability signaling, to reduce the capability signaling overhead. With the experience learnt from 5G, at least the following aspects need to be considered when designing the capability signaling structure in 6GR:

- The explosion of band combinations. The explosion comes from the continued extension of 5G frequency bands, as well as multiple combinations of capability parameters enumerated within a band combination.

- Different types of spectrum utilization, e.g. CA, UL Tx Switching. There are different BC lists specified in 5G to indicate CA combinations and UL Tx switching combinations.

- The asymmetric requirement between DL and UL. In cases where the same DL CA is associated with a single UL band, the pairing of that UL band with each individual DL band needs to be indicated explicitly.

- The capability granularity within a band combination is complicated, while some of the granularities are of less efficient in real deployment, e.g. FeatureSetCombination.

- The fallback principle on some capabilities is not clear in standard, which brings ambiguity on whether capabilities applicable for lower-order BC can be reported in a super BC.

In addition to the capability signaling structure, another aspect on UE capability framework is about UE capability reporting. To address the problems of UE capability overhead, UE capability ID (RACS) was introduced in 5G. There are two kinds of capability ID specified, manufacturer-assigned capability ID and PLMN-assigned capability ID. The assumption of RACS is to use one identifier to support the same set of capabilities with same model/software from the same UE vendor. However, RACS is not supported in 5G commercialization. One reason is the observed number of capability sets in field is much more than expected due to diverse reasons, e.g. user settings, software versions, etc. For example, the number of different UE capability sets observed in a cell during 1 day can be more than 10,000. Besides, obtaining the manufacturer-assigned UE capability ID is contingent upon effective collaboration among operators, network vendors, and UE vendors, which is difficult to achieve in practice.

Observation 6-4: The UE capability ID (RACS) is not commercially supported, because of the huge amounts of capability sets, and the difficulty of effective collaboration among operators, network vendors and UE vendors.

Furthermore, for many 5G features defined in RAN1 and RAN4, the appropriate level for defining capability signaling has sometimes not been thoroughly studied. As a consequence, capability is only reported at the finest granularity (e.g., per band combination per band), that results in excessively large capability signaling volumes, especially as more features and their associated capability parameters are introduced in 5G. It is therefore recommended that TSG RAN and RAN WGs carefully discuss the granularity of each capability when defining feature lists for each release.

Observation 6-5: Due to lack of thorough study, many 5G feature capabilities defined in RAN WGs are just reported at the finest granularity (e.g., per band combination per band), that results in excessively large capability signaling volumes.

In 5G, there are specific use cases where the User Equipment (UE) needs to update its capabilities, such as when experiencing overheating or in Multi-SIM (MUSIM) scenarios. For these use cases, User Assistance Information (UAI)-based mechanisms have been specified in 5G. If enabled by the network, the UE is allowed to indicate its preference regarding the update of specific capability parameters. However, UAI-based mechanisms must first be enabled by the network. To date, these mechanisms have not been commercialized in real-world deployments, primarily due to operator concerns—such as potential impacts on network Key Performance Indicators (KPIs), increased processing load, and other related factors.

According to the analysis above, we think RAN2 should first conduct a study on the issues related to the existing UE capability signaling framework, including capability signaling structure, capability reporting and update, and so on.

Proposal 6: RAN2 should conduct a study on issues related to the existing UE capability signaling framework.

2.7	NTN

6GR intends to enable new business opportunities, reduce total cost of ownership (TCO) and improve user experience. In the updated RAN study item [1], the following objective was agreed:

- Aim at a harmonized 6G Radio design for TN and NTN, including their integration

In 6G, a harmonized Radio design for TN and NTN can help expand markets of both cellular communication and satellite communication from day 1. It gives the chance to use the same ecosystem for TN and NTN to reduce the cost of both network and terminal manufacturing.

Observation 7-1: The harmonized 6G Radio design for TN and NTN can reduce the overall cost by sharing eco-system..

Therefore, RAN2 should strive to maximize the common design between TN and NTN. However, considering the satellite capability, deployment difficulty, and UE capability on RF, some NTN-specific designs are inevitable. For example, satellite coverage is significantly larger than that of terrestrial networks (TN). This extensive coverage may impact system information design, which in turn affects energy consumption on both the network and UE sides. Additionally, unlike TN, low Earth orbit (LEO) satellites provide coverage with fast moving cells. This can influence UE mobility performance, thereby affecting user experience. Another key difference is that UE RF capabilities are not optimized for transmission and reception in non-terrestrial networks (NTN); for instance, a handheld UE can only achieve the intended optimal antenna gain when the user manually aligns the device’s direction with the satellite. Consequently, UEs may miss paging messages due to insufficient link budgets, which further degrades user experience. As such, NTN-specific paging designs require further consideration—for example, extending NTN paging coverage. In summary, on  top of the harmonized 6GR radio design, RAN2 should conduct studies on essential NTN-specific design aspects.

Therefore, we propose:

Proposal 7: RAN2 should strive to maximize common design between TN and NTN, while studying only the essential NTN-specific aspects.

# 3	Conclusion

This contribution makes the following proposals:

Proposal 1: RAN2 should maintain an open approach when discussing candidate technologies, supported by thorough evaluation and analysis.

Proposal 2: RAN2 should prioritize discussions on requirements and solutions for the 6G standalone scenario, including enhanced spectrum utilization and aggregation. Discussions regarding requirements and solutions specific to additional migration mechanisms should await decisions from the RAN plenary.

Observation 3-1: Energy-saving features typically require support from both UEs and networks. The earlier these energy-saving features are supported, the more UEs and networks will be able to adopt them—thus allowing the intended energy-saving gains to be achieved and maximized.

Observation 3-2: The lack of a shared motivation in power-saving design between networks and UEs will reduce the incentives for their commercialization.

Proposal 3: 6GR should support Energy Saving features from Day-1. RAN2 should strive to develop solutions that address both network energy efficiency and UE power saving, to enhance their chance of being commercialized.

Observation 4-1: Immersive communication services have a composite requirement, which means the high data rate, low latency and high reliability need to be satisfied simultaneously for a minimum number of users per TRP.

Observation 4-2：For the new AI services (e.g., AI agent), transforming data into tokens is becoming a popular approach for efficient transmission and processing compared to transmitting raw input data. Token based communication introduces numerous new requirements and unique characteristics.

Proposal 4: RAN2's studies should consider the requirements of new application services emerging in the 6G era, such as immersive communication services and new AI services, by considering their characteristics.

Proposal 5: RAN2's studies should consider the requirements of new AI services to be served by networks/operators (e.g. digital twin).

Observation 6-1: Device types are being discussed in TSG RAN and RAN1 currently. RAN2 should await and align with the conclusions from TSG RAN and RAN1, and further identify the potential impacts to RAN2.

Observation 6-2: Very large UE capability size and number of UEs to handle result in high cost/requirement on storage, transmission and processing resources for the network.

Observation 6-3: Due to large UE capability signaling size and the limit to the message size by the network, the UE may not be able to report its full capabilities to the network, which may result in degradation on user experience and network performance.

Observation 6-4: The UE capability ID (RACS) is not commercially supported, because of the huge amounts of capability sets, and the difficulty of effective collaboration among operators, network vendors and UE vendors.

Observation 6-5: Due to lack of thorough study, many 5G feature capabilities defined in RAN WGs are just reported at the finest granularity (e.g., per band combination per band), that results in excessively large capability signaling volumes.

Proposal 6: RAN2 should conduct a study on issues related to the existing UE capability signaling framework.

Observation 7-1: The harmonized 6G Radio design for TN and NTN can reduce the overall cost by sharing eco-system.

Proposal 7: RAN2 should strive to maximize common design between TN and NTN, while studying only the essential NTN-specific aspects.

# 4	References

- RP-251881, “New SID: Study on 6G Radio”, 3GPP TSG RAN meeting #108, Prague, Czech Republic, June 9-13, 2025.

- RP-252951, Text proposal on spectrum utilization and requirements, Huawei.

- TR 22.870, Study on 6G Use Cases and Service Requirements, Stage 1, Release 20.

- R1-2506456, Moderator summary #2 on AI/ML for 6GR, Samsung (Moderator)

- R2-2506857, Energy efficient and unified RRC state modelling, Huawei, HiSilicon.

- R2-2506856, Discussion on 6G control plane, Huawei, HiSilicon.