3GPP TSG-RAN WG2 Meeting #131-bis	R2-2506856

Prague, CZ, 13th –17th October, 2025

Agenda Item: 10.3.2

Source:	Huawei, HiSilicon

Title:	Discussion on 6G control plane

Document for: Discussion and Decision

1	Introduction

In the WG SID for 6G [1], the 6G control plane and AS security have been listed as of the discussion objectives for radio interface protocol architecture and procedures as below:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we use the 5G control plane functions as the baseline for analysis and provide the initial considerations for the 6G control plane design.

2	Discussion

2.1 Initial access

2.1.1 System information

In 5G, SI scheduling adopts a mechanism based on the SI window. There are three SI scheduling lists and four SI window determination mechanisms for different SIBs or posSIBs summarized as below:

Figure 1. SI window determination mechanisms in 5G

The complexity of the SI window scheduling mechanism stems from its initial design, in which simplicity is overly pursued yet resulting in insufficient utilization of time-domain resources. As the number of SIs increases, the difficulty of window arrangement gradually increases. The window mechanism has been progressively improved to more efficiently utilize unused time-domain opportunities, however, this has led to the current complexity of the SI window mechanism. It would be better to re-design a new SI scheduling mechanism to allow flexible and unified SI scheduling mechanism, rather than to further enhance the current SI scheduling mechanism.

Observation 1-1:	 The current SI scheduling mechanism has become overly complex due to multiple enhancements aimed at better utilizing time-domain opportunities yet at late 5G stage.

In 5G, there are some SI scheduling principles. The details are as follows:

  - Principle1: Only SIBs having the same periodicity can be mapped to the same SI message.

  - Principle2: The SI-windows of different SI messages do not overlap.

Due to the constraints of the above principles, in 5G, SIBs with different periodicity can only be mapped to different SI windows, and these SI windows are sequentially arranged in the time domain. In this case, the network needs to continue sending SI messages over a relatively long period if number of SIs is large or SI window is long, preventing it from entering a sleep state even in low load scenarios.

Observation 1-2:	 The current SI scheduling mechanism restricts the network's ability to enter sleep mode for energy saving.

In 5G, the UE needs to (re)-acquire SIBs upon cell (re)selection, handover or receiving the indication of system information change, etc. However, the change of the cell does not mean the change of the content of SIBs. In the real deployment, the SIBs for most of the intra-frequency cells within a certain area are the same. Parameters of the SIBs are adjusted only for hotspot deployment (more carriers deployed) or some specific cells having different interference situation. UE power could be wasted due to re-acquisition of the SIBs with the same contents. Moreover, unnecessary re-acquisition of the same SIBs may lead to SI request for on-demand SIBs, which could cause more broadcast signaling and energy consumption for the network. Although the feature of system information area was introduced in 5G, it is not widely implemented in the field due to the complexity for the network to coordinate system information for all related cells, i.e., it is difficult for the gNBs to coordinate with each other to manage the system information area. For 6G, a more implementation friendly solution to reduce the unnecessary SI acquisition needs to be considered.

Observation 1-3:	 As the typical implementation for SI acquisition in 5G, UE needs to re-acquire SIBs upon cell change even if the SIBs of the new cell are the same with the stored versions. It causes unnecessary SI broadcast signaling and energy consumption for both UE and network.

Proposal 1: 6G System information design should consider energy saving friendly (for both network and UE) SI scheduling/acquisition mechanism.

In 5G, the TB size for system information doesn't exceed 2976 bits which imposes a limitation on information configured by SIBs especially for SIB1. We have observed scenarios in real-world networks where features configurations are limited due to SIB1 size constraints.

To ensure that the size of SIB1 does not exceed the physical layer SIB1 size limitation, network has to disable certain functions. Considering that 6G targets higher frequencies, it is possible that in the physical layer SIB1 size limitation will not be relaxed. RAN2 should study the issues related to SIB size restriction and corresponding solutions, together with RAN1.

Observation 2-1:	 In 5G, the SIB1 size limitation may prevent the network from enabling certain features in some scenarios.

Proposal 2: RAN2 should study the issues related to SIB size restriction (especially for SIB1).

2.1.2 Paging

In 5G design, PFs/POs are uniformly distributed in time-domain and the network has to frequently transmit paging signals across all time-domain positions, thus the network will be prevented from entering the sleep mode for energy-saving. In 5G-A, in order to enable the adaptation of paging occasions in the time domain for energy saving, features such as N/Ns adjustments have been introduced. The interval between PFs can be increased while maintaining the same number of total POs as in 5G. However, this method can only achieve a limited level of energy saving, and can not achieve optimal energy efficiency among all possible paging cycles. On the UE side, some UEs need to keep awake for a long time to monitor their own POs after synchronization, especially when the PFs/POs are distributed over long paging cycles. Thus, from 6G Day1, it is necessary to consider a paging framework that is beneficial for both network-side and UE-side energy efficiency.

Fig.2. PF/PO distribution in time domain

Proposal 3: 6G paging design should consider support of energy saving (for both network and UE) friendly PF/PO distribution pattern.

2.2 Cell management

In last RAN plenary meeting, following text proposals on spectrum utilization were agreed to be captured in the TR 38.914 as below:

5.3.1 Spectrum Editor note: e.g. spectrum and co-existence related 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

The text proposals above were discussed based on the lessons learned from 5G, including the complexity on cell management, inefficiency on spectrum resource utilization, and inflexibility on spectrum deployment. To address these issues, two directions were proposed in RAN1 in [2]: single carrier-like spectrum aggregation, and DL/UL decoupling. the potential impacts for these two directions need to be studied in RAN2 also.

For single carrier-like spectrum aggregation, i.e. aggregating multiple discrete spectrums within a single carrier (virtual carrier), there would be a new definition of the cell compared with 5G. From RAN2 perspective, with the single carrier-like spectrum aggregation, the legacy cell-specific signaling and procedures, e.g. system information, paging, initial access will be impacted. When there is more than one carrier in a cell, it needs to be studied whether the SIB/paging message can be received on only one carrier within the cell. For initial access, whether one or more carriers can also be configured for RACH to increase the efficiency of cell access needs to be studied.

Another direction discussed in RAN1 is downlink (DL) and uplink (UL) decoupling, which refers to the flexible pairing of DL and UL carriers. To utilize spectrum resources efficiently, the carrier(s) used for UL scheduling can be flexibly paired with those used for DL scheduling across different frequency bands. In other words, there are no restrictions on the number of UL carriers, nor on the frequency positions of UL carriers when paired with DL carriers. The associated DL and UL carriers may be treated as a single cell, or as two separate cells—specifically, a DL-only cell and a UL-only cell. To support DL and UL decoupling, the UE capability signaling and Radio Resource Control (RRC) configuration need to be designed to enable such flexible pairing between DL and UL carriers.

In a word, RAN2 needs to take RAN1 progress into account, and discuss the cell modeling for single carrier-like spectrum aggregation and UL/DL decoupling, including e.g. cell definition, SIB/Paging/RACH carrier mapping, carrier management.

Proposal 4: RAN2 should discuss on the cell modelling/definition for e.g. single carrier-like spectrum aggregation, downlink/uplink decoupling, in collaboration with RAN1.

2.3 RRC signalling design

According to the New SID on 6GR [1], the following objectives should be studied for 6G design:

g) Target scalable and forward compatible design for diverse device types. k) System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

The RRC signalling design is one of the important factors which impact these two objectives. In the NR evolution from Rel-15 to Rel-18, approximately 150 features have been introduced in the RRC spec, which led to the rapid growth of 5G RRC IEs. As illustrated in Figure 1, the number of RRC IEs in Rel-15 NR has increased several times compared to LTE Rel-8. With the evolution of NR, the RRC IEs in Rel-18 have grown more than 4 times further compared to Rel-15. It can be foreseen that the number of RRC IEs in 6GR would further increase, as 6GR is expected to support further new services and new scenarios such as AI and sensing.

Fig.3 The number of RRC IEs in several Releases

In NR RRC signaling design, the IEs for different features are interwoven with one another, as illustrated in Fig.4 below:

Fig.4 Interwoven structure of NR IEs

As a result, implementing RRC IEs for one specific feature unavoidably requires comprehension of whole RRC IEs structure that contains non-relevant features as well in the same release. Furthermore, from the UE side, it is even somehow necessary to comprehend RRC fields and IEs of non-relevant features from the robust signalling processing point of view (e.g. so that the UE can skip the unsupported features in case the NW configures to UE in some abnormal scenario, which was observed in real network occasionally). For example, in order to implement the RRC IEs for feature Z (e.g. IE#3, IE#6 and IE#7), the transmitter and receiver must support the related encoding/decoding operation of preceding IEs, including IE#1 and IE#4 for feature X, and IE#2 and IE#5 for feature Y, since the RRC IEs are encoded/decoded sequentially. Consequently, in terms of UE implementation, it is difficult to tailor the RRC IEs for certain features, i.e. only the RRC IEs for the features to be developed. This prevents the reduction in RRC implementation complexity and memory cost, particularly for low-end UEs which are cost-sensitive. It should be highlighted that even high-end UEs support only about 10-20 out of 150 commercial features in NR, while low-end UEs may support much less features. The additional cost in RRC memory due to the interwoven structure of IEs is considerable.

Observation 5-1: In NR RRC signalling design, implementing RRC IEs for a specific feature requires comprehension of RRC IEs for other features in the same release, due to the interwoven structure of IEs across different features. It is difficult for UE to tailor the IEs only related to the implemented feature, and this is particularly important for low-end UEs, which only implement quite limited features and are sensitive to the memory cost.

In addition, the interwoven structure of RRC IEs across different features also has negative impact on handover performance. Currently, when the UE configuration from the source gNB includes any unrecognized configurations due to ASN.1 version difference, the target gNB must include the full configuration in the handover command. The main reason is that the target gNB fails to interpret the unknown configurations and then can’t indicate the UE to release these configurations accurately, under the interwoven structure of RRC IEs across different features. If RRC IEs for different features could be designed in independent manner, it would be easy for the target gNB to identify the unknown features and to indicate the UE to release the configurations for these features separately, thus full configuration could be avoided during handover.

Observation 5-2: As the result of interwoven structure of IEs, the target RAN has to use full configuration to reconfigure UE if some features in the source configuration are not known by the target RAN during handover.

From the compatibility perspective, one of the lessons learned from NR is that compatibility issues cannot be completely avoided. During NR discussion, NBC changes in RRC signaling are only allowed in the very initial stage of each release of specifications (e.g. before -vXX.3.0). For a number of features which are not deployed in commercial networks, NBC changes can still be beneficial after the first frozen version because they are easier to read and with minimal signaling overhead. However, in the current interwoven structure of RRC IEs across different features, NBC changes for these features would impact other features (e.g., commercialized features) in the same release and then need to be completely avoided after the first frozen version.

Observation 5-3: The interwoven structure of IEs and features are not friendly to signalling updates.

At the beginning of 6G RRC design, RAN2 should identify the problems of NR RRC design and strive to avoid them in 6G RRC design.

Proposal 5: RAN2 should first identify potential issues for existing RRC signalling before considering 6G RRC signalling design.

2.4 AS security

In the September RAN plenary, an LS [3] to SA3 was approved where RAN invites SA to consider adding a June 2026 check point for the alignment between SA3 and RAN2 on any AS security requirements that impact the AS protocol design. It should be SA3 to first discuss on the limitations of AS security first and RAN2 can take further action based on SA3 input if any.

During the discussions at the 6G Workshop and SID (for 6G SA3 security studies), there was question raised on the security protection of MAC CE. Currently, the set of supported MAC CEs in 6G remains unclear. In RAN2 the design targets on how to support new services and features need to be first discussed, and then the potential MAC CE design can be identified. With the potential identified MAC CEs SA3 can conduct a study on the necessity of security protection, and RAN2 can consider whether and how to impact on the detailed protocol design.

Other aspects like control plane security, security for supporting new services, security for user privacy need also to be identified by SA3 first, and RAN2 can consider these requirements on the protocol design.

Proposal 6: RAN2 should first await SA3’s discussion on the requirements for 6G AS security.

2.5 Focus of RRC states

In NR, three RRC states have been defined: RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED, among which RRC_INACTIVE is the additional state introduced compared with LTE. The intention for introducing three states is to balance the performance between UE power consumption and latency to access to the network. However, in reality RRC_INACTIVE was not successfully commercialized. According to our companion contribution [4], the reasons are analyzed as below:

-	Transition from RRC_INACTIVE to RRC_CONNECTED does not provide short enough state transition latency to support services with low latency requirement

-	RRC_INACTIVE does not obtain better performance of UE power consumption compared with RRC_IDLE

In 6G RRC state design in our companion contribution [4], one of the targets for RRC CONNECTED state is to achieve both short latency and power saving, by incorporating two modes (i.e., "no-data low-power mode" and "data transmission mode"), and to support fast switch between these two modes. In addition, we analyze in our companion contribution [4] that RRC IDLE state should be used for the UE’s initial power-on phase and long-term sleep phase.

As for the RRC INACTIVE state, from the requirement perspective, if the 6G RRC CONNECTED state can achieve both short latency and power saving, RAN2 does not need to discuss the RRC INACTIVE state at the beginning. Therefore, when RAN2 investigates the 6G study on the control plane functionalities, it should be sufficient to only consider the essential RRC CONNECTED and RRC IDLE during the initial study phase.

Proposal 7: When studying control plane requirements and functions, as the starting point, RAN2 should focus on 6G CONNECTED state and 6G IDLE state.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1-1:	 The current SI scheduling mechanism has become overly complex due to multiple enhancements aimed at better utilizing time-domain opportunities however at late stage.

Observation 1-2:	 The current SI scheduling mechanism restricts the network's ability to enter sleep mode for energy saving.

Observation 1-3:	As the typical implementation for SI acquisition in 5G, UE needs to re-acquire SIBs upon cell change even if the SIBs of the new cell are the same with the stored versions. It causes unnecessary SI broadcast signaling and energy consumption for both UE and network.

Proposal 1: 6G System information design should consider energy saving friendly (for both network and UE) SI scheduling/acquisition mechanism.

Observation 2-1:	 In 5G, the SIB1 size limitation may prevent the network from enabling certain features in some scenarios.

Proposal 2: RAN2 should study the issues related to SIB size restriction (especially for SIB1).

Proposal 3: 6G paging design should consider support of energy saving (for both network and UE) friendly PF/PO distribution pattern.

Proposal 4: RAN2 should discuss on the cell modelling/definition for e.g. single carrier-like spectrum aggregation, downlink/uplink decoupling, by considering RAN1 progress.

Observation 5-1: In NR RRC signalling design, implementing RRC IEs for a specific feature requires comprehension of RRC IEs for other features in the same release, due to the interwoven structure of IEs across different features. It is difficult for UE to tailor the IEs only related to the implemented feature, and this is particularly important for low-end UEs, which only implement quite limited features and are sensitive to the memory cost.

Observation 5-2: As the result of interwoven structure of IEs, the target RAN has to use full configuration to reconfigure UE if some features in the source configuration are not known by the target RAN during handover.

Observation 5-3: The interwoven structure of IEs and features are not friendly to signalling updates.

Proposal 5: RAN2 should first identify potential issues for existing RRC signalling.

Proposal 6: RAN2 should first await SA3’s discussion on the requirements for 6G AS security.

Proposal 7: When studying control plane requirements and functions, as the starting point, RAN2 should focus on 6G CONNECTED state and 6G IDLE state.

# 4	References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, 3GPP RAN#107, March, 2025.

- R1-2505181 Overview of 6GR air interface, Huawei, HiSilicon

- RP-252891 LS on Early Alignment on Access Stratum security aspects

- R2-2506857	Energy efficient and unified RRC state modelling, Huawei, HiSilicon