3GPP TSG-RAN WG2 Meeting #131bis	R2-2506859

Prague, Czech Replublic, 13th – 17th October 2025

Agenda item:		10.3.2 (Control Plane)

Source:		Lenovo

Title:		6G Radio protocol architecture - Control Plane Aspects

Document for:		Discussion and Decision

# Introduction

3GPP has started working on 6G specification and this will be a first 6G meeting for RAN2. The study in RAN2 will commence according to the SID approved in [1]. This document takes a look at the important control plane aspects, starting with an analysis of 5G CP as background.

# Discussion Background

We are at an interesting point in time where the standardization of 5G will slowly pave way for the next generation, 6G. Comparison between the predecessor and current generations is a norm. Like comparisons have been made between 4G and 5G success, some claiming that the latter has not seen LTE kind of success which arguably may hold some merit since the vision of 5G was mainly centred at the verticals. Fast forward - in years to come 6G will be compared against 5G. From purely subscription point of view 5G has been seeing stronger deployments recently which should keep growing in years to come. Here is some interesting data before we start our technical discussion.

Figure 1: 5G growth expectation until 6G deployment

From, the Figure 1, whether the potential (z-score) for 5G means a regional delay for 6G adoption or rather depicts a bigger Pie for 6G investment can be individually interpreted by the reader!

With this, let’s start our control plane discussion.

The 5G Radio Resource Control (RRC) protocol specifies UE’s Idle/ Inactive state behaviour and how the UE accesses, maintains, and leaves the network. RRC manages UE states (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED) and the associated signaling procedures.

Key aspects include connection establishment and release, where the UE sets up SRBs for control signaling, and system information acquisition, which provides essential parameters for access and mobility. RRC also configures signaling and data bearers (SRBs/DRBs), mapping them to lower layers. It controls radio measurements and reporting, which form the basis for mobility decisions such as handovers, dual connectivity, and conditional handover execution.

The protocol ensures security activation (ciphering and integrity protection) by distributing updated/ refreshed keys to lower layers. It supports QoS management, mapping flows to bearers with appropriate priority and reliability. Failure recovery mechanisms like re-establishment, cell group specific and specific mobility feature specific failure handling/ recoveries are also integral part of the RRC protocol.

With 5G enhancements, RRC now supports advanced mobility (CHO, LTM, SCPAC), network slicing, and energy-saving mechanisms. Overall, the RRC protocol gNB control to guarantee service continuity and efficient radio resource use. In 6G we should accomplish the same and even better while reducing complexity and taking forward compatibility into account.

With this let us take a look at the main CP Aspects as outlined in the agenda for this meeting.

# Main Control Plane Aspects

The following main aspects should be considered from the beginning for 6G control plane:

## RRC Specification Modelling

RRC specification is arguably one of the most lengthy and complex specification instead of efforts pulled over the years from some of the greatest rapporteurs. This is also the fundamental specification when it comes to Access Stratum Control Plane implementation. Efforts are underway to modernize this (and others) specification. Nevertheless, RAN2 should engage in some fundamental changes, going forward to ease a reader’s life. Here are some salient points.

- Very lengthy specification compromises readability esp. due to nested features: A different specification modelling can be considered: Base-specification + individual functional specifications with their own self compile-able ASN.1 (even with importing ASN.1 from the base specification)

- Long list of specified features, some with almost no deployment (potential): Realistic (e.g., commitment based) deployment evaluation for approving features [RAN Plenary]. This should be discussed directly in RAN Plenary based on company contributions and we do not think any RAN2 input at his point is necessary.

Proposal 1: To tackle long, nested RRC specifications (mainly xx.331 and xx.304) RAN2 should study if a different specification modelling (Base-specification + individual functional specifications with their own self compile-able ASN.1 (even with importing ASN.1 from the base specification)) can be pursued. This should happen in the beginning and parallel to the “Modernization” discussions.

## RRC State Modelling

5G NR RRC state modeling defines UE behavior across Idle, Inactive, and Connected states. Idle conserves power with cell reselection; Inactive retains UE context for faster resume with reduced signaling; Connected supports active data transfer and mobility. The model balances energy efficiency, signaling load, and user experience. All 3 states specified in 5G NR have their advantage. RRC Connected and RRC Idle states are indispensable. The latter is the default state after the UE powers on and the UE must continue in this default state until at least the registration procedure with core network is completed.

RRC Inactive state can combine the best of both other states (data transfer possibility of RRC Connected + Power optimized state like RRC Idle) but the deployment of RRC Inactive state is slow. But we note the following points:

- But from this year NA carriers are apparently starting to deploy RRC inactive state.

- With or even without SDT has the potential for harnessing power savings, signalling reduction, better latency for small traffic / IoT / RedCap.

- Limited to a RNAU in 5G.

Observation 2: RRC inactive state has the potential for harnessing power savings, signalling reduction, better latency for small traffic / IoT / RedCap. Initial deployment has started.

Proposal 2: RRC states from 5G could be taken as baseline and a study for the feasibility and need for RRC Inactive State enhancement can be carried out at an early stage of 6G.

## RRC Connection Management

In 5G NR, RRC connection management governs the transition of a UE between idle, inactive, and connected states, balancing signaling overhead, latency, and energy efficiency. When a UE requires data or control signaling, it establishes an RRC connection through an RRC Setup procedure with the gNB. In connected mode, the network configures radio bearers, security, and mobility parameters. Release 15 introduced the RRC Inactive state to reduce signaling and improve resume latency, while preserving context at both UE and gNB. Enhancements in later releases optimize paging, mobility handling, and state transitions, enabling efficient resource use while maintaining user experience. This subtopic can be broken into: Transition to and from RRC Connected + Handovers. As Measurement, Mobility and UE capability is treated under different AI and so, the remaining question for now is as follows:

- Do we need to support handovers to and from LTE (4G)? This is not an urgent question and a note in the SID is already added [1]:

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

So, in end effect nothing new to study immediately. From the connection management perspective, seems 5G NR can be used as a baseline and no immediate need is seen to study something new.

Proposal 3: 5G RRC Connection Management (at least Transition to and from RRC Connected) can be taken as baseline.

## Initial and system access

In a 5G NR system, using initial access a User Equipment (UE) establishes a connection with the network for the first time. This involves cell search, synchronization with the Synchronization Signal Blocks (SSBs), acquiring Master Information Block (MIB), at least System Information Blocks Type 1(SIB1), and performing random access to request resources. The initial access enables the UE to register with the CN (and thereby receive Paging) and maintain connectivity, including authentication, security setup etc. The following aspects are interesting here when we design for 6G:

- Particularly in MRSS (5G-6G Multi-RAT Spectrum Sharing for migration) scenario, which is one of the Migration solution agreed to be studied in the SID, for the cell search and initial camping procedure, whether it is feasible to minimize duplication of potentially common DL transmission (synchronization with the Synchronization Signal Blocks (SSBs)) should be studied. 5G UEs should not be affected as these are already in field. This encompasses PSS, SSS and also MIB/ PBCH information. SSB duplication minimization can save a significant part of otherwise about 3.5% SSB overhead in FR2, 100 MHz BW @ 120 kHz SCS with 64 SSB transmission at 20 ms periodicity. [RAN1 and RAN2]

- RACH simplifications: Evaluation of real RACH load considering both MO and MT accesses – do we need such a complex partitioning/ prioritization mechanism [User Plane].

- Intended cell coverage area(s) and Msg3 size [RAN1]: This is fundamental to understand what Msg3/ MsgA sizes are realizable given the approved deployment scenarios in [2]. This will affect many aspects of our fundamental system access design including how fast the network gets to know the UE (type, identity, establishment cause etc.). RAN2 should discuss the right time to ask for RAN1 opinion on this aspect.

- Access Class and Barring simplification: While SA groups will study the Access classes and Access Categories, RAN2 should focus on cell barring aspects. In 5G NR, release wise we have introduced too many barring bits (cellBarredATG, cellBarred-eRedCap1Rx, Barred-eRedCap2Rx, cellBarredNES, cellBarredNTN, cellBarred-RedCap1Rx, cellBarred-RedCap2Rx). This led to specification complexity and worse to keep out the legacy UEs (since these are not the intended feature specific UEs) the MIB barring was relied upon – which has different implications based on local regulation and devoid legacy UEs of making even emergency calls! For this reason, since we start in 6G with a clean slate, we should collect/ shortlist stringently useful features and implement these from first release of 6G. At the same time, we should develop future compatibility to avoid such issues in 6G.

Observation 4a: SSB duplication minimization in MRSS can save significant overhead (e.g., about 3.5% SSB overhead in FR2, 100 MHz BW @ 120 kHz SCS with 64 SSB transmission at 20 ms periodicity).

Observation 4b: To keep out the legacy UEs (since these are not the intended feature specific UEs) the MIB barring was relied upon in 5G – which has local regulation specific implications and devoid legacy UEs of making even emergency calls!

Proposal 4: For initial access RAN2 should study a) SSB duplication minimization in MRSS and b) means for achieving forward compatibility (minimize a “future” feature deployment requiring to bar a legacy UE).

## Paging

In 5G NR, the paging procedure allows the network to reach idle or inactive UEs for incoming services or system information updates. The gNB transmits paging messages via the Paging Control Channel (PCCH), mapped onto PDSCH, with notifications indicated by the PDCCH. Paging occasions and groups are derived from UE identities and DRX configurations to balance energy saving and latency. Enhancements like Paging early indication (PEI) and paging adaptation from Rel. 19 reduce false alarm for the UE and minimize network paging transmission by reducing the number of POs, beyond what was possible until rel. 18.

The proponent believe that while Rel. 19 could be used as baseline for paging design in 6G, RAN2 should further study Real Paging load and required paging latency – and thereby avoid too much over-dimensioning: UE as well as network energy saving potential based on realistic requirements.

Following are the observations from R2-2406890.

Observation#1a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#1b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Besides, the following aspects can be considered:

- IoT/ Redcap and commercial UEs should be considered separately to see if anything separately needs to be done for these e.g., to help with efficient software download, application pings etc.

- Only one simple schemes for UE power saving can be sufficient to reduce false alarm/ minimize wakeup of main radio i.e., both PEI and LP-WUS may not be necessary. Operator input on deciding the best scheme to adopt in 6G would be very helpful.

- Paging areas/ policy/ propagation: Should check if potential impacts to RAN2 [RAN3]

We can start the 6G study with the following:

Observation#5a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#5b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Proposal 5: Study real Paging load in a cell and required paging latency to avoid too much over-dimensioning of Paging delivery.

## System Information

NR (with no ubiquitous CRS) minimizes always-on transmissions (SSB+SI) compared with LTE. The on-demand SI provisioning further reduces broadcast. Simulation results on how much Network Energy Saving it generates are available in Ch. 6.1.5.2 of 3gpp TR 38.864. One source reveal that with on-demand SIB1 at empty load with baseline of 20ms SSB/SIB1 periodicity, 5.8%~8.6% BS energy savings can be achieved at SIB1 transmission rate of 20%~5% for one SSB beam, and the gains can increase to 32.1%~38.8% for 8 beams case for a same SIB1 transmission rate range.

Observation 6a: BS Energy saving gains of about 30+% can be achieved using on-demand SI provisioning at empty load with baseline of 20ms SSB/SIB1 periodicity for 8 beams case.

However, from the report the contribution of the SSB/ SIB1/ Other SI transmission to the overall “operational” energy consumption (including energy dissipation, light bulbs, cooling system if any etc.) of a base station is not available as the intention of the study was only comparative to the full (radio) load situation. It is possible that these gains are insignificant compared with the total power consumption used to run the BS operations. But the sheer number of the Base Stations already deployed today is estimated upwards of 20 million [3] and if a fully deployed 6G System is to eventually replace (or be upgraded to) most of these in years to come, even a much smaller saving can make significant impact.

Observation 6b: About 20 million Base Station deployed today may be replaced by or upgraded to a 6G base station in fully deployed 6G System. Further work on minimizing broadcast might bring significant ecological gain even for a meagre 1 or 2% further NES saving.

The on-demand system information in 5G NR provides the entire requested SIB and in fact the SI Message containing the SIB. Our initial study of the field logs suspects that rate of change if some System Information is slow – hinting that system information is rather stable. But it is better to study this further and if found true, 6G can design SI distribution which minimizes provisioning the same information repeatedly along with a fraction of changes SI. If much of the system information is stable, there could be other designs minimizing broadcasting it.

Observation 6c: The rate and fraction of SI change is likely (very) low. The complete SI broadcasting can be minimized, even for on-demand SI provisioning.

Proposal 6: Study real on-field rate and fraction of SI change in 5G System. The study could lead to means further reducing System Information broadcast (initial version and/ or SI changes) beyond 5G.

## Dual registration:

Radio aggregation is important for an operator to enable load balancing and likely also to have a better resource utilization if different frequency bands (FR1 vs FR2/ 3) may be better suited for different services. The potential aggregation mechanisms include dual connectivity, dual steering and dual registration. MUSIM can also be looked at as an aggregation solution. In 5G, MUSIM is supported because multi-USIM devices are popular in different countries. For example, a user may have both a personal and a business subscription in one device or has two personal subscriptions in one device for different services. In 6G, we will have similar requirement from the user in some countries. Dual registration allows for data aggregation in the CN. It is applicable regardless of if 6GC is an evolution of 5GC or if both 5GC and 6GC are distinct networks. Connection to both 5G and 6G can be maintained via dual registration. UE can quickly fallback from 6G to 5G. Otherwise, the mobility time from 6G to 5G will be much longer compared to other solutions e.g., Inter-RAT handover. Some may argue that dual RX/TX are required for dual registration. We think that MUSIM-like mechanism can be reused even for a UE with single RX/TX chain. At this early stage it is better to consider dual registration.

Proposal 7: RAN2 should consider if dual registration can be supported from the early stage of 6G.

# Conclusion

In this contribution, we discussed fundamental control plane specific aspects. Based on the discussion, we have the following observation and proposals:

Proposal 1: To tackle long, nested RRC specifications (mainly xx.331 and xx.304) RAN2 should study if a different specification modelling (Base-specification + individual functional specifications with their own self compile-able ASN.1 (even with importing ASN.1 from the base specification)) can be pursued. This should happen in the beginning and parallel to the “Modernization” discussions.

Observation 2: RRC inactive state has the potential for harnessing power savings, signalling reduction, better latency for small traffic / IoT / RedCap. Initial deployment has started.

Proposal 2: RRC states from 5G could be taken as baseline and a study for the feasibility and need for RRC Inactive State enhancement can be carried out at an early stage of 6G.

Proposal 3: 5G RRC Connection Management (at least Transition to and from RRC Connected) can be taken as baseline.

Observation 4a: SSB duplication minimization in MRSS can save significant overhead (e.g., about 3.5% SSB overhead in FR2, 100 MHz BW @ 120 kHz SCS with 64 SSB transmission at 20 ms periodicity).

Observation 4b: To keep out the legacy UEs (since these are not the intended feature specific UEs) the MIB barring was relied upon in 5G – which has local regulation specific implications and devoid legacy UEs of making even emergency calls!

Proposal 4: For initial access RAN2 should study a) SSB duplication minimization in MRSS and b) means for achieving forward compatibility (minimize a “future” feature deployment requiring to bar a legacy UE).

Observation#5a: The capacity utilization (i.e., the total number of RRC Paging records that could be sent with the used paging configuration) is much less than 1% in all observed cases.

Observation#5b: Average number of paging records contained in one RRC message (when Paging is sent) is close to 2. The real average accounting all configured paging occasions will be much less.

Proposal 5: Study real Paging load in a cell and required paging latency to avoid too much over-dimensioning of Paging delivery.

Observation 6a: BS Energy saving gains of about 30+% can be achieved using on-demand SI provisioning at empty load with baseline of 20ms SSB/SIB1 periodicity for 8 beams case.

Observation 6b: About 20 million Base Station deployed today may be replaced by or upgraded to a 6G base station in fully deployed 6G System. Further work on minimizing broadcast might bring significant ecological gain even for a meagre 1 or 2% further NES saving.

Observation 6c: The rate and fraction of SI change is likely (very) low. The complete SI broadcasting can be minimized, even for on-demand SI provisioning.

Proposal 6: Study real on-field rate and fraction of SI change in 5G System. The study could lead to means further reducing System Information broadcast (initial version and/ or SI changes) beyond 5G.

Proposal 7: RAN2 should consider if dual registration can be supported from the early stage of 6G.

# References

RP-252912_6G_WG_SID_cl

RP-252888 Deployment scenario

[3] https://www.mobileworldlive.com/china-mobile/china-5g-base-stations-approach-4-5m/

Based on Ericsson Mobility Report, gsmaintelligence, 5gamericas.org and some Country-level specific data.