3GPP TSG-RAN WG2 Meeting #131bis	R2-2506865

Prague, Czech Republic, Oct. 13th-17th

Source:	CATT

Title:	Discussion on MAC open issues

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

According to the open issues listed for MAC running CR, the following two issues are proposed to be further studied:

Proposal 1: Revisit the UE operation in LP-WUS collision for Option 1-2, which should be based on critical issues in the current RAN2 agreements. (based on contribution) Proposal 6: Further discuss the necessity to explicitly describe “option1-1/1-2” based on 38.300 description. (based on contribution)

In this document, we discuss these two issues.

Discussion

  - UE operation in LP-WUS collision for Option 1-2

It was agreed that in RAN2#131 meeting that [1]:

For Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s). Can discuss if critical issue identified with this mechanism.

According to the agreements above, it can be seen that the lpwus-PDCCH-MonitoringTimer will not be started in collision cases. And based on the RAN1 discussion, the collision cases includes measurement gap, DRX Active time, etc. Then, it may happen that UE misses multiple consecutive LP-WUS occasions due to improper configuration of LP-WUS resources. According to the feature list from RAN1 [2], the periodicity of LP-WUS in Option 1-2 can be tens of slots. Therefore, it is possible that the UE misses the LP-WUS wake-up indication which brings additional transmission latency with improper configuration of LP-WUS resources.

Some companies propose that the NW should be allowed to configure the number of consecutive LOs which the UE is allowed to miss due to collisions before the UE needs to start the timer. In this solution, when collision number is higher than the configured number, the UE is mandatory to start lpwus-PDCCH-MonitoringTimer even though there is no data transmission. This results in low utilization of LP-WUS and minimizing the power saving gain brought by LP-WUS.

Actually, the network is aware of most of the collision cases (RAN1 may continue to study LP-WUS collision cases). Hence, one simple solution is that the network (re)configures suitable configuration for LP-WUS resources avoiding high collision probability. Hence, no enhancement is needed.

Proposal 1: No enhancement is needed for handling on lpwus-PDCCH-MonitoringTimer in LP-WUS collision cases.

  - Terminology of option 1-1/1-2

In MAC, option 1-1 and option 1-2 are implicitly described based on the condition whether lpwus-PDCCH-MonitoringTimer is configured. However, in RRC running, lpwus-PDCCH-MonitoringTimer is mandatory for option 1-2. Some companies think conditions of option 1-2 and lpwus-PDCCH-MonitoringTimer are mutually dependent.

In 38.331, the following principles are applied for conditions.

If conditions are used, a conditional presence table is provided for the message or information element specifying the need of the field for each condition case. The table also specifies whether UE maintains or releases the value in case the field is absent. The conditions clarify what the UE may expect regarding the setting of the message by the network. Violation of conditions is regarded as invalid network behaviour, which the UE is not required to cope with. Hence the general error handling defined in 10.4 does not apply in case a field is absent although it is mandatory according to the CondC or CondM condition.

According to the principle highlighted in yellow, it can be derived that Cond Option12 in LP-WUS RRC CR implies the UE expecting lpwus-PDCCH-MonitoringTimer is mandatory configured for option 1-2. In MAC, checking whether lpwus-PDCCH-MonitoringTimer is configured is used to control whether to start drx-onDurationTimer or lpwus-PDCCH-MonitoringTimer other than checking whether option 1-2 is configured or not. Hence, there is no mutually dependence issue.

Observation: There is no mutually dependence issue on conditions of option 1-2 and lpwus-PDCCH-MonitoringTimer.

Regarding the necessity to explicitly describe “option1-1/1-2” based on TS 38.300, from our perspective MAC has described the procedure clearly. Additionally, companies also propose to replace “option 1-1” and “option 1-2” terminology with description in LP-WUS RRC. Hence, MAC can refer to other spec, e.g. TS 38.331 or TS 38.300, for the definition of “option 1-1/1-2” or description if needed.

Proposal 2: There is no necessity to explicitly describe option1-1 and option 1-2 in MAC. The definitions of option1-1 and option 1-2 in MAC can refer to other spec, e.g., TS 38.331 or TS 38.300, if needed.

Conclusion

According to the analysis in Section 2, our observation and proposals are summarized as follows:

UE operation in LP-WUS collision for Option 1-2

Proposal 1: No enhancement is needed for handling on lpwus-PDCCH-MonitoringTimer in LP-WUS collision cases.

Terminology of option 1-1/1-2

Observation: There is no mutually dependence issue on conditions of option 1-2 and lpwus-PDCCH-MonitoringTimer.

Proposal 2: There is no necessity to explicitly describe option1-1 and option 1-2 in MAC. The definitions of option1-1 and option 1-2 in MAC can refer to other spec, e.g., TS 38.331 or TS 38.300, if needed.

Reference

- RAN2#131 chairman notes.

- R1-2506622 Consolidated Rel-19 higher layers parameters list Post RAN1#122.