3GPP TSG-RAN WG2 Meeting #131bis	R2-2506866

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	8.8.2

Source: 	CATT

Title: 	[C006][C008][C009] Corrections on the smtc5list

Document for: 	Discussion and Decision

Introduction

This contribution discusses the RIL [C006] [C008] and [C009]. These RILs are proposed to correct the configuration of smtc5list and clarify the corresponding UE behaviours.

Discussion

2.1 Discussion on [C006][C008]

# C006 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C006; NTN; 1; Clarify whether the R19 DL CE capable UEs perform measurement as configured in the SMTC4; Yes, R2-250xxxxx; CATT (Da Wang); v008; PropReject. [Description]: Based on the field description of smtc4list and smtc5list under SIB2, it is unclear whether the R19 DL CE capable UEs use the SMTC4 perform measurement as configured in the SMTC4. We agreed that “The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs)”. The yellow highlight text means the R19 DL CE capable UE takes into account all the SMTCs in smtc4list and smtc5list. While the green highlight text means the SMTC5 explict configure SMTCs not configured in SMTC4 and implicit configure SMTCs configured in SMTC4, that the UE only take the entries in SMTC5 into account. There is a contradiction here and the total number of configurable SMTCs across smtc4list and smtc5list may exceed 6. For instance, SMTC4={a,b,c} SMTC5={-,d,e,f,g,h}, the total number of configurable SMTCs across smtc4list and smtc5list is 9, which is against with the yellow highlight part. smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.. [Proposed Change]: remove the limitation of “The total number of configurable SMTCs across smtc4list and smtc5list is 6”. The maximum number configured SMTCs for idle/inactive is 7 can be restricted by the sequence length of SMTC5 naturally. smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.. [Comments]: [Samsung]: we share the same view that the current sentence “The total number of configurable SMTCs across smtc4list and smtc5list is 6.” is not correct and should be removed. [Rapp] It is uncertain why the sentence is not correct as it follows a RAN2 agreement. Please explain further. Another reason why this change cannot be implemented like this relates to the refLocList, please see my reply to V203. # C008 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C006; NTN; 1; Corrections on the smtc5list; Yes, R2-250xxxxx; CATT (Da Wang); v019; PropAgree. [Description]: Same as RIL C006. [Proposed Change]: Modify “The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2.” as “The total number of different SMTC periodicities across smtc and smtc5list is 2.”. Illustrate that “If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.”. smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present. If smtc5list is present, the two SMTC periodicities capable UE ignores smtc4list.. [Comments]: [Rapp] The first change is fine. In our view, the second change is incorrect. RAN2 has agreed delta signalling with smtc4list to precisely avoid configuring two lists (smct4list and smtc5list) with the same information (pci-list and offset). [Qualcomm] The design here is to apply delta configuration so current text is correct that total number does not exceed, may be should say 7. If smtc5 is present, the smtc4 shall be present. But if smtc4 is present, it is not necessary smtc5 to be included.

In RAN2#130 [1] and RAN2#131 [2], the following agreements related to SMTC enhancements of DL CE were reached:

Agreements:

- Introduce support for SMTC enhancements in idle/inactive mode in a backwards compatible manner. FFS signalling details (e.g., new field or extension of SMTC4).

- the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

- To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

Agreements:

- The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 SMTCs).

- We introduce a capability to support SMTC enhancements to FR1 (FFS on FR2 depending on RAN4 progress)

- RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only.

- RAN2 reconfirms the understanding that if the NW configures more than 4 SMTCs the UE in idle/inactive the UE selects up to 4 SMTCs (or less, depending on the UE capability) based on location and only considers them for measurements

- In connected mode the NW will only configure up to 4 SMTC.

Based on these agreements, the field description of smtc4list and smtc5list under SIB2 is defined as follows [3].

smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of configurable SMTCs across smtc4list and smtc5list is 6. The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present.

However, according to the CR, it is unclear how the R19 DL CE capable UE determines the SMTC configurations when the smtc5list is configured. In detail,

- We understand the yellow highlight spec text is intend to reflect the agreement “The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs)”, which means the smtc5list is used to supplement SMTCs not included in smtc4list, and the R19 DL CE capable UE performs measurement according to both the smtc4list and the smtc5list.

- While the green highlight spec text is intend to reflect the agreement “can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications”, which means the smtc5list explicit configure SMTCs not configured in smtc4list and implicit configure SMTCs configured in smtc4list, that the R19 DL CE capable UE performs measurement only according to the smtc5list.

According to the rapporteur’s comment in the ASN.1 comment, it seems we have common understanding on the green highlight part, and our gap is on the yellow highlight part, i.e., why the total number of configurable SMTCs across smtc4list and smtc5list will exceed 6.

The limitation of 6 SMTCs can be fulfilled only when: for R19 DL CE capable UE, the network configures all the SMTCs in the smtc4list and some additional SMTCs in the smtc5list. For instance, the network would like to configure SMTCs {a,b,c} for legacy UEs, and configure SMTCs {a,b,c,d,e,f} for R19 DL CE capable UE. The network should set smtc4list as {a,b,c} and set smtc5list as {-,-,-,d,e,f}. It can be explained as the total number of configurable SMTCs across smtc4list and smtc5list is 6.

However, for most cases, the limitation of “The total number of configurable SMTCs across smtc4list and smtc5list is 6” cannot be fulfilled.

- For instance, the network would like to configure SMTCs {a,b,c} for legacy UEs, and configure SMTCs {a,b,d,e,f,g} for R19 DL CE capable UE. The network should set smtc4list as {a,b,c} and set smtc5list as {-,-,d,e,f,g}. The total number of configurable SMTCs across smtc4list and smtc5list is 7 (3 in the smtc4list and 4 in the smtc5list).

- For another instance, the network would like to configure SMTCs {a,b,c} for legacy UEs, and configure SMTCs {a’,b’,c’,d,e,f} for R19 DL CE capable UE (SMTC a’/b’/c’ means partial of the pci-List, periodicity and offset fields are different from the SMTC a/b/c). The network should set smtc4list as {a,b,c} and set smtc5list as {a’,b’,c’,d,e,f }. E.g.,

|  | smtc4list (periodicity1) | smtc5list | Explanation of smtc5list |
| --- | --- | --- | --- |
| Entry 1 | offset 1, PCI_list1 | -, -, - | offset 1, PCI_list1, periodicity1 |
| Entry 2 | offset 2, PCI_list2 | -, -, periodicity2 | offset 2, PCI_list2, periodicity2 |
| Entry 3 | offset 3, PCI_list3 | -, -, periodicity2 | offset 3, PCI_list3, periodicity2 |
| Entry 4 | / | offset 4, PCI_list4, periodicity1 | offset 4, PCI_list4, periodicity1 |
| Entry 5 | / | offset 5, PCI_list5, periodicity1 | offset 5, PCI_list5, periodicity1 |
| Entry 6 | / | offset 6, PCI_list6, periodicity2 | offset 6, PCI_list6, periodicity2 |
| NOTE: periodicity1 is the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. Periodicity2 is one additional periodicity configured by smtc5list. |  |  |  |

For this instance, we understand the entry 2 and entry 3 of the smtc5list are different SMTCs from same position of the smtc4list. The total number of configurable SMTCs across smtc4list and smtc5list is 8 (3 in the smtc4list and 5 in the smtc5list).

For the cases given above, the SMTCs configured for the R19 DL CE capable UE, reference to the “Explanation of smtc5list” row, is in line with the conclusion, but the total number of configurable SMTCs across smtc4list and smtc5list exceed 6.

Observation 1: It is possible that only partial entry of the smtc4list are reduplicated in the smtc5list; or for an entry of the smtc4list, only partial parameters of the pci-List, periodicity and/or offset fields are reduplicated for the same entry of the smtc5list. For these cases, the SMTCs configured for the R19 DL CE capable UE does not exceed 6 (which is followed the agreement), but the total number of the configured SMTCs across smtc4list and smtc5list exceed 6.

Therefore, it is proposed to delete the limitation of “The total number of configurable SMTCs across smtc4list and smtc5list is 6”, to leave higher flexibility in network configuration., i.e., allows configurations of up to 6 SMTCs with different partial or all parameters for R19 DL CE capable UE. Accordingly, it is proposed that the R19 DL CE capable UE performs measurement only according to the SMTCs in the smtc5list, including the SMTCs explicit configured in smtc5list and the SMTCs implicit configured by referencing smtc4list. Some corrections on the field description of smtc4list, smtc5list under SIB2 are needed:

- [C006] Remove the limitation of “The total number of configurable SMTCs across smtc4list and smtc5list is 6.”;

- [C008] Modify “The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2.” as “The total number of different SMTC periodicities across smtc and smtc5list is 2.”;

- [C008] Illustrate that “If smtc5list is present, the two SMTC periodicities capable UE performs measurement only according to the SMTCs in the smtc5list”.

Proposal 1: Make the following corrections to the field description of smtc4list, smtc5list under SIB2:

- [C006] Remove the limitation of “The total number of configurable SMTCs across smtc4list and smtc5list is 6.”;

- [C008] Modify “The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2.” as “The total number of different SMTC periodicities across smtc and smtc5list is 2.”;

- [C008] Illustrate that “If smtc5list is present, the two SMTC periodicities capable UE performs measurement only according to the SMTCs in the smtc5list”.

The TP to TS 38.331 for Proposal 1 is given in the Annex A.

Proposal 1a: For Proposal 1, adopt the TP provided in the Annex A.

2.2 Discussion on [C009]

# C009 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C009; NTN; 1; clarify how UE determines the periodicity of smtc5list in MO; Yes, R2-250xxxxx; CATT (Da Wang); v019; PropReject. [Description]: We think the periodicity of smtc5list can be optional present. For an entry, if its periodicity is absent, it uses the periodicity from the smtc1. [Proposed Change]: If smtc5list is present, for cells indicated in the pci-List parameter in each SSB-MTC5 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity (if present) and offset parameter in each SSB-MTC5 configuration and use the duration parameter from the smtc1 configuration. If the periodicity parameter is absent for an SSB-MTC5 configuration, the UE uses the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR serving cell meeting the above condition. [Comments]: [Rapp] In the MO, the network can configure a maximum of 2/4 SMTC offsets (depending on the UE capabilities). Therefore, legacy smtc4list can be used for the case where the periodicity is equal to smtc1. Smtc5list was introduced with the purpose of enabling a new periodicity. [Qualcomm] If UE does not support two periodicities, SMTC configuration in connected mode is exactly same as Rel-17.

When the smtc5list is configured in MeasObjectNR, it can include an additional SMTC periodicity to the one indicated in smtc1. According to the signalling structure of IE SSB-MTC5, the periodicity field can be optional present.

SSB-MTC5-r19 ::=             SEQUENCE {

pci-List-r19                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

periodicity-r19              ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}             OPTIONAL,  -- Need M

offset-r19                   INTEGER (0..159)                                                               OPTIONAL   -- Need M [RIL] Z257, NTN

}

If the periodicity parameter of smtc5list is present for an SSB-MTC5 configuration, the UE uses this configured parameter. However, the current CR has not specified the UE behaviour when the periodicity parameter is absent for an entry of the smtc5list. When periodicity parameter is absent, we think the reasonable behaviour is the UE uses the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. For instance,

|  | smtc5list | Explanation of smtc5list |
| --- | --- | --- |
| Entry 1 | offset 1, PCI_list1, periodicity2 | offset 1, PCI_list1, periodicity2 |
| Entry 2 | offset 2, PCI_list2, - | offset 2, PCI_list2, periodicity1 |
| Entry 3 | offset 3, PCI_list3, - | offset 3, PCI_list3, periodicity1 |
| NOTE: periodicity1 is the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. Periodicity2 is one additional periodicity configured by smtc5list. |  |  |

Observation 2: In MeasObjectNR, the periodicity parameter of the SSB-MTC5 is optional present. For the case of the periodicity parameter is absent, the UE behaviour has not been specified.

It is proposed that:

Proposal 2: [C009] For the smtc5list in MeasObjectNR, if the periodicity parameter is present for an SSB-MTC5 configuration, the UE uses this configured parameter; if the periodicity parameter is absent for an SSB-MTC5 configuration, the UE uses the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. Reflect that in section 5.5.2.10.

Proposal 2a: For Proposal 2, adopt the TP provided in the Annex B.

Conclusion

In this contribution, corrections on smtc5list are discussed with the following proposals:

Observation 1: It is possible that only partial entry of the smtc4list are reduplicated in the smtc5list; or for an entry of the smtc4list, only partial parameters of the pci-List, periodicity and/or offset fields are reduplicated for the same entry of the smtc5list. For these cases, the SMTCs configured for the R19 DL CE capable UE does not exceed 6 (which is followed the agreement), but the total number of the configured SMTCs across smtc4list and smtc5list exceed 6.

Proposal 1: Make the following corrections to the field description of smtc4list, smtc5list under SIB2:

- [C006] Remove the limitation of “The total number of configurable SMTCs across smtc4list and smtc5list is 6.”;

- [C008] Modify “The total number of different SMTC periodicities across smtc, smct4list, and smtc5list is 2.” as “The total number of different SMTC periodicities across smtc and smtc5list is 2.”;

- [C008] Illustrate that “If smtc5list is present, the two SMTC periodicities capable UE performs measurement only according to the SMTCs in the smtc5list”.

Proposal 1a: For Proposal 1, adopt the TP provided in the Annex A.

Observation 2: In MeasObjectNR, the periodicity parameter of the SSB-MTC5 is optional present. For the case of the periodicity parameter is absent, the UE behaviour has not been specified.

Proposal 2: [C009] For the smtc5list in MeasObjectNR, if the periodicity parameter is present for an SSB-MTC5 configuration, the UE uses this configured parameter; if the periodicity parameter is absent for an SSB-MTC5 configuration, the UE uses the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. Reflect that in section 5.5.2.10.

Proposal 2a: For Proposal 2, adopt the TP provided in the Annex B.

Reference

- RAN2#130 ChairNotes.

- RAN2#131 ChairNotes.

- R2-2506522  Introduction of NTN Phase 3 enhancements Rel-19 Ericsson	CR	Rel-19

- Annex A: TP for [C006][C008]

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

SIB2 information element

-- ASN1START

-- TAG-SIB2-START

SIB2 ::=                            SEQUENCE {

cellReselectionInfoCommon           SEQUENCE {

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)          OPTIONAL,       -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                     OPTIONAL,       -- Need S

rangeToBestCell                     RangeToBestCell                                 OPTIONAL,       -- Need R

q-Hyst                              ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24},

speedStateReselectionPars           SEQUENCE {

mobilityStateParameters             MobilityStateParameters,

q-HystSF                        SEQUENCE {

sf-Medium                       ENUMERATED {dB-6, dB-4, dB-2, dB0},

sf-High                         ENUMERATED {dB-6, dB-4, dB-2, dB0}

}

}                                                                                   OPTIONAL,       -- Need R

...

},

cellReselectionServingFreqInfo      SEQUENCE {

s-NonIntraSearchP                   ReselectionThreshold                            OPTIONAL,       -- Need S

s-NonIntraSearchQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need S

threshServingLowP                   ReselectionThreshold,

threshServingLowQ                   ReselectionThresholdQ                           OPTIONAL,       -- Need R

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                      OPTIONAL,       -- Need R

...

},

intraFreqCellReselectionInfo        SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                      OPTIONAL,       -- Need R

q-QualMin                           Q-QualMin                                       OPTIONAL,       -- Need S

s-IntraSearchP                      ReselectionThreshold,

s-IntraSearchQ                      ReselectionThresholdQ                           OPTIONAL,       -- Need S

t-ReselectionNR                     T-Reselection,

frequencyBandList                   MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need S

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                    OPTIONAL,       -- Need R

p-Max                               P-Max                                           OPTIONAL,       -- Need S

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S  [RIL]: H251, NTN

ss-RSSI-Measurement                 SS-RSSI-Measurement                             OPTIONAL,       -- Need R

ssb-ToMeasure                       SSB-ToMeasure                                   OPTIONAL,       -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

...,

[[

t-ReselectionNR-SF                  SpeedStateScaleFactors                          OPTIONAL        -- Need R

]],

[[

smtc2-LP-r16                        SSB-MTC2-LP-r16                                 OPTIONAL,        -- Need R

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                    OPTIONAL         -- Cond SharedSpectrum

]],

[[

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                    OPTIONAL         -- Cond SharedSpectrum2

]],

[[

smtc4list-r17                       SSB-MTC4List-r17                                OPTIONAL         -- Need R

]],

[[

frequencyBandList-v1760             MultiFrequencyBandListNR-SIB-v1760              OPTIONAL,       -- Need R

frequencyBandListSUL-v1760          MultiFrequencyBandListNR-SIB-v1760              OPTIONAL        -- Need R

]],

[[

frequencyBandListAerial-r18         MultiFrequencyBandListNR-Aerial-SIB-r18         OPTIONAL        -- Need S

]],

[[

refLocList-r19                      RefLocList-r19                                  OPTIONAL,        -- Need R

smtc5list-r19                       SEQUENCE (SIZE(1..6)) OF SSB-MTC5-r19           OPTIONAL         -- Need R

]] [RIL]E013,NTN, [RIL]X250,NTN, [RIL]: V209

},

...,

[[

relaxedMeasurement-r16              SEQUENCE {

lowMobilityEvaluation-r16           SEQUENCE {

s-SearchDeltaP-r16                  ENUMERATED {

dB3, dB6, dB9, dB12, dB15,

spare3, spare2, spare1},

t-SearchDeltaP-r16                  ENUMERATED {

s5, s10, s20, s30, s60, s120, s180,

s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                   OPTIONAL,       -- Need R

cellEdgeEvaluation-r16              SEQUENCE {

s-SearchThresholdP-r16              ReselectionThreshold,

s-SearchThresholdQ-r16              ReselectionThresholdQ                       OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition-r16     ENUMERATED {true}                               OPTIONAL,       -- Need R

highPriorityMeasRelax-r16           ENUMERATED {true}                               OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]],

[[

cellEquivalentSize-r17                  INTEGER(2..16)                                  OPTIONAL,       -- Cond HSDN

relaxedMeasurement-r17                  SEQUENCE {

stationaryMobilityEvaluation-r17        SEQUENCE {

s-SearchDeltaP-Stationary-r17           ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1},

t-SearchDeltaP-Stationary-r17           ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

},

cellEdgeEvaluationWhileStationary-r17   SEQUENCE {

s-SearchThresholdP2-r17                 ReselectionThreshold,

s-SearchThresholdQ2-r17                 ReselectionThresholdQ                   OPTIONAL        -- Need R

}                                                                                   OPTIONAL,       -- Need R

combineRelaxedMeasCondition2-r17        ENUMERATED {true}                           OPTIONAL        -- Need R

}                                                                                       OPTIONAL        -- Need R

]]

}

RangeToBestCell    ::= Q-OffsetRange

RefLocList-r19 ::= SEQUENCE (SIZE(1..6)) OF ReferenceLocation-r17 [RIL]: N083, NTN

-- TAG-SIB2-STOP

-- ASN1STOP

| SIB2 field descriptions |
| --- |
| absThreshSS-BlocksConsolidation Threshold for consolidation of L1 measurements per RS index. If the field is absent, the UE uses the measurement quantity as specified in TS 38.304 [20]. |
| cellEdgeEvaluation Indicates the criteria for a UE to detect that it is not at cell edge, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.2). |
| cellEdgeEvaluationWhileStationary Indicates the criteria for a UE to detect that it is not at cell edge while stationary, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.4). |
| cellEquivalentSize The number of cell count used for mobility state estimation for this cell as specified in TS 38.304 [20]. |
| cellReselectionInfoCommon Cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection. |
| cellReselectionServingFreqInfo Information common for non-intra-frequency cell re-selection i.e. cell re-selection to inter-frequency and inter-RAT cells. |
| combineRelaxedMeasCondition When both lowMobilityEvaluation and cellEdgeEvaluation criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when either or both of the criteria are met. (See TS 38.304 [20], clause 5.2.4.9.0) |
| combineRelaxedMeasCondition2 When both stationaryMobilityEvaluation and cellEdgeEvaluationWhileStationary criteria are present in SIB2, this parameter configures the UE to fulfil both criteria in order to relax measurement requirements for cell reselection. If the field is absent, the UE is allowed to relax measurement requirements for cell reselection when only the stationary criteria is met. (See TS 38.304 [20], clause 5.2.4.9.0) |
| deriveSSB-IndexFromCell This field indicates whether the UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. If this field is set to true, the UE assumes SFN and frame boundary alignment across cells on the serving frequency as specified in TS 38.133 [14]. |
| frequencyBandList Indicates the list of frequency bands for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3. |
| frequencyBandListAerial Indicates the list of frequency bands for aerial operation for which the NR cell reselection parameters apply. The UE behaviour in case the field is absent is described in clause 5.2.2.4.3. |
| highPriorityMeasRelax Indicates whether measurements can be relaxed on high priority frequencies. If the field is absent, the UE shall not relax measurements on high priority frequencies beyond "Thigher_priority_search" unless both low mobility and not at cell edge criteria are fulfilled (see TS 38.133 [14], clauses 4.2.2.7, 4.2.2.10 and 4.2.2.11). |
| intraFreqCellReselectionInfo Cell re-selection information common for intra-frequency cells. |
| lowMobilityEvaluation Indicates the criteria for a UE to detect low mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.1). |
| nrofSS-BlocksToAverage Number of SS blocks to average for cell measurement derivation. If the field is absent the UE uses the measurement quantity as specified in TS 38.304 [20]. |
| p-Max Value in dBm applicable for the intra-frequency neighbouring NR cells. If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell, TS 38.101-2 [39] in case of an FR2 cell or TS 38.101-5 [75] in case of an NTN cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39] for FR2-1/2 or according to TS 38.101-5 [75] for FR2-NTN. This field is ignored by IAB-MT and NCR-MT. The IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63]. The NCR-MT applies output power and emissions requirements as specified in TS 38.106 [79]. |
| q-Hyst Parameter "Qhyst" in TS 38.304 [20], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on. |
| q-HystSF Parameter "Speed dependent ScalingFactor for Qhyst" in TS 38.304 [20]. The sf-Medium and sf-High concern the additional hysteresis to be applied, in Medium and High Mobility state respectively, to Qhyst as defined in TS 38.304 [20]. In dB. Value dB-6 corresponds to -6dB, dB-4 corresponds to -4dB and so on. |
| q-QualMin Parameter "Qqualmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. If the field is absent, the UE applies the (default) value of negative infinity for Qqualmin. |
| q-RxLevMin Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. |
| q-RxLevMinSUL Parameter "Qrxlevmin" in TS 38.304 [20], applicable for intra-frequency neighbour cells. |
| rangeToBestCell Parameter "rangeToBestCell" in TS 38.304 [20]. The network configures only non-negative (in dB) values. |
| refLocList Indicates a reference location associated to an SMTC configuration in smtc5list. If present, it includes the same number of entries as smtc5list. The first entry in this list corresponds to the first entry in smtc5list, the second entry corresponds to the seccond entry in smtc5list, and so on. [RIL]: V203, NTN |  |
| relaxedMeasurement Configuration to allow relaxation of RRM measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9). In NTN, this field is only applicable for GSO neighbour cells. |
| s-IntraSearchP Parameter "SIntraSearchP" in TS 38.304 [20]. |
| s-IntraSearchQ Parameter "SIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SIntraSearchQ. |
| s-NonIntraSearchP Parameter "SnonIntraSearchP" in TS 38.304 [20]. If this field is absent, the UE applies the (default) value of infinity for SnonIntraSearchP. |
| s-NonIntraSearchQ Parameter "SnonIntraSearchQ" in TS 38.304 [20]. If the field is absent, the UE applies the (default) value of 0 dB for SnonIntraSearchQ. |
| s-SearchDeltaP Parameter "SSearchDeltaP" in TS 38.304 [20]. Value dB3 corresponds to 3 dB, dB6 corresponds to 6 dB and so on. |
| s-SearchDeltaP-Stationary Parameter "SSearchDeltaP-Stationary" in TS 38.304 [20]. Value dB2 corresponds to 2 dB, dB3 corresponds to 3 dB and so on. |
| s-SearchThresholdP, s-SearchThresholdP2 Parameters "SSearchThresholdP" and "SSearchThresholdP2" in TS 38.304 [20]. The network configures s-SearchThresholdP and s-SearchThresholdP2 to be less than or equal to s-IntraSearchP and s-NonIntraSearchP. |
| s-SearchThresholdQ, s-SearchThresholdQ2 Parameters "SSearchThresholdQ" and "SSearchThresholdQ2" in TS 38.304 [20]. The network configures s-SearchThresholdQ and s-SearchThresholdQ2 to be less than or equal to s-IntraSearchQ and s-NonIntraSearchQ. |
| smtc Measurement timing configuration for intra-frequency measurement. If this field is absent, the UE assumes that SSB periodicity is 5 ms for the intra-frequency cells. If the field is broadcast by an NTN cell, the offset (derived from parameter periodicityAndOffset) is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. |
| smtc2-LP Measurement timing configuration for intra-frequency neighbour cells with a Long Periodicity (LP) indicated by periodicity in smtc2-LP. The timing offset and duration are equal to the offset and duration indicated in smtc in intraFreqCellReselectionInfo. The periodicity in smtc2-LP can only be set to a value strictly larger than the periodicity in smtc in intraFreqCellReselectionInfo (e.g. if smtc indicates sf20 the Long Periodicity can only be set to sf40, sf80 or sf160, if smtc indicates sf160, smtc2-LP cannot be configured). The pci-List, if present, includes the physical cell identities of the intra-frequency neighbour cells with Long Periodicity. If smtc2-LP is absent, the UE assumes that there are no intra-frequency neighbour cells with a Long Periodicity. This field is not configured together with smtc4list. |
| smtc4list, smtc5list Measurement timing configuration list for NTN deployments. The offset of each SSB-MTC4 in smtc4list and SSB-MTC5 in smtc5list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in smtc4list and smtc5list, it is up to the UE to select which SMTCs to consider. The total number of different SMTC periodicities across smtc and smtc5list is 2. If an entry in smtc5list is present but the pci-List, periodicity and/or offset fields are absent, the UE applies the value of the corresponding field from the entry at the same position in smtc4list, if present. If smtc5list is present, the two SMTC periodicities capable UE performs measurement only according to the SMTCs in the smtc5list. |
| ssb-PositionQCL-Common Indicates the QCL relation between SS/PBCH blocks for intra-frequency neighbor cells as specified in TS 38.213 [13], clause 4.1. |
| ssb-ToMeasure The set of SS blocks to be measured within the SMTC measurement duration (see TS 38.215 [9]). When the field is absent the UE measures on all SS-blocks. |
| stationaryMobilityEvaluation Indicates the criteria for a UE to detect stationary mobility, in order to relax measurement requirements for cell reselection (see TS 38.304 [20], clause 5.2.4.9.0). |
| t-ReselectionNR Parameter "TreselectionNR" in TS 38.304 [20]. |
| t-ReselectionNR-SF Parameter "Speed dependent ScalingFactor for TreselectionNR" in TS 38.304 [20]. If the field is absent, the UE behaviour is specified in TS 38.304 [20]. |
| threshServingLowP Parameter "ThreshServing, LowP" in TS 38.304 [20]. |
| threshServingLowQ Parameter "ThreshServing, LowQ" in TS 38.304 [20]. |
| t-SearchDeltaP Parameter "TSearchDeltaP" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on. |
| t-SearchDeltaP-Stationary Parameter "TSearchDeltaP-Stationary" in TS 38.304 [20]. Value in seconds. Value s5 means 5 seconds, value s10 means 10 seconds and so on. |

| Conditional Presence | Explanation |
| --- | --- |
| HSDN | The field is optionally present, Need R, if speedStateReselectionPars is present; otherwise the field is not present. |
| SharedSpectrum | This field is mandatory present if this intra-frequency operates with shared spectrum channel access in FR1. Otherwise, it is absent, Need R. |
| SharedSpectrum2 | This field is optionally present if this intra-frequency operates with shared spectrum channel access in FR2-2, Need R. Otherwise, it is absent, Need R. |

- Annex B: TP for [C009]

#### 5.5.2.10	Reference signal measurement timing configuration

The UE shall setup the first SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicityAndOffset parameter (providing Periodicity and Offset value for the following condition) in the SSB-MTC configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the following condition:

SFN mod T = (FLOOR (Offset/10));

if the Periodicity is larger than sf5:

subframe = Offset mod 10;

else:

subframe = Offset or (Offset +5);

with T = CEIL(Periodicity/10).

If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2 configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc configuration for that frequency. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell or serving cell (for cell reselection) meeting the above condition.

If smtc3list is present, for cells indicated in the pci-List parameter in each SSB-MTC3 element of the list in the same MeasObjectNR, the IAB-MT shall setup an additional SS block measurement timing configuration in accordance with the received periodicityAndOffset parameter (using same condition as smtc1 to identify the SFN and the subframe for SMTC occasion) in each SSB-MTC3 configuration and use the duration and ssb-ToMeasure parameters from each SSB-MTC3 configuration.

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR serving cell meeting the above condition.

If smtc5list is present, for cells indicated in the pci-List parameter in each SSB-MTC5 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity (if present) and offset parameter in each SSB-MTC5 configuration and use the duration parameter from the smtc1 configuration. If the periodicity parameter is absent for an SSB-MTC5 configuration, the UE uses the periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR serving cell meeting the above condition.

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).