3GPP TSG-RAN WG2 Meeting #131bis	R2-2506871

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	8.9.4

Source: 	CATT, Samsung, Thales, Google

Title: 	Discussion on relaxation of IDLE mode task based on the S&F monitoring list

Document for: 	Discussion and Decision

Introduction

In RAN2#130, RAN2 discussed potential RAN2 impacts due to S&F monitoring list and left the following FFSs [1]:

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

In RAN2#131, we did not have enough time to discuss this issue, and it was marked come back in the next meeting [2].

Proposal 2: Clarify that the satellite IDs within the S&F monitoring list refers to the satelliteId with long-term valid ephemeris in SIB32. Can come back in the next meeting

This contribution further discusses the necessary cell reselection enhancement based on the S&F monitoring list, and provides corresponding TPs to justify the Spec impacts needed.

Discussion

As illustrated by the coversheet and the changes of the agreed CR [3], SA2 agreed that the S&F Monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Citation from coversheet part [3] Reason for change:; As described in the discussion paper (S2-2500780), this contribution removes this EN: Editor’s Note: How to handle the specification of the “List of satelliteID(s)” and “S&F Monitoring List” as per conclusions in TR 23.700-29 is FFS. The satellite list is a tool which is used by the UE assist with saving power and determining which satellites to synchronise data with, and a useful tool for the network to guide the UEs allowing the network to save resources. While how UE behaves when receiving the satellite list from MME is UE implementation. It is difficult to define the UE behaviour to only follow the received satellite list to access the network. This contribution proposes MME sends S&F Monitoring list to UE and UE behaviour is based on UE implementation and it may use it to determine which satellites to access. For example, if the UE determines a satellite is operating in S&F Mode, the UE may follow the S&F Monitoring List to ensure a successful access; if the UE determines a satellite is not operating in S&F Mode, the UE can access to this satellite even it is not in the S&F Monitoring List..

Citation from TS 23.401, 4.13.9.1 [4] […] - When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause. NOTE 5: When the S&F Wait Timer is running, the power consumption optimization behaviours, if any, are left for UE implementation e.g. whether to listen to paging or deactivate its Access Stratum functions. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE 6: The S&F Wait Timer or S&F Monitoring List doesn't affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE 7: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. […]

Observation 1: SA2 agreed that the S&F monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Since the UE can only receive MT data/signalling from satellites in the S&F monitoring list, it may choose to only camp on those satellites to save power. Considering that the satellites within the S&F monitoring list are not the satellites which provide neighbour cell coverage (i.e. not the next satellite that provides the immediate neighbour cell coverage), the UE could start/stop idle mode tasks right upon the indicated satellite coming and leaving. In more detail, if the UE determines that it is out of coverage of any target satellite indicated by the S&F monitoring list, the UE operating in S&F mode may relax its IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.).

Note that our intention is to allow UE some flexibility to relax IDLE mode tasks for power saving (i.e. using "the UE may relax…"), but not to prohibit the UE from doing these tasks when there is no coverage of satellite configured by MME. Then whether the UE really do this relaxation is eventually still left to UE implementation, and this can address companies' concern whether UE will miss the satellite outside the S&F monitoring list which is still allowed for UE's access: if the UE cannot accept such missing, it simply does not do the relaxation. Anyway, nothing is mandated, and everything is eventually up to UE implementation.

In accordance with the FFS of RAN2 agreement, the above mentioned mechanism can be simply specified in 4.1 of TS 36.304 [5] as an additional optimization on the basis of discontinuous coverage mechanism, given as follows.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. If S&F Monitoring list is configured by upper layers [23], and if the UE has determined that it is out of coverage of all the satellites in S&F Monitoring list (using the available satellite assistance information), the UE may further not need to perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.

This is proposed below, and a TP to TS 36.304 is shown in Annex.

Proposal 1: When a S&F monitoring list is configured by upper layers, the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.) if the UE determines that it is out of coverage of all target satellite(s) indicated by the list. Endorse the TP in Annex.

Conclusion

In this contribution, some leftover issues on support of S&F operation are discussed with corresponding proposals listed as follows:

Observation 1: SA2 agreed that the S&F monitoring list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling.

Proposal 1: When a S&F monitoring list is configured by upper layers, the UE may stop the IDLE mode tasks (e.g. cell (re)selection, paging monitoring, etc.) if the UE determines that it is out of coverage of all target satellite(s) indicated by the list. Endorse the TP in Annex.

Reference

- RAN2#130 ChairNotes.

- RAN2#131 ChairNotes.

- S2-2502513 Support of S&F Monitoring List in S&F Mode.

- 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access" (V19.2.0).

- 3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

- Annex: TP on TS 36.304

## 4.1	Overview

The idle mode tasks can be subdivided into four processes:

-	PLMN selection;

-	Cell selection and reselection;

-	Location registration;

-	Support for manual CSG selection.

The relationship between these processes is illustrated in Figure 4.1-1.

Figure 4.1-1: Overall Idle Mode process

When a UE is switched on, a public land mobile network (PLMN) is selected by NAS. For the selected PLMN, associated RAT(s) may be set TS 23.122 [5]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With the cell selection, the UE searches for a suitable cell of the selected PLMN and chooses that cell to provide available services, further the UE shall tune to its control channel. This choosing is known as "camping on the cell".

For E-UTRA a cell may be associated with more than one CN type (EPC and/or 5GC) and hence the selected cell can be suitable for more than one CN type. The CN type(s) for which the selected cell is suitable are reported to NAS which selects a CN type to be used for camping and for the NAS registration procedure (see below). Note that CN type selection is only applicabe for UE supporting E-UTRA connected to 5GC.

For E-UTRA a cell may be associated with more than one tracking area. The UE reports all the broadcasted tracking area codes in the selected cell to NAS for registration procedure.

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell and as outcome of a successful Location Registration the selected PLMN becomes the registered PLMN TS 23.122 [5].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. Similar to cell selection procedure, if the reselected cell is an E-UTRA cell and the UE supports E-UTRA connected to 5GC, the CN type(s) for which the cell is suitable are reported to NAS which selects one of them. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed. In RRC_INACTIVE state, if the new cell does not belong to the configured RNA, a RNA update procedure is performed.

If necessary, the UE shall search for higher priority PLMNs at regular time intervals as described in TS 22.011 [4] and search for a suitable cell if another PLMN has been selected by NAS.

Search of available CSGs may be triggered by NAS to support manual CSG selection.

If the UE loses coverage of the registered PLMN, either a new PLMN is selected automatically (automatic mode), or an indication of which PLMNs are available is given to the user, so that a manual selection can be made (manual mode).

Registration is not performed by UEs only capable of services that need no registration.

The UE may perform sidelink communication or V2X sidelink communication or sidelink discovery or NR sidelink communication while in-coverage or out-of-coverage for sidelink, as specified in clause 11.

The purpose of camping on a cell in idle mode is fivefold:

a)	It enables the UE to receive system information from the PLMN.

b)	When registered and if the UE wishes to establish an RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped.

c)	If the PLMN receives a call for the registered UE, it knows (in most cases) the set of tracking areas (in RRC_IDLE state) or RNAs (in RRC_INACTIVE state) in which the UE is camped. It can then send a "paging" message for the UE on the control channels of all the cells in this set of tracking areas. The UE will then receive the paging message because it is tuned to the control channel of a cell in one of the registered tracking areas and the UE can respond on that control channel.

d)	It enables the UE to receive ETWS and CMAS notifications.

e)	It enables the UE to receive MBMS services.

If the UE is unable to find a suitable cell to camp on or if the location registration failed (except for LR rejected with cause #12, cause #14, cause #15 or cause #25, see TS 23.122 [5] and TS 24.301 [16]), it attempts to camp on a cell irrespective of the PLMN identity, and enters a "limited service" state.

When NAS indicates that PSM starts, the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while the UE is in PSM it is up to UE implementation whether it performs the corresponding action immediately or the latest when PSM ends. When NAS indicates that PSM ends, the UE shall perform all idle mode tasks.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. If S&F Monitoring list is configured by upper layers [23], and if the UE has determined that it is out of coverage of all the satellites in S&F Monitoring list (using the available satellite assistance information), the UE may further not need to perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.