3GPP TSG-RAN WG2 Meeting #131bis	R2-2506872

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	8.9.2

Source: 	CATT

Title: 	[C001] Correction on the operation mode of neighbour cells

Document for: 	Discussion and Decision

1	Introduction

This contribution discusses the RIL [C001], which is marked as Todo by the rapporteur.

# C001 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. C001; IoTNTN; 2; sf-OperationModeNeigh; Da Wang; V002; ToDo. [Description]: As we comment during the email discussion, we think the intention of agreement is to add S&F mode and nomal mode indication in SIB33, which doesn’t have the intention to include “barred” and “notBarred” in SIB33. Because currently the barred/notBarred information is not provide from neighbour cell to serving cell.

2	Discussion

In RAN2#131, we discussed whether to provide the S&F mode indication of neighbour cells and reached the following agreement [1].

Proposal 5 The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite. - Sateliot would like to confirm the WA and supports the proposal - QC supports this but thinks transition time could be optional (and thinks UE behaviour could left to UE implementation) - Panasonic thinks this should be per satellite ID - vivo thinks it’s not clear how this would be used by the UE. MTK agrees - Ericsson supports this and thinks we should clarify the UE behaviour in terms of cell reselection. IDC agrees - ZTE supports this - Nokia thinks the information would be useful at the application layer The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

In the agreement, the field name sf-OperationMode is used, which is defined for the serving cell’s operation mode and barring status. Based on the agreement, the CR [2] is implemented as follows, i.e., not only the operation mode but also the barring status of the neighbor cells are provided in SIB33.

NeighSatelliteInfo-v19xy ::= SEQUENCE { sf-OperationModeNeigh-r19 ENUMERATED {barred, notBarred} OPTIONAL, -- Need OP t-ModeSwitchingNeigh-r19 TimeOffsetUTC-r17 OPTIONAL -- Need OR }

However, on this agreement, we understand the intention is only to provide the neighbour cell’s operation mode in SIB33, rather than providing the barring status together. If we support providing the neighbor cell’s barring status, the following issues need to be considered:

- A cell cannot obtain its neighbour cell’s barring status. Considering the neighbour cell’s barring status is related to its load situation and is changing dynamically, the neighbour cell’s barring status cannot be preconfigured by O&M. And transmitting the barring status through X2 interface [3] is not supported also. RAN3 enhancement is needed.

- Even if the serving cell could obtain the neighbour cell’s barring status, the changes of barring status will lead to frequent updates of system information. This brings frequently signalling reception and energy consumption for the UE.

- Currently, we only agreed that the UE may deprioritize the neighbor cells operating in the S&F mode. It is unclear how the UE uses the neighbour cell’s barring status.

In summary, providing the neighbour cell’s barring status is currently not feasible and the benefit it brings is unclear. When the UE performs cell (re)selection, it is sufficient for the UE to obtain the neighbour cell’s barring status by reading that cell’s SIB1 according to legacy behavior; further enhancement is not required. The TP is given in Annex.

Observation 1: If we support providing the neighbour cell’s barring status, the following issues need to be considered:

- How could the serving cell obtain the neighbour cell’s barring status is unclear. The enhancement on X2 interface in RAN3 is needed;

- Changes of neighbour cell’s barring status will lead to frequent updates of system information, which is not conducive to UE energy consumption.

- It is unclear how the UE uses the neighbour cell’s barring status.

Observation 2: It is sufficient for the UE to obtain the neighbour cell’s barring status by reading that cell’s SIB1 according to legacy behaviour.

Proposal 1: For the information of neighbour cell in SIB33, only the operation mode of neighbour cell, not the barring status, should be provided. Adopt the TP in Annex.

3	Conclusion

In this contribution, RIL [C001] on neighbour cell’s S&F operation is discussed with observations and proposal listed as follows:

Observation 1: If we support providing the neighbour cell’s barring status, the following issues need to be considered:

- How could the serving cell obtain the neighbour cell’s barring status is unclear. The enhancement on X2 interface in RAN3 is needed;

- Changes of neighbour cell’s barring status will lead to frequent updates of system information, which is not conducive to UE energy consumption.

- It is unclear how the UE uses the neighbour cell’s barring status.

Observation 2: It is sufficient for the UE to obtain the neighbour cell’s barring status by reading that cell’s SIB1 according to legacy behaviour.

Proposal 1: For the information of neighbour cell in SIB33, only the operation mode of neighbour cell, not the barring status, should be provided. Adopt the TP in Annex.

4	Reference

- R2-2506203	Report from Break-out session on NR-NTN and IoT-NTN.

- R2-2506573	Introduction of IoT NTN Ph3.

- TS36.420	Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 general aspects and principles.

5	Annex

–	SystemInformationBlockType33

The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...,

[[	neighSatelliteInfoList-v19xy	NeighSatelliteInfoList-v19xy	OPTIONAL	-- Need OR

]]

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfoList-v19xy ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-v19xy

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

NeighSatelliteInfo-v19xy ::=	SEQUENCE {

sf-OperationModeNeigh-r19		ENUMERATED {true}	OPTIONAL,	-- Need OP

t-ModeSwitchingNeigh-r19		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType33 field descriptions

epochTime Epoch time of the neighbour satellite ephemeris data and common TA parameters, see TS 36.213 [23]. The reference point for epoch time of the neighbour satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell when this field is provided in an NTN cell and the eNB when this field is provided in a TN cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. If this field is absent in an NTN cell, the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. The startSFN indicates the SFN nearest to the frame where the message indicating the epochTime is received. If this field is absent in a TN cell, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB33(-NB) is transmitted.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the (default) value of 0.

neighSatelliteInfoList Neighbour satellite information. If E-UTRAN includes neighSatelliteInfoList-v19xy, it includes the same number of entries and listed in the same order as in neighSatelliteInfoList-r18.

neighValidityDuration Validity duration of the neighbour satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. If this field is absent in an NTN cell, the UE uses validity duration from the serving cell assistance information. If this field is absent in a TN cell, how the UE sets validity duration is left to UE implementation.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

sf-OperationModeNeigh If present, indicates that the neighbour cell associated with the satellite is operating in the Store and Forward Satellite operation mode. If absent, indicates that the neighbour cell associated with the satellite is operating in normal mode.

t-ModeSwitchingNeigh If sf-OperationModeNeigh is present for a neighbour cell associated with the satellite, this field indicates the time information on when this neighbour cell is going to switch from the Store and Forward Satellite operation mode to the normal mode; otherwise, this field indicates the time information on when this neighbour cell is going to switch from the normal mode to the Store and Forward Satellite operation mode.

t-ServiceStartNeigh Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite indicated by satelliteId, see 5.5.3.1, 5.5.8 and 36.304 [4]. This field is only present for the NTN quasi-Earth fixed neighbour cell(s).