3GPP TSG-RAN WG2 Meeting #131	bis			               R2-2506878

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	8.5.3

Source:	Samsung

Title:	Handling of MAC open issues

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss the following open issues.

MAC Issue 1: RA-RNTI for PRACH adaptation (described in LS R1-2506587)

MAC issue 2: whether "initialDownlinkBWP-RedCap" can be configured before SIB1 request procedure (e.g. acquired by UE earlier) and can be used for PDCCH monitoring. If so, can the network know that for a redcap UE performing an OD-SIB1 request.

# Discussion

## 2.1. MAC Issue 1

RAN2 agreement

- No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

RAN1 LS

- There is no proper place in RAN1 specifications to capture the impact on f_id since f_id is not defined in RAN1 specifications. The index nRA in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id.

There is no technical issue with the agreement. The issue is mainly where to capture the agreement. Since it can not be captured in RAN1 specification, it is straightforward to capture in MAC spec.

Proposal 1a: Capture the RAN2 agreement in MAC spec.

Proposal 1b: Adopt the following TP in MAC

### 5.1.3	Random Access Preamble transmission

:

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). The PRACH occasions which are configured by msg1-FrequencyStart-r19 and msg1-FDM-r19 in addlRACH-Config-Adapt of RACH-ConfigCommon and FDMed with other PRACH occasions configured by the msg1-FrequencyStart and msg1-FDM of RACH-ConfigCommon, are sequentially indexed from f_id equals to msg1-FDM to msg1-FDM-r19 minus 1, starting from the lowest frequency indicated by msg1-FrequencyStart-r19.

## 2.2. MAC Issue 2

SIB1 request is based on od-sib1-Config in SIBx. UE applies the downlink time/frequency resource information in SIBx for SIB1 request. Similarly, for the reception of SIB1, UE receives SIB1 in the window (od-sib1-WindowDuration) based on od-sib1-Config in SIBx. UE applies the downlink time/frequency resource information in SIBx for SIB1 request. There is no application of BWP for the SIB1 request/response.

The following text in MAC CR needs to be updated.

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request or SIB1 request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

Proposed change

1>	if the Random Access procedure was not initiated for SIB1 request:

2> if initialDownlinkBWP-RedCap is configured:

3> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

4> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

3> else:

4> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

2> else:

3> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

Proposal 2a: "initialDownlinkBWP-RedCap" is not applicable for SIB1 request procedure. SIB1 request and SIB1 reception upon SIB1 request is based on the od-sib1-Config.

Proposal 2b: Adopt the following TP in MAC

1>	if the Random Access procedure was not initiated for SIB1 request:

2> if initialDownlinkBWP-RedCap is configured:

3> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

4> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

3> else:

4> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

2> else:

3> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

# Conclusion

Proposal 1a: Capture the RAN2 agreement in MAC spec.

Proposal 1b: Adopt the following TP in MAC.

### 5.1.3	Random Access Preamble transmission

:

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). The PRACH occasions which are configured by msg1-FrequencyStart-r19 and msg1-FDM-r19 in addlRACH-Config-Adapt of RACH-ConfigCommon and FDMed with other PRACH occasions configured by the msg1-FrequencyStart and msg1-FDM of RACH-ConfigCommon, are sequentially indexed from f_id equals to msg1-FDM to msg1-FDM-r19 minus 1, starting from the lowest frequency indicated by msg1-FrequencyStart-r19.

Proposal 2a: "initialDownlinkBWP-RedCap" is not applicable for SIB1 request procedure. SIB1 request and SIB1 reception upon SIB1 request is based on the od-sib1-Config.

Proposal 2b: Adopt the following TP in MAC

1>	if the Random Access procedure was not initiated for SIB1 request:

2> if initialDownlinkBWP-RedCap is configured:

3> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

4> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

3> else:

4> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

2> else:

3> monitor the PDCCH on the BWP configured by initialDownlinkBWP.

# References