3GPP TSG-RAN WG2 Meeting #131bis	R2-2506882

Prague, Czech Republic, Oct. 13th-17th

Source: 	China Telecom

Title: 	Discussion of NB-IoT voice over GEO

Agenda Item:	9.7.2

Document for:	Discussion and decision

- Introduction

The WID of R20 IoT NTN has approved in RAN#108 meeting. The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives:

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2]

  - Coordination with SA is required

  - The objective can be updated with further details based on the outcome of the above down-selection

- Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1]

- Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2]

- Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2]

- Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4]

- Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements

- Note: Coordination with SA4 is expected

In this contribution, we provide our consideration on CP and UP solutions for this WI.

- Discussion

For NB-IoT, both CP and UP solution exist for data transmission. As discussed in previous meetings, the selection between these two options does not converge. For CP solution, it is simple for implementation. According to TS 36.331, A NB-IoT UE that only supports the Control Plane CIoT EPS optimisation (see TS 24.301) or the Control Plane CIoT 5GS optimisation (see TS 24.501) does not need to support any DRBs and associated procedures. SRB1bis is used for NAS message transmission.

Observation 1: CP solution is simple for implementation.

For supporting NB-IoT voice over GEO, it is more challenge for legacy NB-IoT CP solutions. First, it is a real time service. The data rate and delay needs to be guaranteed. However, there is no QoS management for SRB. Another problem is the efficiency of scheduling. The voice date packets arrive periodically. For overhead reduction, SPS is needed for semi-persistent scheduling.

Proposal 1: CP solution needs to consider the QoS management and SPS for voice services.

For UP solution, separated DRBs for signalling and voice packets need to be established due to different transport requirements for these two types of packets. Voice packets require consistent jitter and delay handling and may benefit from prioritization or special treatment in satellite links. This separation also aligns with how IMS networks typically handle control and media, enabling better traffic management and potential for differentiated satellite resource allocation.

Current specifications already support the establishment of up to two DRBs for NB-IoT, so this solution is not expected to introduce significant additional changes. Moreover, the UP solution inherently supports SPS based on RRC reconfiguration messages, which is essential for periodic small-packet transmissions such as VoIP, effectively reducing latency and control signalling overhead.

Observation 2: UP solution inherently supports SPS of voice.

With ROHC applied, the overhead of the UP solution can be reduced to 7 bytes, while the CP solution still requires 15 bytes even after optimization. For ultra-narrowband voice with a bitrate of only 1.2 kbps, lower overhead results in a higher proportion of effective data (63.2% for UP vs. 44.4% for CP). This directly translates to greater system capacity potential and contributes to improved voice quality. Thus, we think ROHC is more useful for UP solution.

Proposal 2: UP solution needs to consider the ROHC for overhead reduction.

- Conclusion

This contribution discussed the CP and UP solution of NB-IoT voice over GEO. Proposals and observations are summarized as follows:

Observation 1: CP solution is simple for implementation.

Observation 2: UP solution inherently supports SPS of voice.

Proposal 1: CP solution needs to consider the QoS management and SPS for voice services.

Proposal 2: UP solution needs to consider the ROHC for overhead reduction.