3GPP TSG-RAN WG2 Meeting #131bis	R2-2506884

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.3.2

Source: 	China Telecom

Title:	Discussion on control plane aspects in 6GR

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

- Introduction

In RAN #108 meeting, the New SID for “Study on 6G Radio” was approved [1] aiming to develop 6GR to meet a broad range of use cases, and requirements as defined during the RAN requirements study. Regarding general control plane design in 6GR, the following RAN2-led objectives are listed in the latest SID RP-252912 for further study.

Control plane related objectives in the latest SID RP-252912:

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4].

- Studies on intial access procedure, random access procedures, system information and paging [RAN2,RAN1,RAN4]

In this contribution, we would like to share our opinions on control plane aspects in 6GR, including cell management, RRC state management, system information and paging in 6GR.

- Discussion

Cell management in 6GR

Spectrum serves as the fundamental resource for wireless communication. Drawing from the deployment experiences of 5G spectrum, the mid-low frequency bands will continue to be the coverage foundation of 6G network. However, operators’ spectrums in the mid-low frequency bands usually have limited bandwidth, and due to spectrum re-farming, these spectrums become more and more scattered with narrower bandwidth available for 6G. Thus, it is even more difficult for operators to find continuous large bandwidth resources in 6G. To meet the capacity, data rates, coverage demands of 6G network, it is meaningful to study efficient utilization of non-contiguous spectrum in 6G.

In NR, the main technologies to utilize multi-band spectrum are based on CA or SUL. However, there are many drawbacks and limitations for the CA or SUL based solutions, considering the aggregation of the more fragmented spectrum in 6G:

- The management of multiple cells based on CA framework leads to the signaling overhead, delay, low spectrum utilization efficiency on account of SCell addition/release, and inflexible carrier selection before entering RRC_CONNECTED state. When more small bandwidth carriers especially considering the multiple fragmented spectrums in the mid-low frequency band are aggregated as independent PCell and SCells, handover due to UE mobility, load balance etc. requires further addition/release/activation/deactivation of SCells before/during/after the HO procedure, which causes much signaling overhead and delay. Moreover, since SCells are enabled in RRC_CONNECTED state, the RRC_IDLE/ RRC_INACTIVE state UE can only initiate random access on the selected UL/DL carrier associated with the determined PCell by cell searching procedure. This means that before entering the RRC_CONNECTED state, the available UL/DL carrier is determined only from PCell, which is not beneficial for load balancing and random access performance.

- SUL operation has the limitation on supporting only one available carrier in UL of a single cell at one time, which could not meet the demands to aggregate more than two carriers for the fragmented spectrum. When more than two carriers are required to support some high data rates services such as immersive multimedia, it must be combined with CA.

Therefore, new multi-carrier operation and cell management technologies could be studied to overcome the limitations and utilize the spectrum effectively and efficiently in 6G. From our perspective, potential solutions on multi-carrier operation and cell management in 6GR are discussed as follows.

Single cell multiple carriers

This technique enables multiple UL/DL carriers configured and managed in one serving cell, and these carriers can be intra-band, and/or inter-band. Multiple cells are supported to be aggregated based on CA, for example when the carriers associated to different cells belong to different frequency ranges. Fig. 1 is an illustration for the association between multiple carriers and cells.

Fig. 2 Schematic diagram for single cell multiple carriers

As multiple carriers are regarded as one serving cell, characteristics to be further discussed with anticipated benefits are:

- Random access carrier selection to reduce the congestion and increase the successful rate: The multiple carriers of a cell are visible by a UE in RRC_IDLE/ RRC_INACTIVE state, so that the UE can select a more suitable UL/DL carrier for random access procedure.

- No handover procedure across carriers in a cell and fast carrier activation/deactivation to reduce the latency and signaling overhead when the camping carrier is switched to an intra cell carrier: Intra cell carrier switching can replace conventional inter cell handover, eliminating the need for SCell addition/release/activation/deactivation. This is not only seamless mobility but also has the same benefits for load balancing.

- Taking the spectrum resource of multiple carriers as a virtual large bandwidth resource in a single cell, which can be flexibly and efficiently scheduled with performance improvement: A single TB mapping to multiple carriers in a cell can be scheduled, which brings potential benefits including overhead reduction and higher channel coding gain.

As analysed above, it is worth to study a new framework for multi-carrier operation and cell management in 6GR, not only to utilize the spectrum effectively and efficiently, but also to further reduce latency and system overhead.

Proposal 1: Regarding the cell management in 6GR, a new framework supporting single cell multiple carries could be studied for more efficient spectrum utilization and further reduction of the latency and the system overhead.

Flexible UL DL carrier association

In NR, the UL operating band and DL operating band have a fixed association based on FDD or TDD duplex mode. For an FDD band, the UL and DL spectrums are defined in specification, which are used in pair with fixed one to one association. For a TDD band, UL and DL operate in the same spectrum. Multiple intra band or inter band UL/DL spectrums can be aggregated based on CA or SUL within specification defined band combinations according to the UL DL spectrum association. In order to meet flexible UL/DL traffic patterns and service requirements, the support of flexible UL DL carrier association in 6G could be investigated.

To enable flexible UL DL carrier association, UL and DL carriers could be selected and combined from spectrum resource pools, which can be dynamically or semi-statically configured by the NW. The flexible association between UL carriers with same or different number of DL carriers could be studied. The support of at least a DL carrier in one FDD band associated with a UL carrier in another FDD band, and a UL/DL carrier in one FDD band associated with the carrier in another TDD band can be further discussed, as illustrated in Fig. 2.

Fig. 2 Flexible FDD-FDD, FDD-TDD UL DL carrier association

The flexible association between UL and DL carriers are useful for NW, not only to adapt different UL/DL traffic load, but also to guarantee both capacity and coverage performance with less demand on involved number of carriers. Thus, we suggest

Proposal 2: The flexible association between UL and DL carriers could be studied and supported for the cell management framework in 6GR.

RRC state management in 6GR

In NR, there are three RRC states including RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED. Regarding RRC_INACTIVE state, it is introduced mainly for UE power saving as well as access latency reduction. For 6GR, it is also very important to consider the UE power saving and access latency reduction requirements. An intermediate state between RRC_IDLE and RRC_CONNECTED is still needed in 6GR and we can take the RRC_INACTIVE state in NR as a starting point for further study.

Proposal 3: An intermediate state between RRC_IDLE and RRC_CONNECTED could be supported in 6GR and the RRC_INACTIVE state in NR could be taken as a starting point for further study.

Moreover, the RRC_INACTIVE state in 5G is not widely commercialized. The potential concerns maybe the actual benefit or performance gains brought by RRC_INACTIVE state. In order to introduce an intermediate state that is worthwhile to be used in commercial 6G NW, further enhancement shall be considered to avoid unnecessary state transition and signaling overhead, and to further improve balance of performance between UE power consumption and access latency.

Proposal 4: Necessary enhancement on the intermediate RRC state in 6GR shall be considered to have more performance gains on UE power saving and access latency reduction.

System information and paging in 6GR

According to the SID of 6GR [1], one of the objectives is energy saving both for network and device.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device.

In 5G, system information are divided into Minimum SI (i.e. MIB and SIB1) and Other SI. MIB and SIB1 are periodically broadcast, and Other SI are provided on demand. In 5G-A, SSB-less SCell operation for inter-band CA, on-demand SSB operation and SSB periodicity adaptation are introduced to reduce network energy consumption on SCells in Rel-18 and Rel-19. On-demand SIB1 for UEs in idle/inactive mode is introduced to reduce network energy consumption due to periodic SIB1 transmissions in Rel-19. However, since the network energy saving features are not adopted from day one, to minimize the impact to legacy UEs, the network energy saving techniques are applied to limited scenarios, such as SCell, RRC state. For better network energy saving, in 6G, longer default SSB/SIB1 periodicity can be taken into account. To ensure user experience, flexible additional SSB/SIB1 transmission mechanism can be the supplement. Therefore, it is suggested to study flexible SSB/SIB1 transmission mechanism (e.g. on-demand and/or adaptive SSB/SIB1 transmission) with long default SSB/SIB1 periodicity to balance network energy saving and users’ experience. Regarding SIBs other than SIB1, the on-demand transmission solution can be the start point.

Proposal 5: Study flexible SSB/SIB1 transmission (e.g. on-demand and/or adaptive SSB/SIB1 transmission) with long default SSB/SIB1 periodicity.

Proposal 6: Regarding the transmission mechanism of SIBs other than SIB1, 5G can be a start point.

In 5G, Paging Early Indication is introduced to reduce UE power consumption due to false paging alarms in Rel-17. In 5G-A, adaptation of paging (i.e. the value range for parameter N and the number of POs within one PF are extended to have increased interval between PFs but increased number of POs within one PF) is supported for network energy saving in Rel-19. The PFs and the POs in one PF are determined by UE_ID, UE DRX cycle, and parameters (i.e. parameter N and the number of POs within one PF) configured by the network. However, from the perspective of the network, the distribution of the PFs/POs of UEs may be dispersive and not friendly to base station sleep. Thus we suggest to consider both the base station energy saving and UE energy saving into the 6G paging design.

Proposal 7: Consider both network energy saving and UE energy saving into the 6GR paging design.

- Conclusion

In this contribution, we discuss control plane aspects in 6GR including cell management, RRC state management, system information and paging. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Regarding the cell management in 6GR, a new framework supporting single cell multiple carries could be studied for more efficient spectrum utilization and further reduction of the latency and the system overhead.

Proposal 2: The flexible association between UL and DL carriers could be studied and supported for the cell management framework in 6GR.

Proposal 3: An intermediate state between RRC_IDLE and RRC_CONNECTED could be supported in 6GR and the RRC_INACTIVE state in NR could be taken as a starting point for further study.

Proposal 4: Necessary enhancement on the intermediate RRC state in 6GR shall be considered to have more performance gains on UE power saving and access latency reduction.

Proposal 5: Study flexible SSB/SIB1 transmission (e.g. on-demand and/or adaptive SSB/SIB1 transmission) with long default SSB/SIB1 periodicity.

Proposal 6: Regarding the transmission mechanism of SIBs other than SIB1, 5G can be a start point.

Proposal 7: Consider both network energy saving and UE energy saving into the 6GR paging design.

- Reference

RP-251881	Study Item: Study on 6G Radio