3GPP TSG-RAN WG2 Meeting #131bis	R2-2506885

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.4

Source: 	China Telecom

Title:	Discussion on mobility aspects in 6GR

WID/SID:	FS_6G_Radio

Document for:	Discussion and Decision

- Introduction

In RAN #108 meeting, the New SID for “Study on 6G Radio” was approved [1] aiming to develop 6GR to meet a broad range of use cases, and requirements as defined during the RAN requirements study. Regarding mobility in 6GR, the following RAN2-led objectives are listed in the latest SID RP-252912 for further study.

Control plane related objectives in the latest SID RP-252912:

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

In this contribution, we would like to share our opinions on mobility aspects in 6GR, including intra/inter-RAT mobility for UE in idle/inactive/connected mode.

- Discussion

6GR Mobility management for UE in Idle/Inactive mode

In 6GR, assuming that there are similar RRC states including RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED to NR, when the UE in Idle/Inactive mode moves, cell selection or reselection may occur. Considering the network energy saving requirements for 6GR, it is very likely to further study system information and paging enhancement for the support of network energy saving. Besides, in order to support more efficient spectrum utilization in 6GR, further study on multi-carrier operation and cell management will be considered. All these aspects will impact the general cell selection/reselection procedure design in 6GR.

Proposal 1: Regarding 6GR Mobility management for UE in Idle/Inactive mode, take network energy saving enhancements and multi-carrier enhancements into account for the general cell selection/reselection procedure design in 6GR.

Moreover, network slicing is an important feature in 5G, which can allow differentiated treatment depending on each customer requirements. However, network slicing is supported in the first release of NR only from CN point of view, and the support of RAN slicing is introduced in Rel-17. For 6GR, slicing is also a promising feature for operator’s commercial network, which shall be supported in 6G DAY1. To have better performance, RAN slicing shall be considered in the first release of 6G. Slice aware cell reselection for 6GR could be studied and the design in NR could be taken as a starting point for further study.

Proposal 2: Slice aware cell reselection for 6GR shall be considered and the design in NR could be taken as a starting point for further study.

Regarding the inter-RAT mobility for UE in idle/inactive mode in 6GR, cell reselection between 6G and 5G/4G shall be considered. Considering the deployment of 6G will be step by step and 4G/5G commercial network would coexist with 6G in a period of time, inter RAT cell reselection between 6GR and NR/EUTRA should be supported in 6G DAY1.

Proposal 3: Regarding the inter-RAT mobility for UE in idle/inactive mode in 6GR, cell reselection between 6GR and NR/EUTRA shall be supported in 6G DAY1.

6GR Mobility management for UE in connected mode

With increased frequency carrier in 5G/5G-A evolution, UE in connected mode experiences more frequent handover and PSCell Change in such scenarios. To improve network mobility performance and provide better user experience, mobility enhancement has always been one of the most popular topics through the evolution of NR. As shown in Table.1, from NR R16 to R19 and including the upcoming R20, several mobility enhancements are introduced to reduce HO latency and/or improve robustness.

Table.1 Mobility enhancement from NR R16 to R20

|  | Rel-16 | Rel-17 | Rel-18 | Rel-19 | Rel-20 |
| --- | --- | --- | --- | --- | --- |
| 0ms interruption | DAPS | - | - | - | - |
| Lower latency | - | - | LTM (intra-CU) | LTM (inter-CU) | SCell activation for LTM |
| Higher robustness | CHO/CPC | CPAC | SCPAC | C-LTM (intra-CU) | - |

However, since the above enhancements are not introduced from the first release of NR, it is harder to implement these enhancements in the commercial network. Besides, to minimize the impact to legacy gNBs and UEs, the coexistence among these features are needed to be considered, which makes the overall design on NR mobility management very complex.

Regarding 6GR Mobility management for UE in connected mode, it is still required to support HO with lower latency as well as higher robustness. To have a simple design and make it easy to be implemented in commercial network, it is better to consider a unified framework for 6GR mobility management including L3 HO, LTM and CHO in 6G DAY1.

Proposal 4: Regarding 6GR Mobility management for UE in connected mode, a unified framework for 6GR mobility management including L3 HO, LTM and CHO shall be supported in 6G DAY1.

In NR, early TA acquisition is supported for LTM in order to reduce the handover latency, including the PDCCH order triggered early TA acquisition and UE-based TA measurement. If there is a valid TA value, the UE performs RACH-less LTM so that the HO latency is highly reduced. But for CHO, early TA acquisition and RACH-less HO are not supported. In order to reduce HO latency, early TA acquisition and RACH-less HO shall be supported in 6G DAY1.

Proposal 5: In order to reduce HO latency, early TA acquisition and RACH-less HO shall be supported in 6G DAY1.

Moreover, subsequent LTM is supported in NR without the need to release, reconfigure or add other LTM candidate configurations after each LTM cell switch completion, which can help reduce the signalling overhead. Similarly, subsequent CPAC is also supported in NR. However, subsequent CHO is not supported, which lead to more signalling overhead on RRC reconfiguration in order to execute the next CHO. In order to reduce signalling overhead on RRC reconfiguration for HO, subsequent HO shall be supported in 6G DAY1 without the need to release, reconfigure or add other candidate configurations after each HO completion.

Proposal 6: In order to reduce signalling overhead, subsequent HO shall be supported in 6G DAY1 without the need to release, reconfigure or add other candidate configurations after each HO completion.

Regarding the inter-RAT mobility for UE in connected mode in 6GR, similar to the cell reselection case, HO between 6G and 5G/4G shall be considered due to the coexistence with NR/EUTRA in the early stage of 6G commercial network. At least inter RAT HO between 6GR and NR should be supported in 6G DAY1, while inter RAT HO between 6GR and EUTRA could be treated as second priority.

Proposal 7: Inter RAT HO between 6GR and NR should be supported in 6G DAY1.

- Conclusion

In this contribution, we discuss mobility aspects in 6GR, including intra/inter-RAT mobility for UE in idle/inactive/connected mode. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Regarding 6GR Mobility management for UE in Idle/Inactive mode, take network energy saving enhancements and multi-carrier enhancements into account for the general cell selection/reselection procedure design in 6GR.

Proposal 2: Slice aware cell reselection for 6GR shall be considered and the design in NR could be taken as a starting point for further study.

Proposal 3: Regarding the inter-RAT mobility for UE in idle/inactive mode in 6GR, cell reselection between 6GR and NR/EUTRA shall be supported in 6G DAY1.

Proposal 4: Regarding 6GR Mobility management for UE in connected mode, a unified framework for 6GR mobility management including L3 HO, LTM and CHO shall be supported in 6G DAY1.

Proposal 5: In order to reduce HO latency, early TA acquisition and RACH-less HO shall be supported in 6G DAY1.

Proposal 6: In order to reduce signalling overhead, subsequent HO shall be supported in 6G DAY1 without the need to release, reconfigure or add other candidate configurations after each HO completion.

Proposal 7: Inter RAT HO between 6GR and NR should be supported in 6G DAY1.

- Reference

RP-251881	Study Item: Study on 6G Radio