3GPP TSG-RAN WG2 #131-bis	R2-2506887

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.2

Source:	T-Mobile USA, Ericsson

Title:	SSB transmissions in 6GR NES

Document for:	Discussion

# 1	Introduction

The 6GR Study Item (SI) description was approved in RAN#108 [1]. One of the objectives of this SI is to enhance energy efficiency for both network (NW) and user equipment (UE). Building upon existing energy saving capabilities of 5G NR - most notably its lean time-domain design with respect to LTE - the introduction of 6G Radio (6GR) baseline features offers a rare, once-in-a-decade opportunity to embed energy efficient principles from the outset. Failing to do so could result in restricting the adoption of crucial energy efficient features due to backward-compatibility requirements in later releases.

In this contribution, we discuss the need to specify sparsity across all transmissions and receptions in the first 6GR release as baseline and propose to study the time-domain adaptation of SSB transmissions.

# 2	Discussion

The introduction of a lean carrier design in NR, characterized by reduced always-on transmissions - such as SSBs transmitted with a 20ms periodicity - has enabled notable energy savings for the radio network. This provided a significant improvement over LTE, which requires frequent transmissions of reference signals including CRS, PSS, and SSS. By reducing the time-domain activity, NR facilitates micro-sleep opportunities for base stations, thereby enhancing overall network energy efficiency.

The 20ms SSB periodicity in NR illustrates how lean carrier design enables substantial network energy savings relative to LTE, primarily by reducing always-on transmissions.

In Release 19, some enhancements for broadcast transmission design were introduced in NR, where focus was on prolonging sleep opportunities by extending the intervals between broadcast transmissions (e.g., SSB transmission in SCells, SIB1 transmissions and Paging Occasions in PCells) and reducing the frequency of reception occasions (random access and Scheduling Requests). However, to avoid any impact on performance in general, the intention in Release 19 was to provide such broadcast signalling, when necessary, i.e., on demand, instead. Even though, features introduced within the context of Release 19 Network Energy Saving (NES) WI are shown to be beneficial from network energy saving perspective, benefits are somewhat limited due to the need to avoid impact on legacy UEs.

In 6GR, RAN2 should seize the opportunity to introduce a clean slate design and consider what one can achieve further in the first release when there are no legacy UEs that need to be considered. For broadcast transmissions a lean design, with its full potential, can be realized if sparsity can be maintained across all transmissions and receptions on the same time scale on any carrier. RAN2 should introduce the mechanisms to specify such sparsity in the first 6GR release to avoid any limitation at a later stage due to legacy UEs.

For broadcast transmissions, full benefits of lean design can be achieved if sparsity can be maintained across all transmissions and receptions on the same time scale for any carrier.

Study mechanisms to extend the default SSB periodicity and to introduce on-demand SSB transmissions with the goal to establish sparsity across all transmissions targeting UEs in both connected and non-connected states in the first 6GR release.

## 2.1	Extending the default SSB periodicity

In NR Release 19, enhancements towards lean SSB transmissions were introduced. In 6GR, such enhancements should be adopted but built upon. Improvements are required, as those enhancements do not apply to primary cells (PCells) and do not allow sufficiently sparse SSB transmissions. In particular, the 20ms default SSB periodicity in cells supporting initial access limits opportunities for deeper sleep states for the network, despite efforts to minimize the transmission and reception footprint of signals for UEs in idle and inactive states, such as adaptation of random access and paging occasions.

The 20ms default SSB periodicity in NR in cells supporting initial access, remains a key limiting factor that restricts network’s ability to transition into deep sleep states.

In NR, the 20ms default SSB periodicity originates from initial cell search requirements, ensuring short scan times during blind searches (i.e., without prior knowledge) across multiple frequency bands, as the minimum dwell time per Global Synchronization Channel Number (GSCN) raster point is tied to the SSB periodicity. In practice, however, such completely blind searches are rare, since most UEs use location and history-based strategies and typically retain partial knowledge of serving frequencies or bands. Truly blind scenarios (e.g., those that may be encountered at the airports) are uncommon and should not dictate the default reference signal periodicity in 6GR.

Blind initial cell searches are rare in practice, as most UEs leverage prior knowledge, such as PLMN scanning logic, during cell search.

Constraining adjustments to the default SSB periodicity based on the assumption that UEs frequently perform blind searches is not justified.

In order to unlock the full potential of the lean-carrier concept in the time domain, longer values for default SSB periodicity, e.g., 160ms, should be considered to support UEs in non-connected modes, e.g., idle/inactive modes.

Extended values for default SSB periodicity should be considered for UEs in idle/inactive modes when studying 6GR features.

Note that SSB transmission with a default period of 160ms are mainly intended for UEs in non-connected modes. For UEs in connected mode, additional, i.e., on-demand, SSBs transmissions can be introduced, as discussed in the following section.

## 2.2	On-demand SSB

Extending the default periodicity of “always-on” SSBs may impact UE timing and frequency synchronization. To avoid such impact while maintaining energy savings from a longer default SSB periodicity, additional SSBs, i.e., on-demand SSB, can be transmitted during limited periods.

In NR Release 19, on-demand SSBs are limited to secondary cells (SCells) and the specific use case of SCell activation. Dynamic adaptation of SSB periodicity for PCells was also considered but not pursued in NR Release 19 due to backward compatibility concerns and UE assumptions regarding the default SSB periodicity for initial access. These concerns, however, do not apply to 6GR in the first release.

Therefore, it is worth studying whether on-demand SSBs could be beneficial for a wider range of use cases, supporting UEs in non-connected modes as well as UEs in connected mode on any carrier. Expanding the applicability of on-demand SSBs would enable more adaptive signaling and greater energy savings.

Study on-demand SSB transmission for both connected and non-connected UEs.

# 3	Conclusion

In this contribution we discuss the need to specify sparsity across all transmissions and receptions in the first 6GR release and propose to study the time-domain adaptation of SSB, including the extension of the default SSB periodicity and on-demand SSB. Based on the discussion in the previous sections we made the following observations:

Observation 1	The 20ms SSB periodicity in NR illustrates how lean carrier design enables substantial network energy savings relative to LTE, primarily by reducing always-on transmissions.

Observation 2	For broadcast transmissions, full benefits of lean design can be achieved if sparsity can be maintained across all transmissions and receptions on the same time scale for any carrier.

Observation 3	The 20ms default SSB periodicity in NR in cells supporting initial access, remains a key limiting factor that restricts network’s ability to transition into deep sleep states.

Observation 4	Blind initial cell searches are rare in practice, as most UEs leverage prior knowledge, such as PLMN scanning logic, during cell search.

Observation 5	Constraining adjustments to the default SSB periodicity based on the assumption that UEs frequently perform blind searches is not justified.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Study mechanisms to extend the default SSB periodicity and to introduce on-demand SSB transmissions with the goal to establish sparsity across all transmissions targeting UEs in both connected and non-connected states in the first 6GR release.

Proposal 2	Extended values for default SSB periodicity should be considered for UEs in idle/inactive modes when studying 6GR features.

Proposal 3	Study on-demand SSB transmission for both connected and non-connected UEs.

# References

RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, 3GPP TSG RAN#108, Prague, Czech Republic, June 9-13, 2025.