3GPP TSG-RAN WG2 Meeting #131bis										R2-2506893

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.2

Source: 	Sharp

Title:  	6GR General Aspects

Document for: 	Discussion

# Introduction

3GPP has approved 6G Study Item (SI) which aims to develop one non-backward compatible radio access technology to meet a broad range of use cases and requirements [1]. This document discusses key general aspects of 6GR that should be considered at the early stage of the study.

# Discussion

## RAN Architecture

The need for highly efficient spectrum utilization remains a core driver for 6G. In the RAN-level 6G SI, it was explicitly agreed that the 6G Radio Access Technology (RAT) shall support Spectrum Aggregation to meet extreme throughput and capacity requirements. This aggregation should be available for both uplink and downlink, and applicable to both co-located and non-co-located TRPs.

Historically, Carrier Aggregation (CA) has been the most robust and widely deployed solution for spectrum aggregation since the deployment of LTE. CA is a mature and well-understood technology, having demonstrated its simplicity and effectiveness across commercial networks globally. Due to its established status, CA should be considered the baseline spectrum aggregation option in 6G.

While the 6G study may consider other aggregation options, such as Dual-Connectivity (DC) or multiple active Bandwidth Parts (BWPs), introducing too many independent features with similar fundamental purposes will inevitably increase complexity in the 6G specification and implementation. For the sake of simplification and reduced implementation overhead in the new 6GR design, we should leverage the simplicity of CA as the starting point. The study should then focus on whether the perceived benefits of other, more complex aggregation options genuinely outweigh the benefits of simply enhancing and adapting the mature CA framework for 6G.

Observation 1: CA is a mature and well-understood technology, having demonstrated its simplicity and effectiveness across commercial networks globally.

Proposal 1: Carrier Aggregation is a baseline for spectrum aggregation in 6G.

NR has flexible architecture options in terms of combination of LTE and NR entity/layers. However, it introduced a huge complexity in both standardization and development. In 6G, one or only a few promising options should be considered.

More specifically, NG-RAN is allowed to connect to either EPC or 5G Core. E-UTRAN (LTE) is also allowed to connect to either EPC or 5G Core. Those RAN-CN combination options introduced additional flexibility. For simplified 6G standards, it is reasonable to have a limit to only one RAN-CN architecture option. If 6G CN is standardized in SA, we can assume only 6G-RAN connected to 6G CN.

Observation 2: Multiple RAN-CN combination introduced a huge complexity in both standardization and development.

Proposal 2: RAN2 assumes 6GR/6G-RAN connects to only one core network type (e.g. 6G Core Network).

The 5G CU-DU split was designed to enable flexible C-RAN deployment by centralizing non-real-time functions (CU) and keeping real-time functions local for low latency (DU). However, this architectural promise has not materialized widely in commercial 5G deployments. The implementation complexity and high fronthaul requirements introduced by the CU-DU split often outweighed the theoretical gains in virtualization and resource pooling. Consequently, the overall real-world gain from the CU-DU split may be questionable. Given that 6G aims for massive simplification of RAN architecture, retaining the CU-DU split concept may be counterproductive.

To ensure the simplicity and efficiency of 6GR, the RAN2 6G study should not consider the impact or detailed concept of the CU-DU split. The study should strictly focus on defining essential RAN functionalities needed to meet fundamental 6G requirements from the RAN2 perspective.

Observation 3: Given that 6G aims for massive simplification of RAN architecture, retaining the CU-DU split concept may be counterproductive.

Proposal 3: RAN2 study does not assume CU-DU split in 6GR.

## Capability

In NR, the size of UE capability signaling has become excessive due to the high degree of design flexibility. NR assumed to support various spectrum combinations, aggregation schemes (e.g., CA, DC), and deployment scenarios. This complexity forces UEs to report a massive amount of information to the network. To cope with the large size of signaling, RRC segmentation was introduced. However, segmentation adds latency and consumes valuable radio resources. Critically, this wide flexibility rarely translates into real-world use. Most UEs deploy only a few specific sets of features. The current mechanism may be considered inefficient as it forces the transfer of irrelevant capabilities for the common case.

The shift to the new generation (i.e., 6G) is a crucial opportunity to eliminate this inefficiency. To achieve the goals of simplification, efficiency, and reduced network overhead, the RAN2 6G study should aim to reduce the size of UE capability signaling.

Observation 4: NR capability signaling has become excessive due to the high degree of design flexibility.

Proposal 4: RAN2 study aims to reduce the size of UE capability signaling in 6GR.

To achieve this goal of signaling reduction, the 6GR study should investigate specific optimization mechanisms. One approach is reintroducing a streamlined UE Category system, similar to that used in LTE. This easily allows the network to quickly infer a standard set of capabilities with a minimal signaling footprint. Additionally, the study can explore introducing a capability group concept that avoids the repetitive transfer of individual feature information.

Proposal 5: RAN2 study to consider for 6GR:

- 1) introducing UE category as in LTE

- 2) introducing capability group to cover multiple features.

## 6G Target Services

The 6GR study should be guided by the services of the future. While enhanced Mobile Broadband (eMBB) remains a major requirement, the next generation will be defined by applications needing extreme performance: Ultra-Reliable Low-Latency Communication (URLLC) and Extended Reality (XR). Therefore, the RAN2 study must explicitly target all three (eMBB, URLLC, and XR). URLLC and XR services are transitioning from specialized niches to common and widespread services in the 6G era. URLLC is essential for industrial automation, intelligent V2X systems, and remote operations, and near-perfect reliability. Similarly, XR (VR/AR) requires high throughput combined with extremely low latency to deliver seamless and comfortable immersive experiences.

If 6GR is to succeed in supporting the fully automated and immersive society of the future, its architecture cannot treat URLLC and XR as afterthoughts, which often led to architectural compromises. The initial version of 6GR should fulfill the stringent requirements of URLLC and XR through deep integration of mechanisms for high reliability and ultra-low latency scheduling.

Observation 5: While eMBB remains a major requirement, the next generation will be defined by applications needing extreme performance.

Proposal 6: RAN2 study aims to support eMBB, URLLC and XR as target services of 6G.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals:

Observation 1: CA is a mature and well-understood technology, having demonstrated its simplicity and effectiveness across commercial networks globally.

Proposal 1: Carrier Aggregation is a baseline for spectrum aggregation in 6G.

Observation 2: Multiple RAN-CN combination introduced additional flexibility.

Proposal 2: RAN2 assumes 6GR/6G-RAN connects to only one core network type (e.g. 6G Core Network).

Observation 3: Given that 6G aims for massive simplification of RAN architecture, retaining the CU-DU split concept may be counterproductive.

Proposal 3: RAN2 study does not assume CU-DU split in 6GR.

Observation 4: NR capability signaling has become excessive due to the high degree of design flexibility.

Proposal 4: RAN2 study aims to reduce the size of UE capability signaling in 6GR.

Proposal 5: RAN2 study to consider for 6GR:

- 1) introducing UE category as in LTE

- 2) introducing capability group to cover multiple features.

Observation 5: While eMBB remains a core requirement, the next generation will be defined by applications needing extreme performance.

Proposal 6: RAN2 study aims to support eMBB, URLLC and XR as target services of 6G.

# References

[1] RP-252912	Revised SID: Study on 6G Radio		NTT DOCOMO, CMCC, AT&T, Vodafone