3GPP TSG-RAN2 Meeting #131bis			R2-2506899

Prague, Czech Republic, Oct. 13th-17th

Agenda item:	10.4

Title:	Consideration on the mobility in 6GR

Source:	CMCC

Document for:	Discussion

# 1	Introduction

RAN Plenary#108 agreed the RAN Working Group SID “Study on 6G Radio” in [1], including an objective related to mobility aspects:

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

This paper is to provide our consideration on mobility in 6G.

# 2	Discussion

2.1	5G Mobility solutions in RRC_CONNECTED and deployment/trail

In 5G, multiple mobility mechanisms for RRC_CONNECTED were specified from Release 15 to Release 20, as shown in Table 1.

Table 1: Mobility mechanisms specified in 5G

| Release | Mobility mechanism |
| --- | --- |
| Rel-15 | - Handover - Dual connectivity (DC) |
| Rel-16 | - Conditional Handover (CHO) - Intra-SN Conditional PSCell change (CPC)without MN involvement (SN initiated) - Dual Active Protocol Stacks (DAPS) |
| Rel-17 | - CHO with one target SCG - Conditional PSCell Addition (CPA) - CPC with MN involvement (MN/SN initiated) - SCG/SCell Activation/Deactivation |
| Rel-18 | - Intra-CU L1/L2 Triggered Mobility (LTM) - Subsequent CPAC - CHO with candidate SCG(s) |
| Rel-19 | - Inter-CU MCG/SCG LTM - L1 event triggered measurement reporting - Conditional Intra-CU LTM |
| Rel-20 (Not start) | - LTM SCell Activation - Dynamic L1 measurement and reporting configuration chang |

Based on Tabel 1, it can be seen that some of the mobility solutions are designed for dual connectivity, such as (s)-CPAC and SCG activation/deactivation. Since whether to support DC or NSA architecture in 6G is still under discussion, as an interim milestone is set for this [2], that RAN plenary is to make a decision on additional migration option(s) including DC at TSG#113 (September/2026) :

#### 4.1.1 Interim Milestone Interim results shall be delivered as per the milestones below, in coordination with the Rel-20 RAN Plenary 6G Study (FS_6G_RAN_Scen_Req). TSG#112 (June/2026): RAN1 to provide interim assessment on the following areas: - Waveform, modulation, channel coding: scope of enhancements beyond NR baseline ((2) a, c) - Channel bandwidth (min and max), frame structure, numerology ((2) b, d) - Basic sync signal structure and associated periodicity(ies) ((2) h) For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026. RAN3 to provide interim study results to allow TSGs to make a decision on: - RAN-CN interface: P2P vs SBI - RAN internal interfaces: CU-DU split, CP-UP split. NOTE: It is planned to decide on Release-21 timeline in June/2026. TSG#113 (September/2026): RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

Therefore, RAN2 should first focus on mobility solutions for standalone architecture.

Observation 1: An interim milestone is set as TSG#113 (September/2026) for RAN plenary to  make a decision on additional migration option(s), including 6G-6G DC, therefore, RAN2 can focus on mobility solutions for standalone architecture before that time.

CMCC has deployed 2.59 million base stations on FR1, where L3 measurement based immediate HO is used, and  based on the real network data, the handover success probability is almost 99%, which can ensure UE’s experience very well. Therefore, we think, in 6G, it can be  used as baseline of 6G mobility.

Observation 2: L3 measurement based immediate HO can achieve almost 99% HO success probability on FR1, and it makes sense to take it as baseline of 6G mobility.

For CHO, CMCC has performed field trial for high speed train scenario. The test area was selected as the Suzhou section of the Beijing-Shanghai High-Speed Railway, totaling 58 kilometers. It met the continuous coverage requirements for the 2.6 GHz frequency band, involving 31 dedicated network logical cells and 136 physical points, with an average station spacing of 462 meters. All tests were conducted at the window seats of the Fuxing train. The configuration parameters of the base stations are shown in Table 2.

Table 2 base station configuration parameters of CHO tests

| Frequency | 2515MHz-2615MHz |
| --- | --- |
| Bandwidth | 100MHz |
| TxRx | 8T8R |
| Frame structure | 5ms 8:2 (DD DD DD DS UU) |

Figure 1: Site distribution of field trial

Based on the trial results, the UE configured with CHO may perform handover earlier than legacy HO triggered by network on the place with better channel quality, and this increases the handover reliability, even in the high-speed scenario, it can avoid the deterioration of UE experience. Hence, CHO could also be supported in 6G.

Observation 3: CHO can avoid the deterioration of UE experience and handover successful probability, even in high speed scenario.

As to (C)LTM, it is suitable to frequent mobility in high frequency scenarios, like FR2. Though it is not commercialized or tested since it is introduced in the late releases of 5G, considering in the future 6G, higher frequency spectrum such as U6G will be used, LTM could also be supported from 6G day1 for high frequency scenario.

What’s more, in 5G, some enhancements have been specified to reduce the interruption time of handover, i.e., RACH-less HO and RACH-less LTM, these mechanisms can be inherited for 6G.

Proposal 1: For 6G mobility, it is proposed to take L3 measurement based immediate HO as baseline, besides, RACH-less HO,CHO and (C)LTM are supported from 6G day 1.

What’s more, after reviewing of the evolution history of 5G mobility, we think the following aspects can be further enhanced in 6G.

Measurement:

The 5G NR measurement framework is highly capable, its evolutionary path exhibits a distinct "patchwork" characteristic, leading to issues such as fragmented configurations, disjointed mechanisms, and signaling redundancy. To construct a streamlined, efficient, and future-proof 6G system, an unified redesign of the measurement framework is imperative.

The fragmentation of the current 5G NR measurement framework limits its scalability and efficiency for future 6G scenarios:

- Measurement Resource/Reporting Configuration:

  - Separation between RRM and CSI: RRM measurements for mobility decisions and CSI measurements for link adaptation are decoupled at the RRC signaling level, increasing signaling overhead and UE measurement burden.

  - Inconsistent Event Definitions: Traditional L3 handover relies on cell-level measurement events (e.g., A3/A5), while R19 introduced finer-grained beam-level events (e.g., LTM2/LTM3) for LTM, alongside specialized configurations for scenarios such as NTN and UAV. The coexistence of these event definitions at varying granularities across disparate procedures increases network-side decision complexity and UE implementation challenges.

  - Limited extensibility: To support new features such as LTM, the protocol introduced independent L1 measurement configurations, resulting in fragmented configuration management. For emerging scenarios like NTN and UAV, their unique measurement requirements are also challenging to integrate elegantly into the existing framework.

- Fragmented Measurement Reporting:

  - Multiple Reporting Ways: Conventional HO/CHO measurement reports are transmitted via RRC messages (SRB bearers), which offer high reliability but introduce significant latency. To minimize delay, LTM introduces reporting mechanisms based on MAC CE and UCI (PUCCH/PUSCH), while CSI reporting adheres to an independent and highly complex set of UCI carrying rules.

Mobility configuration:

Along with the mobility discussion, the signaling structure of mobility configuration evolves from supporting only one single target configuration to multiple candidate configurations, and further to support reference configuration with delta configuration(s). The evolution enables the signaling structure to support multiple candidate cells’ configurations and subsequent mobility with low signaling overhead. And there’s similarity among different mobility mechanism’s configuration, for example, configuration provided by target(candidate) cell(s) and execution conditions for UE triggered mobility, and for a specific candidate cell, the related configuration/parameters for different mobility mechanism may be identical, while current separate configuration results in redundancy in RRC reconfiguration. Therefore, a unified mobility configuration framework can be considered in 6G.

LTM signaling structure can be seen as a typical example, in which both candidate configuration and reference configuration are used. Also, it can be associated with execution conditions to achieve UE triggered conditional cell switch. What’s more, it can support functions like early synchronization with candidate cell(s), simplified layer 2 handling and subsequent mobility, which are not supported by HO or CHO. Therefore, it’s better to harmonized LTM signaling structure to support (C)HO configuration, especially if there’s a unified measurement framework in 6G.

Besides, the procedures to accelerate cell switch among multiple cells and simplify UE behavior, including early synchronization and simplified layer2 handling in LTM can also be supported in 6G for the unified mobility mechanism.

Mobility interruption time:

Based on the real network data, there’s service like XR, has low interruption time requirements. Considering 6G is expected to support various services, and provides a more reliable connection, RAN2 should study how to accelerate cell switch procedure and reduce the related interruption time.

Figure 2 is the mobility interruption time for L1/L2 Triggered Mobility (LTM), it is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell, as defined in TS 38.300 [3].

Figure 2: Components of Mobility Latency for LTM

Each component of mobility latency is described in the table below. The value of TRRC is specified in clause 12 of TS 38.331 [4].

Table 3: Components of Mobility Latency

| Component | Meaning |
| --- | --- |
| TRRC | Processing time for RRCReconfiguration carrying candidate configurations |
| TMAC | Time for processing the LTM cell switch command MAC CE |
| TRRC-target | Time for processing and applying the target configuration |
| TDL-sync | Time for DL synchronization |
| TRA | Time until RA transmission |
| TRAR | Time until RAR reception |
| Tfirst-data | Time for UE performs the first DL/UL reception/ transmission on the indicated beam of the target cell, after RAR |

For RACH-less LTM, the interruption time mainly consists of TRRC-target and Tfirst-data, but not 0 interruption time. Based on RAN4 requirements, the minimum interruption time of LTM can almost be 13ms. However, L3 measurement based immediately HO latency is also tens of milliseconds e.g., 20ms, it seems there’s no significant latency gain by applying LTM from UE perspective, but the network cost is quite different, since LTM gain is highly dependent on resource reservation in the network side.

Observation 4: The minimum interruption time of LTM can almost be 13ms, while L3 measurement based immediately HO latency is also tens of milliseconds e.g., 20ms, the benefits of LTM seems not so attractive.

Based on the discussion above, we propose the following design principle for 6G mobility:

- Support both beam level and cell level measurement results based mobility, and study a unified 6G measurement framework to overcome the fragmentation of the 5G measurement system, providing a concise, efficient, and scalable measurement foundation for diverse 6G scenarios;

- Pursue a unified mobility configuration and procedure to avoid redundancy in RRC configuration, and LTM can be taken as baseline;

- Further study to reduce mobility interruption time, e.g., to achieve ms-level mobility interruption time.

Proposal 2: The following aspects should be considered in 6G mobility design:

- Support both beam level and cell level measurement results based mobility, and study a unified 6G measurement framework;

- Pursue a unified mobility configuration and procedure to avoid redundancy in RRC configuration, and LTM can be taken as baseline;

- Further study to reduce mobility interruption time, e.g., to achieve ms-level mobility interruption time.

2.2	Mobility in RRC_IDLE/INACTIVE

Generally, we think 5G mobility procedure in RRC_ IDLE/INACTIVE i.e., cell selection and cell reselection can be inherited for 6G. For cell reselection, absolute priorities of different 6GR frequencies or inter-RAT frequencies may be provided to the UE to derive reselection priorities. In 5G, the cell is associated with only one carrier by default, while for 6G, there could be a single cell with multi carriers as a hyper cell, which can realize efficient and flexible IDLE/INACTIVE mode multiple carriers’ operation, and at the same time, to reduce cell management overhead of existing CA framework. And how to support the cell reselection in the hyper cell scenario should be considered.

Proposal 3: For mobility in RRC_IDLE/INACTIVE, i.e., cell reselection, both single cell associate with one carrier and single cell with multiple carriers (hyper cell) are considered.

2.3	Mobility enhancement for NTN, ATG, UAV and HSDN

2.3.1	NTN

To address the main challenges that longer propagation delay, high speed movements of satellites and  unobvious near-far effect as well as beam hopping scenario in NTN system, we supported the following enhancements for mobility for NR NTN:

- Idle/Inactive mode

- Time-based and location-based measurements on neighbour cells for cell re-selection

- Multiple SMTCs configuration, and UE can adjust SMTCs based on its location and assistance information in SIB19.

- Broadcast TN coverage information to help UE skip TN measurement for NTN->TN cell reselection

- Location-based SMTC selection (network configure multiple SMTCs, each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE)

- Idle mobility from E-UTRAN TN to NR NTN (including reselection and redirection)

- Connected mode

- RRM measurement-based event A4, time-based and location-based for CHO trigger condition

- Support PCI unchanged solution for quasi-Earth fixed scenario and RACH-less HO for HO signaling reduction

- Multiple SMTCs configuration, and network control the adjustment of SMTCs based on UE assistance information (i.e. service link propagation delay difference(s) between serving the cell and neighbour cell(s).) reporting

- UE assisted SMTC configuration (network configure multiple SMTCs based on the N closest reference locations information reported from UE)

- E-UTRA TN to NR NTN handover

However, for NTN-TN handover, there is no dedicated enhancements for seamless service continuity to reduce the interruption time and signalling overhead in 5G. In 6G, to achieve ubiquitous connectivity, NTN-TN mobility including both multi-orbit mobility and NTN<->TN seamless handover could be considered.

For multi-orbit mobility, multi-dimensional matching of orbital characteristics, positioning requirements, transmission power, and other factors. For example, if the service is time-delay insensitive, GEO could be used to perform the data transmission, and once the latency requirements increase, the UE switches to the lower orbit (e.g. LEO).

Figure 3: Multi-orbit mobility

For NTN-TN seamless handover, PCI unchanged solution introduced in 5G could be the baseline. Considering that PCI unchanged solution is just supported in quasi-Earth fixed scenario under transparent payload architecture, it could be further discussed in regenerative payload for intra-NTN and NTN-TN scenarios in 6G.

Figure 4: PCI unchanged for intra-NTN regenerative payload and NTN-TN scenario

Proposal 4: The mobility enhancements  introduced in NR NTN mentioned above (e.g. PCI unchanged, etc.) could be inherited in 6G, and for inter RAT scenarios, it is proposed to prioritize mobility between NR TN/NTN and 6GR NTN/TN. Seamless service continuity for NTN - TN HO and multi-orbit intra-NTN could also be considered in 6G.

2.3.2	ATG

In NR, we supported the following enhancements for ATG mobility:

- Idle/Inactive mode

- gNB reference location of the serving cell and neighbour cell(s) provided in SIB22 can be used for the UE to perform the idle/inactive measurement and mobility.

- Connected mode

- Location based triggered measurement reporting

- Location based conditional handover

- neighbour Secondary Cell measurement skipping

To support ATG communication in 6GR, the mobility enhancements introduced in NR ATG could be inherited.

Proposal 5: For 6G ATG mobility, it is proposed to inherit the enhancements supported in NR.

2.3.3	UAV

In NR, we supported the following enhancements for UAV mobility:

-	Altitude reporting based on the measurement event(s) where the UE's altitude has crossed a network-configured reference altitude threshold;

-	Altitude-dependent configurations which apply only to specific altitude regions (SSB-ToMeasureAltitudeBased configuration for both connected mode and idle/inactive mode UE);

-	Interference detection based on a measurement reporting that is triggered when a configured number of cells (i.e. larger than one) fulfils the triggering criteria simultaneously;

-	Altitude-based CHO enhancement (An Aerial UE can be configured to trigger CHO execution only when both an altitude-dependent condition and an RSRP/RSRQ/SINR-based condition are met );

-	UAV prioritized frequency for Aerial UE cell reselection.

Nowadays, with the rapid development of the low-altitude economy, and UAVs can be used in a wide range of applications, with innovators constantly identifying new, beneficial applications for goods delivery, infrastructure inspection, search and rescue, agricultural monitoring, etc.. In order to enable the low-altitude economy, it is very meaningful and urgent to support UAV in 6G. And mobility is one important feature needed to be considered for 6G UAV. Therefore for 6G UAV mobility, the enhancements supported in NR could be inherited firstly.

Further in NR UAV, NTZ(No Transmit Zones) is supported while without RAN enhancements, and UE is responsible of following any applicable NTZ regulations. For 6GR, to better support UAV communication for low-altitude economy, control on UAV mobility for supervision due to NTZs could be considered. For example, network could provide NTZs related information to help cell (re)selection and handover for UAV UE, then if a cell is operating in the NTZ area, the UAV UE is not be allowed to (re)selection or handover to the cell.

Proposal 6: For 6G UAV mobility, it is proposed to inherit the above enhancements supported in NR. Further, enhancements for NTZs (e.g. network provides NTZs related information) are needed to be considered.

2.3.4	High-speed-railway dedicated network (HSDN)

As mentioned in Section 2.2, in 5G, priorities can be provided to UE’s cell reselection, and this applies to most scenarios. However, when it comes to HSDN, the normal network is not suitable for serving UEs on the high speed train, due to frequently handover. The main purpose of deploying a high-speed-railway dedicated network is to provide better radio connectivity and dedicated optimizations for UEs on the high speed train, which will camp on the dedicated network and therefore may switch to RRC-CONNECTED and initiate service in time. As shown in Figure 5, one of the optimizations for HSDN is that, the HSDN cells are deployed alongside the railway, in which around 8~12 RRUs are concatenated in line and work as only one single cell in order to reduce the frequently handover or reselection and lower the signalling overhead at the same time. In our deployment, HSDN cells and normal cells can be deployed either on the same frequency or different frequency by shifting the SSB center frequency.

HSDN cells always has higher TX power than normal cells in order to compensate the penetration loss of the train body. However, we found a problem in the real network, the railway and HSDN which are deployed in the dense population urban area, low mobility state UEs which are not on the train may reselect to the HSDN cell and access to HSDN cell. When the train is coming, there will be less resources available on the HSDN cell to accept the HO request of the UEs on the train. The issue of low mobility state UE camping on the HSDN cells will frequently cause congestion of the HSDN cells, which is not desired from the network operation point of view. In real network, we observed that such issue results to call drop, low or zero bit rate, handover rejection, RACH failure for the high mobility state UE.

Figure 5: Overlapped coverage between HSDN cells (with Multiple RRUs concatenated as one single cell) and normal cells

In LTE and NR, the same mechanism was introduced as HSDN solution, including 1) providing information for UE’s mobility state estimation; 2) indication whether a cell is a HSDN cell. Based on the information provided by the network, the UE can estimate its mobility state and consider the right cell as highest priority, for example, when the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e. higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e. lower than network configured priorities).

HSDN related solution was not introduced in the first release of LTE and NR, which impacts to the commercial deployment. Considering high speed train railway network is constructed widely in the world, especially in China, we propose to support HSDN solution from 6G day1, that the network provides information for UE’s mobility state estimation and for HSDN cell judgement.

Proposal 7: For HSDN, it is proposed to support NR solutions from 6G day1, in which the network provides information for UE’s mobility state estimation and for HSDN cell judgement to help UE camp on the right cell.

# 3	Conclusion

In this contribution, based on 5G mobility specification and deployment/test results, we provide our view on the mobility solutions supported and potential optimization in 6G and considerations on different mobility scenarios. Following are the observations and proposals:

Observation 1: An interim milestone is set as TSG#113 (September/2026) for RAN plenary to  make a decision on additional migration option(s), including 6G-6G DC, therefore, RAN2 can focus on mobility solutions for standalone architecture before that time.

Observation 2: L3 measurement based immediate HO can achieve almost 99% HO success probability on FR1, and it makes sense to take it as baseline of 6G mobility.

Observation 3: CHO can avoid the deterioration of UE experience and handover successful probability, even in high speed scenario.

Proposal 1: For 6G mobility, it is proposed to take L3 measurement based immediate HO as baseline, besides, RACH-less HO,CHO and (C)LTM are supported from 6G day 1.

Observation 4: The minimum interruption time of LTM can almost be 13ms, while L3 measurement based immediately HO latency is also tens of milliseconds e.g., 20ms, the benefits of LTM seems not so attractive.

Proposal 2: The following aspects should be considered in 6G mobility design:

- Support both beam level and cell level measurement results based mobility, and study a unified 6G measurement framework;

- Pursue a unified mobility configuration and procedure to avoid redundancy in RRC configuration, and LTM can be taken as baseline;

- Further study to reduce mobility interruption time, e.g., to achieve ms-level mobility interruption time.

Proposal 3: For mobility in RRC_IDLE/INACTIVE, i.e., cell reselection, both single cell associate with one carrier and single cell with multiple carriers (hyper cell) are considered.

Proposal 4: The mobility enhancements  introduced in NR NTN mentioned above (e.g. PCI unchanged, etc.) could be inherited in 6G, and for inter RAT scenarios, it is proposed to prioritize mobility between NR TN/NTN and 6GR NTN/TN. Seamless service continuity for NTN - TN HO and multi-orbit intra-NTN could also be considered in 6G.

Proposal 5: For 6G ATG mobility, it is proposed to inherit the enhancements supported in NR.

Proposal 6: For 6G UAV mobility, it is proposed to inherit the above enhancements supported in NR. Further, enhancements for NTZs (e.g. network provides NTZs related information) are needed to be considered.

Proposal 7: For HSDN, it is proposed to support NR solutions from 6G day1, in which the network provides information for UE’s mobility state estimation and for HSDN cell judgement to help UE camp on the right cell.

# 4	References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO;

[2] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone;

[3] TS 38.300, NR and NG-RAN Overall Description; Stage 2 (Release 18);

[4] TS 38.331, Radio Resource Control (RRC) protocol specification; Stage 2 (Release 18);

[5] TR 38.914, Study on 6G Scenarios and Requirements; (Release 20).