3GPP TSG-RAN WG2 Meeting #131bis	R2-2506910

Prague, Czech Republic, Oct 13th –17th, 2025

Agenda Item:	10.2

Source:	CMCC

Title:	General considerations on 6GR

WID/SID:			FS_6G_Radio

Document for:	Discussion

# 1 Introduction

At RAN#108, the SID on 6G Radio Access Technology in 6GR Rel-20 was approved [1] to develop a 6GR access technology that meets a broad range of use cases, including AI/ML-driven services, sensing services, immersive communication, and other requirements defined in TR 38.914 [2], during the RAN requirements study. China Mobile has built the world's largest and most extensive 5G network, deploying 2.59 million 5G base stations,  serving 599 million users and providing the coverage of  various regions such as dense urban areas, urban areas, sub-urban, rural areas, HST, etc. This document addresses the radio architecture aspect, discussing fundamental principles and guidelines to resolve key challenges encountered during 5G commercialization and to satisfy new requirements from a wide range of services, infrastructure, and scenarios in 6G. Considerations on UE capability framework for 6GR are discussed as well .

# 2 Discussion

2.1 Requirements for 6G and Key Issues from 5G

2.1.1 Key requirements for 6G

2.1.1.1 New Services

As global deployment of 5G networks enters a mature phase, the limitations of existing service frameworks have gradually emerged in addressing future societal and technological demands. Current 5G services, while enabling high-speed connectivity and low-latency interactions, struggle to fully support emerging scenarios such as immersive extended reality (XR) experiences, intelligent sensing, and satellite-terrestrial integration. Therefore, 6G is expected to become a foundational infrastructure for the verticals, smart societies, and human-centric intelligent ecosystems. In our view, Integrated Sensing and Communication (ISAC), AI/ML-based services and immersive communication will be the key services in 6GR to unlock the new capabilities to support the next era of digital transformation.

Motivation for 6GR ISAC

ISAC has been identified as one of the six usage scenarios for IMT-2030, providing sensing services for low-altitude economy, traffic surveillance, intrusion detection and micro-deformation detection while also assisting communication services.  TR 22.837[3] identify a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137 [4]. The different use cases of ISAC require the different performance requirements, for example, the performance requirements of sensing results for pedestrian/animal intrusion detection are given in Table 1.

Table 1: Performance requirements of sensing results for pedestrian/animal intrusion detection

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Pedestrian/animal intrusion detection on a highway | Outdoor (Highway) | 95 | ≤1 | N/A | N/A | N/A | N/A | N/A | ≤5000 | ≤ 0.1 | ≤5 | ≤5 |
| NOTE: The terms in Table 5.2.6-1 are found in Section 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |

Motivation for 6GR AI/ML

In 5G-A, 3GPP RAN has investigated various AI for RAN use cases that are aimed to optimize network performance through AI/ML function, for example, AI/ML based CSI feedback enhancement, beam management and positioning enhancement and AI/ML for mobility. For 6G, the potential new use cases of AI for 6G RAN could be identified and investigated, e.g. Joint source/channel coding (JSCC), DMRS design or AI receiver, AI/ML based mobility.

Furthermore, RAN for AI is expected to be investigate and support in 6GR, e.g. RAN provides computing resources, RAN related information or AI data for AI services/applications. AI agent, XR rendering and token communication could be considered as the potential RAN for AI use cases, and the requirements of these use cases are summarized in Table 2.

Table 2: Requirements for RAN for AI use cases

| Use Cases | Uplink KPI |  | Downlink KPI |  |  |
| --- | --- | --- | --- | --- | --- |
|  | E2E Latency | Date Rate | E2E Latency | Date Rate | Load |
| XR rendering | 100 ms | 1.5 Mbit/s | 100 ms | 150 Mbit/s | 1.5 MByte/‌frame |
| Embodied AI/ AI agent | 10 ms | 4.7 Mbit/s | 10 ms | 320 Mbit/s | 40 kByte |

Motivation for 6GR immersive communication

Immersive Communication has been described by ITU-R as a usage scenario that extends the enhanced Mobile Broadband (eMBB) of IMT-2020 and covers use cases that provide a rich and interactive video (immersive) experience to users, including interactions with machine interfaces [5]. Immersive communication use cases will continue to drive requirements on mobile networks. Mixed Reality Gaming is taken as an example, and its performance requirements are shown in Table 3.

Table 3: Performance of Mixed Reality Gaming

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link | Bit rate up link | Application latency | Communication service availability | Overall user density | Activity factor | UE speed |
| Mixed Reality gaming | [50] Mbit/s (note 1) | [50-100] Mbit/s (note 3) | [20] ms (note 2) | Dense Urban [99]% Urban [99]% Rural [98]% | Dense Urban [25,000] /km2 Urban [1000-10,000]/km2 Rural [100]/km2 | 2% (note 4) | 5 km/h |
| NOTE 1: Remote or split rendering for gaming applications, such as rendering of 360-degree video tiles in the cloud and transmitting the images to device, NOTE 2: The local encoding and processing on the glasses is assumed NOTE 3: envisioned 1080P resolution 2 grayscale cameras 1920 x 1080 x 8 bit, a depth sensor 1920 x 1080 x 8 bit with 60 FPS means 1920 * 1080 * 24 * 60 bps = 3Gbps uncompressed. Depending on compression ratios that would lead to 50 Mbps to 100 Mbps NOTE 4: Activity factor 2% is based on 20% uptake of XR service and 10% activity factor for the XR application. |  |  |  |  |  |  |  |

Proposal 1:  RAN2 should study the support of new services, e.g., ISAC, AI/ML-based services and immersive communication, taking NW & UE energy consumption and User experience (QoS/QoE) improvement into consideration.

2.1.1.2 Deployment scenarios and architecture

In RAN#109, the typical deployment scenarios in 6G and their respective attributes have been captured in TR 38.914, including Indoor hotspot, Dense urban, Rural, Urban macro, Sub-Urban macro, High speed, Extreme long distance coverage in low density area, Urban coverage for massive connection, Air-to-Ground Scenario, Non-Terrestrial Network, [Urban grid], [Highway Scenario]. It is proposed that RAN2 should study the support of the 6GR deployment scenarios defined in TR 38.914.

Observation 1: Typical deployment scenarios in 6G and their respective key attributes have been captured in TR 38.914 for the purpose of evaluating RAN performance requirements or features.

For 6G, beyond the typical deployment scenarios, we think that some special scenarios also require further discussion in consideration of service deployment requirements of operators, such as UAV, ATG, HST and NTN.

- The UAV deployment scenario is characterized by high mobility, low latency, large bandwidth for high-definition video transmission, anti-interference capability for emergency communication, and multi-node network interaction. These features impose specific demands on dynamic resource allocation to adapt to real-time UAV position changes, reliable link maintenance to address signal obstruction in complex urban or remote areas, and energy-efficient communication to extend UAV battery life. Therefore, we propose the scenarios and network architectures for UAV should considered integrate both current service needs (e.g., integrated sensing and communication) and future expansion potentials (e.g., deep integration with vertical industries like smart logistics).

- Air-to-ground (ATG) network refers to in-flight connectivity technique, using ground-based cell towers that send signals up to an aircraft’s antenna(s) of onboard ATG terminal. In this network, a direct radio link will be established between BS on the ground and UE mounted in the aircraft. ATG network deployment should to consider the characteristics, e.g,. extremely large inter-site distance (ISD) and large coverage range, flight speed and height range and high throughput demand. In 5G-A, the RRM enhancements have been introduced, for example, measurement enhancement at IDLE/Inactive mode, CHO, neighbour cell measurement skipping at SCC and so on.

- In HST scenario, the user experience is severely compromised due to factors such as ultra-high mobility (speeds up to 400 km/h or higher, leading to frequent cell handovers), Doppler frequency shift and concentrated user demands. To address these issues, dedicated networks are deployed along high-speed rail lines to ensure the performance of users with high mobility. HSDN (High Speed Dedicated Network) is introduced into 5G NR to ensure HSDN capable UE in High-mobility state have priority access to dedicated networks. For 6GR, the corresponding optimization solutions (e.g., HSDN, LTM) for HST scenario still require study and support to guarantee stable user experience.

- The key issues of NTN scenario include large transmission latency, insignificant near-far effect, severe signalling storms due to a sudden surge in the number of UE requiring handover within a short period. Therefore, multiple solutions have been introduced in 5G NR to improve network performance, for example, HARQ enhancement for large transmission latency, time-based and location-based CHO and Cell selection/reselection for near-far effect, RACH-less HO for handover signalling overhead reduction and etc. It needs to be further investigated whether the existing enhancements for NTN could be reused in 6GR.

In the agreements on the requirements of 6G RAN achieved at RAN#109, the following aspects are key focus areas for RAN2.

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall allow non-public networks.

Observation 2: 6GR requirements, including control plane/user plane separation, mobility, slicing, spectrum aggregation, multiple transmission points, service awareness, TN/NTN harmonization and non-public networks are captured in TR 38.914.

Proposal 2: RAN2 should study support of the typical and special deployment scenarios (including UAV, ATG, HST and NTN) and control/user plane separation, mobility, slicing, Spectrum Aggregation, multiple transmission points, service awareness, TN/NTN harmonization and NPN.

2.1.2 Critical Issues from 5G

Issue 1: Protocol Complexity and Innovation Bottleneck

  - The NR RRC configuration and UE capability framework are overly complex, difficult to process on the gNB side, and incur excessive signaling overhead. In NR design, gNB always provides full conﬁguration to UE, especially at the 1st step configuration, which will have big message size with many features enabled.

  - Difficult to maintain, with strong inter-IE coupling: In NR design, RRC specification is very complex and difficult to maintain. Introducing new features necessitates adding new IEs, leading to strong inter-IE coupling. Any modification requires synchronous updates across multiple layers, significantly increasing standardization complexity and time-to-market, thereby hindering rapid innovation.

  - "one-size-fits-all" protocol of 5G: The 5G system has made significant strides in service diversity through technologies like network slicing. However, its UP and CP protocol fundamentally retain a relatively and rigid architecture. And too many L2 header options are in different layers. As we move towards the 6G era of interconnected intelligence, the service scenarios will become highly diversified, encompassing immersive communication, AI services, HRLLC, mMTC, ISAC, among others. These services impose vastly different, and often conflicting, extreme requirements on network throughput, latency, reliability, connection density, and energy efficiency.

  - Functional Redundancies: In NR design, there are always several options for same purpose, for example, retransmission is equipped in PDCP, RLC and MAC layers , which is redundant. For high reliability, various enhancements have been specified, such as, low MCS table, multiple Configured Grants for a given UE, association between CGs and logical channel, PDCP duplication etc.. This introduces implementation complexity when similar functionalities are operated in different protocol layers.

Issue 2: Deployment Cost and Resource Inefficiency: Current 5G base stations are typically deployed with full protocol stack functionality. However, in many specific scenarios (e.g., wide-area IoT coverage, remote area connectivity), base stations do not require all advanced features designed for eMBB. This "over-provisioning" results in wasteful use of expensive hardware resources and energy, driving up operator CAPEX and OPEX.

Issue 3: Performance Limitation: The fixed protocol stack processing pipeline includes functions potentially needed for all scenarios, preventing deep optimization for specific services. For instance, data flow for URLLC traffic cannot bypass certain non-essential functions, limiting the further reduction of end-to-end latency.

Issue 4: Limitation on AI data collection and model transfer/delivery: The 5G-A network architecture remains unchanged to support the enhancements on AI/ML, both legacy CP and UP solutions have restrictions on AI data collection and model transfer/delivery, which is illustrated in [6]:

  - The existing SRB cannot transmit the large volumes of data for model training or AI/ML model (the maximum size of UL/DL RRC signaling is 144kbytes/45kbytes). If a large amount of AI data needs to be transmitted using SRB, the current solution is to use multiple RRC messages, which will lead to signaling overload.

  - The existing DRB cannot be setup between gNB and UE directly, which is always associated with PDU session terminated in UPF. Therefore, the large volumes of data collected by the UE cannot be transmitted to the gNB, or the model trained at NW for two-sided model case cannot be transferred to the UE directly via DRB.

  - On the other hand, the data for model training is collected per use case (e.g. beam management, mobility, CSI prediction) and per model type (e.g. UE-sided model, NW-sided model), which can lead to duplicated data collection and reporting, especially for basic measurement results. For example, L1 beam measurement results can be used for both AI/ML based beam management and AI/ML for mobility use cases. For beam management use case, L1-RSRPs and/or beam-IDs need to be collected by UE for both NW-sided model and UE-sided model training.

Issue 5: Fragmented access control mechanism, including cell barring, UAC and RRC connection rejection, which is illustrated in [7]:

  - For cell barring, except for the common Cell-barred IE in MIB, there are cell bar indications in SIB1 for specific device types, i.e., Redcap, NES. Such design introduce compatibility issue, and extra complexity for some UEs, since they need to additionally receive SIB1 for cell barring decision.

  - For 5G UAC, multiple access categories and access identities are defined by CT1 in NAS layer and it should be noticed that the clarification of access categories and access identities lacks granularity, which cannot meet 6GR’s requirements to support diverse device types, which differs from each other in capability and device form, and these difference may have no or little impact to NAS layer, while huge impact to RAN scheduling.

Issue 6: Dispersed signaling configurations for mobility and measurement results in the redundancy and mess in configuration signaling, which is illustrated in [8]:

  - In 5G, several mobility mechanisms such as L3 immediate HO, CHO, CPAC, and LTM are introduced, while dispersed signaling configurations are used for each other. However, for a specific candidate(target) cell, most parameters and configurations used for a handover/cell switch are the same for different mobility mechanisms, which results in the redundancy in configuration. Therefore, it’s better to design a unified signalling structure to support multiple mobility mechanism. When looking back to the mobility evolution, it evolutes from supporting only single target cell, to support multi candidate cells, then further to support reference configuration, while LTM can be seen as a typical example, since it supports both reference configuration and multiple candidate cells. Hence, LTM signalling structure can be the start point for unified signalling structure study.

Issue 7: Fragmented and complex design of multi-carrier operation results in inefficient utilization of sparse spectrum, inflexible UL and DL carrier association, which is illustrated in [7]:

  - In 5G, multiple multi-carrier operations, which includes carrier aggregation (CA) and supplementary uplink (SUL), are designed. For SUL, it is mainly used to extend the uplink coverage by configuring one low-frequency carrier as SUL carrier, and only one SUL carrier can be configured in one NR cell. In terms of CA, two or more carriers can be aggregated and used by one UE to increase UE’s throughput. However, CA is only applicable to UE in RRC_CONNECTED state, which limits its benefits. Note that for certain scenarios (e.g. uplink coverage extension, uplink transmission rate improvement, etc.), CA and SUL can bring similar benefits, but resulting in additional work for specification since they are different frameworks. Besides, there are some obvious limitations for both SUL and CA, for example, inefficient usage of fragmented spectrum, inflexible UL and DL carrier association, etc.

Proposal 3: The following key issues from 5G commercialization must to be addressed:

- Protocol Complexity and Innovation Bottleneck

- The NR RRC configuration and UE capability framework has been criticized as overly complex, difficult to process and maintain on the gNB-UE both side, consuming excessive signaling overhead;

- Fixed and coupled protocol stack causes high complexity, high cost and performance limitation;

- Limitation on AI/sensing data collection and model transfer

- Fragmented access control mechanism, including cell barring, UAC and RRC connection rejection;

- Dispersed signaling configurations for mobility/measurement;

- Fragmented design of multi-carrier operation results in inefficient utilization of sparse spectrum, inflexible UL and DL carrier association.

2.2 Basic Principles for 6GR design

2.2.1 Simplification

Regarding issue1-3, as increased new features and functions are introduced, the fixed and coupled protocol stack causes high complexity, high cost and performance limitation encountered during the commercialization of 5G. The guidance of pragmatic and simplified 6G standard design is required. Hence, 5G design as starting point for 6G with simplification and to minimize the adoption of multiple options for the same functionality, focusing on practical user experience, including the following aspects:

- L2 sublayer consolidation for reorganization and reduction of redundant functionalities across protocol layer, as shown in figure 1 and will be illustrated in [9]

- RRC configuration and UE capability signaling reduction, will be illustrated in the following sections;

- Avoid excessive nesting of configurations (e.g. avoid BWPs as basic granularity in Physical Radio resource configuration)

Figure 1: Layer 2 Structure

Proposal 4-1: 5G design as starting point for 6G with pragmatic simplification:

- Consolidation L2 sublayers to reduce redundancies across protocol layer

- Simplify RRC configuration and UE capability signaling

- Avoid excessive nesting of configurations (e.g. avoid BWPs as basic granularity in Physical Radio resource configuration)

2.2.2 Extensible and flexible

To address issue 4, it is crucial to consider both the efficiency and complexity of data collection in 6G. Therefore, a unified RAN data collection framework is essential, which should support diverse data collection from 6G new services, such as AI and sensing, which is illustrated in [6].

Figure 2: unified RAN data collection frameworkTo address issue 5, a unified forward-compatible design principle for 6G access control is proposed, including cell barring mechanism, the definition of access categories and access identities in 6GR.

To address issue 6, a unified signaling structure to support multiple mobility mechanism and measurement is demanded. When looking back to the mobility evolution, it evolves from supporting only single target cell, to support multi candidate cells, then further to support reference configuration, while LTM can be seen as a typical example, since it supports both reference configuration and multiple candidate cells. Hence, it is proposed to simplify and unify signaling design by avoiding current approach where L3, CHO and LTM are independent features, where LTM signaling structure can be the start point for unified signaling structure study. Meanwhile,unified measurement framework having the capabilities of the L3- and L1-measurement frameworks from 5G is required as well.

To address issue 7, we propose to design a unified, flexible and concise multi-carrier framework in 6GR to enable efficient multi-carrier operations, which is illustrated in [7]:.

Figure 3: a unified multi-carrier framework

Proposal 4-2: 5G design as starting point for 6G with extensible and flexible:

- Unified forward-compatible design principle for 6G access control, including cell barring mechanism, the definition of access categories and access identities;

- Unified RAN data collection framework for diversified data collected, e.g. AI and sensing;

- Unified mobility mechanism signalling structure with LTM signaling structure as the start point;

- Harmonized measurement framework having the capabilities of the L3- and L1-measurement ;

- Unified multi-carrier framework in 6GR to enable efficient multi-carrier operations.

2.2.3 Excellent Performance

To address issue 2 and meet a broad range of use cases including AI/ML-driven services, sensing service, Immersive communication, and additional requirements in 6G defined in TR 38.914 [2]. 6GR design should take NW & UE energy consumption and User experience improvement into consideration.

2.2.4 Modular

The 5G system has made significant strides in service diversity through technologies like network slicing. However, its UP and CP protocol fundamentally retain a relatively and rigid architecture. As we move towards the 6G era of interconnected intelligence, the service scenarios will become highly diversified, encompassing immersive communication, AI services, HRLLC, mMTC, ISAC, among others. These services impose vastly different, and often conflicting, extreme requirements on network throughput, latency, reliability, connection density, and energy efficiency.

In TR 38.914 [2], from the perspective of scenarios and requirements, it also points out that in addition to providing diverse new 6G services, other factors need to be considered, such as reducing CAPEX/OPEX reduction.

6G aims to support societal advancements and to bring value to society in the 2030s and beyond in secure, resilient, environmentally and economically sustainable ways. In addition to providing diverse and novel 6G services, other considerations are needed, e.g. CAPEX/OPEX reduction, improvement of overall 3GPP system performance, and migration from and interworking with 5G aspects. Therefore, this study aims to identify typical usage scenarios for 6G radio technology and the required capabilities for each scenario.The study will also translate these required capabilities into technical metrics that can be evaluated from a RAN perspective.

Furthermore, 6G necessitates a fundamental paradigm shift from a fixed protocol stack to a lean and modular protocol stack design based on the principles of "service-driven, module-decoupling, profile-based configuration." to meet the requirement of pragmatic and simplified 6G. The core design guidelines are as follows Figure 1:

Figure 4: Modular Protocol Stack

2.2.4.1 Service-Driven Design and Functional Decoupling

- Guideline: Move away from the traditional strict layered model and decouple the protocol stack into fine-grained, self-contained functional modules. These modules cover both Control Plane (e.g., RRC connection management, mobility management, RRM) and User Plane (e.g., security encryption, header compression, segmentation, ARQ/HARQ).

The protocol stack is organized not in "layers" but in "function chains." A Mapping Function entity takes the service use case as input and maps it to specific Control Plane Function Sets and User Plane Function Sets that need to be activated. This supports customizing the protocol stack for common services and lays the foundation for optimizing performance.

2.2.4.2 Modular-Based Configuration

- Guideline: Pre-define optimized configuration profiles for typical scenarios. Each profile represents an organic combination of functional modules and their parameter sets, assigned a unique index. The network can rapidly instruct the terminal and network side to activate the corresponding protocol stack configuration by signaling this index.

Define module such as "Massive IoT Set," "URLLC Set," and "eMBB Set." The Function module set selected by the base station side is signaled to the UE side, ensuring consistent protocol stack behavior on both ends. Significantly simplifies signaling, reduces configuration overhead, ensures consistency, and facilitates the deployment of low-cost base stations (which only need to load lightweight profiles required for their target scenarios). This will be related to the definition of device type mentioned in the following section as well.

This means 6G device categorization should be adaptable for revolving demands of future devices. Each device type should define the minimum set of supported UE capabilities and the allow basic RRC configuration; Knowing the default or minimum set of capabilities associated with each device type can facilitate smooth enable the configuration profile manner with additional delta configuration. And optional UE capability need on top of device type.

Observation 3: 6G device categorization can index the minimum set of functionalities supported by UE capabilities and the profile of basic RRC configuration; Other additional functionalities can be configured with additional delta configuration.

Proposal 5: RAN2 is proposed to discuss a lean and modular protocol stack as a 6G requirement.

2.2.4.3 Comparison with 5G protocol stack features

The proposed modular protocol stack offers features and advantages compared to the conventional 5G protocol stack:

Table 4: Features and advantages of proposed modular protocol stack

| Features | 5G protocol stack | 6G lean and modular protocol stack | Benefits |
| --- | --- | --- | --- |
| Architecture | Fixed, Interlaced | Flexible, Modular, Assembliable | Flexibility, Scalability |
| Configuration | Complex, High Signaling Overhead, Static | Profile-based, Dynamic | Efficiency, Fast Configuration |
| CAPEX/OPEX | "Full-stack" Deployment, Resource Waste | On-demand Activation/Licensing, Function Tailoring | Significant CAPEX/OPEX Reduction |
| Performance | General-purpose Compromise, Hard to Optimize | Tailored to Service, Optimized Trafic Path | Ultimate Performance (Latency, Reliability, etc.)​ |
| Operation | Hard Spec Updating Towards New Features | Modular Insertion of New Features | Towards Flexible Networks Among Protocol Stack |

2.3 UE capability framework

2.3.1 Challenges for UE capability Signaling

Issues of UE capability signaling structure

Compared to LTE, the main enhancements introduced for NR UE capability size reduction were gNB requested band combination signaling, baseband capability extraction and UL/DL bands decoupling.

However, it is proved that a large number of frequency band combinations and complex signaling structures require higher development and maintenance costs for terminals and network. And there is overly redundant UE capability information in multiple band combinations and feature sets. Further there are some new features introduced in the subsequent discussions of different NR releases leading to exploding UE capabilities. And most new features of different NR release are defined as optional, resulting that increasingly more overhead on reporting more complexity to both UE and network are introduced, and some beneficial features are not supported by most UEs for some cost and information leakage considerations.

|  | LTE | NR | 6GR |
| --- | --- | --- | --- |
| Size | TS 36.306: ~160 pages Per UE/Band/BC | TS 38.306: ~400 pages Per UE/Band/BC/FS/FSPC FR1/FR2/FR2-2 | Simplification! |

Observation 4: The existing UE capability framework is overly complex with exploding size, complex band combinations and feature sets increase development and maintenance costs, and explosive growth in UE capability size due to optional features.

Definition of Device type categories with future proof

To enable the network to efficiently support diverse 6G use cases, meet performance requirements, and enhance user experience, it is beneficial for RAN to identify and study representative device types. Device categorization can be based on characteristics such as power class, supported bandwidth, and number of antennas. For each device type, a baseline UE capability profile can be defined to represent mandatory capabilities, while UEs would only report deviations from the baseline.

At least three representative UE types can be identified for 6G:

- Type I: Low-power devices, e.g., LPWA, RedCap, wearables, and 2Rx UEs.

- Type II: Devices with moderate radio capability, similar to traditional eMBB UEs.

- Type III: Devices with high radio capability, e.g., CPE, FWA, and high-end XR devices.

In addition to these categories, 6G is expected to support emerging device types driven by new services and features. The introduction of AI-native functionalities, immersive or multi-modal services, NTN, aerial, and vehicular deployments indicates that categorization based solely on radio capability may be insufficient. These service-specific features may define new or sub-categories of device types that warrant further study.

This means 6G device categorization should be adaptable for revolving demands of future devices. Each device type should define the minimum set of supported UE capabilities and the allow basic RRC configuration; Knowing the default or minimum set of capabilites associated with each device type can facilitate smooth enable the configuration profile manner with additional delta configuration. And optional UE capability need on top of device type.

Observation 5: To reduce the capability signaling, it is proposed to study representative 6G device types and define categories based on both radio capability (e.g., power class, bandwidth, number of antennas) and emerging service-specific features, including AI-native functions, multi-modal support, NTN, vehicular, and aerial deployments. This will be an inter-WG issue, with RAN, RAN1 and RAN4 involved.

Per the RAN #109 discussion, 6GR aims to define a set of mandatory baseline functionalities for all UEs, however, regarding the detail device types and related parameters/factors, further discussion is needed. Then in LTE, per UE Category uplink and downlink capability, including combined UL/DL UE capabilities for Category 1~Category 12 are introduced. However, such fixed definition mechanism is not friendly and can not achieve forward compatibility. Therefore, 6GR needs to consider a more flexible mandatory baseline functionalities based on diverse device types. Further, such mandatory UE Capability need to consider forward compatibility for more possible new device types introduced whereafter, which could improve flexibility compared to LTE per category UE Capability mechanism.

Observation 6: UE categories defined in LTE can facilitate the capabilities' reporting signaling reduction, however, per fixed definition of UE Category uplink and downlink capability in LTE is not friendly and can not cover foreseeable use cases, 6GR needs to consider a definition of Device type categories with future proof.

2.3.2 UE capability signaling structure optimization

Different from the existing too complex UE capability signaling structure including per BC, per FS, per FSPC, 6GR system could consider UE capability signaling structure optimization for 6GR. Specifically, define mandatory UE capability per device type could be considered due to that diverse device types would be introduced in 6GR as mentioned above. And different device types have different related parameters and factors needed to be taken into account for the corresponding Mandatory UE capability. For example, define as more fundamental and comprehensive UE capability sets as possible for all possible device types at 6G initial stage (as shown in the following figure). Then if new types are introduced later, the mandatory baseline UE capability set will be extended in the form of "add on" and based on network configuration to reach forward compatibility. Further in addition to per device type UE capability set, per service UE capability set may be also considered, such as QoS based mandatory baseline UE capability set, etc..

Figure 5: Device type based UE capability framework

Then to further optimize the UE capability structure, index based UE capability (set) could be discussed. As shown in the following table, different UE capability (set) associated with one index, and the detail capability information is pre-defined at network (e.g. RAN, CN node) or the third-party server. When network sends UE capability request signaling, UE could only report corresponding capability index information, which could help to reduce the signaling overhead. Then to further reduce the signaling overhead, UE could only report UE capability information based on network specific request, such as AI related capability, sensing capability, etc..

Table 5: An example for index based UE capability (set)

| Index 1 | UE capability (set) 1 |
| --- | --- |
|  | UE capability (set) 2 |
|  | UE capability (set) 3 |
| Index 2 | UE capability (set) 4 |
|  | UE capability (set) 5 |

To address the interoperability issues due to different understanding on complicated BB/RF features between UE and NW, exploding UE capability size of band combination defines in the specification. Then if the UE is enabled to report explicit RF capability parameters, i.e., RF number supported by UE and corresponding band and bandwidth parameters, it will be beneficial to simplify the UE capability signaling structure.

Observation 7: Enabling the UE to report explicit RF capability parameters will facilitate the simplification of the UE capability signaling structure, i.e., RF number supported by UE and corresponding band and bandwidth parameters.

Proposal 6: RAN2 should study a simplified and future-proof UE capability framework for 6GR, including per device type mandatory UE capabilities, index-based reporting, network-provided filter based UE capability reporting, explicit RF capability parameters, etc., in coordination with RAN, RAN1 and RAN4.

Issues of UE capability update mechanism

For UE capability update, the current mechanism is based on UAI (UE assistance information), including the cases overheating, power saving, and MUSIM, etc. However, such kind of UE capability update is mainly a temporary capability change made for some reasons, which may not be maintained. Further, from the network side, it might not adopt UAI due to that the UAI is merely a suggestion or hope from the UE side.

Observation 8: UAI-based updates are temporary and not guaranteed to be adopted..

UE capability updating mechanism with forward compatibility

In addition to UAI based UE capability update solution, the following more flexible UE capability update mechanism could also be considered.

Based on the index based UE capability definition mentioned above, UE could also report capability update indication instead of detail update information. When UE Capability update request from network is received, UE could report capability indication information instead of detail update capability information to help reduce the signaling overhead. Then UE could just report the capability (set) index information to the network once UE capability update request from network is received, which would reduce the capability signaling overhead compared to detail capability update information reporting.

And with network specific request based UE capability reporting solution, similar UE capability update mechanism could also be discussed. If network needs to obtain some certain specific UE capability update information, it sends UE capability update request information include the specific request to UE, then UE report the UE capability update information based the specific request from network. With this solution, UE could only report the specific capability update information instead of whole UE capability update information, which is also beneficial for signaling overhead reduction.

In order to further reduce the signaling overhead due to multiple UE capability update requests from network, an event based UE capability update mechanism could be discussed. Network configure the UE capability update trigger condition in advance, once the condition is fulfilled, UE report capability update information directly without UE capability update request signaling from network.

Proposal 7: RAN2 could consider more flexible UE capability update mechanisms, including indication-based, network-requested and event-based approaches.

# 3 Conclusion

In this paper, we discuss on basic principles and guidelines on how to address the key issues encountered during the commercialization of 5G and satisfy new requirements from a wide range of services, infrastructure, scenarios in 6G, and UE capability framework for 6GR. We have the following observations and proposals:

Key requirements for 6G

Proposal 1: RAN2 should study the support of new services, e.g., ISAC, AI/ML-based services and immersive communication, taking NW & UE energy consumption and User experience (QoS/QoE) improvement into consideration.

Proposal 2: RAN2 should study support of the typical and special deployment scenarios (including UAV, ATG, HST and NTN) and control/user plane separation, mobility, slicing, Spectrum Aggregation, multiple transmission points, service awareness, TN/NTN harmonization and NPN.

Critical Issues from 5G

Proposal 3: The following key issues from 5G commercialization must to be addressed:

- Protocol Complexity and Innovation Bottleneck

- The NR RRC configuration and UE capability framework has been criticized as overly complex, difficult to process and maintain on the gNB-UE both side, consuming excessive signaling overhead;

- Fixed and coupled protocol stack causes high complexity, high cost and performance limitation;

- Limitation on AI/sensing data collection and model transfer

- Fragmented access control mechanism, including cell barring, UAC and RRC connection rejection;

- Dispersed signaling configurations for mobility/measurement;

- Fragmented design of multi-carrier operation results in inefficient utilization of sparse spectrum, inflexible UL and DL carrier association.

Basic Principles for 6GR design

Proposal 4: 5G design as starting point for 6G with pragmatic simplification:

- Consolidation L2 sublayers to reduce redundancies across protocol layer

- Simplify RRC configuration and UE capability signaling

- Avoid excessive nesting of configurations

-   5G design as starting point for 6G with extensible and flexible:

- Unified forward-compatible design principle for 6G access control, including cell barring mechanism, the definition of access categories and access identities;

- Unified RAN data collection framework for diversified data collected, e.g. AI and sensing;

- Unified mobility mechanism signalling structure with LTM signaling structure as the start point;

- Harmonized measurement framework having the capabilities of the L3- and L1-measurement ;

- Unified multi-carrier framework in 6GR to enable efficient multi-carrier operations.

Proposal 5: RAN2 is proposed to discuss a lean and modular protocol stack as a 6G requirement.

UE capability signaling structure optimization

Proposal 6: RAN2 should study a simplified and future-proof UE capability framework for 6GR, including per device type mandatory UE capabilities, index-based reporting, network-provided filter based UE capability reporting, explicit RF capability parameters, etc., in coordination with RAN, RAN1 and RAN4.

Proposal 7: RAN2 could consider more flexible UE capability update mechanisms, including indication-based, network-requested and event-based approaches.

# 4 Reference

- RP-252476, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone.

- 3GPP TR 38.914 Study on 6G Scenarios and requirements (Release 20)

- 3GPP TR 22.837 Study on Integrated Sensing and Communication (Release 19)

- 3GPP TS 22.137 Integrated Sensing and Communication (Release 19)

- 3GPP TR 22.870 V0.4.0 Study on 6G Use Cases and Service Requirements (Release 20)

- R2-2506909, Requirements on general AI/ML framework and data collection, CMCC.

- R2-2506900, Discussion on 6G control plane, CMCC.

- R2-2506899, Consideration on the mobility in 6GR, CMCC.

- R2-2506905 Consideration on 6GR User Plane, CMCC