3GPP TSG-RAN WG2 Meeting #131bis					R2-2506922

Prague, Czech Republic, Oct. 13th-17th 2025

Agenda item:		9.2.2

Source:			Lenovo

Title:		Discussion for Topology 2 for Rel-20 Ambient IoT

Document for:	 	Discussion and Decision

Introduction

Rel-20 Ambient IoT Phase 2 WID [1] has been approved in RAN#108 meeting. At the same time, a new SID is also approved for Rel-20 Ambient IoT outdoor scenario for active device which is for RAN1 and RAN4 study. In the new WID, objective for Topology 2 will be studied firstly in RAN2 with following constrains

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - According to D2T2-B for Device 1 with carrier-wave transmission for waveform 1 only, per TR 38.769. - No specification support for CW control - According to D2T2-C for active Device(s). - Start after March 2026 - Traffic types DO-DTT and DT, and DO-A (only for active Device(s)). - RAN2 to start on DO-A for D2T2 when sufficient progress has been made on DO-A for D1T1, after a checkpoint in RAN plenary in September 2026 - Representative use cases rUC1 (indoor inventory) and rUC4 (indoor command). - FR1 licensed spectrum in FDD, with R2D in UL spectrum, and D2R and CW in UL spectrum. - Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1. RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

In this contribution, we discussed following aspects for A-IoT Topology 2 scenario, based on the SI conclusion in TR 38.769 [3].

- Architecture

- Resource Allocation Granularity

- Resource Validity

- Resource Allocation Request

- UE Reader Selection

Discussion

Architecture

In the Rel-20 A-IoT WID [1], for Topology 2, the object target to define a single architecture as following

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - … - Specify a single architecture by down-selection between RRC-based architecture and UP-based architecture, in coordination with SA2. - …

According to SA2 agreed TR [3] in SA2#170 meeting, UP based solution is excluded and only RRC based solution is studied, which is as following highlighted part

4Architectural Assumptions and Requirements 4.1Architectural Assumptions The following traffic types for A-IoT Device are to be studied: -DT: Device-terminated; - DO-DTT: Device-originated - device-terminated triggered; and -DO-A: Device-originated - autonomous. Common architectural assumptions: -Architecture reference models defined in TS 23.369 [3] are used as the baseline architecture for this study. -The A-IoT Device is agnostic to topologies, i.e. Topology 1 and Topology 2. Architectural assumptions for support of Topology 2 for Key Issue#1: -No Rel-19 A-IoT Device impact is expected. -The following topology as defined in TR 38.848 [10] is assumed: -BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control -Only Indirect Connectivity (i.e. NG-RAN <-> AMF <-> AIOTF) is applied. -The support of Topology 2 in this study is based on the RRC-based option and the interim conclusions of Rel-19 in TR 23.700-13 [7] is the baseline.

So RAN2 could follow SA2 conclusion that RRC based solution is utilized and studied for A-IoT Topology 2.

Proposal 1: RAN2 study and specify RRC based architecture for Topology 2, according to SA2 conclusion.

The candidate procedure for inventory and command of RRC based architecture for Topology 2 is as following according to [3].

Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 - RRC-based solution

Figure 6.5.3.2-1: Message flow for A-IoT Command in Topology 2 (RRC-based solution)

According to above figures, it was observed that following messages will impact the Uu interface between UE reader and gNB, which could be studied further

- Inventory request message, from gNB to UE-reader

- Inventory response message, from UE-reader to gNB

- Inventory report message, from UE-reader to gNB

- Command request message, from gNB to UE-reader

- Command response message, from UE-reader to gNB

Proposal 2: For RRC based architecture, RAN2 further study the impact of following messages for inventory and command procedure

- Inventory request message, from gNB to UE-reader

- Inventory response message, from UE-reader to gNB

- Inventory report message, from UE-reader to gNB

- Command request message, from gNB to UE-reader

- Command response message, from UE-reader to gNB

Resource Allocation Granularity

It has been agreed that dedicated resource configuration is supported and only given to the UE reader via dedicated signaling from serving gNB. But how the serving gNB controls the A-IoT radio resource, or in the other words what resource allocation granularity could be controlled by the serving gNB to specific UE reader is still open. There’re many options of resource granularity as follows, and each option has its own pros and cons as shown in Table 1.

- Option 1: per UE reader/per service request (i.e., gNB allocates a resource pool/set to the UE reader, and UE reader schedules the specific resource by itself)

- Option 2: per transmission (i.e., including dynamic allocation and semi-static allocation)

- Option 3: per procedure/per sub-procedure

- Option 4: per R2D/D2R transmission pair

- Option 5: per A-IoT paging (round)

- Option 6: per access round

Table 1 Comparison among different options

| Options | Potential Pros | Potential Cons |
| --- | --- | --- |
| Option 1 | - Give UE reader flexibility - Reduce Uu signaling overhead | - The assigned resource may be wasted, thus causing low resource utilization - Add complexity to UE reader |
| Option 2 | - If dynamic allocation, can allocate accurate resource - Allow gNB closely control the resources | - If dynamic allocation, introduce heavy Uu signaling overhead and increase latency - If semi-static allocation, difficult to allocate accurate resource |
| Other options other than option 1 |  | - gNB needs to understand the A-IoT procedure and follow the progress at UE reader side, thus causing additional information interaction btw UE reader and gNB |

For most of A-IoT services (e.g., inventory use case), the message interaction may be frequent between UE reader and devices considering there may be many messages (e.g., Msg1, Msg2, Msg3, NACK, (subsequent) paging, etc.) and plenty of target devices. If the resource granularity is ‘per transmission/message’, it will bring heavy signaling overhead on Uu interface, since UE reader may request resource from gNB for each transmission/message. Thus, in this case, ‘per UE reader’ granularity is more appropriate as it can give UE reader essential flexibility to manage frequent A-IoT transmissions and save signaling on Uu interface.

On the other hand, there’re also some cases where the A-IoT transmissions between UE reader and devices are not frequent (e.g., command message, command response message), and the number of target devices may not be large (e.g., one or several devices). In this case, to avoid wasting assigned resources and improve resource efficiency, it’s feasible to allocate resource per transmission or per R2D/D2R transmission pair.

Proposal 3: RAN2 is suggested to support ‘per UE reader’ resource granularity as baseline, i.e., the gNB allocates resource pool/set per UE reader.

Proposal 4: RAN2 is suggested to further consider other resource granularity options (e.g., per transmission/pair) to support specific procedures (e.g., command procedure).

Resource validity

Resource validity in Scenario 1

In TR 38.769 [3], the resource validity is studied for scenario 1 and scenario 2, and it’s indicated that down-selection can be decided in WI phase. For scenario 1 (e.g., normal case), two options are as follows:

6.3.6 Topology 2 aspects on the interface between UE reader and RAN For the case when the radio resources are allocated semi-statically by the network, the radio resource validities are studied in the following scenarios with candidate options (down-selection can be decided in the WI phase): -Scenario 1: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface: -Option 1: UE reader receives the resources configuration in NR Uu dedicated RRC signalling. The radio resources remain valid until the network releases them explicitly. This would be supported by the NR Uu RRC reconfiguration; and/or -Option 2: UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.

It can be observed that for both option 1 and option 2, gNB may release the allocated A-IoT radio resource by sending RRC reconfiguration message. But it’s still not clear how gNB determines to release the resource. From our point of view, AIoTF and/or UE reader may send some assistance information to gNB as they may know the exact status of the A-IoT procedure (e.g., whether the procedure is completed or not). The assistance information may be the resource release request, the explicit or implicit indication of the end of the procedure/service/paging rounds, etc.

Proposal 5: For resource validity in scenario 1, RAN2 to confirm that at least network explicitly releases radio resources by RRC reconfiguration message is supported.

Proposal 6: For resource validity in scenario 1, gNB determines to release A-IoT radio resource based on the assistance information from AIoTF and/or UE reader. FFS the contents of assistance information.

Regarding whether to configure a time period in which the corresponding resource can be used (i.e., option 2), there were diverse views during SI phase. It’s apparent that option 2 will introduce more spec impact on how to set the time period (e.g., assistance data may be needed), the (re)start and stop of the timer, etc.

Observation 1: Option 2 introduces more spec impact than option 1.

On the other hand, one of the potential advantages of option 2 is that it may be compatible with ‘temporary out of connection’ case. When UE reader is temporarily out of connection, the interaction between A-IoT device and UE reader can continue when the configured time duration is not expired.

Observation 2: The configured time period in option 2 may be used in temporary out of connection case, e.g., the interaction between A-IoT device and UE reader can continue when the configured time duration is not expired even if UE reader is temporarily out of connection.

As for signaling overhead, the comparison between option 1 and option 2 depends on whether the time period of option 2 is set properly or not. If the time period is set too small (i.e., the A-IoT procedure is still ongoing when the timer is about to expire), the A-IoT service may be aborted, or UE reader needs to request additional resource/valid duration from gNB. This will cause worse performance or more signaling than option 1. If the time period is set too long (i.e., the A-IoT procedure ends but the timer is still running), it depends on explicit release from network. In this case, the performance is same as option 1. If the timer period is set properly (i.e., the A-IoT procedure end time is almost consistent with the configured time period), then gNB doesn’t need to explicitly release the A-IoT resource. In this case, signaling is saved compared with option 1.

Observation 3: Only if the configured time duration is set properly, the signaling overhead of option 2 is smaller than option 1.

However, it’s difficult for gNB to set proper time period when providing A-IoT resource configuration at the beginning as the actual duration of A-IoT procedure is dynamic. For example, the uncertain availability of A-IoT devices may cause multiple paging rounds, which may not be known by network in advance. And the response time of D2R messages is difficult to estimate accurately by network due to the uncertain size of command response, uncertain number of segments, etc.

Observation 4: It’s difficult for gNB to set proper valid time duration when providing A-IoT resource configuration at the beginning due to the uncertainty of A-IoT devices and A-IoT procedure.

Based on the above analysis, whether to support option 2 depends on many factors, and RAN2 needs to study its pros and cons.

Proposal 7: For the resource validity in scenario 1, RAN2 is suggested to further study the pros and cons of option 2 (timer-based release), based on the above observations.

Resource validity in Scenario 2

In TR 38.769 [3], it was agreed that in RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases), the details are shown as below:

6.3.6 Topology 2 aspects on the interface between UE reader and RAN -Scenario 2: In RRC_CONNECTED, the UE reader can perform A-IoT operations using the allocated A-IoT radio resource for A-IoT radio interface, upon its temporary out of connection scenarios (e.g., RLF and handover cases): -Option 1: The UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. - Option 2: The UE reader considers that the resources remain valid for a time period. It needs to be further discussed on whether this time period belongs to the one in Option 2 for Scenario 1 or is a different time period. It needs to be further discussed on whether the duration of this time period is related to the ongoing A-IoT operations. NOTE 1: The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.)

- Scenario 2 with Option1

For Option 1 in Scenario 2, the UE reader considers the resources as temporarily invalid during the temporary out of connection condition. The resource may become valid again after the UE recovers from the condition. Considering temporary out of connection includes RLF and HO, the resource validity under these two cases needs to be discussed respectively.

Case#1: RLF case

Based on legacy NR specification TS 38.300 and TS 38.331, there exists the following phases associated with RLF, e.g., recovery and re-establishment. For the recovery phase, when UE receives the indication from physical layer (e.g., out of sync indication, N310), it may start T310. When UE receives N311 consecutive "in-sync" indications from physical layer during T310, it may consider successfully recovered from out of connection. For re-establishment phase, upon T310 expires or upon receiving random access problem indication from source MCG MAC or upon indication from source MCG RLC that the maximum number of retransmissions has been reached, UE declares RLF and starts T311.  Then, UE initiates RRC connection re-establishment procedure to select a suitable cell during T311. If no suitable cell is selected till T311 expiring, UE may enter RRC_IDLE. RAN2 needs to further discuss the resource validity in recovery and re-establishment phases for RLF case if Option 1 for Scenario 2 is supported.

Case#2: Handover case

For the handover case, there exists HO preparation and HO execution phases. In the HO preparation phase, source gNB may send handover request to target gNB.  After target gNB performs admission control, it provides the new RRC configuration as part of the handover request acknowledge message. During HO execution phase, source gNB provides the RRC configuration to the UE by forwarding the RRCReconfiguration message. The UE moves the RRC connection to the target gNB and replies with the RRCReconfigurationComplete. If Option 1 for Scenario 2 is supported, RAN2 needs to further discuss the resource validity in HO preparation and HO execution phases for HO case.

Proposal 8: RAN2 further study the resource validity in different phases if Option 1 for Scenario 2 is supported, e.g.,

- Resource validity in recovery and re-establishment phases for RLF case.

- Resource validity in HO preparation and HO execution phases for HO case.

- Scenario 2 with Option2

Based on the SI phase discussion and Scenario 2 with option2 in [3], UE reader can determine the resource validity for A-IoT air interface at least based on timer-based mechanism, e.g., the UE reader considers that the resources remain valid for a time period. In this option, gNB needs to configure a proper time period for the temporary out of connection scenarios, and gNB may configure two different time periods for RLF and HO cases, respectively. In addition, if Option 2 for Scenario 2 is supported, RAN2 needs further discuss the condition of starting and stopping timer for RLF and HO cases respectively.

Based on the discussion in Scenario 1 with option 2, “UE reader receives the resources configuration in NR Uu dedicated RRC signalling, which configures a time period in which the corresponding resource can be used. The UE reader considers that the resources remain valid for that time period, unless the resource configuration is explicitly released by the network by the NR Uu RRC reconfiguration.”. UE reader may be configured with a resource validity timer regardless of whether UE is in temporary out of connection scenarios or not. Another issue RAN2 needs to be further discussed is the resource validity timer relationship between Scenario 2 and Scenario 1. Considering whether configuring resource validity timer for Scenario 1 and whether the timer (if configured) for Scenario 1 is same as Scenario 2 or not, there may be the following three cases:

- Case#1: No resource validity timer for Scenario 1, and only a resource validity timer for Scenario 2.

- Case#2: Resource validity timers for Scenario 1 and 2 are the same, e.g., a resource validity timer.

- Case#3: Resource validity timers for Scenario 1 and Scenario 2 are different, e.g., two separate resource validity timers.

Proposal 9: RAN2 further study the condition of starting/stopping timer and resource validity time relationship between Scenario 2 and Scenario 1 if Option 2 for Scenario 2 is supported.

Resource Allocation Request

According to the conclusions during study phase, the radio resource used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the BS. To assist the A-IoT radio resource allocation, e.g. resource type and resource size needs to be allocated, assistance information needs to be provided to the BS. It has been concluded in TR38.769 [3] that both the alternatives of CN-based service request and UE reader-based request are studied as the feasible solutions to provide assistance information for BS to configure/allocate A-IoT radio resource.

For the radio resources allocation request, following alternatives are studied: - Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN. NOTE 2: The above two alternatives can co-exist. The configuration for radio resource allocation may also include the validity criteria (if applicable, see the time period in previous option 2 of those scenarios).

Observation 5: Both the cases of CN-based service request and UE reader-based request are studied as feasible solutions to provide assistance information to BS for A-IoT radio resource configuration.

Assistance information from CN

For CN-based service request, the detailed assistance information together with the service operation request from CN to NG-RAN has been concluded in TS 23.360 for R19 topology1 normative work as following:

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests. For Inventory or Command service operation, following Inventory Assistance Information is included in the Inventory Request from AIOTF to NG-RAN: a) A-IoT service type (e.g. Inventory, Command). b) Optionally, approximate number of A-IoT devices based on AF request. c) Size of the Inventory Response message from the A-IoT Device. d) Optionally, time interval for A-IoT Device response aggregation used by the NG-RAN as specified in clause 5.9. For Command service operation, following Command Assistance Information is included in the Command Request from AIOTF to NG-RAN: e) Size of the Command Response message from the A-IoT Device. Bullet c) is determined by AIOTF based on the A-IoT Device ID length for the A-IoT Devices that are expected to respond to Inventory. If not provided by the AF, bullet d) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the A-IoT service provider represented by an AF and the operator. The assistance information is used by the NG-RAN for performing service operations, e.g., radio resource allocation by using bullets b), c) and e), A-IoT Device responses aggregation by using bullet d).

Above conclusions are targeted for Rel-19 with Topology 1 scenario, where the NG-RAN node serves as the reader and allocates the resource to A-IoT device directly; For topology 2, we understood that same assistance information together with the service request is provided from the A-IoT upper layer to the UE reader for performing A-IoT service operation including A-IoT radio resource allocation.

Observation 6: In Rel-19 the AIOTF provides the following assistance information to the NG-RAN together with the service operation requests, for NG-RAN to perform service operations, e.g., radio resource allocation, etc.

- For Inventory or Command service operation:

- A-IoT service type (e.g. Inventory, Command).

- Optionally, approximate number of A-IoT devices based on AF request.

- Size of the Inventory Response message from the A-IoT Device.

- Optionally, time interval for A-IoT Device response aggregation used by the NG-RAN

- For Command service operation:

- Size of the Command Response message from the A-IoT Device.

Regarding whether BS is aware of above CN assistance information may depend on the protocol stack between the UE reader and RAN. As analyzed in section 2.1 Architecture part, we understood that SA2 has already approved that RRC based solution is used between the UE reader and RAN. Then for RRC-based solution, CN indicates the service assistance information for inventory only or for inventory and command service to the UE-reader and explicitly forwarded by the BS via NR Uu RRC message, BS may configure/allocate a set A-IoT radio resource to the UE reader at least based on the CN service assistance information.

Proposal 10: In the case of RRC-based solution is supported, the CN assistance information for resource allocation is transmitted from the CN to the UE-reader and explicitly forwarded by the BS via NR Uu RRC message.

Assistance information from UE-reader

The CN assistance information together with service request is transmitted to the UE reader via RRC signaling. Based on the information, the detailed scheduling parameters e.g., the number of access occasions, the number of frequency resource for each access occasion (value of X, Y), the number of paging rounds, etc. can be determined. For topology1, the scheduling parameters are determined by gNB reader directly based on CN assistance information and included in the A-IoT paging message. However, for topology2 with UE reader, the CN assistance information could be visible for both UE reader and BS. Then whether UE reader or BS to determine the scheduling parameters should be discussed firstly.

- If UE reader determines the scheduling parameters directly based on the CN assistance information, UE reader may need to transmit the determined scheduling parameters to the BS to assist BS to allocate appropriate resource (e.g., determine the resource type, resource size etc.) for A-IoT transmission, or in other case, the UE reader may also determine the preferred resource type and resource size directly based on the scheduling parameters and indicate the preference to BS with the resource allocation request.

- If BS determines the scheduling parameters, BS may determine the scheduling parameters and allocate resource to the UE reader based on the CN assistance information, then indicates the parameters and allocates corresponding A-IoT resource to the UE reader. Whether the explicit request from the UE reader for such resource allocation is needed or not can be further discussed.

Proposal 11: RAN2 to discuss for Topology2, the scheduling parameters e.g., value of X, Y, the number of access occasion, the number of paging round, etc., are determined by the UE reader or BS.

- Opt1: If BS determines the scheduling parameters, BS may determine the scheduling parameters based on CN assistance information and allocate corresponding A-IoT resource to the UE reader.

- Opt2: If UE reader determines the scheduling parameters, UE reader may transmit the determined parameters or preferred resource to the BS, BS allocates A-IoT resource in response to the UE reader request.

The above resource allocation scenarios are targeted for the A-IoT service-initiation phases. During A-IoT data transmission procedure, as agreed in Rel-19 A-IoT system, a more data indication is introduced to indicate whether there are more upper layer data to be sent from the device or not, in the case of UE reader receives a more data indication from the A-IoT device and there has no additional available resource at UE-reader side, the UE-reader may also report such information to the BS, BS then schedules more resource for the subsequent D2R transmission and sends it to UE-reader. Based on above cases, it seems necessary to support BS to configure/allocate the A-IoT resource based on the request from the UE reader, no matter in the A-IoT service initiation phase or during the A-IoT data transmission procedure.

Proposal 12: RAN2 to support the case that BS configures/allocates a set of A-IoT radio resource to the UE reader, in response to a request from the UE reader.

UE Reader Selection

In TR 38.769 [3], it has been concluded that for the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. Currently SA2 is still waiting for the conclusions from RAN3 on this issue. From RAN2 perspective, it is suggested to wait more conclusions on RAN3 and SA2 for the UE reader selection for Topology 2, if BS selects the UE reader is supported, RAN2 can further check and discuss any radio conditions between UE and BS should be involved in the reader selection.

Proposal 13: RAN2 waits the conclusions on RAN3/SA2 for the UE reader selection in topology2, and further study if any RAN2 impacts in the case of RRC solution is supported.

Conclusion

In this contribution, we discussed several aspects for Rel-20 Ambient IoT Topology 2 and made the following observations and proposals:

Architecture

Proposal 1: RAN2 study and specify RRC based architecture for Topology 2, according to SA2 conclusion

Proposal 2: For RRC based architecture, RAN2 further study the impact of following messages for inventory and command procedure

- Inventory request message, from gNB to UE-reader

- Inventory response message, from UE-reader to gNB

- Inventory report message, from UE-reader to gNB

- Command request message, from gNB to UE-reader

- Command response message, from UE-reader to gNB

Resource Allocation Granularity

Proposal 3: RAN2 is suggested to support ‘per UE reader’ resource granularity as baseline, i.e., the gNB allocates resource pool/set per UE reader.

Proposal 4: RAN2 is suggested to further consider other resource granularity options (e.g., per transmission/pair) to support specific procedures (e.g., command procedure).

Resource validity

Resource validity in Scenario 1

Proposal 5: For resource validity in scenario 1, RAN2 to confirm that at least network explicitly releases radio resources by RRC reconfiguration message is supported.

Proposal 6: For resource validity in scenario 1, gNB determines to release A-IoT radio resource based on the assistance information from AIoTF and/or UE reader. FFS the contents of assistance information.

Observation 1: Option 2 introduces more spec impact than option 1.

Observation 2: The configured time period in option 2 may be used in temporary out of connection case, e.g., the interaction between A-IoT device and UE reader can continue when the configured time duration is not expired even if UE reader is temporarily out of connection.

Observation 3: Only if the configured time duration is set properly, the signaling overhead of option 2 is smaller than option 1.

Observation 4: It’s difficult for gNB to set proper valid time duration when providing A-IoT resource configuration at the beginning due to the uncertainty of A-IoT devices and A-IoT procedure.

Proposal 7: For the resource validity in scenario 1, RAN2 is suggested to further study the pros and cons of option 2 (timer-based release), based on the above observations.

Resource validity in Scenario 2

Proposal 8: RAN2 further study the resource validity in different phases if Option 1 for Scenario 2 is supported, e.g.,

- Resource validity in recovery and re-establishment phases for RLF case.

- Resource validity in HO preparation and HO execution phases for HO case.

Proposal 9: RAN2 further study the condition of starting/stopping timer and resource validity time relationship between Scenario 2 and Scenario 1 if Option 2 for Scenario 2 is supported.

Resource Allocation Request

Observation 5: Both the cases of CN-based service request and UE reader-based request are studied as feasible solutions to provide assistance information to BS for A-IoT radio resource configuration.

Observation 6: In Rel-19 the AIOTF provides the following assistance information to the NG-RAN together with the service operation requests, for NG-RAN to perform service operations, e.g., radio resource allocation, etc.

- For Inventory or Command service operation:

- A-IoT service type (e.g. Inventory, Command).

- Optionally, approximate number of A-IoT devices based on AF request.

- Size of the Inventory Response message from the A-IoT Device.

- Optionally, time interval for A-IoT Device response aggregation used by the NG-RAN

- For Command service operation:

- Size of the Command Response message from the A-IoT Device.

Proposal 10: In the case of RRC-based solution is supported, the CN assistance information for resource allocation is transmitted from the CN to the UE-reader and explicitly forwarded by the BS via NR Uu RRC message.

Proposal 11: RAN2 to discuss for Topology2, the scheduling parameters e.g., value of X, Y, the number of access occasion, the number of paging round, etc., is determined by the UE reader or BS.

- Opt1: If BS determines the scheduling parameters, BS may determine the scheduling parameters based on CN assistance information and allocates corresponding A-IoT resource to the UE reader.

- Opt2: If UE reader determines the scheduling parameters, UE reader may transmit the determined parameters or preferred resource to the BS, BS allocates A-IoT resource in response to the UE reader request.

Proposal 12: RAN2 to support the case that BS configures/allocates a set of A-IoT radio resource to the UE reader, in response to a request from the UE reader.

UE Reader Selection

Proposal 13: RAN2 waits the conclusions on RAN3/SA2 for the UE reader selection in topology2, and further study if any RAN2 impacts in the case of RRC solution is supported.

Reference

RP-251885, New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2

RP-251884. New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

TR 38.769 V2.0.1 (2024-12), Study on solutions for ambient IoT (Internet of Things)