3GPP TSG-RAN WG2 Meeting #131bis	R2-2506926

Prague, Czech Replublik, 13th – 17th October 2025

Agenda Item:	8.7.3

Source:	Lenovo

Title:	Discussion on avoiding unnecessary retransmissions

Document for: Discussion and Decision

# 1	Introduction

In this contribution, we will discuss remaining issues related to avoiding unnecessary retransmissions.

# 2. Discussion

5.2.3.2.x Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest. ## 7.1 State variables a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next. ## 7.3 Timers x) t-RxDiscard This timer is used by the receiving side of an AM RLC entity in order to detect discard of AMD PDU(s) (see clause 5.2.3.2.x). If t-RxDiscard is running, t-RxDiscard shall not be started additionally, i.e. only one t-RxDiscard per RLC entity is running at a given time

The definition of RX_Next should be updated in order to be aligned with Actions when t-RxDiscard expires. This issue is proposed in [1]. Our suggestion is: This state variable holds the value of the SN following the last in-sequence completely received RLC SDU also accounting for the SN of RLC SDU(s) which are discarded based on the t-RxDiscard, and it serves as the lower edge of the receiving window.

Proposal 1: [RLC-N01]Update the definition of RX_Next to align with Actions when t-RxDiscard expires, e.g., This state variable holds the value of the SN following the last in-sequence completely received RLC SDU also accounting for the RLC SDU(s) which are discarded based on the t-RxDiscard, and it serves as the lower edge of the receiving window in Appendix A.

### 5.3.4 Status reporting NOTE 2: The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated. NOTE X: The expiry of t-RxDiscard triggers both RX_Next to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Next is updated. When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. - for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet: - include in the STATUS PDU a set of NACK_SN, SOstart and SOend. - for a continuous sequence of RLC SDUs that have not been received yet: - include in the STATUS PDU a set of NACK_SN and NACK range; - include in the STATUS PDU, if required, a pair of SOstart and SOend. - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing and is not discarded due to the expiry of t-RxDiscard in the resulting STATUS PDU. ## 7.1 State variables c) RX_Highest_Status – Maximum STATUS transmit state variable This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

When t-RxDiscard expires, RX_Next is updated to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received, while RX_Highest_Status is according to current specification not updated. RX_Highest_Status holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. To maintain consistency in specification, RX_Highest_Status should be updated to be aligned with the definition of the “ACK_SN”. Therefore, RX_Highest_Status should be updated, e.g., to the SN of the first RLC SDU with SN >= RX_Next for which not all bytes have been received, or the SN of the first RLC SDU with SN >= RX_Highest_Status for which not all bytes have been received, and which is not discarded due to the expiry of t-RxDiscard.

Proposal 2: To maintain consistency in specification, update RX_Highest_Status upon the expiry of t-Discard. Following options can be considered:

- Option 1. Update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next for which not all bytes have been received in Appendix B.

  - Option 2. RX_Highest_Status to the SN of the first RLC SDU with SN >=  RX_Highest_Status for which not all bytes have been received, and which is not discarded due to the expiry of t-RxDiscard.

The expiry of t-RxDiscard triggers both RX_Next to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Next is updated. In this case, there is no RLC SDU that will be included as NACK RLC SDU in status report, and the ACK_SN is just the updated RX_Next, wherein the ACK_SN >= RX_Next > RLC SN of discarded RLC SDU. This issue is also proposed in issue[1]. We also suggest to directly set the ACK_SN to the RX_Next when Status PDU is triggered by the expiry of t-RxDiscard as it is clearer and simpler.

Proposal 3: [RLC-H02] Set the ACK_SN to the RX_Next if Status PDU is triggered by the expiry of t-RxDiscard in Appendix C.

# 3. Conclusion

This contribution makes the following proposals:

Proposal 1: [RLC-N01] Update the definition of RX_Next to align with Actions when t-RxDiscard expires, e.g., This state variable holds the value of the SN following the last in-sequence completely received RLC SDU also accounting for the RLC SDU(s) which are discarded based on expiry of t-RxDiscard, and it serves as the lower edge of the receiving window in Appendix A.

Proposal 2: To maintain consistency in specification, update RX_Highest_Status upon the expiry of t-Discard. Following options can be considered:

- Option 1. Update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next for which not all bytes have been received in Appendix B.

  - Option 2. RX_Highest_Status to the SN of the first RLC SDU with SN >=  RX_Highest_Status for which not all bytes have been received, and which is not discarded due to the expiry of t-RxDiscard.

Proposal 3: [RLC-H02] Set the ACK_SN to the RX_Next if Status PDU is triggered by the expiry of t-RxDiscard in Appendix C.

# 4. Reference

[1] R2-250xxxx, List of open issues for Rel-19 XR 38.322 CR

[2] R2-2506338, Introduction of R19 XR enhancements for RLC spec.

- Appendix A

7.1	State variables

a) RX_Next – Receive state variable

This state variable holds the value of the SN following the last in-sequence completely received RLC SDU also accounting for the RLC SDU(s) which are discarded based on t-RxDiscard, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

c) RX_Highest_Status – Maximum STATUS transmit state variable

This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

- Appendix B

##### 5.2.3.2.x	Actions when t-RxDiscard expires

When t-RxDiscard expires, the receiving side of an AM RLC entity shall:

-	discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any;

-	update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received;

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next for which not all bytes have been received;

-	if RX_Next_Highest > RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-RxDiscard;

-	set RX_Next_Discard_Trigger to RX_Next_Highest.

- Appendix C

### 5.3.4	Status reporting

…

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

NOTE X:	The expiry of t-RxDiscard triggers both RX_Next to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Next is updated.

…

When constructing a STATUS PDU triggered by the expiry of t-RxDiscard, the AM RLC entity shall:

-	set the ACK_SN to RX_Next in the resulting STATUS PDU.

Otherwise, when constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.