3GPP TSG-RAN WG2 Meeting #131bis		                                    R2-2507393

Prague, Czech Republic, 13th – 17th Oct. 2025

Source:	TCL

Title:	Initial considerations on RRC protocol architecture for 6GR

Agenda item:	10.3.2

Document for:	Discussion and decision

- Introduction

At the 3GPP RAN #109 plenary, the updated Study Item “Study on 6G Radio” was approved (SID in RP-252912 [3]). This study establishes high-level objectives for the 6G radio access network (RAN) architecture. Following are part of them:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - ... - j)Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - k)System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - ... - b)Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - ...

In this contribution, we discussed RRC state, Cell management and the harmonized 6G Radio design for TN and NTN and gave initial considerations on those aspect of RRC protocol architecture for 6GR.

- Discussion

  - RRC States

5G NR introduced a three-state RRC model – RRC_IDLE, RRC_CONNECTED, and the newer RRC_INACTIVE state – to balance latency, signaling and power consumption. While this was a step forward, several limitations have been observed in practice:

Signaling Overhead: To reduce the UE’s power consumption, there would be frequent transitions between RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE which can incur significant signaling. UEs in RRC_IDLE must perform a full connection establishment (RRC connection setup) to send or receive data, which involves multiple message exchanges. Even with RRC_INACTIVE, resuming to RRC_CONNECTED requires a resume procedure. These state transitions for sporadic or small data transmissions add control-plane overhead on both UE and network side.

Multi-level paging in RRC_INACTIVE: There are two levels of paging triggers for UEs in RRC_INACTIVE in 5G NR: RAN-initiated paging performed by the NG-RAN (gNB) and core network-initiated paging by the 5GC. The mechanism of two-level of paging could increase UE complexity and power overhead if not managed well.

Limited commercial adoption for RRC_INACTIVE: RRC_INACTIVE state has seen limited commercial adoption. Contributing factors include implementation complexity for vendors, inconsistent UE support (the state is only applicable in 5G Standalone mode and must be indicated in UE capabilities), and the marginal gains observed in some real-world scenarios versus the costs. During the discussion about the lesson from 5G, the principle to simplify the 6G RAN by removing under-utilized options has been accepted widely.

Observation 1: For the three-state RRC model in 5G, there are limitations on significant signaling overhead due to frequent state transition, multi-level paging in RRC_INACTIVE and limited commercial adoption for RRC_INACTIVE.

Moreover, RRC_INACTIVE may be unsuitable for certain UE and service categories—for example, IoT devices with infrequent, small-payload transmissions, and URLLC services with stringent latency constraints. Considering the broader diversity of services and device types expected in 6G, it is proposed that 6GR study a simplified, flexible RRC state model aligned with modular, on-demand design principles. The objective is to minimize the high-level state machine while enabling adaptation via configurable states/sub-states tailored to specific UE and service requirements.

Proposal 1: A simplified and flexible RRC state model with configurable states/sub-states should be studied for 6GR to aligned with specific UE and service requirements.

  - Cell management

In current 4G/5G systems, the fundamental unit of RAN coverage is the cell, typically defined by a base station carrier frequency combination. Each frequency carrier transmitted by a gNB acts as a separate cell entity for idle UEs, broadcasting its own synchronization signals and system information. Idle UEs must search for and camp on one cell per carrier and listen to its broadcast info to access the network on that frequency. This single-carrier-per-cell leads to several inefficiencies:

Idle Mode Energy Overhead: A significant portion of network energy is spent on providing basic coverage and keeping idle UEs informed, rather than on active data traffic. Studies have shown that the power consumed in continuously transmitting cell reference signals and system information across the network can far exceed the incremental energy used for actual user data transfer. In 5G, despite an ultra-lean design philosophy, each active carrier still emits periodic SSBs (typically every 20 ms) and other channel broadcasts by design. If an operator deploys many carriers (including high-frequency bands for capacity), the aggregate idle-mode energy cost multiplies, since each carrier broadcasts its own SSB/SIB even if no UE is actively using that carrier. The overhead is particularly acute for mmWave bands in 6G which might have intermittent usage – keeping them always broadcasting wastes power.

Paging and Broadcast Duplication: Similarly, paging messages and system information have to be sent per carrier. In current 5G multi-band deployments, an idle UE is typically assigned a primary cell where it receives paging; other carriers might not page the UE unless they are configured as secondary cell which only occurs after connection. This architecture simplifies UE design (only need to monitor one paging channel at a time), but it also means that each carrier still maintains the capability for independent paging and broadcast in case a UE camps there. The duplication of broadcasts across carriers increases the overall air-interface load.

- Idle UE Measurement Burden: Because each carrier acts as a separate cell with its own coverage footprint, idle UEs must occasionally search across all supported frequency layers for the best cell (cell reselection). In a multi-layer network (low, mid, high bands), UEs periodically wake and measure each band’s signals. This frequent inter-frequency measurement consumes UE power and introduces latency in cell reselection. A traditional design forces UEs to treat every carrier as a distinct entity to monitor. In 5G, features like speed-dependent measurement relaxation exist, but fundamentally the more carriers deployed, the more work an idle UE has to do to ensure it’s on the optimal cell. This can drain battery and also delay connection establishment if the UE needs to switch cells during random access.

Carrier Aggregation Complexity: In connected mode, 5G supports Carrier Aggregation (CA), where a UE can utilize multiple carriers simultaneously (one primary cell and multiple secondary cells). However, setting up CA requires RRC signaling to add secondary cells (SCells) and each added SCell comes with its own configuration (SCell ID, SSB, reference signals, etc.). The primary cell (PCell) remains the anchor for RRC control, while SCells are mostly user-plane. Still, the UE and network must maintain synchronization and link monitoring on each active SCell. If each SCell is a full-fledged cell, there is overhead in configuration and possibly duplication of some control functions. For example, mobility between frequencies (inter-frequency handover) often involves releasing/adding cells rather than a seamless extension of the existing cell.

Observation 2: For the case deploying many carriers, single-carrier-per-cell design would lead to Idle Mode Energy Overhead, Paging and Broadcast Duplication, Idle UE Measurement Burde and Carrier Aggregation Complexity for connected UE.

As 6G envisions using a wide range of spectrum, the traditional cell concept may become a bottleneck in efficiently managing these resources. To overcome the above challenges, a unified cell architecture in 6G should be studied, wherein a “multi-carrier cell” can span multiple carriers (frequencies) under one logical entity. In essence, a base station could have one broadcast cell identity covering multiple carriers, with a flexible arrangement of coverage carriers for idle UEs vs. capacity carriers for connected UEs.

Proposal 2: In the 6G RAN RRC architecture, introduce a unified multi-carrier cell design for both idle and connected modes UE.

  - Harmonized 6G Radio design for TN and NTN

To achieve the objective of IMT-2030 vision of global ubiquitous connectivity, there is an ongoing discussion in both SA and RAN 3GPP working groups on 6G system and radio access architecture encompassing both terrestrial and non-terrestrial network access/infrastructure from day-one. 3GPP SA WGs are currently discussing several use-cases and requirements such as HAPS-based rapid deployable network for public safety, ubiquitous emergency rescue via UAVs, immersive media services for advanced air mobility enabled by 6G NTN, and enhanced user experience with sparse LEO satellite deployment - demanding a unified 6G TN/NTN design [1]. On the other hand, 3GPP RAN WGs are also aiming at supporting NTN deployment scenarios with clear objectives towards harmonizing 6G Radio design for TN and NTN [2-3]. Having a unified TN/NTN design from day-one is crucial for covering the remaining coverage gaps such as oceans, mountains, rural areas, and even airspace for drones and airplanes connectivity. Moreover, a unified TN/NTN design enables seamlessly support of mobility and services that cross between terrestrial and satellite links without the complexity of adding new technology, features, or components to the 6G initial protocols later on.

Observation 3-1: A unified TN/NTN design from day one is essential for 6G to provide ubiquitous coverage and seamless service continuity, preventing the need for complex and costly integration of non-terrestrial components at a later stage

Unified Radio Interface

A unified 6G air interface must support diverse platforms with vastly different characteristics: propagation delays ranging from milliseconds (TN) to hundreds of milliseconds (GEO), extreme Doppler shifts, heterogeneous link budgets, and distinct channel conditions. Key asymmetries exist in coverage areas, beam characteristics, and dynamic topologies - with LEO constellations and UAVs requiring frequent handovers compared to relatively static GEO and terrestrial networks (Table 1). Current 5G initial access and RRC framework face significant challenges in these mixed environments that demand a flexible waveform and air interface configuration. This creates a fundamental requirement for platform-agnostic unified initial access and connection establishment in the unified air interface.

Observation 3-2: The significant physical layer disparities between TN and NTN platforms necessitate unified access and RRC modelling procedures that can automatically adapt to different waveform and TN/NTN platform signal characteristics.

| Parameter | UAV (Drone) | HAPS | LEO Satellite | MEO Satellite | GEO Satellite | Terrestrial (TN) |
| --- | --- | --- | --- | --- | --- | --- |
| Altitude | 50 m – 10 km | 18–25 km | 500–1,500 km | 8,000–20,000 km | 35,786 km | 10–50 m (Macro) |
| Cell Size (Diameter) | 1–10 km | 50–200 km | 50–500 km | 200–1,000 km | 200–3,500 km | 0.1–3 km (Macro) |
| Beam Type | Narrow (Directional) | Wide (Multi-beam) | Spot Beams (Multi-beam) | Wide Beams | Fixed Continental | Sectorized (120°) |
| Propagation Delay (RTD) | 0.3–3 ms | 0.6–8 ms | 3–15 ms | 50–130 ms | 250–280 ms | 0.1–1 ms |
| Doppler Shift | ±1 kHz | ±5 kHz | ±100 kHz | ±20 kHz | ±1 kHz | ±500 Hz |
| Cell Handover Frequency | Minutes (~0.1–5 km) | Minutes–Hours (~30 km) | 2–10 Seconds (Orbital) | 5–60 min (~3.9 km/s) | None (Geostationary) | User mobility only |
| Atmospheric Loss | Tropospheric | Minimal | Ionospheric (<6 GHz) | Ionospheric | Rain fade (>10 GHz) | Minimal (LOS/NLOS) |
| Bit Error Rate (BER) | 10⁻⁴ – 10⁻⁶ | 10⁻⁵ – 10⁻⁷ | 10⁻⁴ – 10⁻⁶ | 10⁻⁵ – 10⁻⁷ | 10⁻³ – 10⁻⁵ | 10⁻⁶ – 10⁻⁹ |

Table 1: TN/NTN Multi-platform Parameters Variations

Unified Radio Protocol Signalling

To support satellite access in 5G, the RRC layer introduces extended timers (T300, T304) for connection handling and broadcasts critical parameters via SIB19 (NR) and SIB31 (LTE/NB-IoT), including satellite ephemeris data for real-time positioning, Doppler pre-compensation, and timing adjustments to mitigate propagation delays. The MAC layer optimizes scheduling with K-offset adjustments and uplink synchronization mechanisms, while expanding HARQ processes (up to 32 slots) to accommodate long round-trip times. It also reduces active HARQ processes (from 16 to 4) to prevent buffer overflow, extends Timing Advance (up to 0.86 ms), and modifies RACH procedures with longer response windows (up to 40.96 ms). The RLC layer increases the ARQ window size to prevent stalling and adjusts polling frequency, while the PDCP layer extends reordering timers (t-Reordering) and expands duplicate detection windows to handle out-of-order packets from variable satellite delays.

These 5G radio enhancements (continuously sending adjustments or compensation parameters) are implemented per TN and NTN or per GEO and non-GEO (NGO) satellites. However, a radio protocol based on continuous sending of variable parameters per TN/NTN or per GEO and NGO satellites over the radio interface is not optimal for 6G's unified TN/NTN system. This is due to the asymmetrical requirements of each platform in 6G's multi-orbit and multi-platform scenario, which may require excessive information exchange over the air interface (Figure 2).

Observation 3-3: Unlike 5G, which considered TN and NTN separately, 6G radio protocol design must account for and be optimized for the diverse physical parameters and operational characteristics of different TN and NTN platforms from the beginning.

Figure 2: Non-terrestrial Multi-platform Specific Constraints

Proposal 3: NTN/TN unification should be studied from day one for 6GR, including unified initial access and RRC modelling techniques and a unified 6G-radio protocol signalling framework.

- Conclusion

In this contribution, we discussed the following aspect of RRC protocol architecture for 6GR and provided our proposals as follows:

- RRC States

Observation 1: For the three-state RRC model in 5G, there are limitations on significant signaling overhead due to frequent state transition, multi-level paging in RRC_INACTIVE and limited commercial adoption for RRC_INACTIVE.

Proposal 1: A simplified and flexible RRC state model with configurable states/sub-states should be studied for 6GR to aligned with specific UE and service requirements.

- Cell management

Observation 2: For the case deploying many carriers, single-carrier-per-cell design would lead to Idle Mode Energy Overhead, Paging and Broadcast Duplication, Idle UE Measurement Burde and Carrier Aggregation Complexity for connected UE.

Proposal 2:  In the 6G RAN RRC architecture, introduce a unified multi-carrier cell design for both idle and connected modes UE.

- Harmonized 6G Radio design for TN and NTN

Observation 3-1: A unified TN/NTN design from day one is essential for 6G to provide ubiquitous coverage and seamless service continuity, preventing the need for complex and costly integration of non-terrestrial components at a later stage

Observation 3-2: The significant physical layer disparities between TN and NTN platforms necessitate unified access and RRC modelling procedures that can automatically adapt to different waveform and TN/NTN platform signal characteristics.

Observation 3-3: Unlike 5G, which considered TN and NTN separately, 6G radio protocol design must account for and be optimized for the diverse physical parameters and operational characteristics of different TN and NTN platforms from the beginning.

Proposal 3:  NTN/TN unification should be studied from day one for 6GR, including unified initial access and RRC modelling techniques and a unified 6G-radio protocol signalling framework.

- References

- TR 22.870: "Study on 6G Use Cases and Service Requirements".

- TR 38.914: "Study on 6G Scenarios and Requirements".

- RP-252912: " Revised SID: Study on 6G Radio“NTT DOCOMO, CMCC, AT&T, Vodafone Beijing, China, September 15-18, 2025.