3GPP TSG-RAN WG2 Meeting #131bis	R2-2506950

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	10.2

Source: 	OPPO

Title:  General aspects on RAN2 6G

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to present our considerations on some general aspects of RAN2 6G design, including principles/guidelines, architecture and deployment scenarios, and UE capability framework.

For principles/guidelines, some of the principles can be still reused from 5G NR RAN2 design, meanwhile, 5 new aspects are proposed from our perspective considering the lessons we’ve learnt from 5G NR L2 designs, and we also propose a Text Proposal for the TR.

For architecture and deployment scenarios, potential new way/enhancement of aggregating of spectrum is proposed. Meanwhile, we also propose to support common data framework, LCM when considering 6G RAN2 architecture.

Finally, for the capability framework, we discuss several aspects including overhead reduction, signalling report etc, and give our suggestions on how to further improve UE capability framework.

Discussion

  - Principle, Guidance

General Design Principles for 6G Radio protocols and procedures:

- Minimalist and essential design:

  - Avoid over-engineering for rare cases:

    - 5G NR introduced many options (e.g., (NG)EN-DC, NE-DC, eLTE) for interworking with 4G LTE, many of which remain unused commercially. 6G protocols shall prioritize essential functionalities with proven commercial viability, eliminating redundant options to reduce specification complexity and implementation overhead;

    - For vertical services (e.g., URLLC), NR spent lots of efforts on meeting the extreme performance requirements (e.g., ultra low latency), which led to complex protocol functions added on top of eMBB. 6G shall consider a modularized framework [RP-251769] , so that different verticals and device types can be designed in a more efficiency and simple way.

  - Focusing on enhancement for essential design

    - 5G CA framework was mainly inherited from 4G, which handled the aggregation of CCs as an aggregation of cells, and leads to lower efficiency, scatted design, inflexibility of load distribution and etc. So 6G should focus on carrier aggregation, as a key tool for spectrum aggregation, to enhance the related performance.

- Eliminate Redundant Functions:

  - Redundant mechanisms (e.g., retransmission across MAC, RLC, and PDCP layers) shall be consolidated. 6G protocols must streamline layer-specific functions to reduce overhead.

- Enhance security:

  - 5G NR supports security (ciphering/integrity) only in PDCP layer, leaving lower layers (e.g., MAC CE) vulnerable. 6G shall enforce cross-layer security, extending protection to MAC and physical layers to mitigate attacks on sensitive control information/

- Native support for new emerging services:

  - While 5GA initiated AI/ML studies, 6G protocols must natively embed AI-driven functionalities (e.g., AI enabled L2/L3 procedures).

  - Define a standardized protocols/procedures for new services, e.g., AI model/AI data/Sensing data exchanges.

- Energy efficiency:

  - Energy efficiency was not considered well in the 5G day1. Thus, incorporate energy-aware protocols and procedures, like on-demand signal reception, unified power saving schemes to align with global sustainability goals

  - Leverage AI for predictive measurements, minimizing idle-state energy consumption

- Ru-use some of the principle/guidelines adopted in TR 38.804 when necessary.

- For principles of 6GR RAN2 designs, consider new aspects, e.g., minimalist and essential design, eliminating redundant functions, enhanced security, native support for new emerging service, energy efficiency, on top of those which can be re-used from 5G NR principles. Endorse the following TP, as the principles/guidelines:

## x.x	Guidelines

6G Radio Protocols and Procedure should consider the principles as below:

-	Minimalist and essential design: to avoid over-engineering for rare cases and focus on enhancements for essential design.

-  Eliminate redundant functions: redundant mechanisms in 5G protocols (if identified) shall be consolidated.

-  Enhance security: 6G shall enforce cross-layer security, extending protection to MAC and physical layers to mitigate attacks on sensitive control information

-  Native support for new emerging services: to define a standardized protocols/procedures for new services, e.g., AI model/AI data/Sensing data exchanges.

-  Energy efficiency: energy efficiency aware protocols should be considered in 6G Day1.

For both control plane and user plane protocols:

-	Most essential functions (e.g., initial system access) should be future proof and designed to be common to various different use cases, diverse device types and services.

-	NR layer 2 and RRC functions are taken as a baseline for 6GR.

In terms of intra-6GR mobility:

-	Two types of UE states are taken as a baseline: one is network controlled mobility and the other is UE based mobility.

-	Study will start from TN-NTN-common part and consider NTN-specific part.

-	6GR aims to define a unified mobility procedure by taking NR mobility features as starting point.

-	Study schemes to further reduce power and resource consumption to optimize mobility for no-data scenario.

-	Study schemes to enable dynamic load balance for mobility in RRC_IDLE.

In terms of system information delivery:

-	Take 5G NR System information delivery as baseline.

-	Study the on-demand SIB1 delivery for standalone cell to enhance network energy saving.

-	Study more efficient mechanism for handling of system information change.

In terms of UE radio access capabilities, the following issues should be considered in 6GR design:

b)	UE capability signalling overhead reduction.

a)	UE capability updating due to exceptional UE issues, e.g. overheating and MUSIM problems.

  - Architecture and Deployment Scenarios

The first question is about which spectrum would be the main target for 6G

Table 1 Candidate Spectrum for 6G

| Spectrum Range | Detailed Info | Analysis |
| --- | --- | --- |
| Major 6G spectrum candidate 6.425-7.125GHz 7.125-7.400/8.400GHz, 14.80-15.35GHz (4.4-4.8GHz) | Identified as major 6G spectrum candidate, based on WRC-23 (6.425-7.125GHz) and WRC-27 (7.125-7.400/8.400GHz, 14.80-15.35GHz) | Mainly as basic contributor to capacity, but limited coverage compared to 4G/5G spectrum |
| dmW-band as coverage booster Sub-2GHz | Potential refarmed spectrum from legacy network, e.g., 700/800/900MHz, 1.8GHz and etc. | Mainly as coverage enabler, but not capacity enabler |
| mmW-band as capacity booster 26GHz, 40Ghz | The deployment / commercialization of mmW-band is to be seen | For capacity enhancement, yet rather limited coverage. |

So obviously for 6G, the role of spectrum for coverage and capacity diverse. And the relationship between TRP for coverage layer and TRP for capacity layer can be either co-located or non-collocated.

Figure 1 Deployment Scenario for 6G, e.g., via lower / higher frequency for coverage / capacity

- Spectrum-wise, the candidate spectrum for 6G diverse, 
1) cmW-band (6.425-7.125GHz (WRC-23), 7-15GHz (WRC-27)) and potentially mmW-band acts as basic contributor to capacity, but with limited coverage compared to 5G; 
2) while sub-2GHz spectrum refarming helps coverage, but clearly, they are not capacity enabler.

Considering the major candidate spectrum for 6G as summarized above, in our view, what brings real benefit to the consumer is in two aspects

- Robust connection to the network (e.g., carrying control plane data and also maybe essential user plane data like for emergency service), for which a lower frequency band is needed to secure the always-on low-bit-rate link;

- On-demand connection to the network (e.g., carrying high-data-rate traffic data for entertainment service, and/or for load re-distribution), for which temporary available low/medium/high frequency band is required.

Then eventually, what consumer needs is a RAN which is robust and always-on to carry a moderate-bit-rate connection, and also able to take additional load / higher bit-rate traffic when necessary. That is the ultimate goal of efficient usage / aggregation of spectrum.

- R2 study 6G RAN architecture that is able to aggregate spectrum of 1) lower frequency which is to offer robust connection for coverage, together with 2) higher frequency which is to offer on-demand connection for capacity, covering both collocated and non-collocated scenarios.

Since 4G, RAN has deployed multiple operation options to aggregate spectrum, i.e., 1) Carrier aggregation, where the aggregation happens at lower layer-2, and 2) (intra-RAT) Dual connectivity, where the aggregation happens at higher layer-2. Between the two,

- CA has been widely deployed since 4G, supporting both collocated and non-collocated scenario. And it is expected to achieve higher efficiency due to the potential of joint lower layer design (including scheduling, HARQ and etc.), which is especially important when used to aggregation spectrum fragment.

- While commercialization of (intra-RAT) DC is relatively limited, considering the worse performance and the additional complexity (e.g., the complex inter-node coordination design) compared to CA.

- In legacy, the mechanisms to aggregate spectrum are via either Carrier-Aggregation (CA) or Dual-Connectivity (DC), where CA shows superior performance over the latter one, and thus is widely deployed since 4G. On the contrary, the commercialization of DC is rather limited, and is still pending RP decision whether to support it in 6G.

From TS 36.300

- 7.5 Carrier Aggregation When CA is configured, the UE only has one RRC connection with the network. At RRC connection establishment/re-establishment/handover, one serving cell provides the NAS mobility information (e.g., TAI), and at RRC connection re-establishment/handover, one serving cell provides the security input. This cell is referred to as the Primary Cell (PCell). In the downlink, the carrier corresponding to the PCell is the Downlink Primary Component Carrier (DL PCC) while in the uplink it is the Uplink Primary Component Carrier (UL PCC). Depending on UE capabilities, Secondary Cells (SCells) can be configured to form together with the PCell a set of serving cells. In the downlink, the carrier corresponding to an SCell is a Downlink Secondary Component Carrier (DL SCC) while in the uplink it is an Uplink Secondary Component Carrier (UL SCC).

From TS 38.300

- 7.7 Carrier Aggregation When CA is configured, the UE only has one RRC connection with the network. At RRC connection establishment/re-establishment/handover, one serving cell provides the NAS mobility information, and at RRC connection re-establishment/handover, one serving cell provides the security input. This cell is referred to as the Primary Cell (PCell). Depending on UE capabilities, Secondary Cells (SCells) can be configured to form together with the PCell a set of serving cells. The configured set of serving cells for a UE in a cell group always consists of one SpCell and one or more SCells.

It is clear that since 4G, aggregation of carriers has been handled as aggregation of cells. Although it is an easier solution in the sense of keeping the per-CC cell framework. It ends up with some drawbacks, which will be analysed in the following sub-clause.

- The 5G CA framework was mainly inherited from 4G CA framework, which handled the CC aggregation in a way of cell aggregation.

In our view, there are some aspects of 4/5G CA framework can be considered for improvement in 6G

Table 2 Issues of 5G Carrier-Aggregation scheme and potential enhancement in 6G (Part 1)

| Key Dimension | Detailed Aspect | 5G Status | 6G Improvement | Applicable Scenarios |
| --- | --- | --- | --- | --- |
| Spectrum Efficiency | Reduced per-CC overhead, e.g., SSB, SIBx and etc. | Although SIB-less SCell has been defined in 5G, the deployment is limited since the support of inter-band support is introduced late in R18, and non-collocated scenario (OD-SSB) is introduced late in R19. | Reduced always-on signal from Day-1. | At least for intra-band/inter-band and collocated scenario, while on-demand signal can be extended to non-collocated |
|  | Cross-CC/Integrated PDSCH design | Per-CC PDCCH and PDSCH/TB design, even though cross-CC scheduling is supported. | Cross-CC/Integrated PDSCH for less-DCI, less PHY resource fragmentation due to resource allocation restriction, and less L12 overhead for larger TB. | For intra-band and collocated scenario |
|  | Cross-CC/Integrated HARQ design | Per-CC HARQ entity | Cross-CC HARQ to explore diversity across CCs | Both intra/inter-band and (non-)collocated scenarios |
| Load Distribution | Flexible load distribution during initial-access | Dynamic offloading between frequencies is limited to SUL for RACH load balance Otherwise, load balance between frequencies is based on static setting, i.e., cell reselection procedure | Dynamic load balance for frequencies other than SUL, not limited to RACH attempts. | Both intra/inter-band and (non-)collocated scenarios |
| Link Robustness | P/SCC Role alleviation | Critical functions are on PCell, including but not limited to security, RACH, RLM and etc. | Alleviate the P/S-CC role, and thus robust to failure of either CC | The benefit is mainly from non-collocated scenario (if assuming lower-frequency is normally set as PCC) |

Given the aspects above, a further improved CA framework in 6G is critical for the success of 6G system.

- The legacy cell aggregation approach leads to some drawbacks, at least including 1) unnecessary per-cell overhead, 2) unnecessary scheduling overhead/restriction, 3) limited HARQ diversity due to per-cell HARQ entity, 5) Static load balance restricted by per-cell camping and access (other than SUL), and 5) Asymmetric cell functionality.

Besides, it can be considered to use the improved CA framework to accommodate different tools.

Table 3 Issues of 5G Carrier-Aggregation scheme and potential enhancement in 6G (Part 2)

| Key Dimension | Detailed Aspect | 5G Status | 6G Improvement | Applicable Scenarios |
| --- | --- | --- | --- | --- |
| Unified Design NOTE: In the interest of extended coverage, and support of UL-heavy traffic. | Unified design for various aggregation scheme (e.g., CA-like, SUL-like/Carrier switching) | Separate solutions for CA: concurrent operation SUL/Carrier Switching: non-concurrent operation | A same framework for both concurrent and non-current operation | Both intra/inter-band and (non-)collocated scenarios |
|  | DL/UL decoupling | Cross-band DL/UL coupling (other than SUL band) is not supported | Support cross-band DL/UL coupling (e.g., for FDD band) | Both intra/inter-band and (non-)collocated scenarios |

- The 5G spectrum aggregation framework, has 1) separate design for concurrent/non-concurrent operation, and 2) incapable of UL-only operation (other than SUL).

In order to optimize for the aspects above, further study is needed to evaluate the issue and identify potential solution(s).

- R2 study 6G spectrum aggregation architecture improvement on top of 4/5G design, e.g., in terms of 1) improved spectrum efficiency, 2) unified design of various aggregation solution(s), 3) dynamic load balance, and 4) improved link robustness. FFS on the applicable scenario (intra/inter-band, (non)collocated, RRC states and etc.)

AI/ML feature is something new for 3GPP system. Until NR R17, RAN started to study AI/ML feature from RAN3. In the follow-up release, i.e., NR R18/R19/R20, more and more use cases are studied by RAN1/RAN2/RAN3, it seems AI/ML feature attracts more and more attention due to the potential gains compared to non-AI feature. So, no surprise, AI/ML feature is one of the hottest topics for 6G study, but this does not mean that the gains from AI/ML feature are well proved/justified in 5G. At least the following aspects still need further study/clarification:

- A certain degree of gain is observed in B5G AI study, but the potential pain is not well evaluated during the study, especially on energy efficiency aspect, which may degrade the potential gain from AI.

- Due to the fact that NR AI/ML features are not introduced from R15, there is almost no discussion on co-existing and interworking between AI features and non-AI features, which does not match the vision for achieving native AI.

- NR LCM framework design is a good starting point for 6G, but it should be noted that per use case basis LCM discussion is not good way for future proof.

- Divergent data collection frameworks are introduced in NR system, but this design is not efficient considering the procedures for many data collection tasks are quite similar.

To address the concerns above, we think in the very beginning of 6G, the following aspects can be considered as the guidance/principle for 6G RAN AI study:

- 6G RAN design should study the co-existing and interworking between AI and non-AI features, taking into account of comparable energy efficiency;

- 6G LCM framework: taking NR LCM framework design as the baseline, should be future proof and designed to be common to various different use cases；

- 6G data collection framework: should be future proof and designed to be common to various different use cases, e.g., AI/sensing.

- R2 study the AIML architecture/framework to consider: co-existing and interworking between AI and non-AI of a certain feature, LCM framework common to various use cases, and common data framework to be future proof.

  - UE Capability Framework

Since 4G when CA was introduced, UE capability message size grows rapidly. Considering the capability can be categorized as per-band and per-BC (which can be further detailized as per-FS, per-FSPC and etc.), more supported band would lead to the increase of per-band capability in a linear manner, but the increase of per-BC (and per-FS, per-FSPC capability) in an exponential manner.

The following is one example calculation of the signalling overhead for a capability message, for NR SA case, which shows that the main source of signalling overhead is the BC list and/or the FSC list. And it can be understood that larger number of supported band combination and/or the number of supported FSC entries, would lead to higher ratio of overhead.

Table 4 Two examples of UE capability signalling overhead (for NR SA)

| Source of Capability IE/Fields | Case-1 |  | Case-2 |  |
| --- | --- | --- | --- | --- |
|  | Ratio (of the whole capability message) | More details | Ratio (of the whole capability message) | More details |
| supportedBandListNR | 12% | ~20 Bands | 4% | ~10 Band entries |
| supportedBandCombinationList | 50% | ~180 BC:s | 13% | ~80 BC:s |
| featureSetCombinations | 31% | ~110 FSC:s (containing ~660 FSC entries) | 70% | ~80 FSC:s (containing ~2100 FSC entries) |
| featureSet | 5% | ~30 DL FS:s ~20 UL FS:s | 5% | ~35 DL FS:s ~30 UL FS:s |

The same issue is expected for 6G system, due to the support of band combination, i.e., for spectrum aggregation.

Although UE capability is theoretically reported only upon initial registration, there are scenarios when network further enquire UE capability after initial registration, e.g., due to the different capability of network node (like different supported band(s)). It means the signalling overhead issue for capability is not one-shot, but would consume resources in the network frequently, and thus signalling overhead reduction for capability message is always a key issue.

- In 5G, UE capability message size grows quickly with the number of supported band-combinations / FSC entries, which leads to high signaling overhead.

To reduce the signaling overhead, in 5G, 3GPP design the FS/FSC-based method, i.e., to procedure a ‘dictionary’ for the entries that potentially to be repeated multiple times, and thus replace them with an index, which points to the entries defined in the dictionary. In this way, the repetition of entries is replaced by repetition of index, and thus the related signaling overhead is expected to be reduced. Essentially, the key point here is to look for redundancy in the message and to compress it without information loss.

On the other hand, this method did not solve the issue ultimately, and then in later releases 3GPP enables segmentation of capability messages, to the maximum of 16 segmentations (which is later reduced to 6 segmentations, and the allowed number of segmentations is further changed to configurable).

While the question is, in the end, whether is still non-trivial remaining redundancy in the capability message (after adopting the FS/FSC-based method). If so, further compression is helpful / meaningful. If not, unless major change of the signaling framework (which probably ends up with restricted UE/chipset implementation flexibility), there is no much potential expected by the exercise of capability signaling overhead reduction.

To answer this question, we impose some typical compression methods on top of a capability message directly, in order to understand the level of remaining redundancy (if any) after adopting the FS/FSC-based method.

Table 5 Potential capability signalling compression ratio (NR SA)

| Compression Method | Case-1 |  | Case-2 |  |
| --- | --- | --- | --- | --- |
|  | Compressed message size (compared to original size) | Further analysis of the contribution | Compressed message size (compared to original size) | Further analysis of the contribution |
| Deflate | 46% | 40% from LZ77 | 51% | 47% from LZ77 |
|  |  | 6% from Huffman Coding |  | 4% from Huffman Coding |
| LZMA | 50% |  | 53% |  |
| PPMD | 49 |  | 56% |  |

The result showed that the ASN.1 grammar-based compression (i.e., FS/FSC-based method) did not fully explore / remove the redundancy within the message, which then shows the potential to further explore method to reduce capability message size.

- Even though it has been identified that the key source of the capability message overhead is per-FS/FSPC capability and thus the FS/FSC based dictionary + index method has been defined to reduce redundancy, the capability overhead is still large due to remaining redundancy.

If take a step back, thinking about the root cause of this issue, i.e., the redundancy of SRB messages, it can be understood as that the lack of source-encoding (as adopted for DRB, at APP layer) caused this issue.

- Different from DRB, SRB has no source-encoding to reduce redundancy, which is the root cause of the redundancy and thus high signalling overhead of capability message.

And in 5G spec, the segmentation of SRB message hints the concern of large message size, covering

- – DLDedicatedMessageSegment The DLDedicatedMessageSegmentmessage is used to transfer one segment of the RRCResume or RRCReconfigurationmessages. - – ULDedicatedMessageSegment The ULDedicatedMessageSegment message is used to transfer segments of the UECapabilityInformationor MeasurementReportAppLayer message. SRB1 is used at transfer of segments of UECapabilityInformationand SRB4 or SRB5 is used at transfer of segments of MeasurementReportAppLayer.

And also, the segmentation-based method has been adopted to SIB7/8/12/17/17bis/23.

If any compression method to be adopted, it is preferred to be adopted/applied to different SRB messages, rather than design message-specific optimization method.

- For 6G UE capability reporting, R2 study methods to reduce redundancy / overhead for SRB, covering but not limited to capability message(s).

Based on the current 5G design, the UE capability can be updated during RRC_IDLE

<From 23.501> - 5.4.4.1 UE radio capability information storage in the AMF If the trigger to change the UE's NG-RAN or E‑UTRAN UE Radio Capability information happens when the UE is in CM-CONNECTED state, the UE shall first enter CM-IDLE state and then perform the Registration procedure with the Registration type set to Mobility Registration Update and it also includes "UE Radio Capability Update". <From 23.502> - 4.2.2.2 Registration procedures 4.2.2.2.1 General A UE needs to register with the network to get authorized to receive services, to enable mobility tracking and to enable reachability. The UE initiates the Registration procedure using one of the following Registration types: - Initial Registration to the 5GS; - Mobility Registration Update upon changing to a new Tracking Area (TA) outside the UE's Registration Area in both CM-CONNECTED and CM-IDLE state, or when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure with or without changing to a new TA, or a change in the UE's Preferred Network Behaviour that would create an incompatibility with the Supported Network Behaviour provided by the serving AMF, [...]

Due to the difficulty to do that in RRC_CONNECTED: 1) not only about how to do that on-the-fly considering the on-going R1/R2 procedure, 2) but also about how to update the RAN entity and CN entity on-the-fly considering it is a joint RAN and CN procedure.

<From 23.501> The RAN stores the UE Radio Capability information, received in the N2 message or obtained from the UE, for the duration of the UE staying in RRC_CONNECTED or RRC_INACTIVE state. Before any 5G SRVCC handover attempt from NG-RAN to UTRAN, the RAN retrieves the UE's UTRA UE Radio Capabilities from the UE. (For specific requirements for a UE operating in dual-registration mode see clause 5.17.2.1).

- In 4G/5G, UE capability update can be performed during RRC_IDLE, via TAU/MRU procedure.

On the other hand, there are scenarios / reasons that UE would like to do this during RRC_CONNECTED state, like overheating, power saving and MUSIM reasons. The way to handle that is via UAI messages, which is essentially a delta part, on top of the UE capability, and each RAN node, can combine both, to derive the applicable UE capability.

Looking into the content of UAI message, the degrade of UE capability is expressed in a simplified manner

- For overheating: Reduced maximum CC number, Bandwidth, MIMO layer, which is per-CC preference

- For MUSIM: CC(s) to release, SCG release preference, and reduced MIMO layer and bandwidth (per-CC preference) and reduced MIMO layer and bandwidth (Per-FS preference)

There seems some concern raised regarding the optional reaction from network perspective, upon reception of UAI. On the other hand, there seems no logical reason that network ignore such UAI report, given that ignorance would lead to failure / interruption in the end.

But it is worthwhile to re-screen the procedure to solve the concern expressed above, e.g., to compare the pros and cons between the delta-based (via UAI message reporting to revise the capability) method and directly allowing dynamic capability change during RRC_CONNECTED.

- During 5G RRC_CONNECTED, UE capability temporary down-grade can be performed via UAI procedure, for the reasons like overheating, and MUSIM reasons.

For 6G, it would be beneficial to re-screen the procedure, not only for the ignorance issue above, but also to check the impact due to new scenario to handle, e.g., foldable UE.

For foldable UE, the difference is that the impact is limited to MIMO layer (and it is not for sure to happen, if all antennas are deployed at a same side of the UE), but not related to the degradation of support CC/CG/Bandwidth.

Given this difference, one way to handle this rely on lite method, e.g., via L1 report, by reflecting that to be degraded MIMO performance, e.g., higher coherence level between antennas/channels.

Table 6 Comparison between different use cases of capability update

|  | Overheating | MUSIM | Foldable UE | Method for the report |
| --- | --- | --- | --- | --- |
| CC Number, CG Number | Y | Y | N | Capability report |
| Bandwidth | Y | Y | N | Capability report |
| MIMO layer | Y | Y | Y | Capability report + L1 report (RI, CSI) |

- For 6G UE capability reporting, foldable UE only relates to MIMO layer, which can be handled by either capability update or dynamic L1 report as coherence/channel-quality degradation.

Given all the aspects above, it is helpful to compare the pros/cons of different solutions

Table 7 Comparison between different capability update schemes

| Solution | Pros | Cons |
| --- | --- | --- |
| Capability updates only during RRC_IDLE (legacy solution) | No spec effort compared to 5G CN Impact is avoided | Interruption due to the RRC state transition, and lack of effective tools for UE triggered RRC state transition |
| Capability updates during RRC_CONNECTED, via reporting a delta-part/different-version (which is stored/forwarded at RAN side only) | No interruption due to RRC state transition CN Impact is avoided | But the ignorance of the report should be avoided |
| Capability updates during RRC_CONNECTED, via reporting a delta-part/different-version (which is stored/forwarded at RAN side and CN side) | No interruption due to RRC state transition Ignorance of the report can be avoided | CN Impact (relying on dynamic RAN-CN interaction during CM_ CONNECTED) |
| Capability updates during RRC_CONNECTED, by reporting multiple-versions upon initial registration, and indicate the applicable-version dynamically. | No interruption due to RRC state transition Ignorance of the report can be avoided | CN Impact (but no need for dynamic RAN-CN interaction during CM_ CONNECTED) There might be too many combinations to be pre-reported, and it is questionable whether the UE can predict all the degraded capability in advance. Even if it is feasible, it may further increase the capability report size upon initial registration, unnecessarily (since not all degraded capability would happen for sure) |
| Capability updates only during RRC_IDLE, + dynamic L1 report to reflect MIMO layer change | No spec effort compared to 5G CN Impact is avoided No interruption due to RRC state transition | Only applicable to cases with MIMO layer impact only (e.g., foldable UE) |

- For 6G UE capability reporting, R2 study the target use cases for capability update procedure during RRC_CONNECTED state, and taking 5G target scenarios as a starting point.

To achieve the “scalable and forward compatible design for diverse device types” as required in 6GR SID, a scalable feature system can be adopted for 6GR air interface, as shown in Figure 1. A low-cost and energy-efficient “Basic feature set” can be defined for 6GR air interface, which provides “global” mandatory feature for various device types. For a specific device type, a device-type specific feature set can be defined which “grows up” from the Basic feature set.

- Beyond the Basic feature set, device type specific feature sets can be defined for 6G MBB (Smart phone, Wearable IoT), low-end IoT and other verticals.

- The Basic feature set is a lean “global” mandatory feature list for all device types, where each feature in the list shall be well justified to be truly mandatory, and provides basic energy-efficient features for various device types.

- RAN109 discussed the feasibility of defining a mandatory feature set across device types.

The next question to be answered is whether a specific device type can be firstly identified which provides a template for the global mandatory feature set.

Low-end IoT is the lowest end device type in 6G on most of aspects, which is very suitable to provide the template for identifying the global mandatory feature set. And from our perspective, eMBB and low-end IoT are most important device types for the 1st release of 6G system. And each 6G MBB device are naturally a 6G low-end IoT device in “low-data rate” mode. For example, when a smartphone is in a “screen off” status or communicates with IoT devices, only the low-rate “background data” are received and transmitted, and only “IoT-like” traffic is conveyed for the smartphone. Therefore, the 6G low-end IoT provides a proper design template for 6G Basic feature set. Starting from the template, some features specially optimized for low-end IoT devices and not fit to other device types (e.g., extreme coverage enhancement for MTC sensors) can be cut off.

Therefore, a proper working approach is to firstly define the mandatory feature set for 6G low-end IoT, then use the low-end IoT feature set as a template to form the global mandatory feature set, where existing features unjustified for other candidate usage scenarios should be taken out, and necessary “outside” features could be added in.

- RAN109 discussed the requirement of smallest-maximum UE bandwidth and minimum system bandwidth allocation.

Figure 2. Low-end IoT provides a good template for defining Basic feature set

Proposal 6:	Use the low-end IoT feature set as a template/starting point for definition of the “global” mandatory feature set for 6GR.

	Features specially optimized for low-end IoT (e.g., stronger coverage enhancement) does not have to be included in the “global” mandatory feature set.

Based on R109 discussion, the follow conclusion was reached

Proposal 3: To investigate further: Motivations/justifications behind the proposed diverse device types, which should be a limited set Whether/how to have one or more device types for eMBB or 6G IoT Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. Whether/how to explicitly standardize device types Ensuring forward compatibility Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement.

While RP is to further study the related aspects, it would be helpful to speed up the progress at RAN2 level, by initiating some discussion regarding the two bullets above.

- RP has set the scope for further discussion on the diverse device types.

In our view, the two aspects above are coupled with each other, e.g., the benefit (if any) from specifying the device-type in specification.

The lesson we learned from 5G is: when 5G is standardized, it is always a long debate for UE-vendor, chipset-vendor, network-vendor and operator to decide whether/why a specific feature is mandatory or optional, which relates to all WGs and Plenary. Essentially, the difficulty comes from the fact that different capability may be required / not-required for different device-types. From network-vendor / operator perspective, it is safer to mandate one feature if it is required for at least one device-type, while from UE-vendor / chipset-vendor perspective, it is not preferred to mandate a feature if it is not for the concerned device-type.

- In 5G Day-1, the large number of UE capability leads to complexity to both vendors and operators, where one root reason is that different UE capability may be required / not-required for various UE types.

From this perspective, introducing the device-type would be very helpful to guide the mandatory / optional UE capability discussion in different WGs and Plenary. And this benefit comes without affecting signalling aspects, i.e., the capability bits (for IoDT bits and for optional capabilities) can be still reported. While the device-type specific conditionally-mandatory requirement (as adopted in legacy already) simply act as a reference for vendors and operators to decide on associated capability / features. So, there should be less impact to the capability report procedure.

- The impact of diverse-device-type on capability signalling is unclear.

- For 6G UE capability reporting, R2 trigger WG-level discussion (e.g., in terms of capability signaling framework impact, if any) upon RAN Plenary guidance.

Conclusion

Based on the discussion in section2, we have following proposals:

Proposal 1	For principles of 6GR RAN2 designs, consider new aspects, e.g., minimalist and essential design, eliminating redundant functions, enhanced security, native support for new emerging service, energy efficiency, on top of those which can be re-used from 5G NR principles. Endorse the following TP, as the principles/guidelines:

Proposal 2	R2 study 6G RAN architecture that is able to aggregate spectrum of 1) lower frequency which is to offer robust connection for coverage, together with 2) higher frequency which is to offer on-demand connection for capacity, covering both collocated and non-collocated scenarios.

Proposal 3	R2 study 6G spectrum aggregation architecture improvement on top of 4/5G design, e.g., in terms of 1) improved spectrum efficiency, 2) unified design of various aggregation solution(s), 3) dynamic load balance, and 4) improved link robustness. FFS on the applicable scenario (intra/inter-band, (non)collocated, RRC states and etc.)

Proposal 4	R2 study the AIML architecture/framework to consider: co-existing and interworking between AI and non-AI of a certain feature, LCM framework common to various use cases, and common data framework to be future proof.

Proposal 5	For 6G UE capability reporting, R2 study methods to reduce redundancy / overhead for SRB, covering but not limited to capability message(s).

Proposal 6	For 6G UE capability reporting, R2 study the target use cases for capability update procedure during RRC_CONNECTED state, and taking 5G target scenarios as a starting point.

Proposal 7	For 6G UE capability reporting, R2 trigger WG-level discussion (e.g., in terms of capability signaling framework impact, if any) upon RAN Plenary guidance.