3GPP TSG-RAN WG2 Meeting #131bis	R2-2506954

Prague, Czech Republic, Oct. 13th - 17th

Agenda Item:	8.4.3

Source: 	NEC

Title:  	Discussion on LP-WUS MAC remaining issue

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#131 meeting, the following agreement is made:

For Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s). Can discuss if critical issue identified with this mechanism.

In this paper, we further discuss this issue.

- 2 Discussion

- 2.1 Monitoring collision for option 1-2

The background of collision is from DCP case, if DCP monitoring occasion occurs in C-DRX active time, then there is no chance for the UE to monitor DCP, so that the UE does not know whether should to start drx-onDurationTimer or not, in order for the safety, RAN2 has agreed that if the UE is not able to monitor DCP occasion, then the UE should (blindly) start drx-onDurationTimer for PDCCH monitoring. And this mechanism has already been implemented to LP-WUS option 1-1 as shown below in TS 38.304:

Start of change

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else if LP-WUS monitoring is configured:

3>	if lpwus-PDCCH-MonitoringTimer is not configured:

4>	if LP-WUS indication associated with the current DRX cycle received from lower layer indicates to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

4>	if the UE is unable to monitor all LP-WUS monitoring occasion(s), as specified in TS 38.213 [6], due to conflicts with other activities (e.g. all LP-WUS monitoring occasion(s) are associated with the current DRX cycle occurred in Active Time of any DRX group considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last LP-WUS occasion, or during a measurement gap, or during a MUSIM gap or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4)):

5>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

End of change

During [Post130][213][LPWUS], companies discussed monitoring collision case for LP-WUS option 1-2, i.e., when the UE is not able to monitor the LP-WUS occasion(s), whether the UE should (blindly) start lpwus_PDCCHMonitoringTimer. After RAN2#131 meeting discussion, the final agreement is that “For Option 1-2, UE does not start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s). Can discuss if critical issue identified with this mechanism.”

After RAN2#131 meeting, the open issue related to this collision is proposed by company/rapporteur again as the below:

Proposal 1: Revisit the UE operation in LP-WUS collision for Option 1-2, which should be based on critical issues in the current RAN2 agreements. (based on contribution)

Our understanding is that for option 1-2, the UE can periodically monitor the LP-WUS occasion just based on the configured periodicity regardless of C-DRX configuration. The reason why companies do not support (blindly) starting lpwus_PDCCHMonitoringTimer is that normally LP-WUS periodicity for option 1-2 can be configured denser, so that missing some LOs is not a big deal.

However, it is not precise to say whether LP-WUS monitoring occasion is denser or sparser. On the one hand, if LP-WUS periodicity is configured the same as or close to long C-DRX cycle, in this case, there is no reason to not support this collision handling case since option 1-1 already supported it. On the other hand, even though the LP-WUS periodicity is configured denser than long C-DRX cycle, i.e., the LP-WUS periodicity is smaller than long C-DRX cycle, it still depends on the relationship between LP-WUS periodicity and the duration of lpwus_PDCCHMonitoringTimer, as long as the periodicity is larger than LP-WUS_PDCCHMonitoringTimer, this collision handling still works, for example, LP-WUS_PDCCHMonitoringTimer is 5ms while LP-WUS periodicity is 20ms, in this case, the UE can (blindly) start LP-WUS_PDCCHMonitoringTimer when the UE is not able to monitor the LP-WUS, this is just similar to option 1-1, no reason to not support the collision handling. But for the case where LP-WUS_PDCCHMonitoringTimer is 5ms while LP-WUS periodicity is also 5ms, there is no need to handle the collision case then.

Based on the analysis, we still think collision handling case for option 1-2 can be beneficial depending on NW configuration, therefore it can be up to NW control, i.e., NW configures UE whether to start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s).

Proposal-1: RAN2 to reconsider the collision case for option 1-2, NW configures UE whether to start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s).

- 3 Conclusions

Proposal-1: RAN2 to reconsider the collision case for option 1-2, NW configures UE whether to start the lpwus-PDCCH-MonitoringTimer in collision cases, i.e. when the UE is not able to monitor the LP-WUS occasion(s).

- 4 References