3GPP TSG-RAN WG2 Meeting #131bis	R2-2506988

Prague, Czech Republic, October 13-17, 2025

Source: 			Qualcomm Incorporated

Title: 	Considerations on UE capability signalling in 6G

Document for: 	Decision

Agenda Item: 	10.2

# Introduction

In this document, we present our initial thoughts on the UE capability signalling in 6G and try to identify the key discussion points.

# Discussion

  - UE capability signalling overhead

The explosion of UE capability signalling overhead is a well-known problem. RAN2 introduced RRC UL segmentation to allow the UE capability message of a large size to be delivered over the air. It should be noted however the bottleneck was not only the air interface. Later RAN2 identified limitations in the network interfaces and storage. To address the problem, RAN2 introduced the signalling solution where the network can request the UE to limit the number of RRC segments in sending UE capability message.

Proposal 1:	RAN2 to study solutions to reduce UE capability signalling overhead, considering the limitations of air interface, network interfaces and network storage.

  - UE-NR-Capability - breakdown

In order to identity the main contributors to signalling overhead increase, we analysed UE capability message reported by devices in the field. High level breakdown of UE-NR-Capability is shown in the figure below.

Figure-1:	High level breakdown of UE-NR-Capability

It should be noted that the number of supported band combinations increases as the total message size increases to the right in the figure. Some key observations can be made from this analysis.

Observation 1:	Band combination related UE capability parameters are the main contributor to UE capability size increase.

Observation 2:	The size of featureSetCombinations (merely containing pointers to feature sets) becomes prominent factor when the number of supported band combination increases.

Observation 3:	The Feature Set scheme serves the intended purpose to reuse the same set of UE capabilities across different band combinations.

Though not directly shown in the figure, some additional observations are made based on our analysis.

Observation 4:	Roughly the half of RF-parameters is baseband related UE capability parameters, which would be reuseable across band combinations.

Observation 5:	“Multi-dimensional” aspect of UE capabilities, such as CC BW and MIMO-layers, leads to the advertisement of a large number of feature sets within each featureSetCombination (corresponding to one featureSetCombinationId).

  - Dynamic UE capability changes in connected mode

In the current 5G NR standard, the UE can indicate the change of radio capabilities by means of NAS procedure in idle mode. In connected mode on the other hand, the UE can indicate its capability limitations for different use cases, i.e. Overheating, In-Device Coexistence, Power Saving and MUSIM, as summarized in the table below.

| Use Case | Release | UE Indications |
| --- | --- | --- |
| Overheating | Rel-15 | - Maximum bandwidth (FR1/FR2, UL/DL) - Maximum MIMO layers (FR1/FR2, UL/DL) - Maximum number of CCs (UL/DL) |
| In-Device Coex. | Rel-16 | - Affected frequencies - Affected frequency combinations. |
| Power Saving | Rel-16 | - Maximum bandwidth (FR1/FR2, UL/DL) - Maximum MIMO layers (FR1/FR2, UL/DL) - Maximum number of CCs (UL/DL) |
| Overheating (FR2-2) | Rel-17 | - Maximum bandwidth (UL/DL) - Maximum MIMO layers (UL/DL) |
| In-Device Coex. (FDM) | Rel-17 | - Affected frequency ranges. |
| Power Saving (FR2-2) | Rel-17 | - Maximum bandwidth (UL/DL) - Maximum MIMO layers (UL/DL) |
| MUSIM Temporary capability restrictions | Rel-18 | Per serving cell - Maximum bandwidth (UL/DL) - Maximum MIMO layers (UL/DL) Per affected band - Maximum bandwidth (UL/DL) - Maximum MIMO layers (UL/DL) - Maximum number of CCs (FR1/FR2/FR2-2, UL/DL) |

As can be seen, there are many overlaps among the use cases on the UE capability limitations that can be communicated to the network. For 6G, we believe it makes sense to introduce more generic solution allowing the UE to update its radio capabilities in connected mode that is flexible enough to cover all the use cases addressed so far and future use cases.

Proposal 2:	RAN2 to study more generic solution allowing the UE to update its radio capabilities within connected mode.

  - IODT availability vs inter-operability

The UE indicates its support for a given feature only if the feature is sufficiently verified through IODT. This applies to both optional features with UE capability signalling and mandatory features with UE capability signalling. There are however shortcomings with this approach, according to our field experiences.

- The UE has to wait for the IODT with the “slowest” network vendor before activating a capability in operator’s network.

- Even after activating a capability, the UE may encounter inter-operability issues when a new version of network equipment or network equipment of a new vendor is deployed into operator’s network.

Proposal 3:	RAN2 to study solutions allowing the UE to activate a feature capability after IODT with a given set of network vendor, while avoiding inter-operability problems due to the lack of IODT with other network vendors.

  - RACS

RACS (Radio Capabilities Signalling Optimizations) was introduced in Rel-16. The overall goal was to reduce the signalling over Uu and network interfaces.​ There are two variants in RACS.

- Manufacturer assigned UE Radio Capability ID​

- PLMN assigned UE Radio Capability ID

We believe RACS (and its potential enhancements) is a promising day-one 6G feature to address many shortcomings of UE capability signalling today, for example;

- No OTA signalling of UE capability is necessary in case of Manufacturer-assigned UE Radio Capability ID.

- “Full” UE capability can be stored in UCMF (UE Capability Management Function), which can avoid OTA re-enquiry of UE capability upon mobility across RAN nodes of different network vendors.

- UE capability filtering can be applied within the network to reduce the size of UE capability delivered to RAN node, without disclosing the capability of or features supported by the RAN node in the form of UE capability filters.

We suggest RAN2 to work with SA2/CT1 to study RACS and its enhancements.

Proposal 4:	RAN2 to study RACS and its enhancements in coordination with SA2/CT1.

# Conclusion

In this document, we identified the following discussion points for further study.

Proposal 1:	RAN2 to study solutions to reduce UE capability signalling overhead, considering the limitations of air interface, network interfaces and network storage.

Proposal 2:	RAN2 to study more generic solution allowing the UE to update its radio capabilities within connected mode.

Proposal 3:	RAN2 to study solutions allowing the UE to activate a feature capability after IODT with a given set of network vendor, while avoiding inter-operability problems due to the lack of IODT with other network vendors.

Proposal 4:	RAN2 to study RACS and its enhancements in coordination with SA2/CT1.

The following observations were made to assist further study.

Observation 1:	Band combination related UE capability parameters are the main contributor to UE capability size increase.

Observation 2:	The size of featureSetCombinations (merely containing pointers to feature sets) becomes prominent factor when the number of supported band combination increases.

Observation 3:	The Feature Set scheme serves the intended purpose to reuse the same set of UE capabilities across different band combinations.

Observation 4:	Roughly the half of RF-parameters is baseband related UE capability parameters, which would be reuseable across band combinations.

Observation 5:	“Multi-dimensional” aspect of UE capabilities, such as CC BW and MIMO-layers, leads to the advertisement of a large number of feature sets within each featureSetCombination (corresponding to one featureSetCombinationId).