3GPP TSG RAN WG 2 # 131-bis										R2-2506989

Prague, Czech Republic, October 13th-17th, 2025

Agenda Item:	8.8.2

Source:	CSCN

Title:	Discussion on the remaining issue of Downlink Coverage Enhancements

Document for:	Discussion

- Introduction

In RAN2#131, some agreements on downlink coverage enhancement have been achieved as follows [1]:

Agreements: - RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only. Agreements: 1. RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements 2 In connected mode the NW will only configure up to 4 STMC. 3. UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located 4. The procedure for UE assisted SMTC configuration in connected mode is follows: - For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused. - The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations. - The UE can report the N closest reference locations via the RRCReconfigurationComplete message. - The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy). - The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

Some remaining issues are listed as follows:

- Issue#1: whether to support an SSB-index based SMTC selection in IDLE mode;

- Issue#2: whether distance threshold configuration is required for UE assisted SMTC configuration in connected mode;

- Issue#3: whether multiple SMTC periodicities are needed for both intra-frequency neighbour cells and inter-frequency neighbour cells.

In this contribution, we further discuss the remaining issues based on the agreements and discussions [1] from the RAN2#131 meeting.

- Discussion

## Issue#1: whether to support an SSB-index based SMTC selection in RRC_Idle/Inactive mode

During the RAN2#130 meeting, RAN2 agreed to introduce a location-based SMTC selection procedure, where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. A remaining open issue is whether to support an SSB-index based SMTC selection in RRC_Idle/Inactive mode.

The location-based SMTC selection procedure requires UEs to support GNSS and to enable GNSS measurement in idle mode, which increases the power consumption and complexity of UE. Meanwhile, this solution is highly dependent on the availability of GNSS. Considering the GNSS information may be temporarily unavailable, UEs may fail to determine its distance to the neighbour cell.

The location-based SMTC selection is highly dependent on the availability of GNSS.

We introduce an SSB-index based SMTC selection procedure where each SMTC will be associated with an SSB index list of the serving cell. Even if the UE may be at the edge of a beam or the overlapping area of beams, the UE always selects one beam to camp on, depending on the UE implementation as legacy. Once the beam that UE camps on in the serving cell is determined, the UE can use this index to select the SMTCs simultaneously. While the UE moves, it can select new SMTCs corresponding to the new SSB index it camps on. Companies may also be concerned that the mapping between SSB index and SMTC may change frequently. From our point of view, first, NW operator has the responsibility to avoid frequent changes in network topology and connection relationships between the serving cell and neighbour cell(s). Otherwise, it will lead to frequent changes in the neighboring cell(s), and the NW needs to update the neighbour cell information frequently with SIB update. Second, during the time of satellite coverage (i.e. within t-Service), the relationships between the serving cell and neighboring cells are predictable and stable, which means the NW can configure a consistent mapping between SSB index and SMTC for the UEs within the serving cell. Compared to the location-based SMTC selection, SSB-index based SMTC selection is more direct and does not rely on the availability of GNSS. The benefit of SSB-based solution is that it can minimize the impact on UE.

Due to the fact that both one single beam per cell and multiple beams per cell can be considered in network deployment, we need to analyze whether SSB-index based SMTC selection can work in both deployments. For one single beam per cell, network can configure no more than 4 SMTCs based on implementation. However, for multiple beams per cell, SSB-index based SMTC selection can be applied for UE measurements.

Once the UE camps on a beam, it can select the SMTCs based on the association between the SMTCs and the index of the SSB on which it camps in the serving cell.

To support SMTC enhancements on DL CE, UE can select the SMTC based on the association between the SMTC and the index of the SSB UE camps on in the serving cell. Even when considering the maximum number of SSB index in FR2, at most 64 additional bits are required for indicating the SSB index for each SMTC.

For SSB-index based SMTC selection, a list of SSBs containing multiple SSBs configured for each SMTC is feasible for SSB-index based solution.

RAN2 to introduce SSB-based solution in IDLE/Inactive mode. UE can select the SMTC based on the association between the SMTC and the index of the SSB UE camps on in the serving cell.

## Issue#2: whether distance threshold configuration is required for UE assisted SMTC configuration in connected mode

During the last meeting, RAN2 introduced the procedure for UE assisted SMTC configuration in connected mode. For a UE indicating the relevant capability, the network configures the UE via RRCReconfiguration message with a list of reference locations and a parameter N. The UE will report the N closest reference locations to the network. A remaining issue is whether the network needs to configure distance thresholds for each reference location.

From our perspective, only configuring the reference locations and parameter N rise two potential issues. The first issue is that the reference location is insufficient to capture the distribution of neighbour cells with different cell radius. As Figure 1 shows, neighbour cell 1 (NCell1) and neighbour cell 2 (NCell2) have different cell radius. Using the reference location as a basis, the distance between the UE and NCell1 (defined as L1) is computed to be less than the distance between the UE and NCell2 (defined as L2). UE will report this information to the network which may lead the network to determine the UE is closer to NCell1. Nevertheless, as depicted in Figure 1, the UE is actually closer to NCell2 when considering the cell coverage with radius.

Figure 1: Neighbour cells with different radius

Relying solely on the reference location alone is insufficient to reflect the distribution of neighbour cells with different cell radius.

The second potential issue is reference locations configured by the network may not match the UE's current position, especially when the UE is moving at high speed (e.g., airplane). As Figure 2 shows, all the reference locations configured by the network are far from the UE. According to the current agreements, the UE will report N closest reference locations anyway even though the UE is very far from the configured reference location. However, in this case, there is no need to monitor the SSB according to the SMTC configuration based on the reference location.

Figure 2: Improper reference location configuration

The reference locations configured by the network may not match the UE's current position, which resulting in unnecessary SMTC configuration.

To address these two potential issues, we recommend the network configure distance thresholds for each reference location. For the first potential issue, distance thresholds can accurately describe the cell coverage, thereby resolving the issue. For the second potential issue, the UE can determine whether it is within the cell coverage defined by the reference location and distance threshold. If the UE is not within any the cell coverage, then the UE can indicate the network by reporting void or NULL in referenceLocationReport-r19 via RRCReconfiguration or UAI message, and the network won’t configure the SMTC until the update of new reporting.

A distance threshold configuration is required for UE assisted SMTC configuration in connected mode.

If the UE is not within any the cell coverage, then the UE can indicate the network by reporting NULL in referenceLocationReport-r19 via RRCReconfiguration or UAI message, and the network won’t configure the SMTC until the update of new reporting.

## Issue#3: whether multiple SMTC periodicities are needed for both intra-frequency neighbour cells and inter-frequency neighbour cells.

During the last meeting, RAN2 supported to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode. A remaining open issue is whether multiple SMTC periodicities are needed for both intra-frequency neighbour cells and inter-frequency neighbour cells.

Regarding the inter-frequency scenario, there are two main cases: inter-frequency contiguous coverage (case 1) and overlapping coverage (case 2) shown in Figure 3.

Figure 3: two cases of inter-frequency scenario

For idle/inactive mode, UE performs measurements based on the parameters configured by the network in the system information. It is up to network configuration for UE to camp on a cell. Configuring multiple SMTC periodicities for inter-frequency neighbour cells has no impact on the UE measurement.

For connected mode, the UE needs to report the selected N closest reference locations to assist the network configure up to 4 STMC. In our view, the extensibility of multiple SMTCs to inter-frequency depends on whether the network can correctly configure up to 4 SMTCs in connected mode. So this issue is also related to the assistance information configuration in connected mode for inter-frequency. Considering the current reference location may not describe the cell coverage adequately, then the network may not configure the SMTCs appropriately especially in inter-frequency with overlapping scenario. In summary, if distance threshold is also introduced in the assistant information , then multiple SMTCs can be supported for inter-frequency.

If distance threshold is introduced for UE assisted SMTC configuration in connected mode, multiple SMTC periodicities can be supported for inter-frequency.

- Conclusion

Observation 1: The location-based SMTC selection is highly dependent on the availability of GNSS.

Observation 2: Once the UE camps on a beam, it can select the SMTCs based on the association between the SMTCs and the index of the SSB on which it camps in the serving cell.

Observation 3: For SSB-index based SMTC selection, a list of SSBs containing multiple SSBs configured for each SMTC is feasible for SSB-index based solution.

Observation 4: Relying solely on the reference location alone is insufficient to reflect the distribution of neighbour cells with different cell radius.

Observation 5: The reference locations configured by the network may not match the UE's current position, which resaulting in unneccesary SMTC configuration.

Proposal 1: RAN2 to introduce SSB-based solution in IDLE/Inactive mode. UE can select the SMTC based on the association between the SMTC and the index of the SSB UE camps on in the serving cell.

Proposal 2: A distance threshold configuration is required for UE assisted SMTC configuration in connected mode.

Proposal 3: If the UE is not within any the cell coverage, then the UE can indicate the network by reporting void or NULL in referenceLocationReport-r19 via RRCReconfiguration or UAI message, and the network won’t configure the SMTC until the update of new reporting.

Proposal 4: If distance threshold is introduced for UE assisted SMTC configuration in connected mode, multiple SMTC periodicities can be supported for inter-frequency.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #131