3GPP TSG-RAN WG2 Meeting #131bis		R2-2507011

Prague, Czech Republic, 13th – 17th Oct., 2025

Source:	vivo

Title:	Discussion on other open issues for LP-WUS WUR

Agenda Item:	8.4.4

Document for:	Discussion and Decision

- Introduction

In the email discussion on [Post131][211] [LP-WUS] Running CR for 38.304 (CATT), the following open issues are identified:

38304-1: Discuss whether “low mobility” criterion based on LR measurements needs to be introduced for LP-WUS.

38304-3: FFS the impact on legacy low-mobility criteria for MR when the UE exits serving cell measurement offloading case.

- Discussion

  - (38304-1) Low mobility criterion based on LR

During the post meeting email discussion of open issues on Running CR for 38.304, the “low mobility” criterion based on LR measurements for LP-WUS was raised.

38304-1: Discuss whether “low mobility” criterion based on LR measurements needs to be introduced for LP-WUS.

Firstly, we want to point that it is agreed there is no low mobility criterion for Rel-19 LP-WUS RRM relaxation/offloading, thus we think this issue is about low mobility for R16 RRM relaxation for UE supporting LP-WUS, which has also been extensively discussed and RAN2 have reached a conclusion in last meeting [1].

LR measurement is not used for R16/R17 RRM relaxation condition.

On the other hand, if we agree to use the LR measurement for R16/R17 RRM relaxation, it will introduce a large amount of RAN4 work, which may lead a long discussion, e.g., the UE behavior when only one condition is configured, or when two conditions are configured but only one condition is fulfilled, etc. Anyway, since current criteria for RRM relaxation/offloading includes the LR measurements, UE could evaluate whether it is in cell edge quickly, even UE moves fast there is no issue that it could perform RRM relaxation/offloading when it is not in cell edge.

Proposal 1: (38304-1) “low mobility” criterion based on LR measurements is not needed for LP-WUS.

  - (38304-3) FFS the impact on legacy low-mobility criteria for MR with LP-WUS

Regarding the open issue:

38304-3: FFS the impact on legacy low-mobility criteria for MR when the UE exits serving cell measurement offloading case.

The low-mobility criteria for R16 relaxation are captured in TS 38.304 [2]:

5.2.4.9.1 Relaxed measurement criterion for UE with low mobility The relaxed measurement criterion for UE with low mobility is fulfilled when: - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows: - After selecting or reselecting a new cell, or - If (Srxlev - SrxlevRef) > 0, or - If the relaxed measurement criterion has not been met for TSearchDeltaP: - The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

In our understanding, with the highlight wording restriction, the UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell after exits offloading and the condition “If the relaxed measurement criterion has not been met for TSearchDeltaP” is fulfilled. Obviously, the current specification is sufficient, and there is no room to optimize, so there is no specification impact for R16 low mobility criterion when UE exits fully offloading.

Proposal 2: (38304-3) There is no specification impact for R16 low mobility criterion when UE exits fully offloading.

- Conclusion

In this contribution, we discuss the open issues identified for Rel-19 LPWUS 38.304 Running CR. Based on the discussion, we have the following proposals.

(38304-1) Neighboring cell measurement relaxation criterion

Proposal 1: (38304-1) “low mobility” criterion based on LR measurements is not needed for LP-WUS.

(38304-3) FFS the impact on legacy low-mobility criteria for MR with LP-WUS

Proposal 2: (38304-3) There is no specification impact for R16 low mobility criterion when UE exits fully offloading.

- References

- RAN2#131 Chair minutes

- TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state