3GPP TSG-RAN WG2 Meeting #131bis		R2-2507012

Prague, Czech Republic, 13th – 17th Oct., 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2123 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on MAC for Mob Ph4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TBD |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TBD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TBD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1xxxx |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

Next change

5.12	MAC Reset

If a reset of the MAC entity is requested by upper layers upon receiving RRCResume or RRCSetup, the MAC entity shall:

1>	stop the MBS multicast DRX timers;

1>	flush the soft buffers for all DL HARQ processes used for MBS multicast;

1>	for each DL HARQ process used for MBS multicast, consider the next received transmission for a TB as the very first transmission.

Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall:

1>	if the MAC reset is not due to SCG deactivation:

2>	initialize Bj for each logical channel to zero;

1>	initialize SBj for each logical channel to zero if Sidelink resource allocation mode 1 is configured by RRC;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is configured for the deactivated SCG:

2>	stop (if running) all timers except beamFailureDetectionTimer associated with PSCell and timeAlignmentTimers.

1>	else if upper layers indicate the reset is triggered by reconfiguration with sync:

2>	stop (if running) all timers, except MBS broadcast DRX timers, ltm-Candidate-TimeAlignmentTimer, and ltm-Candidate-TimeAlignmentTimerTAG2, if configured;

2>	consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2;

1>	else:

2>	stop (if running) all timers, except MBS broadcast DRX timers;

2>	consider all timeAlignmentTimers, inactivePosSRS-TimeAlignmentTimer, and cg-SDT-TimeAlignmentTimer, if configured, as expired and perform the corresponding actions in clause 5.2;

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	sets the NDIs for all HARQ process IDs to the value 0 for monitoring PDCCH in Sidelink resource allocation mode 1;

1>	stop, if any, ongoing Random Access procedure;

1>	discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any, except the contention-free Random Access Resources signalled in the LTM Cell Switch Command;

1>	flush Msg3 buffer;

1>	flush MSGA buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Delay Status Reporting procedure;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	cancel, if any, triggered consistent LBT failure;

1>	cancel, if any, triggered Sidelink consistent LBT failure;

1>	cancel, if any, triggered BFR;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure;

1>	cancel, if any, triggered Pre-emptive Buffer Status Reporting procedure;

1>	cancel, if any, triggered Timing Advance Reporting procedure;

1>	cancel, if any, triggered Recommended bit rate query procedure;

1>	cancel, if any, triggered Configured uplink grant confirmation;

1>	cancel, if any, triggered configured sidelink grant confirmation;

1>	clear, if any, configured sidelink grants;

1>	cancel, if any, triggered Desired Guard Symbol query;

1>	cancel, if any, triggered Positioning Measurement Gap Activation/Deactivation Request procedure;

1>	cancel, if any, triggered SDT procedure;

1>	cancel, if any, triggered IAB-MT Recommended Beam Indication query;

1>	cancel, if any, triggered Desired DL TX Power Adjustment query;

1>	cancel, if any, triggered Desired IAB-MT PSD range query;

1>	cancel, if any, triggered Case-6 Timing Request query;

1>	cancel, if any, triggered SL-PRS resource request;

1>	cancel, if any, triggered Event Triggered L1 Measurement Report;

1>	flush the soft buffers for all DL HARQ processes, except for the DL HARQ process being used for MBS broadcast;

1>	for each DL HARQ process, except for the DL HARQ process being used for MBS broadcast, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI;

1>	clear, if any, Differential Koffset;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is not configured; or

1>	if the MAC reset is not due to SCG deactivation:

2>	reset all BFI_COUNTERs;

1>	reset all LBT_COUNTERs;

1>	reset TTT for event triggered L1 measurement report triggering condition evaluation;

1>	reset all MR_SENT_COUNTER;

1>	clear all BEAM_ENTERING_LIST;

1>	clear all BEAM_LEAVING_LIST;

1>	clear all BEAM_REPORTED_LIST;

1>	clear all MR_LIST.

If a Sidelink specific reset of the MAC entity is requested for a PC5-RRC connection by upper layers, the MAC entity shall:

1>	flush the soft buffers for all Sidelink processes for all TB(s) associated to the PC5-RRC connection;

1>	consider all Sidelink processes for all TB(s) associated to the PC5-RRC connection as unoccupied;

1>	cancel, if any, triggered Scheduling Request procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink CSI Reporting procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink DRX Command Indication procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink IUC-Request transmission procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink IUC-Information Reporting procedure associated to the PC5-RRC connection;

1>	stop (if running) all timers associated to the PC5-RRC connection;

1>	reset the numConsecutiveDTX associated to the PC5-RRC connection;

1>	initialize SBj for each logical channel associated to the PC5-RRC connection to zero.

Next change

5.x	L1 measurement and event triggered report

5.x.1	Introduction

The network may configure an RRC_CONNECTED UE to perform L1 beam level measurements for LTM candidate cell(s) and/or serving cell. The network may configure the UE to report them in accordance with the event triggered L1 measurement configuration. The measurement report is used for indicating to serving gNB of the L1 measurement results from the serving cell and/or candidate cell(s). The measurement configuration is provided by means of RRC dedicated signalling.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	SS/PBCH block(s) resource indicator (SSBRI).

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	CSI-RS resource indicator (CRI).

The RRC configures the following parameters in the LTM-CSI-ReportConfig for event triggered L1 measurement and corresponding reporting procedure:

-	LTM-CSI-ReportConfig for the event-triggered measurement report;

-	eventTriggered for the event-triggered measurement report;

-	eventLTM2, eventLTM3, eventLTM4, eventLTM5: events for the event-triggered measurement report;

-	timeToTrigger: time during which an entering/leaving condition needs to be consistently satisfied for reporting event triggered L1 measurement report or for cell switch execution to be met;

-	ltm-CandidateReportConfigList: List of report configurations for LTM candidate IDs;

-	ltm-EventTriggeredPeriodicReport: whether the event triggered L1 measurement report is sent periodically if an LTM event is triggered;

-	reportOnLeave: whether the event triggered L1 measurement report shall be triggered when the leaving condition for an event is satisfied;

-	ltm-EventTriggeredReportReportContent: the content of the event triggered L1 measurement report.

5.x.2	Performing measurement

An RRC_CONNECTED UE obtains L1 beam level measurement results by measuring one or multiple RSs as configured by the network as specified in TS 38.214 for the LTM candidate cell(s) with the candidate ID configured in ltm-CandidateReportConfigList for evaluation of reporting criteria or of execution condition. For each L1 beam level measurement result in RRC_CONNECTED, the UE applies the layer 1 filtering by implementation, before using the measured results for evaluation of reporting criteria and measurement reporting or of execution condition. When the UE has two indicated TCI-states, the UE uses the best beam of serving cell is used for LTM event evaluation. It is up to the UE implementation how to choose the best beam. The MAC entity performs the evaluation of reporting criteria as specified in 5.x.3 or of execution condition as specified in 5.y.2 based on the L1 measurement results from lower layer. For the LTM candidate cell(s) with the candidate ID not configured in ltm-CandidateReportConfigList, the UE is not required to perform and the event evaluation in 5.x.3 on the RSs belonging to the candidate ID for the corresponding event.

For L1 beam level event triggered measurements report, the network can configure SS/PBCH block(s) or CSI-RS as event evaluation RS type, and L1-RSRP as trigger quantity. Reporting quantity is the same as the trigger quantity.

5.x.3	Measurement report triggering

5.x.3.1	General

The UE maintains the following UE variables for event triggered L1 measurement and report procedure:

-	MR_LIST: includes the list of event triggered L1 measurement report information, including RS resource index of LTM candidate cell(s), L1 measurement result, and type of reporting RS(s) as defined in 6.1.3.x, for which the L1 measurement report triggering conditions have been met for TTT. Each entry in the list is associated with a ltm-CSI-ReportConfigId;

-	MR_SENT_COUNTER: represents the number of event triggered L1 measurement report performed by UE if the triggering condition for the corresponding event is met for TTT for each ltm-CSI-ReportConfigId;

-	BEAM_ENTERING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report entering conditions have been met for TTT for the triggered L1 measurement report;

-	BEAM_LEAVING_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have been met for TTT for the triggered L1 measurement report;

-	BEAM_REPORTED_LIST: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which has been reported in the (Truncated) L1 measurement report MAC CE, and the L1 measurement report leaving conditions have not been met for TTT.

The RRC configures the following parameters in the LTM-CSI-ReportConfig for event triggered L1 measurement and corresponding reporting procedure:

-	candidateSpecificOffset: offset for event condition that is applicable for all the reference signals belonging to the candidate cell with the candidate cell ID ltm-CandidateReportConfigId;

-	candidateSpecificOffsetS: offset for event condition that is applicable for all the reference signals belonging to the serving cell with the candidate cell ID ltm-CandidateReportConfigId.

Unless explicitly specified otherwise, it is up to UE implementation how to store these variables.

The MAC entity shall for LTM event evaluation procedure:

1>	for each ltm-CSI-ReportConfigId included in the serving cell:

2>	if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration:

3>		if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId:

4> consider only the current beam of serving cell, i.e. the beam corresponds to the RS configured in the indicated TCI State or the RS QCLed with the RS configured in the indicated TCI State indicated by TCI State in the serving cell as defined in clause 5.1.5 in TS 38.214, with the same RS type as the beam of LTM candidate cell, i.e. the RSs configured in LTM-CSI-ResourceConfig which associated with this ltm-CSI-ReportConfigId, to be applicable;

3>		if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId:

4>	if ltm-CandidateReportConfigList is configured:

5> consider any beam of LTM candidate cell (except the serving cell) configured in ltm-CandidateReportConfigList, i.e. the RSs configured in LTM-CSI-ResourceConfig which is associated with this ltm-CSI-ReportConfigId, to be applicable;

4>	else:

5> consider any beam of LTM candidate cell (except the serving cell), i.e. the RSs configured in LTM-CSI-ResourceConfig which associated with this ltm-CSI-ReportConfigId, to be applicable;

2>	if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during TTT defined for this event:

3>	if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event):

4>	include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId;

3> if the beam is not in BEAM_LEAVING_LIST:

4>	include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> if the beam is in BEAM_LEAVING_LIST:

4> remove the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

4> if the SSBRI or CRI of the concerned beam(s) has been removed from BEAM_REPORTING_LIST:

5> include the SSBRI or CRI of the concerned beam(s) in the BEAM_REPORTING_LIST for this ltm-CSI-ReportConfigId;

3>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.x.4.

2>	else if the leaving condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams included in the BEAM_ENTERING_LIST or BEAM_REPORTED_LIST for the measurement from lower layer during TTT defined for this event:

3> if the beam is in BEAM_ENTERING_LIST:

4> remove the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> if the beam is in BEAM_REPORTED_LIST:

4> remove the concerned beam(s) in the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

4> include the SSBRI or CRI of the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

3>	if reportOnLeave is set to true for this ltm-CSI-ReportConfigId:

4>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.x.4;

2>	upon expiry of the periodical reporting timer for this ltm-CSI-ReportConfigId:

3>	consider L1 measurement reporting to be triggered for the ltm-CSI-ReportConfigId, as specified in 5.x.4.

NOTE x:	TTT is not restarted if the current beam of serving cell changes and the entry condition is still met with the new current beam.

NOTE y:	To evaluate the L1 measurement reporting triggering event, the UE uses the latest L1-RSRP measurement from lower layer.

5.x.3.2	Event LTM2 (Beam of serving cell becomes worse than threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition LTM2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition LTM2-2, as specified below, is fulfilled;

Inequality LTM2-1 (Entering condition)

Ms + Hys < Thresh

Inequality LTM2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the beam measurement quantity of the serving cell based on SS/PBCH block or CSI-RS, not taking into account any offsets. The beam associated with this event is the current beam, i.e. corresponding to the RS configured in the indicated TCI state or the RS QCLed with the RS configured in the indicated TCI state in the serving cell as defined in clause 5.1.5 in TS 38.214, with the same RS type as the beam of LTM candidate cell, i.e. the RSs configured in LTM-CSI-ResourceConfig.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within LTM-CSI-ReportConfig for this event).

Thresh is the threshold parameter for this event (i.e. ltm2-Threshold as defined within LTM-CSI-ReportConfig for this event).

Ms is expressed in dBm in case of RSRP.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

5.x.3.3	Event LTM3 (Beam of candidate cell becomes offset better than beam of serving cell)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition LTM3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition LTM3-2, as specified below, is fulfilled;

Inequality LTM3-1 (Entering condition)

Mn + Obn – Hys > Ms + Obs + Off

Inequality LTM3-2 (Leaving condition)

Mn + Obn + Hys < Ms + Obs + Off

The variables in the formula are defined as follows:

Mn is the beam measurement quantity of the LTM candidate cell based on SS/PBCH block or CSI-RS, not taking into account any offsets.

Obn is the offset of the LTM candidate cell (i.e. candidateSpecificOffset as defined in LTM-CSI-ReportConfig for this event). One offset is applied to all beam(s) associated with the LTM candidate cell.

Ms is the beam measurement quantity of the serving cell based on SS/PBCH block or CSI-RS, not taking into account any offsets. The beam associated with this event is the current beam, i.e corresponding to the RS configured in the indicated TCI state or the RS QCLed with the RS configured in the indicated TCI State in the serving cell as defined in clause 5.1.5 in TS 38.214 in the serving cell.

Obs is the offset of the beam of the serving cell (i.e. candidateSpecificOffsetS as defined in LTM-CSI-ReportConfig for this event).

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within LTM-CSI-ReportConfig for this event).

Off is the offset parameter for this event (i.e. ltm3-Offset as defined within LTM-CSI-ReportConfig for this event).

Mn, Ms are expressed in dBm in case of RSRP.

Obn, Obs, Hys, Off are expressed in dB.

5.x.3.4	Event LTM4 (Beam of candidate cell becomes better than absolute threshold)

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition LTM4-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition LTM4-2, as specified below, is fulfilled.

Inequality LTM4-1 (Entering condition)

Mn + Obn – Hys > Thresh

Inequality LTM4-2 (Leaving condition)

Mn + Obn + Hys < Thresh

The variables in the formula are defined as follows:

Mn is the beam measurement quantity of the LTM candidate cell based on SS/PBCH block or CSI-RS, not taking into account any offsets.

Obn is the offset of the beam of the LTM candidate cell (i.e. candidateSpecificOffset as defined in LTM-CSI-ReportConfig for this event). One offset is applied to all beam(s) associated with the LTM candidate cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within LTM-CSI-ReportConfig for this event).

Off is the offset parameter for this event (i.e. ltm4-Offset as defined within LTM-CSI-ReportConfig for this event).

Mn is expressed in dBm in case of RSRP.

Obn, Hys are expressed in dB.

Thresh is expressed in the same unit as Mn.

5.x.3.5	Event LTM5 (Beam of serving cell becomes worse than threshold1 and Beam of candidate cell becomes better than threshold2)

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition LTM5-1 and condition LTM5-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition LTM5-3 or condition LTM5-4, i.e. at least one of the two, as specified below, is fulfilled;

Inequality LTM5-1 (Entering condition 1)

Ms + Hys < Thresh1

Inequality LTM5-2 (Entering condition 2)

Mn + Obn – Hys > Thresh2

Inequality LTM5-3 (Leaving condition 1)

Ms – Hys > Thresh1

Inequality LTM5-4 (Leaving condition 2)

Mn + Obn + Hys < Thresh2

The variables in the formula are defined as follows:

Ms is the beam measurement quantity of the serving cell based on SS/PBCH block or CSI-RS, not taking into account any offsets. The beam associated with this event is the current beam, i.e corresponding to the RS configured in the indicated TCI state or the RS QCLed with the RS configured in the indicated TCI State in the serving cell, as defined in clause 5.1.5 in TS 38.214.

Mn is the beam measurement quantity of the LTM candidate cell based on SS/PBCH block or CSI-RS, not taking into account any offsets.

Obn is the offset of the LTM candidate cell (i.e. candidateSpecificOffset as defined in LTM-CSI-ReportConfig for this event). One offset is applied to all beam(s) associated with the LTM candidate cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within LTM-CSI-ReportConfig for this event).

Thresh1 is the threshold parameter for this event (i.e. ltm5-Threshold1 as defined within LTM-CSI-ReportConfig for this event).

Thresh2 is the threshold parameter for this event (i.e. ltm5-Threshold2 as defined within LTM-CSI-ReportConfig for this event).

Mn, Ms are expressed in dBm in case of RSRP.

Obn, Hys are expressed in dB.

Thresh1 is expressed in the same unit as Ms.

Thresh2 is expressed in the same unit as Mn.

5.x.4	Measurement report

The purpose of this procedure is to transfer L1 measurement results from the UE to the network.

RRC controls the event triggered L1 beam level measurement reporting by configuring the following parameter:

-	reportInterval: the periodicity of the event-triggered periodic measurement report;

-	reportAmount: number of measurement reports needs to be transmitted after the event is triggered;

-	maxNumberOfReportedBeams: number of beams whose measurements can be reported in the event triggered L1 measurement report by MAC CE regardless whether or not the report includes the current beam;

-	allowReportAnyBeam: whether the UE can report the measurement results for the beams not satisfying the conditions of the events;

-	reportCurrentBeam: whether the UE is required to report the measurement result of the current beam;

-	ltm-CandidateReportConfigId: LTM candidate cell ID for which the UE is required to measure reference signal and perform LTM event evaluation;

-	reportQuantity: the report quantity for the CSI report.

For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the serving cell, the MAC entity shall:

1>	if at least one L1 measurement report has been triggered for the ltm-CSI-ReportConfigId as specified in 5.x.3 and not cancelled:

2>	if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as befined in clause 6.1.3.x according to the measurement report information in the MR_LIST;

3> if reportAmount is configured in the LTM-EventTriggeredPeriodicReport by RRC:

4> if at least one L1 measurement report associated with the ltm-CSI-ReportConfigId has been triggered due to the reason other than the expiry of the periodical reporting timer and the L1 measurement report is not cancelled:

5>	set the MR_SENT_COUNTER to 0 for this ltm-CSI-ReportConfigId;

4> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId by 1;

4> stop the periodical reporting timer, if running;

4> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount:

5> restart the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig;

3> include the SSBRI or CRI in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId, if any, into the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId;

3> clear the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId;

3> clear the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId;

3>	if the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId is empty:

4>	remove the measurement reporting entry within the MR_LIST for this ltm-CSI-ReportConfigId;

4>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running;

3> cancel the triggered L1 measurement report;

2>	else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST by selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists;

2> else:

3>	if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured:

4>	trigger the SR using the dedicated SR configuration for L1 measurement report;

3>	else:

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR;

3>	stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running.

NOTE X:	After sending a Truncated L1 measurement report MAC CE, if the subsequent UL grant is still not big enough to fit all the remaining beams, it is up to UE implementation to select the beam(s) which were not included in the previous MAC CE.

NOTE Y: If more than one triggering events for L1 measurement report are pending and the UL grant is not sufficient for all regular L1 measurement report MAC CEs, it is up to UE implementation how to handle/include the L1 measurement report MAC CEs and/or Truncated L1 measurement report MAC CE.

NOTE Z: When a measurement report is triggered by entry condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, another measurement report is triggered by leaving condition for the same RS(s), all the corresponding measurement reports are cancelled. When a measurement report is triggered by leaving condition for one or more RS(s), and included in the BEAM_LEAVING_LIST, another measurement report is triggered by entry condition for the same RS(s), all the corresponding measurement reports are cancelled.

Next change

6.1.3.x	Event Triggered L1 Measurement Report MAC CE

Event triggered L1 measurement report MAC CE consists of either:

-	event triggered L1 measurement report format (variable size); or

-	truncated event triggered L1 measurement report format (variable size).

The event triggered L1 measurement report formats are identified by MAC subheaders with an eLCIDs as specified in Table 6.2.1-2b.

For a truncated event triggered L1 measurement report MAC CE, at least the following fields should be included: report ID field, at least one triggered RS with corresponding measured quantity, and the current RS of serving cell as described in TS 38.215 [24] with corresponding measured quantity, if UE is configured to report the measurement result of current RS of the serving cell by reportCurrentBeam.

The fields in the (truncated) event triggered L1 measurement report MAC CE are defined as follows:

-	Report ID: This field indicates corresponding measurement report ID for this ltm-CSI-ReportConfigId associated with this event triggered measurement report. The length of the Report ID field is 6 bits;

-	Typei: This field indicates the type of the RS i of LTM candidate cell included in the event triggered L1 measurement report. The field is set to 00 to indicate the RS(s) that has satisfied the entry condition of the event associated with the report ID for TTT, triggers this measurement report MAC CE, and included in the BEAM_ENTERING_LIST; it is set to 01 to indicate the RS(s) that has satisfied the leaving condition of the event associated with the report ID for TTT, triggers this measurement report MAC CE, and included in the BEAM_LEAVING_LIST; it is set to 10 to indicate the RS(s) has been reported in the (Truncated) L1 measurement report MAC CE, and included in the BEAM_REPORTED_LIST associated with the report ID as specified in clause 5.x.3; it is set to 11 to indicate the RS(s) not satisfying the event for TTT, if configured by network by allowReportAnyBeam specified in TS 38.331 [5], i.e. the RS(s) with Type set to neither 00, 01, nor 10. The RS(s) not satisfying the event for TTT is selected based on the decending order of measured quantity. The RS(s) included in the truncated event triggered L1 measurement report MAC CE are selected based on a decreasing order of the priority for the type of beam: 00, 01, 10, 11. If the (truncated) event triggered L1 measurement report MAC CE cannot accommodate all RS(s) with the same priority, the RS(s) are selected based on a decreasing order of measured quality. The length of the field is 2 bits;

NOTE:	For the measurement report triggered by LTM2, the RS with Type of 00 or 01 is the current beam, which is always included in the last octet.

-	RSRIi: This field indicates the reference signaling resource index of the beam i of LTM candidate cell for the event triggered L1 measurement report (i.e. SS/PBCH Block Resource indicator (SSBRI) or CSI-RS resource indicator (CRI)). The maximum number of non-serving RS reported, i.e. M value, is configured by maxNumberOfReportedBeams if the measurement of current RS of serving cell is not included, or is maxNumberOfReportedBeams-1 if the measurement of current RS of serving cell is included. The first RS is the RS with the highest measured quality for non-serving RS in this measurement report MAC CE. The length of the RSRI index field is 9 bits;

-	RSRP1: This field indicates the measured quantity of the first beam based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) as described in TS 38.215 [24]. The length of the RSRP1 field is 7 bits;

-	DiffRSRPi: This field indicates the derived differential measured quantity for the beam i of LTM candidate cell based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) as described in TS 38.215 [24], with the reference of measured quality of the first beam. The length of the DiffRSRPi field is 4 bits;

-	RSRPserving: This field indicates the measured quantity based on SS/PBCH block or CSI-RS (i.e. the L1-RSRP) for current RS of serving cell as described in TS 38.215 [24] used for LTM event evaluation in clause 5.x.2, if UE is configured to report the measurement result of current RS of the serving cell by reportCurrentBeam. The length of the RSRPserving field is 7 bits;

-	R: Reserved bit, set to 0.

Figure 6.1.3.x-1: event triggered L1 measurement report and truncated event triggered L1 measurement report MAC CE

Next change

End of chang