3GPP TSG-RAN WG2 Meeting #131bis		R2-2507020

Prague, Czech Republic, 13th -17th Oct., 2025

Source:	vivo

Title:	Discussion on RLC open issues for R19 XR

Agenda Item:	8.7.3

Document for:	Discussion and Decision

- Introduction

In this paper, we will discuss the remaining open issues for the UP enhancement in relation to RLC for R19 XR, including DSR data volume calculation and the RLC enhancements.

- Discussion

  - [RLC-X01] non-delay reporting data volume calculation in RLC

According to post meeting email discussion [POST130][508][XR] RLC running CR and open issues (vivo), there is an open issue as below:

| RLC-13 | Whether the delay-reporting or non-delay-reporting information is visible or not to RLC? |
| --- | --- |

At RAN2#131, the following agreement were reached in relation to this open issue:

- [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold - [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC

These agreements were captured in the latest PDCP/RLC running CRs. In the post meeting email discussion [POST131][507][XR] Final 38.322 CR (vivo), one issue was raised whether the related change is feasible. The major concern regarding removal of the visibility of non-delay reporting data to RLC is:

If all the RLC SDUs that RLC needs to calculated are indicated from PDCP, then RLC can indeed calculate the data based on PDCP’s indication. Then RLC calculation of the none delay reporting data can be removed. However, is that possible that RLC put some packets which are not know in PDCP before the data of one portion or is there possible that before DSR is triggered, there has already some data before one portion? Then in this way, since PDCP is not aware of data in RLC’s buffer, then I guess the part that RLC calculation of the none delay reporting data still can not be removed.

[RLC-X01] There are the following two concerns for RLC DSR data volume calculation for non-delay-reporting data purely relying on the DSR data indication from PDCP:

    - Concern 1: whether RLC puts some packets not known by PDCP ahead of the data of one portion in RLC buffer.

    - Concern 2: whether there could be some data before one portion before DSR triggering.

Regarding Concern 1, the following should be considered:

- FIFO rule for data transmission: as RLC does not generate packet by itself, all packets in RLC buffer come from the PDCP layer, i.e. there is no data packet in RLC buffer that is not known to PDCP. For UL data TX, the common understanding is that all data packets will be transmitted according to the first-in-first-out criteria, i.e. RLC will transmit the buffered data packets in the same order as they are delivered from PDCP to RLC.

- Shared memory space between PDCP and RLC at UE side: As far as our understanding, Concern 1 is also related to the buffer design for RLC entity and PDCP entity in the UE side. Logically, there are respective buffers for PDCP entity and RLC entity in the specification. However, at the UE side in real field, both PDCP entity and RLC entity are always in the same device. This gives the opportunity that the UE may only save one copy for each IP packet, while the PDCP layer and the RLC layer only maintains the respective PDCP header and RLC header for each packet. By doing so, the UE does not have to physically copy one packet from PDCP buffer to RLC buffer, which may save not only the memory space but also the operation complexity to copy packet from one physical memory to another physical memory.

[RLC-X01] All RLC data packets come from PDCP and are transmitted in first-in-first-out order, i.e. there is no RLC data packet that is not known to PDCP.

[RLC-X01] At UE side, the reasonable design for the UE only saves single copy for each packet in a shared memory for PDCP and RLC, while PDCP and RLC maintain respective protocol headers, i.e. PDCP and RLC has the same data packet placement order.

Regarding Concern 2, it can happen that there is one data before one portion before DSR triggering. The key issue is whether RLC can be timely updated by PDCP when the association with DSR-ReportingThreshold changes for a non-delay reporting packet. According to the following part of the latest PDCP CR [1],

The transmitting PDCP entity provides a DSR data indication associated with the i:th DSR-ReportingThreshold for the PDCP Data PDU to lower layers when: - the PDCP Data PDU has already been submitted to lower layers and the corresponding PDCP SDU becomes a delay-reporting PDCP SDU or non-delay-reporting PDCP SDU (if dsr-ReportNonDelayCriticalData is configured) associated with the i:th DSR-ReportingThreshold; or - the PDCP Data PDU is submitted to lower layers and the corresponding PDCP SDU is already a delay-reporting PDCP SDU or non-delay-reporting PDCP SDU (if dsr-ReportNonDelayCriticalData is configured) associated with the i:th DSR-ReportingThreshold.

Based on the above specification part, there are the following understanding:

- PDCP will provide a first DSR data indication to RLC for a delivered non-delay reporting packet ahead of the i:th DSR-ReportingThreshold when it becomes the non-delay reporting data ahead of the i:th portion.

- Later, when this packet becomes non-delay reporting packet ahead of the (i-1):th DSR-ReportingThreshold (smaller than i:th DSR-ReportingThreshold), PDCP will send a second DSR data indication to RLC.

Wherein the first and the second DSR data indication indicate the association with the i:th and the (i-1):th DSR-ReportingThreshold respectively for this packet.

[RLC-X01] At the UE side, PDCP will send an DSR data indication at each time when a non-delay reporting packet associate with a different DSR-ReportingThreshold.

If the above understanding is correct, the PDCP will know exactly the packet placement order in the memory, no matter the packet has been delivered to RLC or not. RLC will always be updated with the DSR data indication for each non-delay reporting data. That is, the latest running CR is good enough and no change is needed.

- [RLC-X01] It is feasible the RLC to calculate DSR data volume data purely based on the DSR data indication from PDCP, i.e. no further change on RLC is needed.

  - Remaining issues for avoiding unnecessary retransmissions

    - Window stalling issue

In RAN2#130, the following agreements were made regarding window stalling issue with polling retransmission:

RAN2#130 Agreements for RLC-11

- Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

- Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

And in RAN2#131, RAN2 further confirmed that there is no need to poll or retransmit the poll in this case:

RAN2#131 Agreement for RLC-11

- [RLC-11] In case “there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with”, RAN2 confirm no need to poll or retransmit the poll for any acknowledgement (no spec change).

However, how to resolve the window stalling issue when only SDUs whose transmissions have been stopped remain in the buffer has not been fully addressed.

In our view, window stalling is already rare; combined with the condition that the buffer contains only stopped SDUs, the likelihood is even lower and this is a corner case. If it does occur, we can rely on network implementation to address it. For example, the network can trigger an RLC re-establishment for the corresponding DRB to resolve the window stalling.

- Regarding the window stalling issue when only SDUs with stopped (re)transmissions are buffered, it is up to network implementation to resolve it.

    - [RLC-V01] Impact on PDCP t-Reordering timer for avoiding unnecessary RLC retx

When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, due to the fake ACK, the transmitting PDCP entity will consider these RLC SDUs as being successfully received based on the STATUS report, while the receiving PDCP entity will still be waiting to receive the RLC SDUs from the RLC entity until the t-Reordering timer expires. In this case, HFN asynchronization between the transmitting PDCP entity and the receiving PDCP entity may occur. For example, the transmitting PDCP entity may consider the current HFN for a PDCP SDU to be X+1 based on the fake ACK from the STATUS report, while the receiving PDCP entity may consider it to be X.

[RLC-V01] HFN asynchronization between the transmitting PDCP entity and receiving PDCP entity may occur when the feature of avoiding unnecessary RLC retransmission is configured.

Furthermore, if t-RxDiscard expires while t-Reordering is still running and the PDCP entity is waiting for the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery.

[RLC-V01] If the t-RxDiscard expires while t-Reordering is still running and receiving PDCP entity is waiting for the outdated SDUs from the RLC layer, it will cause unnecessary packet delivery delay.

To solve this problem, a straightforward solution is to make the receiving RLC entity indicate SNs of obsolete RLC SDUs when the t-RxDiscard expires, so that the receiving PDCP entity can update its state variable to maintain HFN synchronization with the transmitting PDCP entity and also deliver the buffered PDCP SDUs quickly to the upper layer.

- [RLC-V01] The receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires for HFN synchronization and avoiding packet delivery delay.

  - [RLC-V02] Coexistence of remaining time based RLC retransmission and ARQ

In RAN2#131 meeting, it was agreed to clarify in the RLC spec that “the pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU” as below:

- Can try to clarify together with the previous agreement. [That: “The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU”]

However, this has not been captured due to less consensus in the post email discussion. We could continue to discuss how to capture it in the RLC spec to ensure the packets which have already been positively acked will not be retransmitted.

- [RLC-V02] RAN2 to discuss how to capture the agreement “The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU” in the RLC spec.

- Conclusion

In this contribution, we discuss the remaining open issues for the UP enhancement in relation to RLC for R19 XR, and we have the following observations and proposals:

Regarding RLC-X01

- [RLC-X01] There are the following two concerns for RLC DSR data volume calculation for non-delay-reporting data purely relying on the DSR data indication from PDCP:

    - Concern 1: whether RLC puts some packets not known by PDCP ahead of the data of one portion in RLC buffer.

    - Concern 2: whether there could be some data before one portion before DSR triggering.

[RLC-X01] All RLC data packets come from PDCP and are transmitted in first-in-first-out order, i.e. there is no RLC data packet that is not known to PDCP.

[RLC-X01] At UE side, the reasonable design for the UE only saves single copy for each packet in a shared memory for PDCP and RLC, while PDCP and RLC maintain respective protocol headers, i.e. PDCP and RLC has the same data packet placement order.

[RLC-X01] At the UE side, PDCP will send an DSR data indication at each time when a non-delay reporting packet associate with a different DSR-ReportingThreshold.

- [RLC-X01] It is feasible the RLC to calculate DSR data volume data purely based on the DSR data indication from PDCP, i.e. no further change on RLC is needed.

Regarding Window stalling issue

- Regarding the window stalling issue when only SDUs with stopped (re)transmissions are buffered, it is up to network implementation to resolve it.

Regarding RLC-V01

[RLC-V01] HFN asynchronization between the transmitting PDCP entity and receiving PDCP entity may occur when the feature of avoiding unnecessary RLC retransmission is configured.

[RLC-V01] If the t-RxDiscard expires while t-Reordering is still running and receiving PDCP entity is waiting for the outdated SDUs from the RLC layer, it will cause unnecessary packet delivery delay.

- [RLC-V01] The receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires for HFN synchronization and avoiding packet delivery delay.

Regarding RLC-V02

- [RLC-V02] RAN2 to discuss how to capture the agreement “The pending remaining time based RLC retransmission should be cancelled when the RLC entity receives a positively ACK for the corresponding RLC SDU” in the RLC spec.

- References

- R2-2506337, Introduction of R19 XR enhancements for PDCP spec.

- R2-2507017, List of RLC open issues for R19 XR, vivo.