3GPP TSG-RAN WG2 Meeting #131bis		R2-2507021

Prague, Czech Republic, 13th – 17th Oct., 2025

Source:	vivo

Title:	Discussion on MAC open issues for UEI BMR

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In the email discussion on [Post131][217][MIMO_Ph5] CR for TS 38.321 (Samsung), some MAC open issues related to UEI BMR is listed in [1]. After some offline discussion, the first open issue is still controversial. In this contribution, we will discuss this open issue.

- Discussion

  - MAC open issues

In the offline discussion, the issue is described as:

[Issue Description]: The PUCCH resource of a UEI report configuration of a SCell can be configured on a PUCCH Cell associated with a different TAG from the SCell: Currently, in this case, when TAT of a TAG of the SCell expires and PUSCH transmission for the CSI report becomes unavailable, the PUCCH resource of a UEI report configuration of the SCell is not released when the PUCCH resource is configured on a PUCCH Cell with a different TAG. This will cause unnecessary PUCCH transmission and the PUCCH resource will be occupied by the UE and cannot be reallocated for other uses. In addition, if RAN2 decides to not release PUCCH resource in this case, behavior of UEI beam reporting should be clarified when no PUSCH is available for CSI report (i.e., whether the UE still transmits the PUCCH when no PUSCH is available for CSI report). To address this open issue, several options have been identified as below: Upon STAG TAT expiry associated with a SCell configured with a UEI report configuration, if the PUCCH resource of the UEI report configuration is configured on a PCell or PUCCH-SCell of a different TAG with a running TAT, the UE behavious includes: - Option 1: releases the PUCCH resource. - Option 2a: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE transmits the PUCCH and does not transmit the PUSCH for the UEI beam report. - Option 2b: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE does not transmit the PUCCH for the UEI beam report.

In our understanding, this issue only occurs for mode-B because CG-PUSCH carries the beam report is 1-to-1 associated with PUCCH resource carries trigger indication and the trigger indication is used to inform the network to monitor the corresponding CG-PUSCH. Thus, if the CG-PUSCH resource is not available for the UE, whether to transmit/release the PUCCH resource carrying the trigger indication needs further clarification. However, in Mode A, the PUSCH carrying beam report is scheduled dynamically by the network after the UE receives the UEIRI. Therefore, whether the TAT corresponding to the CC on which the PUSCH is transmitted expires or not would not make any impact on the transmission of the PUCCH associated with other TAGs.

For option 1, the PUCCH resource is not dedicated to UEIRI. HARQ-ACK and SR can also be transmitted on the PUCCH resource. If it is released upon the TAT of the associated CG-PUSCH expires, the HQRA-ACK/SR transmission will be affected a lot. For option 2a/2b, either is fine with us.

Upon STAG TAT expiry associated with a SCell configured with a UEI report configuration with Mode B, if the PUCCH resource of the UEI report configuration is configured on a PCell or PUCCH-SCell of a different TAG with a running TAT, the UE:

- Option 2a: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE transmits the PUCCH and does not transmit the PUSCH for the UEI beam report.

- Option 2b: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE does not transmit the PUCCH for the UEI beam report.

- Conclusion

In this contribution, we discuss the MAC open issues on UE-initiated/event-driven beam management, identified in the post meeting email discussion. Based on the above discussion, we have the following proposals.

MAC open issue:

- Upon STAG TAT expiry associated with a SCell configured with a UEI report configuration with Mode B, if the PUCCH resource of the UEI report configuration is configured on a PCell or PUCCH-SCell of a different TAG with a running TAT, the UE:

- Option 2a: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE transmits the PUCCH and does not transmit the PUSCH for the UEI beam report.

- Option 2b: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE does not transmit the PUCCH for the UEI beam report.

- References

- R2-2507497, Report of Rel-19 MIMO MAC open issues for maintenance, Samsung