3GPP TSG-RAN WG2 Meeting #131bis		R2-2507028

Prague, Czech Republic, 13th -17th Oct., 2025

Source:	vivo

Title:	Discussion on DSR triggering for R18 XR

Agenda Item:	7.0.2.16

Document for:	Discussion

- Introduction

In RAN2#131 meeting, whether DSR can be triggerd for the PDCP SDUs with some segments have been transmitted in MAC PDU and some segments have not has been discussed as below. It was postponed due to companies have different understandings on the current specification, which may also impact the product implement.

R2-2505408 Correction on DSR triggering vivo CR Rel-18 38.321 18.6.0 2099 - F NR_XR_enh-Core Postponed - Xiaomi does not see strong need to change. It is an optimization. LGE agrees. - Nokia thinks this makes sense. - LGE thinks this would require to also change DSR cancellation condition. We can leave as is. - Samsung agrees with LGE and Xiaomi. Network knows about the segment so will schedule the transmission. - Ericsson does not think we need a change. - Sharp agrees that an issue can happen. - After offline discussion vivo clarifies that companies have different understanding on the current specification and would like to check with product teams.

In this contribution, we further discuss this issue.

- Discussion

According to the current MAC specification, DSR triggering is determined by the smallest remaining time value of PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU.

However, there may be cases where a PDCP SDU is segmented at the RLC layer, with some RLC SDU segments already transmitted in a MAC PDU while others remain in the buffer. According to the current specification, these remaining RLC SDU segments cannot trigger DSR because they are “PDCP SDUs that have been transmitted partially” while not qualitied as "PDCP SDUs that have not been transmitted in any MAC PDU."

We understand that this kind of RLC SDU segments should also be able to trigger DSR if the remaining time is below the threshold. But during the discussion in RAN2#131, companies have different understandings:

- alt1: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been completely transmitted in any MAC PDU”. -> SDU segment could trigger DSR

- alt2: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been partially transmitted in any MAC PDU”. -> SDU segment cannot trigger DSR

With this, it is better to discuss which alternative is the correct understanding on the current specification, and whether any change is needed.

Proposal 1: RAN2 to discuss which is the correct understanding on the current specification, and whether any change on MAC is needed:

- alt1: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been completely transmitted in any MAC PDU”. -> SDU segment could trigger DSR

- alt2: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been partially transmitted in any MAC PDU”. -> SDU segment cannot trigger DSR

Besides, during the offline, companies have different understandings on how to change it: if the specification needs to be updated, from which release should it start, i.e. Rel-18 or Rel-19.

Proposal 2: If the change on MAC is needed in P1, from which release should it start, i.e. Rel-18 or Rel-19.

- Conclusions and Proposals

In this contribution, we discussed the DSR triggering for R18 XR. Based on the discussion, we have the following proposals.

Proposal 1: RAN2 to discuss which is the correct understanding on the current specification, and whether any change on MAC is needed:

- alt1: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been completely transmitted in any MAC PDU”. -> SDU segment could trigger DSR

- alt2: “PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU” in 5.4.9 in MAC means “PDCP SDUs that are buffered for the LCG but have not been partially transmitted in any MAC PDU”. -> SDU segment cannot trigger DSR

Proposal 2: If the change on MAC is needed in P1, from which release should it start, i.e. Rel-18 or Rel-19.

- References

R2-2505408, Correction on DSR triggering, vivo

Annex – Text Proposal on TS 38.321

Start of change

### 5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been completely transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been completely transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the logical channel:

2>	trigger a DSR for the logical channel.

If there is at least one DSR pending, the MAC entity shall:

1>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72.

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE 1:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

End of change