3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507031

Prague, Czech Republic, 13th –17th October, 2025

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	A-IoT remaining issues related to other WGs

Document for: Discussion and Decision

1	Introduction

This contribution focuses on several remaining issues related to other WGs:

- Security parameter in paging message;

- Data not available case;

- Permanent device ID length (SA2 LS);

- Max NAS message size (CT1 LS).

2	Discussion

2.1	Security parameter in paging message

It is one RAN2 remaining issue on how to implement the security parameter from last meeting RAN2 agreement:

- RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not.

Some aspects are analysed in the joint paper [1]. Moreover, in MAC layer procedure, when the reader sends the subsequent paging to address the Msg3 transmission failure after the NACK feedback, the reader can determine that the device has already received the previous paging message content. Then, there is no need to repeat the 128-bit rand number in subsequent paging if the intention is purely for triggering re-access.

Observation 1:	It is unnecessary to always include the 128-bit rand number in subsequent paging, when triggering the re-access.

Therefore, from RAN2 signaling design perspective, it is proposed:

Proposal 1:	Use 1-bit to indicate the presence of the 128bit random number field in paging message.

2.2	Data not available case

One remaining issue from last RAN2 meeting is to check if there is the case that “NAS doesn’t provide a response at all” in CT1.

Firstly, we echo the view that there is such case. In fact, we identified two possible cases, which may cause no NAS response at all, due to the upper layer failures. For example, if the device failed the integrity protection of the received NAS message, it will not generate any NAS response. Also, if the device NAS layer receives a NAS message with message type not defined/implemented, the device will ignore the NAS message and therefore generate no NAS response.

Proposal 2:	RAN2 to confirm the valid case that “NAS doesn’t provide a response at all”.

Given that there are two possible cases where the reader does not get the immediate D2R response, it is reasonable to differentiate the two cases. The “delayed NAS response” case requires reader to re-schedule the device later, while the “no NAS response at all” case does not.

Since the “SDU=0” already implies the “no immediate D2R response”, we can use the MDI field to indicate the different cases. In our understanding, “more data” (i.e., MDI=1) can imply the device may have more data later, while “no more data” (i.e., MDI=0) is suitable to imply there is no response at all.

Proposal 3:	As to the data not available scenario, RAN2 agrees that “MDI=0 and SDU=0” refers to the “delayed NAS response” case and “MDI=1 and SDU=0” refers to “no NAS response at all” case. (flip the agreed MDI value setting from last meeting agreement)

Moreover, as to the possible “delayed NAS response” due to write operation, the suitable D2R TBS for the initial schedule should be quite small (e.g., just 2bytes), since there is no immediate SDU expected. Therefore, it is useful to let the reader know the command type from CN, so that the reader implementation can determine the suitable D2T TBS based on the command type.

Proposal 4:	RAN2 suggests to RAN3 and SA2 that the CN should indicate the command type (including read, write, disable) to the reader.

2.3	Permanent device ID length (SA2 LS)

Based on the SA2 LS in about the AIoT Device permanent ID, it is suggested to consider supporting support 256-bit and 496-bit permanent device ID [2].

As part of the Ambient IoT standard, the AIoT Device permanent ID needs to consider the requirement from various industries. SA2 notices the current design on AIoT Device permanent ID lengths (i.e. Identification Information only supports two options: either 96 bit, or 128 bit) has constrains and cannot meet the requirement for some industries like healthcare, global trade, etc. SA2 has discussed extending the length options of the AIoT Device permanent ID (i.e., the Identification Information part) to support 256-bit and 496-bit. The rationale for this extension has received general support. SA2 would like to clarify that the other parts of the AIoT Device permanent ID (i.e. ID type, domain information) is not impacted. SA2 roughly estimated that the filtering information will be around 600-bit in case the Identification Information is 496-bit, and around 360-bit in case the Identification Information is 256-bit. The maximum length of filtering information will be decided by CT4 WG. In addition to filtering information, SA2 understands the overall length of the paging message may need to consider other information like security parameters, etc. SA2 would like to check with RAN2 on whether extending the AIoT Device permanent ID to 256-bit and 496-bit is feasible from RAN2 point of view.

As mentioned in the LS, the rationale for this extension has received general support. In addition, SA2 ask whether extending the AIoT Device permanent ID to 256-bit and 496-bit is feasible from RAN2 point of view.

Note that this extension means the maximum paging ID size could be up to 600-bit.

Considering the maximum size of paging message as 125bytes and 128bits (16bytes) for the security parameters, then the remaining maximum allowed paging ID size is:

- In CBRA: 125byte- about 6byte AS filed – 16byte security field= 824 bits

- In CFA: 125byte- about 6byte AS filed – 16byte security field= 824 bits

Figure 2.3-1: MAC PDU of A-IoT Paging message indicating CBRA

Figure 2.3-2: MAC PDU of A-IoT Paging message indicating CFA

Observation 2:	There are about 800bits left for the paging ID, which can accommodate the maximum size 600bits requested by SA2.

Therefore, RAN2 can confirm the feasibility of this paging ID size extension, if needed. Of course, the actual coverage performance may be less optimal, when network chooses to use this 256/496bits device ID.

Proposal 5:	RAN2 to reply SA2 LS to confirm the feasibility of the extension for AIoT Device permanent ID.

2.4	Max NAS message size (CT1 LS)

CT1 asked RAN2 to clarify the possible maximum NAS message size via LS [3]:

CT1 is currently working on the coding aspects of AIoT NAS command messages. CT1 kindly requests clarification on the maximum supported AIoT NAS container length in both R2D and D2R directions, including segmentation, if supported.

Figure 2.4-1: MAC PDU of R2D Upper Layer Data Transfer message containing Data SDU

Figure 2.4-2: MAC PDU of D2R Upper Layer Data Transfer message

Considering the maximum size of MAC PDU as 125bytes for both directions, based on the current A-IoT MAC specification, the remaining maximum size for MAC SDU is (in theory):

- R2D maximum NAS container size=125byte max TBS - 6byte taken by the other MAC fields;

- D2R maximum NAS container size = 125byte max TBS - 2byte taken by the other MAC fields;

However, some margin should be left in case of future extension of some possible new MAC fields. For example, 2-3 bytes for the future extension.

Note that the D2R MAC segmentation cannot enable larger NAS container size. D2R segmentations requires the essential indicator on Received Data Size field, which can only indicate maximum 125 bytes with 7bits, as in TS 38.391. It means the reader cannot guarantee the successful and in-order reception of the NAS container larger than 125 bytes.

Particularly, the R2D message may also carry the write command. Note that the supper large write command may lead to supper long write operation delay, which could cause device power-off during the write operation. Namely that larger size write command, the more possibility of write operation failure. In addition, large write command requires large device buffer capability, which may be not supported by all different device buffer budget. Also, if there is really the case of large write command, it can still be written by several separated write commands.

Therefore, we propose follow replies, by considering the MAC specified fields, margin for possible future extension and device limitation of write operation.

Proposal 6:	RAN2 to reply CT1 LS that the maximum size of NAS container can be:

Maximum size of one R2D NAS container =125byte max TBS - 6byte taken by the other MAC fields – 3byte margin for future MAC fields, i.e., 116 bytes;

  - In addition, it is recommended that the NAS message for the write command be reasonably shortened.

Maximum size of one D2R NAS container = 125byte max TBS - 2byte taken by the other MAC fields- 3byte margin for future MAC fields, i.e., 120 bytes.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	It is unnecessary to always include the 128-bit rand number in subsequent paging, when triggering the re-access.

Observation 2:	There are about 800bits left for the paging ID, which can accommodate the maximum size 600bits requested by SA2.

Security parameter in paging message

Proposal 1:	Add a 128-bit field in paging message to carry the security parameter, which is optionally present with 1-bit to indicate its presence.

Data not available case

Proposal 2:	RAN2 to confirm the valid case that “NAS doesn’t provide a response at all”.

Proposal 3:	As to the data not available scenario, RAN2 agrees that “MDI=0 and SDU=0” refers to the “delayed NAS response” case and “MDI=1 and SDU=0” refers to “no NAS response at all” case. (flip the agreed MDI value setting from last meeting agreement)

Proposal 4:	RAN2 suggests to RAN3 that the CN should indicate the command type (including read, write, disable) to the reader.

Permanent device ID length (SA2 LS)

Proposal 5:	RAN2 to reply SA2 LS to confirm the feasibility of the extension for AIoT Device permanent ID.

Max NAS message size (CT1 LS)

Proposal 6:	RAN2 to reply CT1 LS that the maximum size of NAS container can be:

Maximum size of one R2D NAS container =125byte max TBS - 6byte taken by the other MAC fields – 3byte margin for future MAC fields, i.e., 116 bytes;

  - In addition, it is recommended that the NAS message for the write command be reasonably shortened.

Maximum size of one D2R NAS container = 125byte max TBS - 2byte taken by the other MAC fields- 3byte margin for future MAC fields, i.e., 120 bytes.

# 4	References

- R2-2506902, Issue 1-7 Security parameter in A-IoT paging, CMCC, China Unicom, China Telecom, Huawei, HiSilicon, ZTE Corporation, Sanechips, Nokia, Xiaomi, vivo, Panasonic, LGE, Fujitsu, Transsion Holdings, Quanray

- S2-2507793, LS on AIoT Device Permanent ID Length

- C1-255679, LS on the maximum supported AIoT NAS container length