3GPP RAN WG2 Meeting #131-bis                                                                                R2-2507036

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	10.3.3

Source:	HONOR

Title:	Discussion on Common User plane and Control plane for 6GR

Document for:	Discussion and Decision

- Introduction

In RAN#108 meeting, the industry has approved the 6G Study Item (RP-251881[1]), marking the first concrete blueprint for the next generation of wireless system. As a potential key technology of 6G, RAN2 group is responsible for discussing and promoting the study of radio interface protocol architecture and procedures for 6GR. One of the objectives is,

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we try to analyze the possible functionalities for 6GR to meet the requirements of Data Handling.

- Discussion

  - Overview

In 5G NR, several use cases have been studied on leveraging AI/ML to improve network performance, such as CSI feedback enhancement, beam management and positioning enhancement. In each scenario, data plays a crucial role in supporting AI/ML model training, model inference, and model monitoring/management. The corresponding data handling mechanisms are also captured in the framework of AI/ML [2], including data collection—a process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference.

In addition to data collection, data delivery is another crucial part of the data handling procedure to support AI/ML training. The data could be AI/ML model for inference or datasets for model training, and it could be delivered from one entity to another entity in any manner. For example, in the context of AI/ML-based CSI compression using two-sided models in Rel-19, the sharing of dataset/model parameters from NW-side to UE-side training entity was discussed. In this case, the RAN node should support the generation of dataset and then transfer the collected data to target nodes, e.g., OAM and CN node.

For sensing in NR, sensing entity performs the sensing service requested by sensing function (SnF). In general, the SnF will generate the sensing results and send them to the consumer. In certain cases, the SnF may request the sensing entity (e.g., RAN node) to process the sensing data and generate sensing result or partial result in the report. Therefore, the sensing entity collects the sensing data and delivers the sensing data or results to the SnF.

Observation 1: Data collection and data delivery are important functionalities of AI/ML and sensing, and NR has been discussing data collection and data delivery mechanisms for specific use cases.

With the rapid advancement of AI/ML technology, new use cases are being explored to improve the performance of 6GR, such as AI for energy saving and AI for mobility. In our view, 6G Day-1 RAN design should take the existing and potential use cases for data services into account to design the signaling and procedures for data collection and data delivery with addressing both universal and scenario-specific requirements. From the RAN node’s perspective, the corresponding functionalities could be summarized as:

–	Data Collection: The RAN node could collect data from the UE side or generate data directly.

–	Data Delivery: The RAN node supports the delivery of collected or received data to the target nodes (e.g., UE, CN node and OAM).

As more use cases evolve, the RAN node may need to accumulate large amount of data for various purposes, potentially leading to the duplicated collection of same or similar data, as well as the wastage of energy and resources. In addition, NR is studying how to leverage AI to optimize RAN’s performance and management within the network. It would be beneficial if the RAN node could be aware of the data content and adapt accordingly for performance improvement. To facilitate the issue, data management should be studied with potential framework enhancement.

Observation 2: 6GR shall support the data handling framework which includes Data Collection, Data Delivery and Data Management to support efficient data collection and utilization.

Proposal 1: RAN2 to study how to support the Data Collection, Data Delivery, and/or Data Management to fulfil the data service requirements.

–	Data Collection: The RAN node could collect data from the UE side or generate data directly.

–	Data Delivery: The RAN node supports the delivery of collected or received data to the target nodes (e.g., UE, CN node and OAM).

–	Data Management: The RAN node could be aware of the data and utilize data for various purposes, e.g., performing AI-enabled data analysis and adapt accordingly for performance improvement.

  - Functionalities of Data Collection

2.2.1 Motivation (6G Use cases)

In the TR 22.870 [3], SA WG1 has defined the 6G system data which is generated and controlled by the 6G system. 6G system data, e.g., AI/ML data, sensing data, energy efficiency information and NF context, is different from user traffic data which is application-level data transmitted transparently between the UE and DN, thus presenting new requirements for 6G data collection framework. Take AI/ML data as an example,

- AI/ML data

  - Various data types: AI/ML data includes raw data or processed datasets for training/testing/validating AI models, AI models and parameter data, AI inference input data and output data (e.g., AI analytic results), AI monitoring data, and assistance information (e.g., for categorizing the data like timestamps). It can be found that there is a large amount of data types defined in 3GPP apart from those non-standardized ones.

  - More data providers and consumers: Not only UE, but also RAN/CN/AF can provide the data. Meanwhile, UE, RAN and CN can be the data consumers [3].

  - Massive data: The scale of AI/ML data volume is usually large (from 10k to 100M) [3].

  - Different latency requirements for data collection: In the Rel-18 discussion of AI/ML data collection [2], there is no latency requirement for data collection in all types of offline model training (i.e., UE-/NW-/two-sided model training). However, for model inference, when required data comes from other entities, a latency requirement exists. Similarly, for the real-time performance monitoring, when monitoring data (e.g., performance metrics) comes from other entities, a latency requirement is also necessary.

  - In-network data storage and sharing: Unlike most of the 5G data, which is typically used once and rarely stored (e.g., measurement data reported by UEs), 6G networks require the collected data be accessible to multiple data consumers within 3GPP system to support various services, e.g., measured data can also be analyzed and utilized by native AI for flexible resource allocation and optimized network performance.

As 6G networks progressively integrate advanced capabilities like AI/ML and multi-dimensional sensing, they will need to collect and manage massive amounts of multi-source, heterogeneous data (from diverse formats and data resource) generated by 6G system. Therefore, 6G networks shall consider these new data characteristics and support efficient data collection and control mechanism to meet both universal and scenario-specific requirements across diverse use cases, such as AI, sensing, XR services, and digital twin networks.

Proposal 2: 6G system shall consider the new data characteristics (e.g., various data types, new data provider/consumer and large volumes) derived from heterogeneous use cases (e.g., AI, sensing) and support the efficient data collection and control mechanism.

2.2.2 Impact for 6G data collection functionalities

From the RAN side’s perspective, facing these new data characteristics, the 6G network capability of data collection needs to be extended based on the 5G network, including enhancements in controllability, UE selection, and visibility.

- Controllability

Data collection is crucial for enabling various AI/ML-assisted use cases. In 5G NR AI/ML air interface discussions, it is agreed that data collection initiation and configuration for data collection are under network control, a principle that should be carried over to 6GR. Specifically, the RAN node can determine if and when to start or stop data collection and configure whether the UE is allowed to request data collection. For some use cases, such as beam management, the RAN provides the data collection configuration for training a UE-side model. In other cases, such as positioning, the configuration may come from outside the RAN (e.g., LMF). Even in these latter cases, some level of RAN awareness and control is necessary to ensure that UE performance is not adversely affected by data collection measurements.

- UE selection

In the Rel-18 study on AI/ML air interface, for network (NW)-side model training [2], a set of general data collection principles involving UE capability demands is expected to be considered, including:

-	UE to support data logging,

-	UE to report the collected data periodically, event-based, and on-demand,

-	The UE memory, processing power, energy consumption, signaling overhead should be considered.

Therefore, when the network-side training entity requests data from RAN nodes, the RAN may need to select appropriate UEs based on data requirements. Moreover, in the current SA2 discussions [4], the UE-side training entity can instruct specific UEs, groups of UEs, or any UE to collect and report data, which implicitly involves the UE selection process. As mentioned before, the network sided data collection involves buffer management: when the UE reaches its buffer limitation, it stops the logging measurements. Our understanding is that the same principle applies to the UE selection during UE-side data collection.

- Visibility

As concluded in Rel-19 on AI/ML for NR air interface, various directions and options for inter-vendor collaboration were investigated [5]. In Rel-20, Direction A sub-option 4-1, where NW-side training entity sends datasets to UE-side for training the actual encoder is prioritized for study, as it introduces less complexity if the dataset and its exchange method/procedure are standardized. Currently, RAN1 is discussing the standardization of dataset for the CSI compression use case [6]. From the RAN side, the dataset should be understandable by the RAN node and the NW-side training entity to ensure interoperability and efficient implementation. This requires clear definitions of data format, data content for different data types to support consistent training and inference across different vendors.

Proposal 3: For 6GR, RAN2 to study the possible functionalities of Data Collection, which include: controllability, UE selection, and visibility.

  - Functionalities of Data Delivery

Two alternatives (non-OTA, OTA approach) (as shown in Fig. 1 [7]) for the sharing of NW-side dataset/model parameters have been discussion in Rel-19 and initial progress has been made.

Figure 1 Non-OTA and OTA approaches for dataset/model parameter transfer

For alternative 2 (OTA approach), NW-side dataset/model parameters collection entity located at gNB/CN/OAM is responsible for transmitting dataset/model parameters towards UE-side training entity (a server inside MNO or an OTT server) via UE. From RAN2’s perspective, alternative 2 involves more standard impacts, and thus we will focus on the OTA approach for data collection in the UE part of the two-sided model.

We know that the generalization ability of a model can not only improve its performance, but also enhance resource efficiency and model robustness. Therefore, it is desirable for the dataset/model parameters to be applicable across multiple UEs. Simultaneous delivery of these dataset/parameters to a group of UEs is an energy-efficient approach for the RAN node. Thus, the existing MBS multicast/MBS broadcast mechanisms can be further considered in this case. However, considering the complexity of MBS multicast/broadcast, the issues such as UE selection and group scheduling are involved. Therefore, it’s recommended to take unicast as the baseline in the initial discussion stage, and further study the possibility of multicast or broadcast.

Observation 3: For the transmission from RAN node to UE side, MBS multicast/broadcast scheme may introduce additional complexity in the case of AI/ML dataset/model parameters transferring.

Current specification has specified serval functions for control data transmission and user data transmission. Once the data is received in AS layer, the AS layer starts preparing the data transmission, such as QoS mapping, packet encapsulation, security policy, compression, discard, transmission/retransmission, segmentation, in-order delivery, etc.

According to the requirement analysis in the section 2.2, the data volume of the AI/ML data and sensing data is typically large. During AI model/sensing data/results transmissions, the reliability of data transmission should be guaranteed. Meanwhile, sensing data and AI/ML data (e.g., for inference) need to be transmitted within specific delay constraints. Therefore, to accommodate the transmission of different data types, some fundamental functions derived from 5G NR should be supported for data transmission on 6GR.

Proposal 4: For 6GR, RAN2 to study the possible functionalities of Data Delivery, which can include: QoS mapping, security policy, compression, transmission/retransmission, segmentation, in-order delivery, etc.

  - Consideration on Protocol Stack

In general, the data is quite different with traditional UE connectivity-based service with new traffic characteristics, and requirements for RAN node. Considering a system-level solution, the data handling should also include data sharing and consistency within and across RAN, CN and OAM domains, aiming to reduce duplication and simplify integration—requirements not typically seen in traditional services. Meanwhile, the 6G system shall support mechanisms for data handling that minimize the impact to traditional services. For 6G Day-1, 6GR should study whether and how to introduce the data plane to support the functionalities for data handling.

Observation 4: 6GR should study whether and how to introduce the data plane to support the functionalities for data handling, which include Data Collection, Data Delivery and Data Management.

In NR, RAN2 has evaluated how to support data collection and delivery of AI/ML via enhancing of control plane and user plane. However, it is hard to define an efficient mechanism constrained by backward compatibility. For 6GR, it is possible to natively support a scalable and forward-compatible data handling with capture data handling functionalities in Control Plane and User Plane.

Proposal 5: For 6GR, RAN2 to study whether and how to introduce the data plane, or how to enhance the user plane or control plan to support the functionalities of data handling.

The 4G/5G protocol stack design applies a well-defined protocol layered structure, where each layer provides its own services. If a new plane is introduced, the protocol layered structure should be also used. Given that the new plane is expected to handle data with different QoS requirements, the possible protocols and their corresponding functions can be as following:

- A protocol supporting QoS mapping, segmentation, ect.

- PDCP protocol supporting security handling, compression, in-order delivery/out-of-order delivery, ect.

- RLC protocol supporting TM/UM/AM, transmission/retransmission, segmentation, ect.

- MAC protocol supporting multiplexing/demultiplexing, scheduling, ect.

Proposal 6: RAN2 to study the possible protocol stack and functionalities of each sublayer if the new Data plane is introduced.

  - Coordination with the other groups

There should be a system-level solution to ensure the efficient and scalable handling of various types of data (including AI/ML data, sensing, etc.). Currently, the 6G data framework is also studied in SA2, with a scope that may include identifying the data type and requirements, designing the end-to-end system architecture, defining the procedures and protocols and establishing policy/Qos for 6G data framework. This would introduce new requirements for RAN, and RAN2 should keep coordination with SA2 to be aware of the requirements to provide an efficient data handling solution.

From the RAN’s aspect, an AS security scheme is necessary to ensure secure transmission, with the detailed algorithms defined in SA3. User consent and privacy are important considerations in the initial phase and fall within the scope of SA3. RAN should consider the progress of SA3 to decide whether and how to support the security issue in the first phase. Therefore, coordination with SA3 is required for user consent, privacy, security and data governance related topics.

Proposal 7: It is necessary to coordinate with SA2 and SA3 to design a system-level and efficient data handling scheme.

- Conclusions

In this contribution, we discussed the possible requirements and functionalities of data handling for 6GR, and made the following proposals:

Observation 1: Data collection and data delivery are important functionalities of AI/ML and sensing, and NR has been discussing data collection and data delivery mechanisms for specific use cases.

Observation 2: 6GR shall support the data handling framework which includes Data Collection, Data Delivery and Data Management to support efficient data collection and utilization.

Proposal 1: RAN2 to study how to support the Data Collection, Data Delivery, and/or Data Management to fulfil the data service requirements.

–	Data Collection: The RAN node could collect data from the UE side or generate data directly.

–	Data Delivery: The RAN node supports the delivery of collected or received data to the target nodes (e.g., UE, CN node and OAM).

–	Data Management: The RAN node could be aware of the data and utilize data for various purposes, e.g., performing AI-enabled data analysis and adapt accordingly for performance improvement.

Proposal 2: 6G system shall consider the new data characteristics (e.g., various data types, new data provider/consumer and large volumes) derived from heterogeneous use cases (e.g., AI, sensing) and support the efficient data collection and control mechanism.

Proposal 3: For 6GR, RAN2 to study the possible functionalities of Data Collection, which include: controllability, UE selection, and visibility.

Observation 3: For the transmission from RAN node to UE side, MBS multicast/broadcast scheme may introduce additional complexity in the case of AI/ML dataset/model parameters transferring.

Proposal 4: For 6GR, RAN2 to study the possible functionalities of Data Delivery, which can include: QoS mapping, security policy, compression, transmission/retransmission, segmentation, in-order delivery, etc.

Observation 4: 6GR should study whether and how to introduce the data plane to support the functionalities for data handling, which include Data Collection, Data Delivery and Data Management.

Proposal 5: For 6GR, RAN2 to study whether and how to introduce the data plane, or how to enhance the user plane or control plan to support the functionalities of data handling.

Proposal 6: RAN2 to study the possible protocol stack and functionalities of each sublayer if the new Data plane is introduced.

Proposal 7: It is necessary to coordinate with SA2 and SA3 to design a system-level and efficient data handling scheme.

- References

- RP-251881

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP

- TR 22.870, “Study on 6G Use Cases and Service Requirements”, 3GPP

- TR 23700-04, “Study on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML); Phase 2”, 3GPP

- RAN1#117 Chairman notes

- RAN1#122 Chairman notes

- RAN2#129-bis Chairman notes