3GPP RAN WG2 Meeting #131-bis                                                                                R2-2507037

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	10.4

Source:	HONOR

Title:	Discussion on Mobility management for 6GR

Document for:	Discussion and Decision

- Introduction

RAN plenary #108 meeting discussed the objectives for 6GR and provide the initial version for the 6GR SID which is revised in RAN plenary #109 meeting [1]. As shown in the following, RAN2 would lead the study of mobility for 6GR and start the discussion in RAN2#131-bis meeting.

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

- User Plane architecture and protocol design [RAN2]

- Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3]

- Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

- Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4]

- Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

(4)	Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In this contribution, we will provide our initial thinking for the mobility management for 6GR for different RRC states and RRM.

- Discussion

Mobility is a key function for the cellular network to provide a continuous service, similar to 5G, the study of mobility management aims to copy with the UE with different RRC state and different phase of mobility management (cell switch/beam switch, measurement, failure detection and handling, etc.).

In 5G, in addition to the baseline cell switch mechanism, different mobility enhancements are introduced to copy with different scenarios or meet different requirements. For example, to support seamless service during mobility, DAPS is introduced to enable the UE to communicate with both source and target during handover. To improve the robustness due to failed signalling exchange, handover execution could be triggered by UE based on the NW-preconfigured configuration and event based on the conditional handover mechanism. To reduce the latency caused by RRC signalling and the interruption caused by L2/L3 reset/reconfiguration, LTM is introduced as a mechanism triggered by MAC CE based on the L1 measurement report. Furthermore, LTM is also extended to a conditional mechanism based on preconfigured event to further reduce the latency caused by the signalling interaction.

Observation 1: The 5G cell switch mechanisms triggered by different node (i.e., NW triggered, or UE triggered) based on cross layer mechanism (e.g., L3 HO or L1/L2 based cell switch) aim to different scenarios and requirements.

For 6G Radio Access Technology, to achieve a transformative leap in capability, efficiency, and simplicity, the study of mobility should consider the anticipated diverse service classes and try to a unified yet flexible framework for all RRC state and different phase (e.g., RRM). We would like to discuss the mobility for different RRC states and the measurement related issues.

  - Mobility for UEs in connected RRC state

NW based mechanism is the baseline to switch the UE to the target cell which NW takes account in the information (e.g., measurement result) reported from the UE and the NW status (e.g., load balance, energy efficiency). However, the signalling may be lost due to the fast channel quality degradation. Compared with NW based mechanism, the UE triggered mechanism could be more applicable in some scenarios. For example, high speed UE could trigger the execution by the preconfigured condition without the signalling interaction which increase the robustness for the cell switch. Furthermore, UE with fixed trajectory would have a higher HO success rate and resource utilization rate since the preconfigured resource could be used when the UE moving along the predictable path. To meet the full range of 6G use case, both the NW signalling based and UE triggered mechanisms could be considered from the Day 1 for the 6G study.

Proposal 1: Support both UE triggered switch mechanism and NW signalling based switch mechanism from day 1. Further discuss whether a unified configuration framework could be used.

In 5G, when the UE moves along, the target cell and source cell could be intra-DU, inter-DU intra-CU and inter CU. Based on the relationship between the target cell and the source cell, the UE could perform different level of protocol reset. Different level of protocol reset could be discuss based on the NW architecture to further reduce the interruption caused by cell switch. Furthermore, the lower layer signalling could further reduce the latency caused by signalling deliver but only limited information could be indicated to the UE upon cell switch. Higher layer signalling (e.g., RRC) could carry the complete configuration but signalling interaction and message decoding would cause larger latency. For the measurement report, the L1 measurement and report could provide prompt information while L3 measurement could provide more stable results. Cross layer design could utilize the advantages for different layer. Thus, we propose that

Proposal 2: Introduce a cell switch mechanism for cross layer switch (e.g., L1/L2/L3 cell switch) based on a unified framework.

In 5G, beam management is to switch the serving beam within the serving cell based on the L1 measurement reported from the UE. However, when introducing LTM in the later release of 5G, UE could perform the L1 measurement for the candidate cells and report the L1 results to the serving cell. MAC CE is used by the source DU to trigger the UE to apply the preconfigured L3 configuration of the target candidate cell and indicate the target beam for accessing the target cell. Beam specific operation and L1 measurement are not limited within the serving cell. The cell is covered by multiple beams which may be transmitted by multiple TRPs. Based on our understanding, beam switch could be triggered if the UE is moving within a limited area (e.g., the serving cell) and the cell switch is only triggered when the UE leaving the previous limited area. In another word, when the LTM candidate cell becomes the serving cell of the UE, it could be switched to the preconfigured beam when it moves within this candidate cell and could be switched to the preconfigured beam of another candidate cell when it moves out of this candidate cell. Similarly, for the study of 6G, the cell switch could be target to a specific beam and the beam management could be a cross cell operation. A unified design for the cell switch and beam management may be beneficial to reduce the redundant configurations and options for the same functionality.

Proposal 3: Consider the unified the design for the cell switch and beam management. Inter-WG work with RAN1 is required.

  - Mobility for UEs in non-connected RRC state

Mobility management for non-connected UEs is primarily implemented through cell selection and reselection. Cell selection and reselection identify the cell that the UE should camp on, ensuring coverage continuity and service reachability of mobile communication system. The UE shall select a suitable cell based on RRC_IDLE or RRC_INACTIVE state measurements and cell selection criteria. When camped on a cell, it shall regularly search for a better cell and try to select a more suitable cell according to the RRC_IDLE or RRC_INACTIVE state measurement results and the cell reselection criteria. The 5G cell reselection process mainly includes the configuration of parameters for measurement and evaluation, the sorting of cell/frequency priorities, the processing of measurement results and the criteria for cell evaluation. As the main method of non-connected UE mobility management, cell reselection should be studied from 6G Day1. It is recommended to consider 5G cell reselection process as a starting point and try to optimize the cell reselection process to simplify the design complexity, adapting to diverse 6G scenarios.

Proposal 4: Consider 5G cell selection/reselection process as a starting point and try to optimize the cell selection/reselection process to simplify the design complexity, adapting to diverse 6G scenarios.

Cell selection/reselection involves measurements of sync signals and parameters carried in system information in the serving and neighbour cells. However, for network energy saving requirements, the architecture of sync signals and system information is likely to change. For example, the number of always-on signals may be further reduced, or they are transmitting more sparsely. It is necessary to identify the impact of these changes on the cell reselection process.

Proposal 5: Identify the impact from the design of 6G system information and sync signals on the cell selection/reselection process.

2.3 Measurement

In order to achieve the unified mobility, a unified measurement framework would also be pursued. Generally speaking, a unified measurement framework for 6G should cover 5G functions of L1 and L3 measurement and report.

In 5G, two separate mechanisms are introduced for L1 measurement and L3 measurement. Take the measurement report for example. The configuration for triggering measurement report includes the configuration for periodic/aperiodic/semi-persistent report and the configuration for event-triggered report. Both the configuration for triggering the report and the configuration for measurement are configured independently for L1 measurement and L3 measurement. Due to the overlap between the configuration related to L1 measurement and report and that related to L3 measurement and report, this will result in larger signalling overhead. In addition, due to the possible redundancy between the L1 reporting and the L3 reporting, the impact of signalling overhead will be further increased.

To address the issues caused by the 5G design, unified L1 and L3 measurement and report can be considered for unified design.  That is, make a unified mobility decision based on L1 and L3 measurement report. A common measurement configuration and a common 	trigger mechanism for L1and L3 measurement and report can be considered. If the common trigger mechanism is applied, it is natural that both L1 and L3 measurement report will be triggered by the same one condition. To further unified the measurement and report, a general measurement framework merges L1 and L3 measurement and report. With this enhancement, there is no need to distinguish L1 and L3 measurement and report. However, the detailed discussions are still needed in the subsequent study.

Proposal 6: Unified measurement framework for L1 /L3 measurement and report should be introduced.

- Conclusions

In this contribution, we discussed mobility related issues and made the following observations and proposals:

Observation 1: The 5G cell switch mechanisms triggered by different node (i.e., NW triggered, or UE triggered) based on cross layer mechanism (e.g., L3 HO or L1/L2 based cell switch) aim to different scenarios and requirements.

Proposal 1: Support both UE triggered switch mechanism and NW signalling based switch mechanism from day 1. Further discuss whether a unified configuration framework could be used.

Proposal 2: Introduce a cell switch mechanism for cross layer switch (e.g., L1/L2/L3 cell switch) based on a unified framework.

Proposal 3: Consider the unified the design for the cell switch and beam management. Inter-WG work with RAN1 is required.

Proposal 4: Consider 5G cell selection/reselection process as a starting point and try to optimize the cell selection/reselection process to simplify the design complexity, adapting to diverse 6G scenarios.

Proposal 5: Identify the impact from the design of 6G system information and sync signals on the cell selection/reselection process.

Proposal 6: Unified measurement framework for L1 /L3 measurement and report should be introduced.

- References

- RP-252912 Revised SID: Study on 6G Radio