3GPP RAN WG2 Meeting #131-bis                                                                                R2-2507038

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	9.2.2

Source:	HONOR

Title:	Discussion on A-IoT resource allocation for T2

Document for:	Discussion and Decision

- Introduction

R20 A-IoT WID [1] has revealed the consideration of A-IoT radio resource allocation for the UE reader in A-IoT topology 2 (T2). In the RAN2#128 meeting [2], A-IoT T2 was discussed and following agreements have been made:

Agreements

- Resource validity across multiple cells is not supported in the initial release.

- Include in the TR the resource validity is under network control.  CB which option we include.

- We will capture the following options in the TR for resource validity in different cases when the resources are allocated semi-statically by the network:

- Reader UE performing A-IoT operation in RRC_CONNECTED (downselection or support of both options will be decided in the WI phase):

- Option 1: UE receives resources in dedicated signalling.  The resources remain valid until the network releases them explicitly (understanding is this would be supported by RRC reconfiguration)

- Option 2: UE receives resources in dedicated signalling along with a time period in which they can be used.  The resources remain valid for that time period.

- Reader UE performing A-IoT operation in RRC_INACTIVE (downselection or support of both options will be decided in the WI phase):

- Option 1: UE receives resources in dedicated signalling before moving to RRC_INACTIVE.  The resources remain valid until the network releases them explicitly.

- Option 2: UE receives resources and a time period in dedicated signalling before moving to RRC_INACTIVE.  The resources remain valid for that period of time.

- Reader UE performing A-IoT operation in RRC_CONNECTED and upon “temporary out of connection” during the operation. (downselection between the following options will be performed in the WI phase)

- Option 1: Resources remain valid at the reader for a time period (FFS whether the same time period as in RRC_CONNECTED options or a different time period.  FFS whether the time period is related to the ongoing A-IoT operation).

- Option 2: Resources are temporarily invalid at the reader during the “temporary out of connection” condition but may become valid again if the UE recovers from the condition.

- If reader UE supports A-IoT operation in RRC_IDLE, options of RRC_INACTIVE are candidates.

- Support of RRC_IDLE (other than in temporary out of connection) is down-prioritized.   During WI can consider it if feasible with minimum spec impact

For all options

The resources remain valid only in the same cell for which the resources were configured.

In this contribution, we provide our views on A-IoT resource allocation of UE reader in T2.

- Discussion

  - Resource Allocation Modes

As outlined in RAN2#127 agreement [3], the network (e.g., gNB) is in control of resources to be used by A-IoT air interface. Based on the current specification [4],  three baseline assumptions for A-IoT resource allocation can be considered: dynamic grant (e.g., DCI via PDCCH indicating resources in every transmission time interval), semi-statical/persistent (e.g., uplink configured grant type 1/2 where RRC provides period, time offset and frequency resource configurations with or without activation signaling) and the shared resource allocation (e.g., Sidelink Mode 2 with a shared resource pool).

RAN2#127-bis has agreed that only dedicated resource configuration instead of shared resource pool should be provided to the UE reader via dedicated signaling, and R20 WID [1] revealed the dedicated signaling is RRC signaling. Therefore, our understanding is that RAN2 shall consider the configured grant (CG)-like resource allocation to a specific UE reader.

R20 A-IoT WID

The work is subject to the following constraints:

RAN2

- The objective includes specification of necessary Uu procedures to enable the A-IoT operations.

- RRC CONNECTED UE reader only, including RLF, without specific enhancement for other temporary out-of-connection scenarios.

- A-IoT radio resource of the UE reader is only valid within the serving cell.

- Resource allocation to the UE Reader shall be via Uu RRC signaling.

Referring to the configured grant scheduling introduced in Rel-16, we believe that at least these three configured grant-like A-IoT resource allocation methods should be supported in T2:

- CG-like Type 1: The RRC signaling includes all resource parameters for A-IoT communication, such as period, time offset, frequency domain resources, and possible modulation and coding methods for R2D/D2R transmission. It becomes effective immediately upon the UE’s correct reception of the A-IoT resources.

- CG-like Type 2: The RRC signaling initially provides partial A-IoT communication resource parameters (e.g., frequency domain). The remaining configuration (e.g., time domain) is subsequently added.

- CG-like Type 3: The RRC signaling encompasses all A-IoT communication resource parameters (e.g., frequency, cycle). Activation or deactivation of the configuration is controlled by a subsequent message (e.g., DCI, MAC CE, RRC signaling). These resources remain valid until an explicit deactivation message is received.

Proposal 1: RAN2 should agree that the configured grant-like resource allocations for A-IoT communications are supported, which may include

- CG-like Type 1: The RRC signaling contains all A-IoT resource parameters and takes effect immediately upon correct reception by the UE.

- CG-like Type 2: The RRC signaling initially provides partial A-IoT resource parameters, with the remaining added later.

- CG-like Type 3: The RRC signaling includes all A-IoT resource parameters, which are activated by a subsequent message and remain valid until deactivation.

Additionally, a gNB’s coverage area is typically much larger than that of UE reader. Ideally, a gNB should select only one UE reader within a single or multiple adjacent A-IoT area (as defined in RAN3 WG). This enables UE readers located far apart under the same gNB to reuse the same A-IoT time-frequency resources. Thus, the gNB can consider the A-IoT resource reuse based on the proximity of UE readers, thereby enhancing the system capacity within the gNB. Specifically, UE can provide its detailed positions (e.g., the A-IoT area ID where it is located) to assist the gNB in A-IoT resource configuration; and the gNB can provide a predefined A-IoT resource configuration related to specific A-IoT serving area for UE selection via SIB or dedicated signaling (e.g., A-IoT service request).

Proposal 2: gNB can support the A-IoT resource allocation based on the UE readers’ positions.

  - Information Obtained by the gNB for allocation

RAN2#127 agreement has indicated that the resource allocation is the gNB’s responsibility. According to the SA2’s discussion [5], the following assistance information is visible to gNB, which could also be helpful for gNB to estimate and configure resource for A-IoT air interface between UE reader and A-IoT devices in T2. For example, the gNB will allocate resource according to service type or the number of target devices, e.g., more resources will be allocated for full inventory or a large number of devices. Moreover, gNB could consider the message size for (expected or maximum) D2R and R2D transmissions as an important factor for resource configuration, especially when the upper layer packet size is not fixed or the packet sizes for different A-IoT services (e.g., inventory or read/write command) differ.

TS 23.369 clause 5.4

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests.

For Inventory or Command service operation, following Inventory Assistance Information is included in the Inventory Request from AIOTF to NG-RAN:

a) A-IoT service type (e.g. Inventory, Command).

b) Optionally, approximate number of A-IoT devices based on AF request.

c) Size of the Inventory Response message from the A-IoT Device.

d) Optionally, time interval for A-IoT Device response aggregation used by the NG-RAN as specified in clause 5.9.

For Command service operation, following Command Assistance Information is included in the Command Request from AIOTF to NG-RAN:

e) Size of the Command Response message from the A-IoT Device.

Proposal 3: The assistance information provided to the gNB should be used as a reference for A-IoT resource allocation by the gNB.

In addition to AIOTF, UE reader can also serve as an assistance provider to enhance the A-IoT resource allocation. For example, it can provide the UE’s location (e.g., identified by the A-IoT area ID) and measure results (e.g., received signal strength from A-IoT devices).

Proposal 4: In addition to CN, UE reader can provide the assistance information (e.g., UE’s location and measurement results) to enhance the A-IoT resource allocation.

  - Resource Validity for RLF case

R20 A-IoT WID has agreed RAN2 will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). In RAN2#128 discussion, for UE reader performing A-IoT operation in RRC_CONNECTED and upon “temporary out of connection” during the operation, two options of resource validity are proposed for down-selection in the WI phrase.

Reader UE performing A-IoT operation in RRC_CONNECTED and upon “temporary out of connection” during the operation. (downselection between the following options will be performed in the WI phase) - Option 1: Resources remain valid at the reader for a time period (FFS whether the same time period as in RRC_CONNECTED options or a different time period. FFS whether the time period is related to the ongoing A-IoT operation). - Option 2: Resources are temporarily invalid at the reader during the “temporary out of connection” condition but may become valid again if the UE recovers from the condition.

In the legacy RRC re-establishment procedure, the UE will start T311 after transmitting the RRCReestablishmentRequest. If the UE receives an RRCReestablishment or RRCReestablishmentReject before T311 expires, T311 will stop. If the T311 expires, the UE will transition to the RRC_IDLE state. Currently, the T311 value options are: 1000ms, 3000ms, 5000ms, 10000ms, 15000ms, 20000ms, 30000ms, with the default value of 30000ms [6]. The time required for the RRC re-establishment process may be long even compared to the entire A-IoT procedure, and UE can continue the A-IoT operation during the RLF to avoid the waste of resources and time, particularly in scenarios where RLF occurs repeatedly (e.g., the channel quality of the serving cell is poor). Therefore, we prefer Option 1 for resource validity in the case of RLF.

Proposal 5: Support Option 1 for resource validity during RLF case.

In addition, we believe that the choice of option may depend on whether the UE needs to respond the previously received A-IoT service results to the gNB after recovering from RLF to the RRC-connected state. For example, if the UE is expected to send the A-IoT data that was received from A-IoT devices but not transmitted to gNB after re-establishing the connection, it would be beneficial for the UE to continue the A-IoT operation during RLF. However, if the UE discards the previous data upon RLF, then setting an additional resource validity duration for the RLF case (Option 1) would also be meaningless.

Proposal 6：RAN2 to discuss whether the UE could send the received A-IoT data after recovering from RLF.

- Conclusions

In this contribution, issues related to A-IoT resource allocation for T2 have been discussed and made the following proposals:

Proposal 1: RAN2 should agree that the configured grant-like resource allocations for A-IoT communications are supported, which may include

- CG-like Type 1: The RRC signaling contains all A-IoT resource parameters and takes effect immediately upon correct reception by the UE.

- CG-like Type 2: The RRC signaling initially provides partial A-IoT resource parameters, with the remaining added later.

- CG-like Type 3: The RRC signaling includes all A-IoT resource parameters, which are activated by a subsequent message and remain valid until deactivation.

Proposal 2: gNB can support the A-IoT resource allocation based on the UE readers’ positions.

Proposal 3: The assistance information provided to the gNB should be used as a reference for A-IoT resource allocation by the gNB.

Proposal 4: In addition to CN, UE reader can provide the assistance information (e.g., UE’s location and measurement results) to enhance the A-IoT resource allocation.

Proposal 5: Support Option 1 for resource validity during RLF case.

Proposal 6: RAN2 to discuss whether the UE could send the received A-IoT data after recovering from RLF.

- References

- RP-251885

- RAN2#128 Chairman notes

- RAN2#127 Chairman notes

- TS 38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- TS 23.369, “Architecture support for Ambient power-enabled Internet of Things; Stage 2”, 3GPP

- TS 38.311, “NR; Radio Resource Control (RRC); Protocol specification”, 3GPP