3GPP TSG-RAN WG2 Meeting #131bis                                                              R2-2507050

Prague, 13th –17th October, 2025

Agenda Item:	8.5.2

Source: 	Xiaomi

Title:  	RIL X200, X201, X202, X203 and remaining open issues for NES

Document for:	Discussion and Decision

1	Introduction

During the ASN.1 review, we have raised several RIL issues. In addition, there are several open issues identified by Rapporteur. In this contribution, we would like to discuss these issues and provide corresponding text proposals.

2	Discussion

2.1 RILs

2.1.1 X200

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X200 | NES | 1 | SMTC handling for OD-SSB | R2-25xxxxx | Xiaomi (Li Zhao) |  | V003 | ToDo |

[Description]: The existing text in 5.5.2.10 still needs to be further polished to reflect the mapping between the SMTC and the OD-SSB periodicity. However, there are different Alts to reflect this mapping.

Alt1: there is a 1-1 mapping between the OD-SSB periodicity value and the SMTC, no matter whether there is an OD-SSB configuration for a certain OD-SSB periodicity

Alt2: there is a 1-1 mapping between the SSB periodicity of OD-SSB configuration(s) and SMTC, only the OD-SSB periodicity associated with an OD-SSB configuration has a corresponding SMTC.

Based on the comments from the other companies, Alt2 is preferred. This is because if there is a periodicity for which there is no corresponding OD-SSB periodicity, following Alt1, the SMTC has to be configured within smtcxlist (e.g., if there are OD-SSB configuration for ms5 and ms20, but there is no OD-SSB configuration for ms10), leading to unnecessary signaling overhead.

Actually, we think both Alt1 and Alt2 can work, for Alt1, even there is a SMTC for each OD-SSB periodicity, considering there are only six candidate OD-SSB periodicity values, the signalling overhead is not a big issue. In addition, it is not sure if Alt2 can describe the intention clearly. Thus we propose TPs for both Alts and we can make down selection based on majority view.

TP for Alt1:

5.5.2.10 Reference signal measurement timing configuration <text omitted> “If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as the first OD-SSB periodicity value of od-ssb-Periodicity-r19; the UE shall setup SMTC according to the second configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as the second OD-SSB periodicity value of od-ssb-Periodicity and so on”

TP for Alt2:

5.5.2.10 Reference signal measurement timing configuration <text omitted> “If smtcxlist is present, when OD-SSB is activated and the serving cell is activated, the UE shall setup SMTC according to the first configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as the first value among OD-SSB periodicity value(s) in OD-SSB configuration(s) for the serving cell; the UE shall setup SMTC according to the second configured field in smtcxlist for serving cell measurements on the corresponding configured measurement object as specified in 5.5.3.1, if the SS/PBCH block reception periodicity is configured as the second value among OD-SSB periodicity value(s) in OD-SSB configuration(s) for the serving cell and so on”

Proposal 1: RAN2 to discuss and down select between Alt1 and Alt2 for the mapping between OD-SSB periodicity and SMTC.

- Alt1: 1-1 mapping between the OD-SSB periodicity value and the SMTC, no matter whether there is an OD-SSB configuration for a certain OD-SSB periodicity

- Alt2: 1-1 mapping between the SSB periodicity of OD-SSB configuration(s) and SMTC, only the OD-SSB periodicity associated with an OD-SSB configuration has a corresponding SMTC

2.1.2 X201

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X201 | NES | 1 | Serving cell MO handling for OD-SSB | R2-25xxxxx | Xiaomi (Li Zhao) |  | V003 | ToDo |

[Description]: The existing text in 5.5.3.1 still needs to be further polished to make it more readable and solve the conflict between the level 2 “absoluteFrequencySSB is not configured” and level 3 “and absoluteFrequencySSB is configured in ServingCellConfigCommon”

Thus, it is proposed to relocate the level 2 conditions to solve the conflict between the level 2 “absoluteFrequencySSB is not configured” and level 3 “and absoluteFrequencySSB is configured in ServingCellConfigCommon”. Based on the comments from the other companies, there was a concern that the one of “if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated” should be relocated, but that should rely on servingCellMO-OD so should combine with the branch of servingCellMO-OD.

The key issue is for case 1 (no AO-SSB), when OD-SSB is activated, whether the measurement is based on servingCellMO or servingCellMO-OD. Actually, there are two situations

Situation 1: the ssbFrequency in servingCellMO is same as od-ssb-absoluteFrequency

Situation 2: the ssbFrequency in servingCellMO is different from od-ssb-absoluteFrequency

For situation 1, there is no need to configure servingCellMO-OD and when OD-SSB is activated, the measurement is based on servingCellMO while for situation 2, servingCellMO-OD is configured and the measurement is based on servingCellMO-OD. If so, we need more branches to differentiate situation 1 and situation 2 in the text procedure. Another alt is to mandate using servingCellMO (rather than servingCellMO-OD) for case 1 (no AO-SSB), i.e., does not pursue the situation where the ssbFrequency in servingCellMO is different from od-ssb-absoluteFrequency.

Proposal 2: RAN2 to agree for case 1 (no AO-SSB) when OD-SSB is activated, the measurement is based on servingCellMO, i.e., does not pursue the situation where the ssbFrequency in servingCellMO is different from od-ssb-absoluteFrequency.

The proposed TP is shown below. RAN2 is suggested to discuss and agree the TP.

5.5.3 Performing measurements 5.5.3.1 General <text omitted> The UE shall: 1> whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows: 2> if the OD-SSB-Config is not configured 3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO, and absoluteFrequencySSB is configured in ServingCellConfigCommon: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 4> derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3; 3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a; 4> derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3; 2> if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is not configured, or: 2> if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated, or: 2> if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is not activated: 3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 4> derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3; 3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a; 4> derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3; 2> else if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is activated: 3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO-OD: 4> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb: 5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 4> derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3; 3> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO-OD: 4> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs: 5> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a; 4> derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3; 1> for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity: 2> if the OD-SSB-Config is not configured: 3> if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO, and absoluteFrequencySSB is configured in ServingCellConfigCommon: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 4> derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3; 3> if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a; 4> derive serving cell SINR based on CSI-RS, as described in 5.5.3.3; 2> if the OD-SSB-Config and absoluteFrequencySSB are configured and od-ssb-absoluteFrequency is not configured, or: 2> if the OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated, or: 2> if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is not activated: 3> if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 4> derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3; 3> if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a; 4> derive serving cell SINR based on CSI-RS, as described in 5.5.3.3; 2> else if the OD-SSB-Config, absoluteFrequencySSB and od-ssb-absoluteFrequency are configured and OD-SSB transmission is activated: 3> if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO-OD: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 4> derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3; 3> if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO-OD: 4> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 5> derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a; 4> derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;

2.1.3 X202

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X202 | NES | 1 | Classify parameters of OD-SIB1 | R2-25xxxxx | Xiaomi (Li Zhao) |  | V003 | ToDo |

[Description]: According to the parameter list from RAN1, some parameters related to OD-SIB1 should be configured per od-sib1-Config instead of per SIB1-RequestConfig, e.g., sib1-rsrp-ThresholdSSB-r19, locationAndBandwidth-r19, absoluteFrequencyPointA-r19, ul-FrequencyBandList-r19, ul-SubCarrierSpacing-r19, etc.

Even there is no functional difference as all parameters are in the highest level, however, it is not sure if there is any issue regarding future extension. In addition, there is no motivation to deviate from the RAN1 parameter list. Thus, we suggest to follow the signalling structure provided by RAN1 and agree the TP below. During the email discussion, there was an argument that the current gouping is to support SUL. However, we think to support SUL, a separate sib1-RequestConfigSUL within OD-SIB1-Config and a rsrp-ThresholdSSB-SUL within SIB1-RequestConfig is sufficient and aligned with the legacy on-demand SI configuration. Please note in legacy, to support SUL for on-demand SI request, a separate si-RequestConfigSUL configured within SI-SchedulingInfo is defined and refers to the IE SI-RequestConfig which only includes the rach-OccasionsSI, si-RequestPeriod and si-RequestResources, see below.

#### –	SI-RequestConfig

The IE SI-RequestConfig contains configuration for Msg1 based SI request without Msg1 repetition. For the random access parameters (e.g. prach-RootSequenceIndex, msg1-SubcarrierSpacing, etc.) not configured in SI-RequestConfig, the UE applies the parameters configured in rach-ConfigCommon corresponding to the RACH resource set selected upon RACH initialization (as specified in TS 38.321 [3]), of the initial uplink BWP unless otherwise specified.

SI-RequestConfig information element

-- ASN1START

-- TAG-SI-REQUESTCONFIG-START

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SI-REQUESTCONFIG-STOP

-- ASN1STOP

Thus, we think some paraemeters within SIB1-RequestConfig, e.g., sib1-rsrp-ThresholdSSB-r19, locationAndBandwidth-r19, absoluteFrequencyPointA-r19, ul-FrequencyBandList-r19, ul-SubCarrierSpacing-r19 has noting to do with SUL and should be moved out.

Proposal 3: RAN2 to agree to follow the signalling structure provided by RAN1 parameter list for OD-SIB1 configuration.

#### –	SIBxx

SIBxx contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-sib1-ConfigList-r19         SEQUENCE (SIZE(1..maxNrofOD-SIB1-r19))  OF OD-SIB1-Config-r19,

lateNonCriticalExtension       OCTET STRING                                                     OPTIONAL,

...

}

OD-SIB1-Config-r19  ::=       SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellIdList-r19                       SEQUENCE (SIZE(1.. maxPCI-OD-SIB1-r19)) OF PhysCellId,

offsetToCarrier-r19                      INTEGER (0..2199),

carrierBandwidth-r19                     INTEGER (1..maxNrofPhysicalResourceBlocks)                                 OPTIONAL, -- Need R

offsetToPointA-r19                       INTEGER (0..2199)                                                          OPTIONAL, -- Cond TDD

frequencyBandList-r19                    MultiFrequencyBandListNR-SIB                                               OPTIONAL, -- Need R

ssb-Periodicity-r19                      ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2}            OPTIONAL, -- Need R

od-sib1-WindowDuration-r19               ENUMERATED {ms20, ms40, ms80, ms160, ms320, spare1, spare2, spare3}        OPTIONAL, -- Need R

od-sib1-windowStartOffset-r19            ENUMERATED {sl0, sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

k-ssb-r19                                INTEGER (1..23),

controlResourceSetZero-r19               ControlResourceSetZero,

searchSpaceZero-r19                      SearchSpaceZero,

sib1-RequestConfig-r19                   SIB1-RequestConfig-r19,

sib1-RequestConfigSUL-r19                SIB1-RequestConfig-r19                                                     OPTIONAL, -- Need R

sib1-PDCCH-RestrictionToPRACH-r19       ENUMERATED {true},

locationAndBandwidth-r19                 INTEGER (0..37949)                                                            OPTIONAL, -- Need R

absoluteFrequencyPointA-r19              ARFCN-ValueNR                                                                 OPTIONAL, -- Cond FDD

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB                                                  OPTIONAL, -- Need R

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480-v1700, kHz960-v1700}   OPTIONAL, -- Need R

p-Max-r19                                P-Max                                                                         OPTIONAL, -- Need R

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}          OPTIONAL,  -- Need R

ra-SearchSpace-r19                      SearchSpace                                                                    OPTIONAL, -- Need S

n-TimingAdvanceOffset-r19               ENUMERATED { n0, n25600, n39936, spare1 }                                      OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19              ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}      OPTIONAL, -- Need R

prach-RootSequenceIndex-r19             CHOICE {

l839                                       INTEGER (0..837),

l139                                       INTEGER (0..137)

}                                                                                                                    OPTIONAL, -- Need R

sib1-rsrp-ThresholdSSB-r19               RSRP-Range                                                                    OPTIONAL, -- Need R

}

SIB1-RequestConfig-r19 ::=                SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                  SEQUENCE {

ssb-perRACH-Occasion-r19                  ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

},

sib1-RequestPeriod-r19                  ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}                  OPTIONAL, -- Need R

sib1-RequestResources-r19               SIB1-RequestResources-r19,

rach-ConfigSIB1-r19                     RACH-ConfigGeneric,

rsrp-ThresholdSSB-SUL                    RSRP-Range                                                                    OPTIONAL, -- Cond SUL

sib1-TDD-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                     OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower-r19                   INTEGER (-60..50)                                                          OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                 SEQUENCE {

inOneGroup                             BIT STRING (SIZE (8)),

groupPresence                          BIT STRING (SIZE (8))                                                      OPTIONAL  -- Cond FR2-Only

}

}

SIB1-RequestResources-r19 ::=             SEQUENCE {

sib1-RA-PreambleStartIndex-r19               INTEGER (0..63),

sib1-RA-AssociationPeriodIndex-r19           INTEGER (0..15)                                                      OPTIONAL,   -- Need R

sib1-RA-SSB-OccasionMaskIndex-r19            INTEGER (0..15)                                                      OPTIONAL    -- Need R

}

-- TAG-SIBxx-STOP

-- ASN1STOP

2.1.4 X203

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X203 | NES | 1 | Using NUL/SUL for OD-SIB1 request | R2-25xxxxx | Xiaomi (Haitao) |  | V003 | ToDo |

[Description]: Description on OD-SIB1 request for NUL and SUL repeats quite much and make spec messy.

We suggest to merge the SUL and NUL parts to simplify the spec. below is the suggestest TP.

5.2.2.3.3x Request for on-demand SIB1 The UE shall, while SDT procedure is not ongoing: 1> if od-SIB1-Config for this cell in stored valid version of SIBxx includes sib1-RequestConfig and/or sib1-RequestConfigSUL: 2> trigger the lower layer to initiate the Random Access procedure on the selected uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfig (if normal uplink is selected) or in sib1-RequestConfigSUL (if supplementary uplink is selected) included in od-sib1-Config for this cell in stored valid version of SIBxx; 3> if indication that maximum number of PRACH attempts as configured in sib1-RequestConfig (if normal uplink is selected) or in sib1-RequestConfigSUL (if supplementary uplink is selected) is reached is received from lower layers as defined in TS 38.321 [3]: 4> perform the actions as specified in clause 5.2.2.5. 3> if acknowledgement for SIB1 request is received from lower layers: 4> acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately; 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2; 1>

Proposal 4: Adopt the above TP to merge the SUL and NUL parts.

2.2 Measurement gap (open issue 38300-1)

Issue description: Capturing measurement gap behaviour for OD-SSB, either in a Note, or by normative text. Clarification what is the meaning of “SSB associated to the initial DL BWP” in the measurement gap section in 9.2.4, whether it refers to AO-SSB or OD-SSB.

Possible options to resolve the issue:

- Normative text as e.g. provided by rapp in CR v04 (Appendix TP1)

- Note as e.g. provided by in the collection of comments in Apple001 (Appendix TP2)

- No modification of the measurement gap paragraph

Based on the offline comments, there were different interpretations on the following RAN1 spec.

When the first SS/PBCH blocks in a configured DL BWP can be used to obtain SIB1 and the frequency location of the first SS/PBCH blocks corresponds to the GSCN of a synchronization raster entry, the UE expects

-	a frequency location of the second SS/PBCH blocks to be different from the frequency location of the first SS/PBCH blocks and not to correspond to the GSCN of a synchronization raster entry

-	frequency resources of the second SS/PBCH blocks not to overlap with frequency resources of the first SS/PBCH blocks

-	the second SS/PBCH blocks to be within the configured DL BWP as the first SS/PBCH blocks

-	the second SS/PBCH blocks are not used to obtain SIB1

When the first SS/PBCH blocks in a configured DL BWP cannot be used to obtain SIB1, the UE expects

-	a same frequency location for the second SS/PBCH blocks and for the first SS/PBCH blocks

-	a same PBCH payload, other than the SFN index and the half frame index, for a first SS/PBCH block, from the first SS/PBCH blocks, and for a second SS/PBCH block, from the second SS/PBCH blocks, with same SS/PBCH block index as the first SS/PBCH block

Some companies think there is no case, for a SSB-based Scell, there is a configured BWP, which covers one SSB but not the other, i.e., all BWP are configured with both AO-SSB and OD-SSB. However, we don’t think there is a such a restriction. This is because the text from RAN1 requires the OD-SSB to be on the same BWP as AO-SSB for SIB1-BWP while on the same frequency as AO-SSB for non-SIB1 BWP if there is OD-SSB, however, it does not mean OD-SSB is mandatory configured on all the BWP. Based on this assumption, we think it is necessary to clarify which SSB frequency is utilized to determine the necessity of measurement gap considering the condition is “if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP”. We prefer the note based solution.

Proposal 5: RAN2 to agree to add a note that when the serving cell is associated with both SSB and OD-SSB in different frequency, the SSB associated to the initial DL BWP includes both SSB and OD-SSB.

3. Conclusions

Based on the above discussion, we propose following proposals:

Proposal 1: RAN2 to discuss and down select between Alt1 and Alt2 for the mapping between OD-SSB periodicity and SMTC.

- Alt1: 1-1 mapping between the OD-SSB periodicity value and the SMTC, no matter whether there is an OD-SSB configuration for a certain OD-SSB periodicity

- Alt2: 1-1 mapping between the SSB periodicity of OD-SSB configuration(s) and SMTC, only the OD-SSB periodicity associated with an OD-SSB configuration has a corresponding SMTC

Proposal 2: RAN2 to agree for case 1 (no AO-SSB) when OD-SSB is activated, the measurement is based on servingCellMO, i.e., does not pursue the situation where the ssbFrequency in servingCellMO is different from od-ssb-absoluteFrequency.

Proposal 3: RAN2 to agree to follow the signalling structure provided by RAN1 parameter list for OD-SIB1 configuration.

Proposal 4: Adopt the above TP to merge the SUL and NUL parts.

Proposal 5: RAN2 to agree to add a note that when the serving cell is associated with both SSB and OD-SSB in different frequency, the SSB associated to the initial DL BWP includes both SSB and OD-SSB.

- Reference

[1] ASN.1 review comment file

[2] Remaining 38300 open issues