3GPP TSG-RAN Meeting #131-bis	R2-2507056

# Prague, Czech, 13th  – 17th October,  2025

Agenda item:	8.7.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on remaining issues for RLC in R19 XR

Document for:	Discussion and Decision

- Introduction

We discuss on the remaining issues for RLC spec for R19 XR

- Discussion

In section 5.3.4, the content when STATUS PDU is triggered by other triggering conditions are reused. However, when t-RxDiscard expires, RX_Next is updated to RX_Next_Discard_Trigger and after the update, RX_Next might be larger than RX_Highest_Status.

Hence, the following condition is wrong, which always implies that the RX_Next< RX_Highest_Status.

- for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

When the status report is triggered by t-RxDiscard, because the RX_Next is updated to the position of RX_Next_Discard_Trigger, all the RLC SDUs below RX_Next_Discard_Trigger will be discarded, a straightforward solution would be just to set the ACK_SN to the updated RX_Next. Then, the SDUs before RX_Next are indicated as ACK

The TP can be follows:

When constructing a STATUS PDU not triggered by the expiry of t-RxDiscard, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: =====omitted=== When constructing a STATUS PDU triggered by the expity of t-RxDiscard, the AM RLC entity shall: - set the ACK_SN as the updated RX_Next

Hence, we propose the following

Proposal1: (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Specify the content of the RLC STATUS PDU triggered by the t-RxDiscard expiry in a separate condition.

In legacy RLC AM, since RLC SDU will never be lost, the RLC SDUs with SN smaller than RX_Next can be considered as completely received, as given below [2]:

The receiving side of each AM RLC entity shall maintain the following state variables: a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

However, the introduction of t-RxDiscard enables the Rx RLC entity to update RX_Next even the RLC SDU is not received and hence the definition of the variable should be updated according. To avoid impacting the legacy behaviour of the Rx RLC entity, it is suggested to separately introduce a new definition when stopReTxDiscardedSDU is configured, as given below:

The receiving side of each AM RLC entity shall maintain the following state variables: a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received or discarded (as specified in clause 5.2.3.2.x) RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

Proposal2: (RLC-N01) Additional definition of RX_Next should be introduced according to behaviour on expiry of t-RxDiscard without impacting legacy definition.

Moreover, when stopReTxDiscardedSDU is configured, there can be an issue that there is only outdated RLC SDU buffered at Tx RLC entity when t-PollRetransmit expires. To address the issue, it was agreed that t-PollRetransmit can be stopped and reset as explained in the newly added note below [3]:

5.3.3.3 Reception of a STATUS report Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall: - if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit. NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

As the issue is introduced by the configuration of stopReTxDiscardedSDU, it is suggested to clarity the case of stopping and resetting of the running t-PollRetranmsit is about configuration of stopReTxDiscardedSDU.

5.3.3.3 Reception of a STATUS report Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall: - if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit. NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Proposal3: (RLC-N02) The UE procedure captured in the NOTE for stopping and resetting the running t-PollRetransmit due to avoiding unnecessary retransmission should only be applicable when stopReTxDiscardedSDU is configured.

- Conclusion

This contribution makes the following proposals:

Proposal1: (RLC-H02) Confirm that when t-RxDiscard expires, RX_Next might be larger or equal than RX_Highest_Status. Specify the content of the RLC STATUS PDU triggered by the t-RxDiscard expiry in a separate condition.

Proposal2: (RLC-N01) Additional definition of RX_Next should be introduced according to behaviour on expiry of t-RxDiscard without impact on legacy definition.

Proposal3: (RLC-N02) The UE procedure captured in the NOTE for stopping and resetting the running t-PollRetransmit due to avoiding unnecessary retransmission should only be applicable when stopReTxDiscardedSDU is configured.