3GPP TSG-RAN WG2 Meeting #132															R2- 2507058

Prague, Czech, 13-17 Oct, 2025

Agenda item:	8.7.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on non-delay-reporting PDCP SDU definition

Document for:	Discussion and Decision

1. Introduction

After RAN2#131, RAN2 has made a lot of progress. In this contribution, we further discuss the remaining issues for XR with respect to PDCP.

2.	Definition of non-delay-reporting SDU

The PDCP data volume calculation for DSR procedure is specified as below [1]:

- the delay-reporting PDCP data volume is determined in ascending order of DSR-ReportingThreshold, and

- the data volume of non-delay-reporting PDCP SDUs are also considered as part of delay-reporting PDCP data volume if dsr-ReportNonDelayCriticalData is configured.

For the purpose of MAC multiple entry delay status reporting, the transmitting PDCP entity shall evaluate the delay-reporting PDCP data volume in ascending order of DSR-ReportingThreshold, and consider the following as delay-reporting PDCP data volume associated with the i:th DSR-ReportingThreshold: - the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th DSR-ReportingThreshold where k < i; - the PDCP Data PDUs that contain the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th DSR-ReportingThreshold where k < i; - if dsr-ReportNonDelayCriticalData is configured: - the non-delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold for which no PDCP Data PDUs have been constructed, and are not considered as delay-reporting PDCP data volume associated with any of the k:th DSR-ReportingThreshold where k < i; - the PDCP Data PDUs that contain the non-delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold and have not been submitted to lower layers, and are not considered as delay-reporting PDCP data volume associated with any of the k:th DSR-ReportingThreshold where k < i; - if i = 1, the PDCP Control PDUs; - if i = 1, for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - if i = 1, for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

It can be seen that for the case where dsr-ReportNonDelayCriticalData is configured, if the data volume of a non-delay-reporting PDCP SDU is calculated to kth DSR-ReportingThreshold, then it will not be considered as a non-delay-reporting PDCP SDU associated with ith DSR-ReportingThreshold where k < i.

However, in the current running CR, the definition for non-delay-reporting SDU as below [1]:

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold and that is not a delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold.

In the above definition, the PDCP SDUs that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold are considered as the non-delay-reporting SDUs if they are not considered as delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold. However, this definition does not exclude the SDUs associated with the k:th DSR-ReportingThreshold where k < i, including the delay-reporting SDUs associated with the k:th DSR-ReportingThreshold and the non-delay-reporting SDUs associated with the k:th DSR-ReportingThreshold. It is not aligned with the text procedure as specified in the data volume calculation.

To align with the text procedure for data volume calculation, the definition of non-delay-reporting PDCP SDU should be simply changed as: “a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<=i.”.

Proposal 1:	To align with the procedure description as in data volume calculation, the definition of non-delay-reporting PDCP SDU should be changed as “a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<=i”

3. Conclusion

We summarize the discussions in the paper as follows:

Proposal 1:	To align with the procedure description as in data volume calculation, the definition of non-delay-reporting PDCP SDU should be changed as “a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k<=i”

4. Reference

[1]. R2-2506337 Introduction of R19 XR enhancements for PDCP spec, LG Electronics Inc., RAN2#131 Bangalore, India, 25 – 29 August 2025

Appendix. TP for TS 38.323

Start of change

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AM DRB: a data radio bearer which utilizes RLC AM.

AM MRB: an MRB associated with at least one AM RLC bearer for PTP transmission.

Broadcast MRB: a radio bearer configured for MBS broadcast delivery.

DAPS bearer: a bearer whose radio protocols are located in both the source gNB and the target gNB during DAPS handover to use both source gNB and target gNB resources.

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

Delay-reporting PDCP SDU: if pdu-SetDiscard is not configured, a delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU for which the remaining time till discardTimer expiry is less than the i:th DSR-ReportingThreshold and larger than or equal to the i-1:th DSR-ReportingThreshold (if i>1) or larger than zero (if i=1). If pdu-SetDiscard is configured, a delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is a PDCP SDU belonging to a PDU Set of which the PDU Set remaining time is less than the i:th DSR-ReportingThreshold and larger than or equal to the i-1:th DSR-ReportingThreshold (if i>1) or larger than zero (if i=1).

MBS Radio Bearer: a radio bearer that is configured for MBS delivery.

Multicast MRB: a radio bearer configured for MBS multicast delivery.

Multi-path: Mode of operation of a remote UE in RRC_CONNECTED configured with one direct path on which the UE connects to the gNB using NR Uu and one indirect path on which the UE connects to the same gNB via another UE using PC5 unicast link or Non-3GPP Connection.

Multi-path Primary Path: In multi-path for a split DRB, the primary path is configured by RRC to be either the direct path or the indirect path. In multi-path for a split SRB, the primary path is always the direct path.

Multi-path Secondary Path: In multi-path for a split DRB, the path (either direct or indirect) which is not configured by RRC as the primary path. In multi-path for a split SRB, the secondary path is always the indirect path (SL or N3C).

Multi-path split bearer: In multi-path, a bearer in which one PDCP entity is mapped to one or more (direct) Uu RLC entities and either one SRAP entity of a SL indirect path or a non-3GPP connection.

N3C indirect path: In multi-path, the indirect path using Non-3GPP Connection between remote UE and relay UE.

Non-delay-reporting PDCP SDU: a non-delay-reporting PDCP SDU associated with the DSR-ReportingThreshold is a PDCP SDU that will be transmitted prior to any of the delay-reporting PDCP SDUs associated with the i:th DSR-ReportingThreshold but after all delay-reporting PDCP SDUs associated with the i-1:th DSR-ReportingThreshold, and that is not a delay-reporting PDCP SDU associated with any of the k:th DSR-ReportingThreshold where k <= i.

Non-split bearer: a bearer whose radio protocols are located in either the MgNB or the SgNB to use MgNB or SgNB resource, respectively.

NR sidelink communication: AS functionality enabling at least V2X communication as defined in TS 23.287 [13] and ProSe communication (including ProSe non-Relay, UE-to-Network Relay, and UE-to-UE Relay communication) as defined in TS 23.304 [18], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay Discovery, ProSe UE-to-Network Relay discovery, and ProSe UE-to-UE Relay discovery for Proximity based Services as defined in TS 23.304 [18] between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink transmission: any NR Sidelink-based transmission, including both transmission for NR sidelink discovery and transmission for NR sidelink communication.

PDCP data volume: the amount of data available for transmission in a PDCP entity.

PDU Set: one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. frame(s) or video slice(s) etc. for XR services), as defined in TS 23.501 [23]. A PDU in the PDU Set corresponds to a PDCP SDU.

PDU Set remaining time: the shortest remaining time till discardTimer expiry among the remaining time of the PDCP SDUs belonging to the PDU Set.

SL indirect path: In multi-path, the indirect path on which the L2 U2N Remote UE connects to the network via a L2 U2N Relay UE.

Split bearer: in dual connectivity, a bearer whose radio protocols are located in both the MgNB and the SgNB to use both MgNB and SgNB resources.

Split secondary RLC entity: in dual connectivity, the RLC entity other than the primary RLC entity which is responsible for split bearer operation. If the PDCP entity is associated with two RLC entities, the split secondary RLC entity is the RLC entity other than the primary RLC entity. If the PDCP entity is associated with more than two RLC entities, the split secondary RLC entity is configured by upper layers. In multi-path, the split secondary RLC entity is the RLC entity on the direct path which is responsible for split bearer operation when the MP primary path is the indirect path. When the PDCP entity on the direct path is associated with one RLC entity, the split secondary RLC entity is that RLC entity. When the PDCP entity is associated with more than one RLC entity, the split secondary RLC entity is configured by upper layers.

UM DRB: a data radio bearer which utilizes RLC UM.

UM MRB: an MRB associated with only RLC UM.

U2N Relay UE: A UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: A UE that communicates with the network via a U2N Relay UE.

U2U Relay UE: A UE that provides functionality to support connectivity between two U2U Remote UEs

U2U Remote UE: A UE that communicates with another UE via a U2U Relay UE

End of change