3GPP TSG-RAN2#131bis	Tdoc R2-2507071

Prague, Czech Republic, 2025-10-13 - 2025-10-17

Agenda Item:	10.3.1 – User plane aspects

Source:	Ericsson

Title:	User plane: Let's keep it simple!

Document for:	Discussion, Decision

# 1	Introduction

The evolution of 5G New Radio (NR) technology has fundamentally transformed the landscape of wireless communication. As 5G deployments mature and move beyond initial commercial implementations, there emerges a critical need to refine and optimize the user plane protocol stack architecture to meet the demanding performance, energy efficiency, and flexibility requirements of next-generation applications.

To address these requirements, the following detailed objectives are relevant for the RAN user plane as part of the 6G RAN study item description [1]:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

…

k) System simplification, including reducing configuration complexity, enabling more efficienCell/UE management, etc.

(3)	 Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3]

a)	User Plane architecture and protocol design [RAN2]

# 2	Discussion

The current 5G user plane protocol stack, comprised of the Physical (PHY), Medium Access Control (MAC), Radio Link Control (RLC), Packet Data Convergence Protocol (PDCP), and Service Data Adaptation Protocol (SDAP) layers, represents a sophisticated yet complex architecture that has proven its capability in delivering 5G’s promised performance. However, the inherent complexity of this approach, while providing functional separation and modularity, introduces considerable challenges in terms of processing overhead, memory access patterns, and energy consumption that become increasingly critical in foreseen resource-constrained environments and battery-powered devices.

As an example, the NR PUCCH HARQ feedback illustrates this pattern perfectly - starting as a simple, fast control channel, it required RLC ARQ overlay for reliability, then accumulated additional features for TDD, MIMO, and carrier aggregation support. Paradoxically, these “improvements” made PUCCH itself less reliable while degrading PUSCH performance through rate matching conflicts and power splitting inefficiencies. This suggests 6G development should prioritize holistic stack redesign over piecemeal evolution, examining cross-layer interactions and seeking unified architectures that consolidate feedback mechanisms rather than layering multiple complex solutions that ultimately undermine each other’s effectiveness.

The pursuit of protocol stack design in 6G thus presents a compelling opportunity to address these challenges through strategic architectural modifications creating a leaner protocol stack that maintain functional integrity while improving efficiency. The elimination of dual connectivity (DC) assumptions opens new possibilities for protocol simplification, allowing for streamlined processing paths and reduced computational complexity.

Equally critical is the enhancement of Quality of Service (QoS) and scheduling mechanisms, particularly in the context of Logical Channel Prioritization (LCP) and Buffer Status Reporting (BSR) procedures. The current token-bucket based approach, while functionally sound, exhibits performance limitations that can lead to starvation scenarios and suboptimal resource allocation across priority flows, potentially resulting in Radio Link Failures (RLF) despite adequate service levels for individual Logical Channels (LCH). Moreover, integration of Active Queue Management (AQM) and Explicit Congestion Notification (ECN) mechanisms into L2 offers a pathway to more in-band intelligent traffic regulation, enabling the network to provide timely feedback to endpoints and optimize resource utilization in high-demand scenarios.

It is beneficial to study necessary modifications for a leaner 6G protocol stack that preserves functional integrity while improving efficiency

## 2.1 	6G user plane protocol stack

The 5G QoS framework provides a suite of functionalities to identify and map traffic flows into different QoS flows (in CN) and in RAN to map these different QoS flows to different DRBs (making use of the QFI field in SDAP protocol). As such the 5G QoS framework provides an elaborate baseline when studying potential improvements for 6G.

On DRB level, differentiated flow treatment in RAN is possible in multiple aspects for example in UL logical channel prioritization (LCP), reliable mode (retransmission in RLC) , in-order delivery (in PDCP), and recovery at handover (PDCP). Inherently, by utilization of different DRBs, traffic flows are un-coupled, i.e. head of line blocking by common re-ordering behaviour is avoided.

One significant advantage of dividing functionality between the CN and the RAN is the ability to leverage each domain’s specific expertise and reuse capabilities. For example, 5G CN features can be applied across both 5G and 6G networks. The 5G CN is capable of traffic identification, is being further enhanced with APIs to provide even greater application awareness and already has the business logic in place to facilitate these functions. Furthermore, the introduction of support for 'URSP' (User Equipment (UE) Route Selection Policy, see TS 23.503) enables PDU Session based network controlled steering of application specific data flows onto QoS flows with appropriate packet treatment. Meanwhile, the RAN in 5G is well equipped with the necessary tools to guarantee differentiated service handling, particularly for the air interface, which remains the primary bottleneck in the network. And as such the 5G RAN functionality provides the baseline for 6G design.

For 6G’s differentiated services treatment, 5G’s QoS framework in CN and RAN provides the baseline.

One aim of the 6G study item [1] is to minimize specification of multiple options for the same functionality, as well as system simplification. RAN2 should thus strive to reduce overhead and redundant functionality within the user plane protocols by reconsidering the functionality distribution among layers.

For example, for standalone deployments i.e. not based on dual connectivity (DC), 5G NR protocols are built to support DC, and thus do not exploit the one-to-one mapping between RLC and PDCP packets for the case without DC. While in LTE one RLC protocol data unit (PDU) with its own SN concatenated multiple PDCP PDUs, the RLC PDU in 5G can only include at most one PDCP PDU. Still, both PDCP PDU and RLC PDU in 5G include their own SN, which unnecessarily adds overhead and processing.

Furthermore, in 5G, several inter-layer indications are specified to synchronize protocol states locally between entities in each transmitter and receiver sides. For example, in the transmitter side, RLC AM indicates ACKs to PDCP, PDCP indicates discards to RLC, and at the receiver side, state within RLC and PDCP can get un-synchronized, given their different individual reordering and status reporting mechanisms.

Another starting point for simplification is the retransmission schemes separately specified in PDCP, for handover recovery, and RLC for ARQ.

Complexities in 5G’s protocol stack stem from separated state variables, packet queues, retransmission schemes and reordering mechanisms among protocol layers, affecting both transmitter and receiver side.

Assuming that there is no DC support in 6G according to the SID [1], 6G should thus study how to reduce such complexities in functional distribution and related inter-layer indications between RLC, PDCP and also SDAP.

Another area for streamlining of the specification are the rather static different RLC operational modes: E.g. Acknowledged Mode (AM) differs from Unacknowledged Mode (UM). RLC specifies their algorithms separately, e.g. at the receiver side, however, both undergo the same PDCP receiver algorithm that eventually determines the behaviour how packets are delivered to higher layers. The algorithms could potentially be consolidated into a common one, and AM and UM modes would differ only by configuring support for polling, status reports, and retransmissions, but not in their algorithms. Furthermore, as discussed in Section 2.3, local feedback form HARQ could trigger retransmissions in addition to status reporting.

Study necessary modifications for a leaner UP protocol stack by improving efficiency, protocol layer interworking and removing functional redundancy across protocol layers.

From a processing efficiency point of view, it is preferable to process incoming packets only once, and upon arrival at the protocol stack to the largest possible extent. This reduces required processing in the time-critical scheduling loop, i.e. upon submission via the radio interface. This “pre-processing” may include PDCP SN assignment, encryption, integrity protection, and preparing of PDCP-, RLC- and MAC-sub headers, which can thereafter be stored in memory, and should be an integral building block for the 6G design.

Pre-processing capability on transmitter side is an essential component in 5G and for 6G.

With active queue management (AQM), the transmitter can control the size and latency of the queue awaiting TX over the radio link. It aims for a small queue to keep the E2E latency low while storing sufficient data to fully utilize the underlying link and thereby maximize the throughput. AQM achieves this by issuing signals to the end-to-end (E2E) transport protocol such as TCP and Quick UDP Internet Connections (QUIC), e.g. indicated via explicit congestion notifications (ECN) [RFC3168][2] if supported, or via artificial dropping of packets, upon which the E2E transport protocol detects and reduces its transmit rate. AQM works most efficiently if these indications are triggered timely and reach the traffic source as fast as possible. In 5G this is not the case, as it conflicts with pre-processing and reordering behaviour. After pre-processing, AQM can no longer access the IP packets’ ECN fields to set the ECN congestion experienced (ECN-CE) bit, as they are ciphered. Also, AQM packet dropping after pre-processing with SN assignment would lead to an SN gap noted by the PDCP receiver which starts its reordering timer. Only expiry or the reordering timer, subsequent packets would be delivered, meaning that the indication would be delayed. In 5G, this is partially addressed with the Rel-18 feature of PDCP SN gap reporting. However, such gap indication signaling in PDCP would need to undergo queueing in RLC which also comes with a delay.

Indications to manage queueing latency in RAN to transport layer are subject to delays considering 5G’s need for pre-processing.

AQM should be an integral part of the 6G layer 2. Since at the transmitter side, one cannot access encrypted IP fields of packets in its queue, we suggest setting a “CE flag” in its own layer 2 header to mark the packet front-of-queue upon packet submission. When this indication is received, the receiver “copies” it into the IP header’s ECN-CE field after de-ciphering and thereby notifies the E2E protocols instantaneously. Hence, critical communication services could also be better supported. If the E2E path does not support ECN, the AQM algorithm should rather drop a packet. To avoid latency spikes caused by reordering, the transmitter-side protocol should have means to inform its receiving peer that PDUs have been dropped and that the reordering function at the receiver shall deliver subsequent packets immediately. The PDCP SN drop indication introduced for 5G in 3GPP Rel-18 should thus become another integral part of a 6G layer 2 protocol.

Study support for faster queue management as an integral component of 6G RAN (e.g. based on queue indications).

## 2.2	Scheduling enhancements

When trying to improve the end-to-end (E2E) performance, 3GPP tends to focus on enhancing the data rate that the Uu connection offers to a user or to all users in the system. However, we observe in simulation and field data that the e2e performance is often limited by the e2e round-trip-time. For small data transfers and for the initial phase of larger data transfers the latency impacts the perceived performance. This is visible in applications like web-browsing, video streaming and real-time communications, i.e., for both UL- and DL heavy applications. Most traffic also requires quick turnaround times for DNS enquiry, TCP handshakes, and other connection-establishment procedures.

With increasing L1 data rates and decreasing backhaul/internet delays, the delays in obtaining the initial grant for transmission via SR/BSR is the dominant contributor to latency. When UL data arrives in the UE’s previously empty queue the UE has typically no valid UL grant and does not expect to receive one either. In this situation, which recurs repeatedly especially for DL-heavy traffic, the NR UE transmits a scheduling request (SR) on PUCCH to the gNB. The SR is just a single bit indication which the PUCCH encodes efficiently. But since dedicated SR resources must be assigned to all UEs, they recur typically only every 5-20 ms. Denser allocations would cost prohibitively many UL resources that could not be used for user data. Upon receiving an SR, the gNB replies with a small grant allowing the UE to send a BSR in the allocated PUSCH resource. Upon reception of the BSR, the gNB allocates a larger grant for the remainder of the data. The latency introduced by this procedure becomes the dominant delay component when the UE transmits non-contiguous UL data. It should be noted that this is also the case for DL heavy traffic which, especially during the early phase of the connection, requires fast transport layer feedback (TCP or QUIC).

The latency introduced by the SR/BSR procedure to obtain the initial grant in NR is the dominant contributor to delay and hence limits the end-to-end performance of UL- and DL-heavy applications.

The XR Rel-18 WI specified a refined BSR to provide precise, fine-grained buffer information even for larger buffer and included the delay status report (DSR) to assist the network scheduler in prioritizing among UEs. For effective dynamic scheduling, accurate reports are important. However, these enhancements did not decrease reporting latency. Therefore, uplink buffer information reporting procedures in 6G should strive to reduce latency while maintaining accuracy and providing sufficient information to support optimal scheduling decisions.

For 6G we have investigated the possibility to replace the dedicated SR resource by a small, but frequently recurring PUSCH allocation shared by many/all UEs in a cell. When a UE has available UL data but no valid/expected grant, the UE sends a BSR and its identity (e.g. C-RNTI) on the shared PUSCH resource in a contention-based manner. This eliminates the waiting time for the SR resource and one Uu round trip time. A robust MCS minimizes decoding errors while leaving enough room for a detailed BSR even on a small PUSCH allocation (1-2 PRB). Due to the sparse utilization, collisions are found to be rare. Even if collisions occur, random DMRS selection from a small set of configured DMRS sequences can help the gNB to decode at least some of the colliding PUSCHs anyway. UEs that don’t get a timely response should re-attempt the BSR transmissions - similarly to NR’s SR retransmissions.

Simulations with realistic channel-, protocol- and traffic-models revealed that sending BSRs on a contention based PUSCH improves the performance of UL- and DL heavy applications by reducing the UL latency and by consuming fewer UL resources than dedicated SR resources on PUCCH. This was the case for both low and high system load, i.e., the contention based PUSCH scaled better with increasing load than dedicated SR resources. Furthermore, the gNB’s (blind) decoding effort can be kept reasonably low since all UEs use the same small set of DMRS sequences.

It is favourable in terms of latency, performance and system capacity if UEs send their BSRs on a frequently recurring PUSCH in a contention-based fashion compared to NR’s traditional two-step SR+BSR.

Study means to reduce latency and improve accuracy for uplink buffer information reporting and thereby decrease latency and enhance e2e performance.

Another important aspect for improving uplink QoE and enabling differentiated connectivity as part of the uplink scheduling framework is the Logical Channel Prioritization (LCP) procedure. These improvements become even more important in the 6G timeframe, where UEs are expected to handle a growing mix of concurrent MBB, rate adaptive and delay-sensitive services simultaneously – a trend that we observe recently already in 5G.

In such configurations we noticed that the legacy token bucket model suffers from performance issues, with Logical Channel (LCH) starvation being one of the important concerns. Complete or partial starvation can render applications on the affected LCHs unusable. LCP configurations are designed to avoid starvation, often set a smaller Prioritized Bit Rate (PBR) on the highest priority LCH. This reduces flexibility and loss of fine-grained control over specific QoS characteristics, impacting QoE. These limitations in LCP configuration result in inefficient resource distribution among LCHs and can cause misalignment with the intended gNB uplink scheduling strategy.

Further, the requirements of the application’s traffic are dynamic i.e., changes over time depending on factors like network conditions, activity patterns and the nature of the service itself. Such variability can lead to different requirements in bit rate, latency and QoS expectations over the application’s operational phase. However, given the largely static nature of the LCP configuration, the legacy uplink scheduling framework cannot accommodate such changes often leading to suboptimal resource allocation.

For delay-sensitive traffic such as XR, video streaming, and cloud gaming — which require stringent latency control and rate enforcement — efficient uplink scheduling is essential for maximizing system capacity. This need was recognized in the 3GPP Rel-19 XR WI, which for example also highlighted limitations of the legacy LCP and the need for an evolved framework for more fine-grained control of uplink logical channel scheduling.

Legacy LCP procedure leads to LCH starvation, inefficiency and inability to adapt to dynamic traffic requirements and delay-sensitive traffic.

Thus, an evolved framework should ensure that all configured LCHs maintain acceptable QoE levels for their respective applications or services, collectively balancing priorities rather than applying uniform or overly strict treatment to each channel. Another important aspect for scheduling is how to align the gNB scheduling strategy and UE resource split.

Study uplink scheduling framework to ensure QoE fairness with fine-grained control over QoS characteristics among LCH(s).

## 2.3 	HARQ and CSI Feedback

4G and 5G UEs send their UL HARQ-FB and CSI reports on a tailor-made physical channel (PUCCH). To save signalling overhead, the HARQ-FB reports comprise only of a set of ACK/NACK bits without explicit process- or carrier IDs. UE and network derive the HARQ process IDs from the previous scheduled HARQ processes, from DAI fields in the DL grants and from the PUCCH’s transmissions time. While this was feasible in early LTE days (FDD only) the PUCCH signalling became increasingly complex in later LTE releases (carrier aggregation, TDD) and in NR (more CA, mixed numerologies, code-block groups, …). The effort for book-keeping, inter-carrier coordination and resolving race conditions (e.g. in case of lost DL grants or ACK/NACK errors) increases substantially with the number of carriers and numerologies.

The allocation of PUCCH resources (PRI), and the setting of K1- and DAI values must be coordinated across the DL grants that the gNB sends for different carriers. This would be complex but feasible if a single scheduler thread could take all scheduling decisions for all UEs in all carriers sequentially. However, with more carriers of various characteristics, scheduling is typically split across several parallel threads or processes. In such a setup the coordination of K1- and DAI values and PUCCH resources across carriers puts tight requirements on the inter-thread or inter-process communication. In addition, the HARQ feedback is multiplexed into the UE’s PUSCH resources when the gNB issues an UL grant for data in the same slot. This further couples the UL scheduler with the DL scheduler as the gNB should also indicate DAI in the uplink scheduling DCI.

5G HARQ feedback in carrier aggregation requires tight coordination among involved schedulers which is very challenging in practical implementation.

Similarly, the channel state information (CSI) evolved from a simple “CQI+rank” in LTE to a diverse set of CSI report types of various sizes and with different processing complexity. The latter results in different processing delays which restrict when the network may schedule those reports and regular data. For example, if the gNB requests an aperiodic CSI report, the UE needs 72 slots (almost 6 slots) on a 30 kHz carrier to generate the report. Generating a pure PUSCH HARQ transmission takes only ~2 slots.

The selection and coordination of PUCCH formats becomes even more complicated when different UCI types (HARQ FB and CSI reports) must be multiplexed into the same slot. For this purpose NR offers standardized specific multiplexing rules for all combinations of different UCI and resource types. These also led to complex timeline requirements when scheduling PUCCH and PUSCH resources.

Furthermore, NR allows multiplexing UCI with UL data. In this case the UE encodes UCI and data separately maps the encoded UCI bits into a subset of the PUSCH resource elements (REs) and fills the remaining REs with the encoded data. This leads to decoding errors when the UE did not detect all DCIs and hence derived a different UCI/PUCCH format than the gNB expected. From a UE perspective, the HARQ feedback multiplexing also means frequently overriding PUCCH resource which is a big burden. The above-mentioned processing constraints require the gNB to schedule an UL data transmission earlier if it coincides with a requested CSI report. And in such cases the gNB cannot send another UL data grant in a subsequent PDCCH occasion for an earlier UL slot – UL grants shall not overtake each other. And finally, the rules by which the encoded UCI steals PUSCH resource elements from the encoded transport block increases the HARQ BLER especially when the UE is in challenging radio conditions.

5G HARQ feedback multiplexing in PUCCH or PUSCH is very complex due to tailor-made L1 multiplexing rules and timeline requirements.

The complexity of HARQ-FB and CSI reports and the way they are transmitted in NR became one of the biggest pain points for network and UE implementation. The complexity hinders the swift introduction of new carrier aggregation variants or advanced CSI types.

On the other hand, NR- and LTE UEs multiplex other feedback such as BSR and PHR into the MAC PDU and transmit them in the PUSCH transport block which is protected by CRC and HARQ. This leads to additional delays in case of HARQ retransmissions but avoids cumbersome race conditions and minimizes impact to scheduling algorithms.

MAC already has a generic framework of multiplexing a various type of data without strict timeline requirements and cross-scheduler coordination.

What would happen if 6G UEs multiplex HARQ FB and CSI reports into MAC PDUs? A tailor-made PUCCH could be abandoned. The reports should contain explicit references to HARQ processes and CSI report configurations so that the gNB can interpret them irrespective of the time at which the UE generated them. This would relieve the gNB from DAI coordination and from choosing UCI- and PUCCH formats. And it would leave more freedom to the UE when generating and multiplexing reports. The above-mentioned scheduling restrictions in case of CSI types that require long processing delays would vanish. The gNB would just schedule a dynamic PUSCH grant when it expects HARQ FB or CSI reports and the UE multiplexes the prepared reports like any other control- or user data. The in-built CRC protection and UL HARQ would resolve NACK<->ACK errors that made LTE’s and NR’s DL HARQ unreliable.

On the downside, the HARQ-FB and CSI-formats would be larger than NR’s optimized UCI formats due to explicitly signalled IDs, MAC Sub-Headers and CRCs. Especially compared to NR’s small PUCCH formats the link budget of a PUSCH transmission would be noticeably worse. However, NR’s coverage is in practice not determined by the link budget of the PUCCH but rather by PUSCH. In other words, if an NR UE cannot transmit UL user data, TCP ACK and RRC messages at a reasonable data rate, it does not help if it could still transmit PUCCH.

In simulations with realistic channel-, protocol- and traffic-models we observed that despite the substantial simplification of the protocol stack, the e2e performance is not affected negatively when mapping CSI reports into UL MAC PDUs rather than on a tailor-made PUCCH. Sending also UL HARQ feedback inside MAC PDUs shows a noticeable performance improvement for most traffic types. This unexpected result is caused by a positive free-ride effect: When the NW sends UL grants that are supposed to carry expected HARQ feedback for previously scheduled DL HARQ processes, the UE can often fit additional UL data or at least a BSR into the same grant. This effect occurs often since most data transfers are bi-directional (TCP/QUIC ACKs, …) and it accelerates the UL scheduling noticeably. This free-ride effect also helps to keep the PDCCH utilization reasonably low, i.e., the PDCCH load increases only marginally due to additional UL grants that the gNB must send for CSI- and HARQ reports.

Study ways to simplify HARQ- and CSI reporting for gNB and UE especially considering challenging cases such as multi-carrier-multi-numerology configurations and advanced CSI types.

## 2.4	HARQ and ARQ Interworking

In 5G, HARQ and RLC AM together provide fast and efficient transmissions with very high reliability.

NR’s HARQ protocol offers fast retransmissions and soft-combining under tight scheduler control. However, it has limited reliability, primarily due to errors in DL grants and UL feedback. HARQ is also limited to operate within one carrier. RLC AM on the other hand, has very high reliability in terms of packet errors, it can also distribute data over multiple carriers. RLC provides also the possibility to differentiate retransmission behaviour on a per bearer basis i.e. for those services that need it. RLC AM is however relatively slow as it operates on its one state variables and timers, independently from underlying HARQ protocol. Furthermore, RLC is configured per bearer, parameters cannot be adopted to the characteristics of different CCs. This was also observed and studied in NR Rel-19 to enhance XR support.

In 5G, HARQ and RLC ARQ operate independently. HARQ is driven by Layer 1 signaling and feedback on a per‑component carrier basis, enabling fast retransmissions but leaving some residual losses. In contrast, RLC ARQ is timer‑driven and works on a per‑bearer basis, making it slower but highly reliable.

For 6G, with the objectives of striving for simplification and removal of redundant functionality, the properties of these two retransmission schemes and their required feedback mechanisms should be carefully analysed. In this respect, also tighter interworking between the two schemes should be studied, such as basing both protocols on the same feedback. For example, if the 6G UE would map its UL HARQ feedback into MAC PDUs (see section 2.3) it would become as reliable as NR’s RLC status reports. Based on the reliable HARQ feedback, the scheduler could decide to abandon a continuously failing HARQ process and to trigger a (possibly re-segmented) RLC retransmission. This could reduce latencies and reduce overhead from otherwise needed frequent RLC status reporting.

Study benefits for tighter interworking of retransmission protocol layers.

# 3	Conclusion

In this paper, we have reached these observations:

Observation 1	It is beneficial to study necessary modifications for a leaner 6G protocol stack that preserves functional integrity while improving efficiency

Observation 2	For 6G’s differentiated services treatment, 5G’s QoS framework in CN and RAN provides the baseline.

Observation 3	Complexities in 5G’s protocol stack stem from separated state variables, packet queues, retransmission schemes and reordering mechanisms among protocol layers, affecting both transmitter and receiver side.

Observation 4	Pre-processing capability on transmitter side is an essential component in 5G and for 6G.

Observation 5	Indications to manage queueing latency in RAN to transport layer are subject to delays considering 5G’s need for pre-processing.

Observation 6	The latency introduced by the SR/BSR procedure to obtain the initial grant in NR is the dominant contributor to delay and hence limits the end-to-end performance of UL- and DL-heavy applications.

Observation 7	It is favourable in terms of latency, performance and system capacity if UEs send their BSRs on a frequently recurring PUSCH in a contention-based fashion compared to NR’s traditional two-step SR+BSR.

Observation 8	Legacy LCP procedure leads to LCH starvation, inefficiency and inability to adapt to dynamic traffic requirements and delay-sensitive traffic.

Observation 9	5G HARQ feedback in carrier aggregation requires tight coordination among involved schedulers which is very challenging in practical implementation.

Observation 10	5G HARQ feedback multiplexing in PUCCH or PUSCH is very complex due to tailor-made L1 multiplexing rules and timeline requirements.

Observation 11	MAC already has a generic framework of multiplexing a various type of data without strict timeline requirements and cross-scheduler coordination.

Observation 12	In 5G, HARQ and RLC ARQ operate independently. HARQ is driven by Layer 1 signaling and feedback on a per‑component carrier basis, enabling fast retransmissions but leaving some residual losses. In contrast, RLC ARQ is timer‑driven and works on a per‑bearer basis, making it slower but highly reliable.

We propose:

Proposal 1	Study necessary modifications for a leaner UP protocol stack by improving efficiency, protocol layer interworking and removing functional redundancy across protocol layers.

Proposal 2	Study support for faster queue management as an integral component of 6G RAN (e.g. based on queue indications).

Proposal 3	Study means to reduce latency and improve accuracy for uplink buffer information reporting and thereby decrease latency and enhance e2e performance.

Proposal 4	Study uplink scheduling framework to ensure QoE fairness with fine-grained control over QoS characteristics among LCH(s).

Proposal 5	Study ways to simplify HARQ- and CSI reporting for gNB and UE especially considering challenging cases such as multi-carrier-multi-numerology configurations and advanced CSI types.

Proposal 6	Study benefits for tighter interworking of retransmission protocol layers.

# 4	References

RP-251881, New SID: Study on 6G Radio, 3GPP RAN#108, Prague, Czech Republic, June 9-13, 2025 [online]

Internet Engineering Task Force (IETF), RFC 9330, “Low Latency, Low Loss, and Scalable Throughput (L4S) Internet Service: Architecture”, December 2023

R2-2507074, Common User Plane and Control Plane Aspects for 6G, 3GPP RAN2#131bis, Prague, Czech Republic, October 13-17, 2025.