3GPP TSG-RAN WG2 Meeting #131bis					R2-2507090

Prague, Czech Republic, October 13 – 17th, 2025

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Remaining issues in LCM for BM use case

Document for:	Discussion and Decision

## Introduction

In this contribution, we provide our view on some RRC RIL issues that the spec rapporteur suggests to discuss in contribution and also discuss to solve one remaining FFS on associated ID.

## RIL open issues

## E040

RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. E040; AIML; 2; Ensure that UE does not request a candidate UE-side data collection configuration for which it already has a UE-side data collection configuration; R2-25xxxx; Ericsson (Andra); V022; ToDo. [Description]: According to the current procedural text in 5.7.4.3, the UE may request a candidate UE-side data collection configuration in UAI (received in otherConfig), even if it already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig) for that candidate configuration. This is not a desired behaviour and it should be fixed. [Proposed Change]: The issue and some possible solutions will be discussed in more detail in a Tdoc. Some possible solutions are: - change only the procedural text to make it clear that the UE does not request candidate configurations for which it already has a corresponding UE-side data collection configuration; - change the ASN.1 (and corresponding procedures) so the network can indicate which candidate configuration a real UE-side data collection configuration corresponds to. [Comments]:

In our assumption, UE sends a preference request when a candidate configuration is preferred to be configured. It will implicitly mean that the corresponding configuration is not yet configured in CSI-ReportConfig. As long as the parameters in the candidate configuration is matched in the CSI-ReportConfig, UE could assume that the preferred configuration is configured.

Therefore, it would be considered as a wrong implementation if UE requests a candidate configuration is already configured in CSI-ReportConfig.

Proposal 1: (E040) no spec change is needed.

## E041

# E041 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. E041; AIML; 2; Associated ID validity; R2-25xxxx; Ericsson (Jens); V022; ToDo. [Description]: According to current specifications, once an associated ID value is signaled from the network to the UE, the meaning of that value at the UE (or UE-side entity) corresponds to the same beam property/deployment forever. There is no way for the network to reset/reassign that value to other beam properties and inform the UE that should forget the old meaning of that value. We think it is unreasonable to assume that a given meaning holds for a given associated ID forever. It may happen that e.g. some hardware is replaced and the old hardware is not used again anywhere else in the network, or there is a failure/reset in the network entity storing/assigning associated IDs, after which the legacy associated ID meanings (assigned to deployment characteristics) cannot be recovered. Although such events may be very unlikely, they may still happen at a certain point in the future. Thus, there should be a way for the network and UE to make sure that they have the same understanding of an associated ID value. This is important for the UE (or UE-side entity) to be able to understand if a model exists and/or is trained for the same deployment assumed by the network for an associated ID. [Proposed Change]: We will discuss this issue in more detail and present possible solutions in a Tdoc. [Comments]:

Technically, it is a valid point. However, given that we introduce a quite big range of associate ID range, it is not clear how likely there is a need to re-use the same associated ID to different Network additional conditions. It is not clear how often this NW side additional condition has changed and how much the difference affects in model performance. Furthermore, model training can also adjust model training based on the performance monitoring. So, unless there is big change in NW deployment, there may not be need to have validity handling.

If it is really necessary, we could consider a simple solution e.g. a cell specific value tag to indicate whether the configured associated IDs are refreshed.

However, considering the WI is completed, it would be more reasonable to not consider a solution to solve this issue.

Proposal 2: (E041) no spec change is needed.

## N021/N003

# N021 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. N021; AIML; 2; Remove the CHOICE hierarchy from the configuration for CSI prediction and beam prediction; R2-25xxxx; Jerediah Fevold; vnnn; ToDo. [Description]: The extra hierarchy introduced by forcing a choice between predictionConfiguration-r19 and configurationForChannelMonitoring-r19 is unnecessary and does not have the advantage of grouping many parameters together. It should be obvious to the configuring entity that one or the other should be configured. [Proposed Change]: Eliminate one level of hierarchy by removing the choice element predictionConfiguration from CSI-ReportConfig and bring predictionConfiguration-r19 and configurationForChannelMonitoring-r19 to the main level of CSI-ReportConfig. [[ nrofReportedRS-v19xy ENUMERATED {n6, n8} OPTIONAL, -- Need R reportQuantity-r19 ReportQuantity-r19 OPTIONAL, -- Need R csi-InferencePrediction-r19 ENUMERATED {true}, configurationForChannelPrediction-r19 SEQUENCE { resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofReportedPredicted-RS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {n1, n2, n4, n8} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL, -- Need R ... } OPTIONAL, -- Need R configurationForChannelMonitoring-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, nrofBestBeamForMonitoring-r19 ENUMERATED {n1, n2} OPTIONAL, -- Need R nrofTransmissionOccasion-r19 ENUMERATED {n1, n3, n7, n15} OPTIONAL, -- Need R timeInstanceFor-RS-PAI-r19 ENUMERATED {n1, n2, n8, spare1} OPTIONAL, -- Need R mappingToResourcesForChannelPrediction-r19 BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OPTIONAL, -- Need R timeInstanceFor-SGCS-r19 ENUMERATED {n1, spare3, spare2, spare1} OPTIONAL, -- Need R ... } OPTIONAL -- Need R ]] [Comments]: [WI CR rapporteur-v022]: We suggest to discuss the restructuring of this part in a Tdoc, since there are also other solutions that other companies raised in the past and in RIL H003.

# H003 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. H003; AIML; 2; Configuration for UE data collection; R2-25xxxx; Dawid; v017; ToDo. [Description]: This is related to the following EN captured ubnder CSI-ReportConfig information element code: Editor's Note: FFS whether/how to group the parameters (and whether/how to update the field descriptions) for prediction, monitoring, and UE-side data collection based on the beam management and CSI prediction use cases. Currently “configurationForChannelPrediction-r19” is used for both inference and UE data collection. However, the notation “ForChannelPrediction” and “predictionConfiguration-r19” suggests that this parameter is only for inference. We think it is better to add a new parameter for data collection (e.g. configurationForUE-DataCollection-r19) and make “configurationForChannelPrediction-r19” applicable only for inference [Proposed Change]: configurationForUE-DataCollection-r19 SEQUENCE { resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {n1, n2, n4, n8} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL, -- Need R ... } [Comments]: [WI CR rapporteur-v022]: We agree that the suggested change is a possibility to group the parameters and thus make the type of configuration clear. In the past we received input from some companies that we should avoid redundancy (the parameters for inference and those for data collection are the same), so we removed the separate CHOICE branch for UE-side data collection. Thus, we propose to discuss this issue in Tdocs, as for RIL N021, which is related.

We agree with N021 that there is no need to have it in CHOICE structure especially for csi-InferencePrediction-r19. And also for prediction and performance monitoring can be distinguishable with reportQuantity-r19.

Regarding H003, we don’t see a need to have a separate configuration for data collection. In reportQuantity-r19, none-BM-r19 and none-CSI-r19 can be used to distinguish the same configuration. In addition, two operations use the same set of configurations according to RAN1 agreements.

Another issue to raise is that configurations for prediction accuracy indicator (PAI) are quite different for BM and CSI. In addition, BM and CSI PAI is distinguished and also inference configuration for BM is distinguished via reportQuantity-r19 (rs-PAI-r19 and csi-PAI-r19/sgcs-r19). Therefore, there is no reason to have a CHOICE structure under predictionConfiguration-r19.

Proposal 3: (N021/N003) it is not necessary to have CHOICE structure. There is no need to separate configuration for both inference and UE data collection. It is necessary to separate performance monitoring for BM use case and CSI prediction.

The below is our proposed structure.

csi-InferencePrediction-r19         ENUMERATED {true}                                          OPTIONAL,   -- Need R

configurationForChannelPrediction-r19   SEQUENCE {

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                    OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                        OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                        OPTIONAL,   -- Need R

nrofReportedPredicted-RS-r19                ENUMERATED {n1, n2, n3, n4}                             OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}                                        OPTIONAL,   -- Need R

timeGap-r19                                 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL,   -- Need R

...

}	OPTIONAL,   -- Need R,

configurationForBM-PAI-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                                         OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}                                OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {n1, n2, n8, spare1}                             OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet))    OPTIONAL,   -- Need R

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}                     OPTIONAL,   -- Need R

...

}	OPTIONAL,   -- Need R

configurationForCS-PAI-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}                     OPTIONAL,   -- Need R

...

}	OPTIONAL,   -- Need R

## E042

# E042 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. E042; AIML; 2; Necessity of dedicated flag to configure applicability reporting via UAI; R2-25xxxx; Ericsson (Andra); V022; ToDo. [Description]: RAN2#130 agreed the following: “2 Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified)” After adding the sets of inference related parameters for option B in otherConfig, it was not discussed how to combine this flag in ASN.1 with the configurations for option B. [Proposed Change]: We will provide a more detailed discussion and solutions in a Tdoc. Some possible solutions are: - Remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig. - Keep the flag reportApplicabilityUAI-r19 within applicabilityReportConfig, as it is. - Move the flag reportApplicabilityUAI-r19 one level up, directly under otherConfig. [Comments]:

Solution 1 and solution 2 are equivalent but it is just difference whether there is explicit RRC parameter.

In that sense, solution 1 provides less signaling overhead and we slightly prefer solution 1.

Proposal 4: (E042) RAN2 agree with Solution 1) i.e. remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

## Z001

# Z001 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. Z001; AIML; 1; Preference suggestion to the functionality that is reported as inapplicable in RRCReconfiguration; R2-25xxxx; Fei; V015; ToDo. [Description]: Currently, for the applicability reporting via RRCReconfigurationComplete, it has been defined as below: 4> for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference; For the functionality that is provided in the OtherConfig whose applicability has been changed, there is no need for UE to provide the information of the releaseConfigurationPrefernece in the RRCReconfigurationComplete as there is no hurt to keep it still in the otherConfig. [Proposed Change]: #### 5.3.5.3 Reception of an RRCReconfiguration by the UE /omit for short/ 2> if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured which has not been successfully transmitted since entering RRC_CONNECTED state: 3> include measConfigReportAppLayerAvailable; 2> if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6: 3> include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration; 2> if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or 2> if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or 2> if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation); or 2> if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation): 3> for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference; 4> for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; [Comments]: [WI CR rapporteur-v022]: We think this issue should be discussed based on Tdocs, since there are changes in the UE behaviour for applicability reporting in RRCReconfigurationComplete versus UAI.

Even in case of option B (i.e. OtherConfig), it would be useful for gNB to know whether a certain set of inference related parameters are not possible to change status in the near future. Eventually, it is up to gNB whether the set of inference related parameters should be released or not. Therefore, it is desirable to provide in releaseConfigurationPreference.

Proposal 5: (Z001) no change is needed.

## S045

We would like to discuss on our RIL (i.e., S045):

# S045 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. S045; AIML; 2; Correction on dataCollectionSart; TBD; Seung-Beom; v026; ToDo. [Description]: We have the following agreement: 3 On stop/start indication - The UE can send start indication (without a preferred list) to indicate preference to start data collection - The UE can send preferred list implying that it would like to start data collection on those configuration - The UE can send stop indication for all or a given actual CSI report config ID. - Rapporteur will determine best way of signaling. This doesn’t preclude merging 1 and 2.. In our understanding, UE can indicate its preference for the following three cases via UAI: Case 1) Absence of Start indication (i.e., dataCollectionStart-r19): UE has no preference to start data collection Case 2) Presence of Start indication “without” preferred list: UE has preference to start data collection, but none of given candidates is preferred Case 3) Presence of Start indication “with” preferred list: UE has preference to start data collection, and the indicated list among given candidates is preferred Here, we want to clarify UE could have different preference for each serving cell. For example, - For serving cell 1, UE has no preference to start data collection (i.e., Case 1), leading to absence of start indication - For serving cell 2, UE has preference to start data collection (i.e., Case 2 or 3), leading to presence of start indication However, according to the current ASN.1, the start indication is defined commonly to all serving cells. Therefore, there is no way for UE to indicate different start indication (i.e., absence vs. presence) for different cells. [Proposed Change]: Start indication is defined per serving cell, and corresponding TP could be also updated as follows: UEAssistanceInformation-v19xy-IEs ::= SEQUENCE { applicabilityReportList-r19 ApplicabilityReportList-r19 OPTIONAL, dataCollectionPreference-r19 DataCollectionPreference-r19 OPTIONAL, loggedDataCollectionAssistance-r19 LoggedDataCollectionAssistance-r19 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } DataCollectionPreference-r19 ::= SEQUENCE { dataCollectionStart-r19 ENUMERATED {start} OPTIONAL, dataCollectionPreferredConfigurationList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19 OPTIONAL, dataCollectionStopConfigurationList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19 OPTIONAL, ... } DataCollectionCandidateList-r19 ::= SEQUENCE { dataCollectionServCellIndex-r19 ServCellIndex, dataCollectionStart-r19 ENUMERATED {start} OPTIONAL, dataCollectionCandidateIdList-r19 SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigId-r19 OPTIONAL } DataCollectionList-r19 ::= SEQUENCE { dataCollectionStopServCellIndex-r19 ServCellIndex, dataCollectionIdList-r19 SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId OPTIONAL }. #### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message The UE shall set the contents of the UEAssistanceInformation message as follows: <…> 1> if transmission of the UEAssistanceInformation message is initiated to report the UE preference to be configured with radio resources to perform UE-side data collection according to 5.7.4.2: 2> include dataCollectionPreference in this UEAssistanceInformation message; 2> if the UE prefers to be configured with radio resources to perform data collection: 3> for each serving cell for which UE prefers to be configured with radio resources to perform data collection: 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> set dataCollectionStart to start; 5> if dataCollectionCandidateConfigParameterList is configured for the serving cell and the UE has one or more preferred radio resource configuration(s) among the dataCollectionCandidateConfigParameterList: 6> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList; 3> set dataCollectionStart to start; 3> for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s): 4> include an entry in dataCollectionPreferredConfigurationList and set the content as follows: 5> set the dataCollectionServCellIndex to the serving cell index of the cell; 5> include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;.

What we want to clarify is Case 2 (i.e., Presence of Start indication “without” preferred list) is used for UE to request NW new candidate configuration(s) (e.g., a new candidate list or update of previous candidate list).

Meanwhile, X003 proposed to remove the start indication (i.e., dataCollectionStart-r19) simply, while keeping the TP as it is. However, as clarified above, in X003, there is no way for UE to request NW new candidate configuration(s) (e.g., a new candidate list or update of previous candidate list).

Therefore, we suggest to adopt S045, instead of X003.

Proposal 6: (S045) the start indication (i.e., dataCollectionStart-r19) is introduced per serving cell, and the TP proposed in S045 is adopted.

## X004 and S041

# X004 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. X004; AIML; 1; candidate data collection configuration; R2-25xxxx; Xing; V012; ToDo. [Description]: Currently, the inference related configuration and candidate data collection configuration is provided via a list in OtherConfig. However, with a list, NW has to provide the full list upon modification or release for a certain entry. ToAddMod structure can be used to reduce the signalling upon modification or release for a certain entry. [Proposed Change]: Change current structure of candidate data collection configuration to ToAddMod structure. UE behaviour should also be added. [Comments]: [WI CR rapporteur-v022]: Since this implies several changes, companies are encouraged to discuss this topic and provide TPs in Tdocs, as for the similar RILs C083/C084.

# S041 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. S041; AIML; 2; ToAddModList and To ReleaseList for candidate configuration; Seung-Beom; v028; ToDo. [Description]: According to current ASN.1 structure, when NW adds/releases/modifies each serving cell, NW has to provide all configuration list. i.e., No delta signaling is supported. For example, UE is configured with three DataCollectionCandidateConfig-r19 for 3 serving cell. When NW adds one more serving cell to UE (i.e., totally 4 serving cells), there is no way for NW to provide only one new DataCollectionCandidateConfig-r19 for the new cell. i.e., NW has to not only provide the new DataCollectionCandidateConfig-r19 but also the old three DataCollectionCandidateConfig-r19 again. [Proposed Change]: DataCollectionPreferenceConfig-r19 ::= SEQUENCE { dataCollectionCandidateConfigList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19 OPTIONAL, -- Need R dataCollectionCandidateConfigToAddModList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19 OPTIONAL, -- Need N dataCollectionCandidateConfigToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need N ... }

DataCollectionPreferenceConfig-r19 ::= SEQUENCE { dataCollectionCandidateConfigList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19[RIL]: S041, AIML OPTIONAL, -- Need R ... } DataCollectionCandidateConfig-r19 ::= SEQUENCE { dataCollectionServCellIndex-r19 ServCellIndex, dataCollectionCandidateConfigParameterList-r19 SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigParameters-r19 OPTIONAL, -- Need R[RIL]: X004 AIML ... } DataCollectionCandidateConfigParameters-r19 ::= SEQUENCE { dataCollectionCandidateConfigId-r19 DataCollectionCandidateConfigId-r19, resourcesForChannelMeasurement CSI-ResourceConfigId OPTIONAL, -- Need R resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R ... }

As specified in S041, candidate list for multiple serving cells (i.e., dataCollectionCandidateConfigList) could be added/modified/released in accordance with addition/modification/release of serving cell(s). For example, UE is configured with three DataCollectionCandidateConfig-r19 for 3 serving cell. When NW adds one more serving cell to UE (i.e., totally 4 serving cells), there is no way for NW to provide only one new DataCollectionCandidateConfig-r19 for the new cell. i.e., NW has to not only provide the new DataCollectionCandidateConfig-r19 but also the three old DataCollectionCandidateConfig-r19 again.

Similarly, candidate list for each serving cell (i.e., dataCollectionCandidateConfigParameterList) could be also added/modified/released in accordance with NW’s needs or UE’s preference. For example, for a serving cell, UE is configured with three DataCollectionCandidateConfigParameters-r19. NW would release one of them, after the candidate is configured as actual configuration (i.e., via CSI-ReportConfig) to UE after UE’s request via UAI. However, there is no way for NW to release only one DataCollectionCandidateConfigParameters-r19. i.e., NW has to provide two old DataCollectionCandidateConfigParameters-r19 again to release the other one.

Therefore, we would like to suggest to update both dataCollectionCandidateConfigList (i.e., candidates for multiple serving cells) and dataCollectionCandidateConfigParameterList (i.e., candidates for each serving cell) with ToAddModList and ToReleaseList.

DataCollectionPreferenceConfig-r19 ::= SEQUENCE { dataCollectionCandidateConfigList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19 OPTIONAL, -- Need R dataCollectionCandidateConfigToAddModList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19 OPTIONAL, -- Need N dataCollectionCandidateConfigToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex OPTIONAL, -- Need N ... } DataCollectionCandidateConfig-r19 ::= SEQUENCE { dataCollectionServCellIndex-r19 ServCellIndex, dataCollectionCandidateConfigParameterList-r19 SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigParameters-r19 OPTIONAL, -- Need R dataCollectionCandidateConfigParameterToAddModList-r19 SEQUENCE (SIZE (1.. maxCandidateConfig-r19)) OF DataCollectionCandidateConfigParameters-r19 OPTIONAL, -- Need N dataCollectionCandidateConfigParameterToReleaseList-r19 SEQUENCE (SIZE (1.. maxCandidateConfig-r19)) OF DataCollectionCandidateConfigId-r19 OPTIONAL, -- Need N ... }

Proposal 7: (X004 and S041) update both dataCollectionCandidateConfigList (i.e., candidates for multiple serving cells) and dataCollectionCandidateConfigParameterList (i.e., candidates for each serving cell) with ToAddModList and ToReleaseList.

## S049

In RAN2#131, RAN2 decided to include applicability reporting via RRCResumeComplete message, and this is captured in the RRC specification. So UE needs to consider the applicability reported in ResumeComplete while reporting the change in applicability in RRCReconfigurationComplete or UAI.

While reporting UAI, 1> if configured to report assistance information about the applicability of configurations subject to the applicability determination procedure: 2> if the applicability status of configurations subject to the applicability determination procedure has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or in UEAssistanceInformation or RRCResumeComplete): 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to report assistance information about the applicability of configurations subject to the applicability determination procedure; While reporting ReconfigurationComplete: 2> if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or 2> if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or 2> if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or UEAssistanceInformation or RRCResumeComplete); or 2> if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (in RRCReconfigurationComplete or UEAssistanceInformation or RRCResumeComplete): 3>for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows: 4>set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (RRCReconfigurationComplete or UEAssistanceInformation or RRCResumeComplete): 5>include an entry in the applicabilityInfoReportList and set the content as follows: 6>set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId; 6>set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6>if the applicabilityStatus is set to inapplicable: 7>if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference; 4> for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (RRCReconfigurationComplete or UEAssistanceInformation or RRCResumeComplete): 5>include an entry in the applicabilityInfoReportList and set the content as follows: 6>set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6>set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

Proposal 8: [S049] Consider applicability reported in RRCResumeComplete while reporting applicability in UAI or RRCReconfigurationComplete. S049 is agreed.

## S050

If the UE has sent a UAI and within 1 second a handover has occurred or if the UE has sent a UAI and a CHO or LTM cell switch has occurred, UE is required to resend the UAI. This is to avoid the transmission of UAI from source to target cell or to prepare CHO/LTM candidate cells, which can interrupt ongoing mobility. For applicability reporting, RAN2 has agreed that UE will send the new applicability in RRCReconfigurationComplete after handover. Thus, there is no need to retransmit the UAI.

This can be captured as below in the specification:

2> if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup: 3> if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or 3> if the RRCReconfiguration message is applied due to a conditional reconfiguration execution or an LTM cell switch procedure, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2: 4> if the UE assistance information was not initiated to report assistance information about the applicability of configurations subject to the applicability determination procedure: 5> initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information; 5> start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration; 4> start or restart the leave without response timer (if exists) or the wait timer (if exists) with the timer value set to the value in musim-CapabilityRestrictionConfig;

Proposal 9: [S050] UE does not resend the UAI for applicability reporting in below cases:

  - If the UAI was send 1s before the handover execution.

  - After LTM cell switch or CHO execution.

## FFS on associated ID

Agreements on associated ID 1 Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells). 2 Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment. FFS is we should have signalling indicating multi-cell. 3 We will not define areas. The Associated ID is 24 bits. 4 If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

In the last meeting, there was a view to introduce an explicit indication to distinguish single cell and multi-cells specific associated ID.

Currently, 24 bits are defined for associated ID, which can distinguish 16,777,216 different NW side additional conditions. Although it is hard to estimate the exact number of different NW configurations, we think it is enough to allocate cell specific associated ID.

In addition, if we introduce an explicit indication, we also need to introduce a new UE behavior to discard and apply a new associated ID for a given CSI-ReportConfig during HO.  Considering the WI is completed, it is difficult to spend more efforts to introduce additional feature.

Proposal 10: No explicit indication to distinguish cell specific and multi-cell specific associated ID.

## Conclusion

Based on the discussion on RIL open issues and FFS on associated ID, we propose the following.

Proposal 1: (E040) no spec change is needed.

Proposal 2: (E041) no spec change is needed.

Proposal 3: (N021/N003) it is not necessary to have CHOICE structure. There is no need to separate configuration for both inference and UE data collection. It is necessary to separate performance monitoring for BM use case and CSI prediction.

Proposal 4: (E042) RAN2 agree with Solution 1) i.e. remove the separate flag reportApplicabilityUAI-r19 and assume that applicapility reporting via UAI is implicitly configured when applicabilityReportConfig is included in otherConfig.

Proposal 5: (Z001) no change is needed.

Proposal 6: (S045) the start indication (i.e., dataCollectionStart-r19) is introduced per serving cell, and the TP proposed in S045 is adopted.

Proposal 7: (X004 and S041) update both dataCollectionCandidateConfigList (i.e., candidates for multiple serving cells) and dataCollectionCandidateConfigParameterList (i.e., candidates for each serving cell) with ToAddModList and ToReleaseList.

Proposal 8: [S049] Consider applicability reported in RRCResumeComplete while reporting applicability in UAI or RRCReconfigurationComplete. S049 is agreed.

Proposal 9: [S050] UE does not resend the UAI for applicability reporting in below cases:

  - If the UAI was send 1s before the handover execution.

  - After LTM cell switch or CHO execution.

Proposal 10: No explicit indication to distinguish cell specific and multi-cell specific associated ID.