3GPP TSG-RAN WG2 Meeting #131bis	R2-2507093

Prague, Czech, October 13th – 17th, 2025

Agenda Item:	8.6.2

Source: 	OPPO

Title: 	RRC open issues for R19 mobility

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to present our considerations on RRC open issues.

Discussion

Compression continuation during inter-CU LTM

In current RRC specification, the UE behaviours in case of inter-CU LTM is specified as follows:

##### 5.3.5.18.6 LTM cell switch execution … 1> if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange: … 2> at the end of the procedure, for each drb-Identity value that is part of the current UE configuration: 3> if the LTM cell switch is triggered on the MCG; or 3> if the LTM cell switch is triggered on the SCG and this DRB is using the secondary key; or 3> if the LTM cell switch is triggered on the SCG and the keyToUse for this DRB is changed: … 4> if drb-ContinueROHC is included in pdcp-Config: 5> indicate to lower layer that drb-ContinueROHC is configured; 4> if drb-ContinueEHC-DL is included in pdcp-Config: 5> indicate to lower layer that drb-ContinueEHC-DL is configured; 4> if drb-ContinueEHC-UL is included in pdcp-Config: 5> indicate to lower layer that drb-ContinueEHC-UL is configured; 4> if drb-ContinueUDC is included in pdcp-Config: 5> indicate to lower layer that drb-ContinueUDC is configured; 4> re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

As yellow-highlighted, the UE determines whether to continue or reset header/uplink data compression protocols subjecting to the indications (i.e., drb-ContinueROHC/ drb-ContinueEHC-DL/ drb-ContinueEHC-UL/ drb-ContinueUDC) included in PDCP-Config for each DRB.

While according to the field descriptions of these fields, it is specified that the field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the bearer is configured as DAPS bearer. This field can be configured for both DRB and multicast MRB.

drb-ContinueEHC-DL Indicates whether the PDCP entity continues or resets the downlink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

drb-ContinueEHC-UL Indicates whether the PDCP entity continues or resets the uplink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

uplinkDataCompression Indicates the UDC configuration that the UE shall apply. Network does not configure uplinkDataCompression for a DRB, if headerCompression or ethernetHeaderCompression is already configured or outOfOrderDelivery or DAPS is configured for the DRB. The maximum number of DRBs where uplinkDataCompression can be applied is two. The network reconfigures uplinkDataCompression only upon reconfiguration involving PDCP re-establishment. If the field is set to drb-ContinueUDC, the PDCP entity continues the uplink data compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is set to drb-ContinueUDC only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated.

For LTM, whether there is termination point change for a DRB is unfixed, i.e., due to the support of subsequent LTM, there is a case that the PDCP termination point for a DRB is changed when the UE switches from Cell B to Cell A, but remains unchanged when the UE switches from Cell C to Cell A. Since the trajectory is unpredictable, the indications for header/uplink data compression continuation can not be preconfigured. The issue is similar to what we have discussed for other L2 reset handlings such as RLC/PDCP re-establishment.

During LTM execution, the UE performs PDCP re-establishment when the ltm-NoSecurityChangeID of the target cell is not equal to the value of ltm-ServingCellNoSecurityChange which means that there is always PDCP termination point change for the concerned DRBs. In this case, the indications for header/uplink data compression continuity should not be configured according to the field descriptions. Furthermore, as in current LTM execution procedure, there is no case that the UE needs to perform PDCP re-establishment while the PDCP termination point is not changed. Therefore, drb-ContinueROHC/ drb-ContinueEHC-DL/ drb-ContinueEHC-UL/ drb-ContinueUDC should not configured in PDCP-Config if it is included in LTM candidate cell configuration and the UE does not need to check whether these fields are configured during LTM execution.

drb-ContinueROHC/drb-ContinueEHC-DL/drb-ContinueEHC-UL/drb-ContinueUDC are not configured in the PDCP-Config if it is part of an RRCReconfiguration message within the LTM-Config.

The UE does not need to check whether drb-ContinueROHC/drb-ContinueEHC-DL/drb-ContinueEHC-UL/drb-ContinueUDC are configured during inter-CU LTM execution.

RAN2 to adopt the Text Proposal 1 in Annex.

RRC and MAC misalignment issues

For the timer used to maintain the TA for CLTM candidate cell, it is named as ltm-Candidate-TimeAlignmentTimer/ ltm-Candidate-TimeAlignmentTimerTAG in MAC while ltm-TimerAlignmentTimer/ ltm-TimeAlignmentTimerTag2 in RRC. For alignment, the name used in MAC is preferred since it can more directly reflect the usage of the timer.

MAC spec ## 5.2 Maintenance of Uplink Time Alignment RRC configures the following parameters for the maintenance of UL time alignment: … - ltm-Candidate-TimeAlignmentTimer which controls how long the MAC entity considers the CLTM candidate cell associated with this timer to be uplink time aligned. Each ltm-Candidate-TimeAlignmentTimer is associated with one CLTM candidate cell; - ltm-Candidate-TimeAlignmentTimerTAG2 which controls how long the MAC entity considers the CLTM candidate cell associated with this timer to be uplink time aligned for TAG with ID tag2-Id. This timer is configured if two TAGs are configured for the CLTM candidate cell.

RRC spec

Align the name of the timers used to maintain the TA for CLTM candidate cell in MAC and RRC specifications, e.g., ltm-Candidate-TimeAlignmentTimer/ ltm-Candidate-TimeAlignmentTimerTAG are adopted.

In last RAN2 meeting, the following agreement was made on how to handle the ltm-Candidate-TimerAlignmentTimer when the corresponding CLTM candidate cell is released.

- Stop the running ltm-Candidate-TimeAlignmentTimer for the corresponding candidate cell and release the stored TA value for the candidate cell if the corresponding CLTM candidate configuration is released.

The agreement is captured in current MAC specification as follows:

## 5.2x Maintenance of UL Synchronization for CLTM candidate cell The MAC entity shall for each CLTM candidate cell: … 1> when the CLTM candidate configuration(s) is released as specified in TS 38.331 [5]: 2> stop the running ltm-Candidate-TimeAlignmentTimer and ltm-Candidate-TimeAlignmentTimerTAG2 associated with the corresponding CLTM candidate cell(s), if any; 2> release the stored TA value for the corresponding CLTM candidate cell(s), if any.

While in our understanding, MAC may not be aware of whether the CLTM candidate cell is released as the LTM configuration is managed by the RRC layer. To make the procedure clearer, it is proposed that the RRC explicitly informs the lower layer of the release of the CLTM candidate cell configuration. Upon receiving this indication, the MAC layer shall stop the corresponding CLTM timers if they are running.

RRC indicates the lower layers that the CLTM candidate cell configuration is released, based on which MAC stops the corresponding CLTM timers.

RAN2 to adopt the Text Proposal 2 in Annex.

MAC F02

As indicated by MAC rapporteur, MAC-F02 should be considered as an RRC issue.

| Ofinno | In 5.y.3, UE selects the valid CG for L3-based CLTM based on the selected beam above a threshold. In our understanding, RAN2 did not introduce any new threshold for this. However, the selection seems to be based on beam being above cg-LTM-RSRP-ThresholdSSB. Further, this field is not defined in 38.331. 3> if cg-LTM-Configuration is configured for the CLTM target cell (i.e. the SpCell corresponding to the target configuration indicated by Target Configuration ID), if two TAGs are not configured for the CLTM target cell, and if at least one of the SSB(s) of the CLTM target cell with SS-RSRP above cg-LTM-RSRP-ThresholdSSB and amongst the SSBs associated with the CG is available: 4> select the SSB(s) with SS-RSRP above the cg-LTM-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured uplink grant; 4> if the ltm-Candidate-TimeAlignmentTimer associated with the CLTM target cell is running in the first available CG occasion corresponding to one of the seleted SSB for initial uplink transmission according to clause 5.8.2: 5> process the stored Timing Advance Command associated with the running ltm-Candidate-TimeAlignmentTimer (see clause 5.2); 5> consider the RACH-less CLTM cell switch to be ongoing; We suggest to either re-use cg-RRC-RSRP-ThresholdSSB or define cg-LTM-RSRP-ThresholdSSB in 38.331. In either case, there is change required in 38.331. [Rapp]: This should be defined in RRC, as we discussed and concluded before. Similar threshold as cg-RRC-RSRP-ThresholdSSB should be introduced for LTM. This is an RRC issue. [Rapp2]: After coordination with RRC Rapp/Tony, let’s discuss this issue in MAC. | Issue Type: Not essential not important How to address it: based on companies’ contribution. Issue Number: MAC-F02 |
| --- | --- | --- |

Further discussion is needed on how to configure the RSRP threshold for SSB selection during L3-based CLTM execution, i.e., whether to reuse cg-RRC-RSRP-ThresholdSSB or introduce cg-LTM-RSRP-ThresholdSSB.

In current specification, cg-RRC-RSRP-ThresholdSSB is used for SSB selection for RACH-less handover and is configured in CG-RRC-Configuration. The CG-RRC-Configuration is a common field that can be referred by cg-LTM-Configuration for LTM and cg-RRC-Configuration for RACH-less handover. In R18, since the UE does not need to select an SSB based on the RSRP threshold during RACH-less LTM, it is specified that the cg-RRC-RSRP-ThresholdSSB field is absent in cg-LTM-Configuration. In R19, SSB selection based on RSRP threshold is introduced for L3-based CLTM, we see no issue to reuse the cg-RRC-RSRP-ThresholdSSB in CG-RRC-Configuration.

cg-RRC-RSRP-ThresholdSSB is reused for SSB selection during L3-based CLTM execution. MAC and RRC make alignment for name of the parameter.

Conclusion

Based on the discussion in section2, we have following proposals:

Proposal 1	drb-ContinueROHC/drb-ContinueEHC-DL/drb-ContinueEHC-UL/drb-ContinueUDC are not configured in the PDCP-Config if it is part of an RRCReconfiguration message within the LTM-Config.

Proposal 2	The UE does not need to check whether drb-ContinueROHC/drb-ContinueEHC-DL/drb-ContinueEHC-UL/drb-ContinueUDC are configured during inter-CU LTM execution.

Proposal 3	RAN2 to adopt the Text Proposal 1 in Annex.

Proposal 4	Align the name of the timers used to maintain the TA for CLTM candidate cell in MAC and RRC specifications, e.g., ltm-Candidate-TimeAlignmentTimer/ ltm-Candidate-TimeAlignmentTimerTAG are adopted.

Proposal 5	RRC indicates the lower layers that the CLTM candidate cell configuration is released, based on which MAC stops the corresponding CLTM timers.

Proposal 6	RAN2 to adopt the Text Proposal 2 in Annex.

Proposal 7	cg-RRC-RSRP-ThresholdSSB is reused for SSB selection during L3-based CLTM execution. MAC and RRC make alignment for name of the parameter.

Annex

- Text Proposal 1

5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

1>	if this procedure is triggered due to fulfilment of LTM cell switch execution conditions:

2> if more than one LTM candidate configuration has triggered this procedure:

3>	select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution;

1>	if the LTM cell switch is triggered on the MCG; or

1>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID, and VarLTM-ServingCellNoSecurityChange;

-	the ltm-Config and ltm-ConfigNRDC (if configured);

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

3>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

-	the ServingCellConfigCommon of the PCell;

1>	if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group(s) for which the RRCReconfiguration message is applied due to the triggered LTM cell switch procedure, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group(s) for which the RRCReconfiguration message is applied due to the triggered LTM cell switch procedure;

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	if the LTM cell switch is triggered on the MCG:

3>	update the master security key by performing the AS security key update procedure as specified in 5.3.5.7;

2> else if the LTM cell switch is triggered on the SCG:

3>	consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange associated to the the field ltm-NoSecurityChangeID as the selected sk-Counter value, and update the secondary key by performing security key update procedure as specified in 5.3.5.7;

3>	remove the selected sk-Counter value from the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange;

2>	at the end of the procedure, for each drb-Identity value that is part of the current UE configuration:

3>	if the LTM cell switch is triggered on the MCG; or

3>	if the LTM cell switch is triggered on the SCG and this DRB is using the secondary key; or

3>	if the LTM cell switch is triggered on the SCG and the keyToUse for this DRB is changed:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KgNB) or secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

4>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

3>	else if LTM cell switch is triggered on the SCG and this DRB is using the master key:

4>	if the RLC entity of an RLC bearer associated with this DRB is re-established or released during LTM cell switch execution:

5>	if this DRB is an AM DRB:

6>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

----Unrelated parts are omitted---

#### –	PDCP-Config

The IE PDCP-Config is used to set the configurable PDCP parameters for signalling, MBS multicast and data radio bearers.

PDCP-Config information element

-- ASN1START

-- TAG-PDCP-CONFIG-START

PDCP-Config ::=         SEQUENCE {

drb                     SEQUENCE {

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200,

ms250, ms300, ms500, ms750, ms1500, infinity}       OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup1

pdcp-SN-SizeDL          ENUMERATED {len12bits, len18bits}                               OPTIONAL, -- Cond Setup2

headerCompression       CHOICE {

notUsed                 NULL,

rohc                    SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0001           BOOLEAN,

profile0x0002           BOOLEAN,

profile0x0003           BOOLEAN,

profile0x0004           BOOLEAN,

profile0x0006           BOOLEAN,

profile0x0101           BOOLEAN,

profile0x0102           BOOLEAN,

profile0x0103           BOOLEAN,

profile0x0104           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

uplinkOnlyROHC          SEQUENCE {

maxCID                  INTEGER (1..16383)                                      DEFAULT 15,

profiles                SEQUENCE {

profile0x0006           BOOLEAN

},

drb-ContinueROHC            ENUMERATED { true }                                 OPTIONAL    -- Need N

},

...

},

integrityProtection     ENUMERATED { enabled }                                          OPTIONAL,   -- Cond ConnectedTo5GC1

statusReportRequired    ENUMERATED { true }                                             OPTIONAL,   -- Cond Rlc-AM-UM

outOfOrderDelivery      ENUMERATED { true }                                             OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond DRB

moreThanOneRLC          SEQUENCE {

primaryPath             SEQUENCE {

cellGroup               CellGroupId                                                 OPTIONAL,   -- Need R

logicalChannel          LogicalChannelIdentity                                      OPTIONAL    -- Need R

},

ul-DataSplitThreshold   UL-DataSplitThreshold                                           OPTIONAL,   -- Cond SplitBearer

pdcp-Duplication            BOOLEAN                                                     OPTIONAL    -- Need R

}                                                                                           OPTIONAL,   -- Cond MoreThanOneRLC

t-Reordering                ENUMERATED {

ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40,

ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220,

ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250,

ms1500, ms1750, ms2000, ms2250, ms2500, ms2750,

ms3000, spare28, spare27, spare26, spare25, spare24,

spare23, spare22, spare21, spare20,

spare19, spare18, spare17, spare16, spare15, spare14,

spare13, spare12, spare11, spare10, spare09,

spare08, spare07, spare06, spare05, spare04, spare03,

spare02, spare01 }                                          OPTIONAL, -- Need S

...,

[[

cipheringDisabled       ENUMERATED {true}                                                   OPTIONAL    -- Cond ConnectedTo5GC

]],

[[

discardTimerExt-r16     SetupRelease { DiscardTimerExt-r16 }                                OPTIONAL,    -- Cond DRB2

moreThanTwoRLC-DRB-r16  SEQUENCE {

splitSecondaryPath-r16  LogicalChannelIdentity                                          OPTIONAL,   -- Cond SplitBearer2

duplicationState-r16    SEQUENCE (SIZE (3)) OF BOOLEAN                                  OPTIONAL    -- Need S

}                                                                                           OPTIONAL,   -- Cond MoreThanTwoRLC-DRB

ethernetHeaderCompression-r16  SetupRelease { EthernetHeaderCompression-r16 }               OPTIONAL    -- Need M

]],

[[

survivalTimeStateSupport-r17   ENUMERATED {true}                                            OPTIONAL,   -- Cond Drb-Duplication

uplinkDataCompression-r17      SetupRelease { UplinkDataCompression-r17 }                   OPTIONAL,   -- Cond Rlc-AM

discardTimerExt2-r17           SetupRelease { DiscardTimerExt2-r17 }                        OPTIONAL,   -- Need M

initialRX-DELIV-r17            BIT STRING (SIZE (32))                                       OPTIONAL    -- Cond MRB-Initialization

]],

[[

pdu-SetDiscard-r18             ENUMERATED {true}                                            OPTIONAL,   -- Need R

discardTimerForLowImportance-r18   SetupRelease { DiscardTimerForLowImportance-r18 }        OPTIONAL,   -- Cond DRB2

primaryPathOnIndirectPath-r18  ENUMERATED {true}                                            OPTIONAL,   -- Cond SplitBearerMP

sn-GapReport-r18               ENUMERATED {true}                                            OPTIONAL    -- Need R

]]

}

EthernetHeaderCompression-r16 ::=  SEQUENCE {

ehc-Common-r16                     SEQUENCE {

ehc-CID-Length-r16                 ENUMERATED { bits7, bits15 },

...

},

ehc-Downlink-r16               SEQUENCE {

drb-ContinueEHC-DL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL,   -- Need M

ehc-Uplink-r16                 SEQUENCE {

maxCID-EHC-UL-r16              INTEGER (1..32767),

drb-ContinueEHC-UL-r16         ENUMERATED { true }                                      OPTIONAL,   -- Need N

...

}                                                                                           OPTIONAL    -- Need M

}

UL-DataSplitThreshold ::= ENUMERATED {

b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800,

b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400,

b6553600, infinity, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

DiscardTimerExt-r16 ::= ENUMERATED {ms0dot5, ms1, ms2, ms4, ms6, ms8, spare2, spare1}

DiscardTimerExt2-r17 ::= ENUMERATED {ms2000, spare3, spare2, spare1}

UplinkDataCompression-r17 ::= CHOICE {

newSetup                      SEQUENCE {

bufferSize-r17                ENUMERATED {kbyte2, kbyte4, kbyte8, spare1},

dictionary-r17                ENUMERATED {sip-SDP, operator}                            OPTIONAL    -- Need N

},

drb-ContinueUDC           NULL

}

DiscardTimerForLowImportance-r18 ::= ENUMERATED {ms0, ms2, ms4, ms6, ms8, ms10, ms12, ms14, ms18, ms22, ms26, ms30, ms40, ms50, ms75, ms100}

-- TAG-PDCP-CONFIG-STOP

-- ASN1STOP

PDCP-Config field descriptions

cipheringDisabled If included, ciphering is disabled for this DRB regardless of which ciphering algorithm is configured for the SRB/DRBs. The field may only be included if the UE is connected to 5GC. Otherwise the field is absent. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms10 corresponds to 10 ms, value ms20 corresponds to 20 ms and so on. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

discardTimerExt Value in ms of discardTimer specified in TS 38.323 [5]. Value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1ms and so on. If this field is present, the field discardTimer is ignored and discardTimerExt is used instead.

discardTimerExt2 Value in ms of discardTimerExt specified in TS 38.323 [5]. Value ms2000 corresponds to 2000 ms. If this field is present, the field discardTimer and discardTimerExt are ignored and discardTimerExt2 is used instead.

discardTimerForLowImportance Value in ms of discardTimerForLowImportance specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms2 corresponds to 2 ms and so on. The value of this timer for a PDCP entity is always configured shorter than discardTimer, discardTimerExt or discardTimerExt2, whichever is used for the PDCP entity. The presence of this field indicates that PSI-based SDU discard is configured for a DRB. The network configures at most 8 DRBs with discardTimerForLowImportance.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. This field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the bearer is configured as DAPS bearer, or if the PDCP-Config IE is part of an RRCReconfiguration message within the LTM-Config IE. This field can be configured for both DRB and multicast MRB.

duplicationState This field indicates the uplink PDCP duplication state for the associated RLC entities at the time of receiving this IE. If set to true, the PDCP duplication state is activated for the associated RLC entity. The index for the indication is determined by ascending order of logical channel ID of all RLC entities other than the primary RLC entity indicated by primaryPath in the order of MCG and SCG, as in clause 6.1.3.32 of TS 38.321 [3]. For MP, the index for the indication is determined by ascending order of direct path (where i is ascending order of logical channel ID of secondary RLC entities) and indirect path, as in clause 6.1.3.32 of TS 38.321 [3]. If the number of associated RLC entities other than the primary RLC entity is two, UE ignores the value in the largest index of this field. If the field is absent, the PDCP duplication states are deactivated for all associated RLC entities.

ethernetHeaderCompression This fields configures Ethernet Header Compression. This field can only be configured for a bi-directional DRB or a bi-directional multicast MRB. The network reconfigures ethernetHeaderCompression only upon reconfiguration involving PDCP re-establishment and with neither drb-ContinueEHC-DL nor drb-ContinueEHC-UL configured. Network only configures this field when uplinkDataCompression is not configured.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configured ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC and EHC can be both configured simultaneously for a DRB or a multicast MRB. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment or involving PDCP entity reconfiguration to configure DAPS bearer(s), and without any drb-ContinueROHC. Network configures headerCompression to notUsed when outOfOrderDelivery is configured. Network only configures this field when uplinkDataCompression is not configured.

initialRX-DELIV Indicates the initial value of RX_DELIV during PDCP window initialization for multicast MRB as specified in TS 38.323 [5]. The first/leftmost bit of the bit string contains the most significant bit.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The network configures all DRBs with the same PDU-session ID with same value for this field. The value for this field cannot be changed after the DRB is set up.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5]. The total value of MAX_CIDs across all bearers for the UE should be less than or equal to the value of maxNumberROHC-ContextSessions parameter as indicated by the UE.

moreThanOneRLC This field configures UL data transmission when more than one RLC entity is associated with the PDCP entity. This field is not present if the bearer is configured as DAPS bearer.

moreThanTwoRLC-DRB This field configures UL data transmission when more than two RLC entities are associated with the PDCP entity for DRBs.

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the radio bearer is established.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. For PDCP entity with more than two associated RLC entities for UL transmission, this field is always present. If the field moreThanTwoRLC-DRB is present, the value of this field is ignored and the state of the duplication is indicated by duplicationState. For PDCP entity with more than two associated RLC entities, only NR RLC bearer is supported.

pdcp-SN-SizeDL PDCP sequence number size for downlink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdcp-SN-SizeUL PDCP sequence number size for uplink, 12 or 18 bits, as specified in TS 38.323 [5]. For SRBs only the value len12bits is applicable. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

pdu-SetDiscard If set to true, the UE shall perform PDU Set based discarding for this PDCP entity, as specified in TS 38.323 [5]. This field is only configured for a DRB.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 [5], clause 5.2.1 for UL data transmission when more than one RLC entity is associated with the PDCP entity. In this version of the specification, only cell group ID corresponding to MCG is supported for SRBs, except for the split SRB2 of the IAB-MT, and except when the UE is required to set the primaryPath to refer to the SCG or indirect path as specified in clause 5.7.3b.4. In this last case, if the network sends an RRCReconfiguration message (in NR-DC) or an EUTRA RRCConnectionReconfiguration message (in (NG)EN-DC) keeping SRB1 as split SRB, the network explicitly configures the primaryPath for the PDCP entity of SRB1 to refer to the MCG. In this version of the specification, only cell group ID corresponding to MCG is supported for DRBs when the SCG is deactivated. In MR-DC, the NW indicates cellGroup for split bearers using logical channels in different cell groups. The NW always indicates logicalChannel if CA based PDCP duplication is configured in the cell group indicated by cellGroup of this field. This field is also used for configuring Multi-path Primary Path defined in [5] for MP operation. In MP, when the primary path is indirect path, the field cellGroup and logicalChannel are absent, and the field primaryPathOnIndirectPath is set to true.

primaryPathOnIndirectPath Indicates that the primary path is SL indirect path, or primary path is associated with the N3C indirect path, for DRB when MP is configured.

sn-GapReport Indicates whether the PDCP entity is configured to send a PDCP SN gap report in the uplink, as specified in TS 38.323 [5]. This field is only configured for DRBs. This field can be included only if outOfOrderDelivery is not configured.

splitSecondaryPath Indicates the LCID of the split secondary RLC entity as specified in TS 38.323 [5] for fallback to split bearer operation when UL data transmission with more than two RLC entities is associated with the PDCP entity. This RLC entity belongs to a cell group that is different from the cell group indicated by cellGroup in the field primaryPath. This RLC entity belongs to the cell group of the direct path if primaryPathOnIndirectPath is set to true in MP case.

statusReportRequired For AM DRBs, AM MRBs and DAPS UM DRBs, indicates whether the DRB or the multicast MRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For DAPS AM DRBs, it also indicates whether the DRB is configured to send a second PDCP status report in the uplink, as specified in TS 38.323 [5].

survivalTimeStateSupport Indicates whether the DRB associated with this PDCP entity has survival time state support. If this field is configured to be true, all associated RLC entities are activated for PDCP duplication upon reception of a retransmission grant addressed to CS-RNTI, as specified in TS 38.321 [3].

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0 ms, value ms20 corresponds to 20 ms, value ms40 corresponds to 40 ms, and so on. When the field is absent the UE applies the value infinity. The value for this field cannot be changed in case of reconfiguration with sync, if the bearer is configured as DAPS bearer.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bytes, value b100 corresponds to 100 bytes, value b200 corresponds to 200 bytes, and so on. The network sets this field to infinity for UEs not supporting splitDRB-withUL-Both-MCG-SCG and when the SCG is deactivated or for multi-path U2N Remote UEs not supporting splitDRB-WithUL-BothDirectIndirect. If the field is absent when the split bearer is configured for the radio bearer first time, then the default value infinity is applied.

uplinkDataCompression Indicates the UDC configuration that the UE shall apply. Network does not configure uplinkDataCompression for a DRB, if headerCompression or ethernetHeaderCompression is already configured or outOfOrderDelivery or DAPS is configured for the DRB. The maximum number of DRBs where uplinkDataCompression can be applied is two. The network reconfigures uplinkDataCompression only upon reconfiguration involving PDCP re-establishment. If the field is set to drb-ContinueUDC, the PDCP entity continues the uplink data compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is set to drb-ContinueUDC only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the PDCP-Config IE is part of an RRCReconfiguration message within the LTM-Config IE.

EthernetHeaderCompression field descriptions

drb-ContinueEHC-DL Indicates whether the PDCP entity continues or resets the downlink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the PDCP-Config IE is part of an RRCReconfiguration message within the LTM-Config IE.

drb-ContinueEHC-UL Indicates whether the PDCP entity continues or resets the uplink EHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. The field is configured only in case of resuming an RRC connection or reconfiguration with sync, where the PDCP termination point is not changed and the fullConfig is not indicated. The network does not include the field if the PDCP-Config IE is part of an RRCReconfiguration message within the LTM-Config IE.

ehc-CID-Length Indicates the length of the CID field for EHC packet. The value bits7 indicates the length is 7 bits, and the value bits15 indicates the length is 15 bits. Once the field ethernetHeaderCompression-r16 is configured for a DRB or a multicast MRB, the value of the field ehc-CID-Length for this DRB or multicast MRB is not reconfigured to a different value.

ehc-Common Indicates the configurations that apply for both downlink and uplink.

ehc-Downlink Indicates the configurations that apply for only downlink. If the field is configured, then Ethernet header compression is configured for downlink. Otherwise, it is not configured for downlink.

ehc-Uplink Indicates the configurations that apply for only uplink. If the field is configured, then Ethernet header compression is configured for uplnik. Otherwise, it is not configured for uplink.

maxCID-EHC-UL Indicates the value of the MAX_CID_EHC_UL parameter as specified in TS 38.323 [5]. The total value of MAX_CID_EHC_UL across all bearers for the UE should be less than or equal to the value of maxNumberEHC-Contexts parameter as indicated by the UE.

UplinkDataCompression field descriptions

bufferSize This field indicates the buffer size applied for UDC as specified in TS 38.323 [5]. Value kbyte2 means 2048 bytes, kbyte4 means 4096 bytes and so on.

dictionary This field indicates which pre-defined dictionary is used for UDC as specified in TS 38.323 [5]. The value sip-SDP means that UE shall prefill the buffer with standard dictionary for SIP and SDP defined in TS 38.323 [5], and the value operator means that UE shall prefill the buffer with operator-defined dictionary.

| Conditional presence | Explanation |
| --- | --- |
| DRB | This field is mandatory present when the corresponding DRB/multicast MRB is being set up, absent for SRBs. Otherwise this field is optionally present, need M. |
| DRB2 | This field is optionally present in case of DRB, need M. Otherwise, it is absent for SRBs and MRBs. |
| Drb-Duplication | For SRBs, this field is absent. For DRBs, this field is absent if duplication is not configured. Otherwise, this field is optional, need R. |
| MoreThanOneRLC | This field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer (except for multicast MRB) with more than one associated logical channel and upon RRC reconfiguration with the association of additional logical channels to the PDCP entity. The field is also mandatory present in case the field moreThanTwoRLC-DRB is included in PDCP-Config. Upon RRC reconfiguration when a PDCP entity is associated with multiple logical channels, this field is optionally present need M. Otherwise, this field is absent. Need R. |
| MoreThanTwoRLC-DRB | For SRBs, this field is absent. For DRBs, this field is mandatory present upon RRC reconfiguration with setup of a PDCP entity for a radio bearer with more than two associated logical channels and upon RRC reconfiguration with the association of one or more additional logical channel(s) to the PDCP entity so that the PDCP entity has more than two associated logical channels. Upon RRC reconfiguration when a PDCP entity is associated with more than two logical channels, this field is optionally present, Need M. Otherwise, the field is absent, Need R. |
| Rlc-AM | For RLC AM, the field is optionally present, need M. Otherwise, the field is absent. |
| Rlc-AM-UM | In case of DRB, for RLC UM (if the UE supports DAPS handover) or RLC AM, the field is optionally present, need R. In case of multicast MRB, if multicast MRB is associated with at least one RLC AM entity, the field is optionally present, need R. Otherwise, the field is absent. |
| Setup | The field is mandatory present in case of DRB setup. Otherwise the field is optionally present, need M. |
| SplitBearer | The field is absent for SRBs. Otherwise, the field is optional present, need M, in case of radio bearer with more than one associated RLC mapped to different cell groups. |
| SplitBearer2 | The field is mandatory present, in case of a split bearer except MP split bearer with primary path on direct path. Otherwise the field is absent. |
| SplitBearerMP | The field is absent for SRBs. Otherwise, the field is optionally present, need R, when MP is configured. |
| ConnectedTo5GC | The field is optionally present, need R, if the UE is connected to 5GC. Otherwise the field is absent. |
| ConnectedTo5GC1 | The field is optionally present, need R, if the UE is connected to NR/5GC or if the UE supports user plane integrity protection when connected to E-UTRA/EPC (as specified in TS 33.401 [30]). Otherwise the field is absent. |
| Setup1 | This field is mandatory present in case of DRB setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| Setup2 | This field is mandatory present in case for radio bearer setup for RLC-AM and RLC-UM. Otherwise, this field is absent, Need M. |
| MRB-Initialization | This field is mandatory present in case of multicast MRB setup or in case UE configured with multicast reception in RRC_INACTIVE resumes the RRC connection. In case of PDCP re-establishment for multicast MRB, this field is optionally present, Need N. Otherwise, this field is absent, Need N. |

- Text Proposal 2

RRC

##### 5.3.5.18.2	LTM candidate configuration release

The UE shall:

1>	for each ltm-CandidateId value included in the ltm-CandidateToReleaseList that is part of the current UE configuration:

2>	remove the corresponding LTM-Candidate.

2> if executionCondition is configured:

3> inform lower layers that the LTM candidate configuration is released;

MAC

## 5.2x	Maintenance of UL Synchronization for CLTM candidate cell

The MAC entity shall for each CLTM candidate cell:

1>	when an LTM Candidate Timing Advance Command MAC CE described in clause 6.1.3.4x is received:

2>	if two TAGs are configured for the CLTM candidate cell:

3>	store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell for the indicated TAG as specified in clause 6.1.3.4x;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the indicated LTM candidate cell for the indicated TAG as specified in clause 6.1.3.4x;

2>	else:

3>	store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell as specified in clause 6.1.3.4x;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer associated with the indicated LTM candidate cell as specified in clause 6.1.3.4x.

1>	when the CLTM candidate configuration(s) is released as indicated by upper layers:

2>	stop the running ltm-Candidate-TimeAlignmentTimer and ltm-Candidate-TimeAlignmentTimerTAG2 associated with the corresponding CLTM candidate cell(s), if any;

2>	release the stored TA value for the corresponding CLTM candidate cell(s), if any.

NOTE:	If the UE receives more TA values than it can store, it is up to the UE implementation which TA values to store or discard.