# GPP TSG RAN WG2 Meeting #131bis      	                        R2-2507101
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	8.2.2

Source:	Apple

Title:	Discussion on remaining open issues of A-IoT

WID/SID:	Ambient_IoT_Solutions_Ph2 – Release 20

Document for:	Discussion and Decision

# 1 Introduction

In this paper, we discuss some remaining issues for the normative work for Ambient IoT MAC layer design for Rel-19 Ambient IoT.

# 2 Discussion

### 2.1 A-IoT NAS “response unavailable” issue

In RAN2#131 meeting [1], RAN2 has made the following agreements on the “delayed NAS” issue:

Agreements on no data available due to delay in NAS

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

- The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device.

- The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command.

- RAN2 would like to check if there is a case where NAS doesn’t provide a response at all.   If this case exists, RAN2 will discuss this issue.

Based on CT1 specification of A-IoT TS 24.369 [2], the A-IoT NAS layer may ignore the incoming A-IoT NAS PDU, which is equivalent to a “silent discard”. For example, as shown below, if the message is too short, or message contain unknown message type, or the message is wrong in semantics and there is no specified reaction in A-IOT NAS layer of the UE, the NAS message will be dropped silently by the UE.

### 6.2.1 Message too short When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3]. ### 6.2.2 Message too long Editor's note: Maximum AIoT NAS message length is FFS and to be defined by RAN3. ## 6.3 Unknown or unforeseen message type If the AIoT device or the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message. NOTE: A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3]. ## 6.4 Non-semantical mandatory information element errors When on receipt of a message, a) an "imperative message part" error; or b) a "missing mandatory IE" error; is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall ignore the AIoT NAS message and the network shall: a) try to treat the message (the exact further actions are implementation dependent); or b) ignore the message. ## <text omitted> ## 6.7 Messages with semantically incorrect contents When a message with semantically incorrect contents is received, the AIoT device shall perform the foreseen reactions of the procedural part of clause 5. If, however no such reactions are specified, the UE shall ignore the message. The network should follow the same procedure.

With the same gist of A-IoT NAS specification, we think if upper layer ignores the A-IoT NAS message and notifies the AS layer (i.e., A-IoT MAC layer), A-IoT MAC layer shall simply do the same as “ignoring” and do nothing else. It is wrong and risky for the device to further communicate with the reader in this case because the “reader” may be a faked or invalid reader.

Also, when MAC layer does not signaling anything to the reader, the NW side may attempt to re-transmit the “Command” and this retransmission is better to be a full version with a correctly formed command instead of an “empty” command as RAN2 agreed to use in “0 SDU” case. From this perspective, the behavior to not sending anything to reader is a reasonable choice

Proposal 1: 	If NAS layer informs MAC layer that NAS command is ignored, the MAC layer of A-IoT device will do nothing (i.e., not sending any signaling to the reader in D2R grant scheduled).

Proposal 2: 	When A-IOT command (R2D Upper layer Data transfer) message receives no response from the device, the reader informs A-IoT Function. It is up to A-IoT Function to decide whether to re-initiate the command procedure. No MAC layer impact is expected.

Therefore, although there is a case that NAS layer does not provide any response, this has no RAN2 impact because there is no expected AS layer signaling to be generated. In other words, the agreements made in RAN2#131 [1] can be kept unchanged. To be specific, for all the following case, the “0 SDU with MDI=0” can be used by the A-IoT device as a proper response:

- NAS layer informs MAC layer that it needs more time to process the received A-IoT command message;

- NAS layer does not inform MAC layer anything.

- NAS layer informs MAC layer it is unsure whether it will generate a response or not.

In any of the above cases, it is fine for the A-IoT reader to send the follow-up R2D Upper Layer Data Transfer w/o actual command to check again at a later time.

Proposal 3: 	RAN2-agreed solution (“0 SDU”) is used for all of the following cases:

1) NAS informs MAC layer a response will be delayed;

2) NAS layer inform nothing to MAC layer;

3) NAS layer informs MAC layer it is unsure whether there will be a “delayed response” or not.

From the reader perspective, there is a potential corner issue that when it sends R2D Upper layer data transfer message w/o command (i.e., “empty command”), the A-IoT device may not generate response at all. This may be due to packet loss in either UL or DL or other unforeseen reasons. In this case, the MAC layer of A-IoT reader’s MAC layer may choose to re-transmit the “empty command” message again or inform the A-IoT Function. This is up to reader implementation and has no MAC specification impact

Proposal 4: 	It is up to reader implementation on how to handle “no response” after sending an “R2D Upper layer Data Transfer w/o command” message. There is no MAC spec impact.

### 2.2 Maximum supported AIoT NAS container length

### There is an incoming LS from CT1 [3] inquiry about the maximum NAS container the AS layer can provide, with LS content copied below:

CT1 is currently working on the coding aspects of AIoT NAS command messages.

CT1 kindly requests clarification on the maximum supported AIoT NAS container length in both R2D and D2R directions, including segmentation, if supported.

### For R2D direction, segmentation is not supported. The R2D messages have the maximum size as 1000bits. Based on the MAC PDU format of R2D Upper Layer Data Transfer message containing Data SDU in TS 38.391 [4], six octets are used in MAC header. Therefore, there are 119 bytes can be used to contain a NAS layer “command” message.

Proposal 5: 	Maximum supported A-IoT NAS PDU size in R2D direction is 119-byte.

### For D2R direction, since MAC layer supports segmentation, there is no maximum size limit in A-IoT MAC layer to carry an upper layer message. CT1 can decide the D2R NAS layer message size itself.

Proposal 6: 	There is no size limit for A-IoT NAS PDU from RAN2 perspective in D2R direction. It is up to CT1 to decide a proper maximum size.

### 2.3 Paging ID length

### In the SA2 LS to RAN2, SA2 has asked whether Paging ID length of 256 or even 496 bit can be considered, as

SA2 would like to check with RAN2 on whether extending the AIoT Device permanent ID to 256-bit and 496-bit is feasible from RAN2 point of view.

### In our view, the 496-bit permanent ID does not make too much sense. That means a 2.05*10149 devices worldwide, which is a super big number. As only low-cost, low-power, low-complexity Type 1 device is supported in Rel-19, there is no expectation of this device will last for a long time, given that the building material is to be cheap and not endurable. Logically, if a device is not cheap, then it cannot be mass-deployed due to the high cost, then it will not need a very, very long ID. If the device is cheap, then the device itself has a short life-span, and the ID can always be reused after some time. Hence, we fail to understand how this super cheap, short-lifespan device can be reliably used in “healthcare and global trade”, which is the main reason to require a 496-bit Permanent ID.

### Also, SA2 has designed “disable” mechanism to kill the device if a device is no longer needed. This can also help to recycle the ID usage if ID space is in shortage.

### On the other hand, from RAN2 signaling design perspective, we plan to support multiple paging IDs in future signaling. Support 496-bit paging ID render that infeasible.

So, RAN2 should tell SA2 to only consider 256-bit Paging ID.

Proposal 7: 	RAN2 reply to SA2 that only 256-bit Paging ID can be considered.

# 3 Conclusion

In this contribution, we discuss the remaining open issues for Ambient IoT, and have the following proposals:

Proposal 1: 	If NAS layer informs MAC layer that NAS command is ignored, the MAC layer of A-IoT device will do nothing (i.e., not sending any signaling to the reader in D2R grant scheduled).

Proposal 2: 	When A-IOT command (R2D Upper layer Data transfer) message receives no response from the device, the reader informs A-IoT Function. It is up to A-IoT Function to decide whether to re-initiate the command procedure. No MAC layer impact is expected.

Proposal 3: 	RAN2-agreed solution (“0 SDU”) is used for all of the following cases:

1) NAS informs MAC layer that a response will be delayed;

2) NAS layer inform nothing to MAC layer;

3) NAS layer informs MAC layer it is unsure whether there will be a “delayed response” or not.

Proposal 4: 	It is up to reader implementation on how to handle “no response” after sending an “R2D Upper layer Data Transfer w/o command” message. There is no MAC spec impact.

Proposal 5: 	Maximum supported A-IoT NAS PDU size in R2D direction is 119-byte.

Proposal 6: 	There is no size limit for A-IoT NAS PDU from RAN2 perspective in D2R direction. It is up to CT1 to decide a proper maximum size.

Proposal 7: 	RAN2 reply to SA2 that only 256-bit Paging ID can be considered.

# 4 References

[1] Chairman’s note R2-131

[2] TS 24.369 v1.0.0
[3] C1-255679 LS on the maximum supported AIoT NAS container length

[4] TS 38.391 v19.0.0

[5] S2-2507793 LS on AIoT Device Permanent ID Length