3GPP TSG-RAN WG2 Meeting #131bis	R2-2507113

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	10.3.1

Source:	Apple

Title:	Views on Directions of 6G User Plane Enhancements

WID/SID:	FS_6G_Radio

Document for:	Discussion / Decision

# Introduction

With the advent and growing maturity of various computing technologies and applications, such as AI, XR, robotics, and self-driving vehicles etc, it is anticipated that 6G radio access technology should be developed to meet the cellular connectivity requirements of these emerging use cases. In particular, to make sure that 6GR user plane protocols can transfer the data over the air in a practical manner, it is crucial to conceive how the user plane should be evolved in 6G based on the experiences we have in 5G product implementation, as well as the observations made in the field deployment. From our perspectives, we think RAN2 should strive to develop a User Plane protocol for 6GR that is simple, efficient, and can deliver optimal user experiences:

- Simplicity: In the name of flexibility, 5G has introduced multiple options for the same functionality. Moreover, some of the mechanisms may be considered redundant throughout the L2 protocol layers. This has resulted in implementation complexity of 5G products. Hence, 6GR should aim to simplify the user plane protocols to reduce processing burdens.

- Efficiency: 5G has introduced many features that have promised to achieve higher efficiency, but many of these features have not been deployed in a sensible way. Consequently, we have observed excessive UE power wastage in the field. By exploiting the recent advancement of AI/ML and tighter coordination between radio and Application, a better energy efficiency could be realized in 6GR.

- User Experience: For emerging use cases and applications such as Immersive XR, user experience is of paramount importance. The 6GR cellular connectivity should be capable of handling the instantaneous need of Application to deliver a truly immersive experience. This is essential to consider end-to-end QoS requirements and characteristics of these new use cases for the design of 6GR user plane protocol.

This paper elaborates some of our perspectives on 6G user plane enhancement in these three different aspects (namely Simplification, User Experience, and Efficiency), along with some suggestions about the high-level directions that we think RAN2 can further discuss in this Study Item.

# Simplification of User Plane Operations

## Functionality Re-Arrangement

To ensure that robust, reliable, and secure communication can be supported to fulfil diverse requirements anticipated in 6G applications, the fundamental functionalities of 5G NR L2 protocols including sequence number management, Automatic Repeat reQuest (ARQ), segmentation, reassembly, and security protection, remain to be essential for 6G.

On the other hand, it is also desirable to streamline the user plane processing by minimizing some of the redundant operations. For example, it can be noted that payload of a packet on a 5G radio bearer may be retransmitted over the air several times via different mechanisms across multiple protocol layers. To be specific, it may be retransmitted by MAC layer based on HARQ, by RLC layer based on L2 ARQ (with RLC-AM), and by PDCP layer based due to some RRC procedures (e.g. handover). This is reasonable as lossless communications are needed for traditional TCP transport protocols. Nevertheless, we have observed that more and more traffics are operating based on QUIC nowadays. Thus, the need to balance the gap between transport network (TCP slow start/congestion, IP layer driven aspects) and mobile links may be reduced in 6G. Instead, to achieve the target reliability, we foresee that 6G will rely even more on MAC/PHY improvements including HARQ enhancement and distributed multi-TRP communications etc. With this mind, even though L2 ARQ remains to be a crucial functionality especially for control plane messages, it may be deemed to be less pivotal for user plane traffics in 6G.

Thus, RAN2 may consider moving some functionalities such as L2 ARQ to the higher layer. In one instance, 6G protocol stack could be simplified by merging the ARQ functionality of RLC-AM into PDCP layer, and all SDU-based retransmissions can be jointly performed by PDCP rather than distributed across multiple L2 sublayers. We suggest that RAN2 can study if/how some L2 functionalities can be reshuffled/re-arranged to provide a more streamlined user plane protocol that is more suitable for the traffics that are foreseeable in 6G.

Proposal 1: 6GR User plane should retain the functionalities including sequence number (SN) management, ARQ, segmentation, reassembly, security protection, and in-sequence delivery, but RAN2 can study if some of these functionalities and protocol layers may be re-arranged or simplified in 6GR to better accommodate the recent trends of transport layer traffic.

## Processing Overhead Minimization

According to the 6G study item description [1], it is important to reduce UE burden and minimize the number of configuration options for the same functionality. We think these aspects are indeed crucial in terms of decreasing 6G user plane processing overhead.

Currently, since PDCP SDU and PDCP PDU is one-to-one mapping, in UL the UE requires to perform security handling and header addition to each of the incoming PDCP SDU, which is obviously inefficient from UE processing perspective, especially when many packets with small payload size arrive within a short period of time. This results in high header processing overhead, as header is added to each SDU at multiple L2 sublayers. Furthermore, the CPU load can surge when the UE needs to process many small packets at the same time. Thus, we think RAN2 should study the possibility of mapping multiple SDUs to one PDU via packet concatenation, which can significantly decrease the UE burden of processing.

On the other hand, 5G NR has provided flexibility of data structure through many different packet header formats and associated options, that are optimized for different requirements. It is questionable whether 6G should support so many different options of L2 headers, considering that packet header processing can be costly for implementation of both UE and network. If we assume 6GR can offer higher data rates (spectral efficiency), transmission of a few extra bits could become more affordable. With this in mind, it is beneficial to have a more unified header structure in 6G by reducing the options of e.g. sequence number (SN) length, segment offset in RLC, logical channel ID in MAC etc. Moreover, it may be redundant to have separate SN space for PDCP and RLC, considering that they are almost the same. Obviously, the overhead can be further optimized if only one common SN shared by the two layers is included in the header.

Proposal 2: RAN2 should study how L2 protocol and procedures can be modified to reduce User Plane processing overhead in 6GR, the directions such as SDU concatenation and header optimization may be considered.

# Optimization of User Experience

## Coordination between L2-ARQ and HARQ

Many 6G use cases such as immersive communications may require both high reliability and low latency. In Rel-19 work item of NR enhancement for XR, a couple of new RLC-AM mechanisms have been introduced to speed up the operation, including remaining time-based autonomous retransmission and poll triggering. However, since the triggering condition is based on the remaining time till PDCP discard timer expiry, these Rel-19 features rely on the presence of PDCP discard timer, which is not always configured. Therefore, it is desirable to reduce retransmission latency while minimizing L2 message exchange to support ARQ (e.g. Status Report), without too much inter-dependency to other functionalities.

To simplify L2 ARQ operations while reducing the feedback delay, we think it is beneficial to utilize the HARQ feedback as the enablement of L2 ARQ retransmission.  In particular, the L2 ARQ functionality may determine whether a SDU has been successfully delivered based on the HARQ feedback status in L1. This allows the transmitter to react (e.g. perform retransmission of a SDU if needed) much more quickly, while minimizing (or even eliminating) the need of L2 status report signalling.

Nevertheless, in 5G NR there is no explicit indication of HARQ ACK/NACK for UL; it is implied via allocation of UL grants for initial retransmission or retransmission associated to different HARQ processes. On the other hand, we cannot expect the HARQ feedback to be 100% reliable, and so there may be situations where e.g. the TX incorrectly decode a NACK as ACK and advance the window inappropriately. To address this issue, we think RAN2 may collaborate with RAN1 to see how much the HARQ reliability can be improved in 6G. Alternatively, RAN2 may also study and identify solutions that enable better coordination between L2 ARQ and HARQ without any RAN1 impacts.

Proposal 3: RAN2 should study to investigate if L2 ARQ efficiency can be enhanced in 6GR based on tight coordination with HARQ.

## Improvements of Scheduling Latency and Flexibility

Based on the current mechanism, the UE may trigger scheduling request (SR) when it has data in the buffer for transmission, but no UL-SCH resource is available. Furthermore, even if the SR is signaled, the network may not know the actual buffer size at the UE side, so it may first allocate a UL resource that is sufficient to accommodate a BSR MAC CE. Once the BSR is provided, the network can allocate a grant that is large enough to reflect the UE’s UL buffer size, so the UE can transmit the data. Consequently, the resultant scheduling delay can be quite long due to several rounds of handshake between UE and the network. This may impact the user experiences in delay-sensitive applications such as immersive communications. In 5G NR, even though configured grant (CG) has been introduced to minimize scheduling latency, it is rarely enabled in 5G deployment due to concerns of potential inefficiency.

To reduce the scheduling delay, it is useful if the network can obtain at least some knowledge about the UE’s current buffer status as soon as possible. Against this backdrop, there are several directions that RAN2 can examine for 6GR, for examples:

- More versatile scheduling request: The SR is an 1-bit indication informing the network that the UE has something to transmit, but it does not provide too much other details about e.g. how much data is in the buffer. In light of this, we think SR can be enhanced in 6G by e.g. increasing the number of bits for the SR code point, such that some information related to the buffer status can be directly conveyed by SR. However, such approach may have some RAN1 impacts due to the need of new UCI design.

- Multiple SR configurations per Logical Channel: In 5G NR, a logical channel is associated to up to one SR configuration, and the UE may trigger the SR according to the associated configuration when data becomes available in this logical channel. For 6GR, we may allow a logical channel to be mapped to multiple SR configurations corresponding to different PUCCH resources, where each SR configuration may be linked to a condition related to the buffer status (e.g. whether the buffer size exceed a threshold). Thus, the UE can select a SR configuration to trigger a SR based on the current buffer status of the logical channel, and so the buffer status can be implicitly indicated via the PUCCH resource that is used to transmit this SR.

- SR/BSR based on Predicted Data Arrival: Instead of triggering SR/BSR after the UL data becomes available in the buffer, the UE may predict the data is going to arrive in the near future and send SR or BSR in advance, which facilitates the network to allocate UL resource in a more timely fashion (e.g. right after the actual packet arrival) to minimize scheduling delay between resource allocation and data transmission. Some simulation results in our companion paper [2] have shown that AI/ML can be employed to predict future traffic arrival with high accuracy.

In addition to minimization of scheduling delay, we think another issue relevant to UL scheduling to be studied in 6G is whether logical channel prioritization (LCP) should be modified to allow better implementation flexibility that bring some benefits to user experience. In 5G NR, the UE selects logical channel (as well as the amount of data from each logical channel) in the LCP procedure based on Bj, which is a parameter that the UE needs to continuously maintain for each logical channel, and it is derived based on network configurations of PBR and BSD. Nevertheless, the UE is in a much better position to know the data from each logical channel that should be allocated to maximize application performance. It is debatable whether 6G should continue to impose these constraints to the UE, given that the UE can have a much clear view about how to multiplex a MAC PDU based on the instant need of Applications.

Proposal 4: RAN2 should study how to reduce UL scheduling delay and increase LCP flexibility, in order to deliver an optimized user experience in 6G.

## Adaptive QoS Handling for Emerging Use Cases

In 5G, the QoS flow has been deemed to be the finest granularity for differentiated QoS requirement. All packets on the same QoS flow are expected to have the uniform radio treatment, so a QoS flow can only be mapped to up to one DRB. For 6G, this may be beneficial to have more dynamic adaptation of radio treatment for packets within a QoS flow (or multiple QoS flows that are mapped to the same DRB), based on the nature of some emerging applications and use cases. Specifically, in the context of immersive communications, we have identified the following scenarios where dynamic QoS adaptation within a QoS flow or DRB may be advantageous:

- Different Packet Importance in the same QoS flow: In Rel-18, the concept of PDU Set Importance (PSI) has been defined by SA4 for XR, which was mainly used for purpose of discarding when congestion is present. According to 3GPP TS 26.522 [3], the PSI of a PDU Set may be related to if there are many other packets depending on this Application data unit. That is, when a PDU Set with high importance is lost, potentially many other packets would become useless from Application point of view. Apparently, other than discarding, it also makes sense to make sure the PDU Sets with higher importance can be delivered higher reliability. Since PSI is used to indicate the relative importance of packets within the same QoS flow, some dynamic adaptation of target reliability within a DRB may be needed.

- Variable Processing Time for different Generative AI Tasks: It can be expected that Generative AI (GenAI) would play a crucial role in future immersive communications. Nevertheless, the highly variable processing time for different Generative AI tasks may bring some challenges to deliver the truly immersive experience. Ideally, the user should receive the generated response within 400ms after the AI prompt is sent, according to the theory of Doherty Threshold [4]. But since some types of GenAI tasks can have much longer processing time than the other types (e.g. comparison between image-generation and text-generation), it may be beneficial to dynamically adapt the delay budget of UL prompts within a flow/DRB based on the expected inference time, to make sure the overall round trip time is harmonized across various types of AI tasks.

- Random Jitter in Tethering Link: In tethering use cases, where the user application resides on a remote device that is tethered to the UE via e.g. WiFi or Bluetooth, each UL packet may experience different amount of random jitter over the tethering link. This may impact the end-to-end latency when the jitter is large. Thus, some dynamic QoS adaptation may be beneficial if the UE is able to adjust the radio treatment within a QoS flow or DRB on a per-packet basis to compensate the delay.

We suggest that RAN2 can coordinate with SA2/SA4 to identify the need of supporting more adaptive QoS handling for some emerging use cases and Applications in 6G.

Proposal 5: RAN2 should study (based on coordination with SA) how differentiated radio treatment can be provided to packets within the same QoS flow/DRB, to accommodate the characteristics of some emerging use cases in 6G.

## Issues Related to Head of Line (HoL) Blocking

Given that multiple QoS flows may be mapped to the same DRB (which is a very common deployment in the field), there may be some head of line (HoL) blocking issues which can delay the data of more important traffic on the same bearer. For examples, at the transmitter side, some high priority packets cannot be transmitted immediately due to window stalling caused by packets of some background traffic that arrive earlier. Also, at the receiver side, some already-received high priority packets may not be delivered to the upper layer immediately, as the reordering timer is still running for some missing packets of some other less-important traffic flows within the same DRB. These problems may be even more severe when network congestion is present.

To optimize user experience in 6G, the data associated to more critical QoS flows should be transmitted/delivered as soon as possible, rather than being jeopardized by some other less essential packets that are in front of the queue. Thus, RAN2 should aim to identify the mechanisms to reduce the delay for high priority QoS flows by addressing the HoL blocking issues in 6GR.

Proposal 6: RAN2 should study how 6G user plane protocols can be enhanced to prevent or reduce the delay of more critical QoS flows(s) caused by head of line blocking issues within a DRB.

# Improvement of Efficiency

## C-DRX Enhancement

For 6GR, power efficiency improvement undoubtably remains to be a key area that 3GPP should continue to further explore. Several UE power saving schemes have been introduced throughout different releases in 5G/5G-A, including DRX, DCP-based C-DRX, and LP-WUS based C-DRX. However, all these mechanisms have static behaviour regardless of the traffic type to be transmitted, which has drastically reduced the power saving gain. In particular, C-DRX in 5G operates based on timers such as On-Duration Timer, Inactivity Timer, HARQ RTT timer, and HARQ Retransmission timer, and these timer values are fixed once they are configured. In many cases, the UE needs to stay awake based on these timer configurations even when there is no data activity, or when the data to be transmitted or retransmitted is no longer valid (e.g. already discarded and no longer useful for Applications). Consequently, the UE would unnecessarily waste an excessive amount of energy.

Furthermore, according to the field data, we have often observed that the network would schedule some radio resources even before data arrival (i.e. Pre-Scheduling), which requires the UE to start/restart Inactivity Timer and stay awake, but actually no meaningful data is transmitted on these scheduled resources. As shown in Figure 1, which is a snapshot of real data collected in the field, we see that the UE receives consecutive DL assignments that do not really translate into any data throughput in the higher layer, but the UE needs to stay awake most of the time because of such scheduling. This is indeed inefficient from both radio resource and power perspective.

Figure 1 A snapshot of field data that shows unnecessary UE power wastage due to inappropriate scheduling

Thus, we think for 6GR the DRX scheme can be enhanced to be more adaptive and flexible as compared to 5G, by allowing more dynamic adjustment of DRX timer values or UE behaviour. For example, a UE may be pre-configured with multiple different OnDuration Timer values and/or Inactivity Timer values, and the WUS signal can indicate which timer value that the UE should apply when the UE wakes up based on this WUS. Similarly, the UE can also further optimize its power saving gain if it can determine the setting of HARQ RTT Timer and/or HARQ Retransmission Timer based on the content of MAC PDU stored in the corresponding HARQ process. If the data multiplexed in a MAC PDU is already outdated, the UE can have the flexibility to terminate these timer operations early because HARQ retransmission for this MAC PDU is no longer needed. In our views, RAN2 should discuss how C-DRX can be enhanced in 6GR to realize UE power saving gain in a more intelligent manner.

Proposal 7: RAN2 should study how C-DRX flexibility can be enhanced to improve UE power efficiency.

# Conclusions

This contribution presents some of our initial views about the directions that RAN2 should study for 6G User Plane. We have the following proposals:

Proposal 1: 6GR User plane should retain the functionalities including sequence number (SN) management, ARQ, segmentation, reassembly, security protection, and in-sequence delivery, but RAN2 can study if some of these functionalities and protocol layers may be re-arranged or simplified in 6GR to better accommodate the recent trends of transport layer traffic.

Proposal 2: RAN2 should study how L2 protocol and procedures can be modified to reduce User Plane processing overhead in 6GR, the directions such as SDU concatenation and header optimization may be considered.

Proposal 3: RAN2 should study to investigate if L2 ARQ efficiency can be enhanced in 6GR based on tight coordination with HARQ.

Proposal 4: RAN2 should study how to reduce UL scheduling delay and increase LCP flexibility, in order to deliver an optimized user experience in 6G.

Proposal 5: RAN2 should study (based on coordination with SA) how differentiated radio treatment can be provided to packets within the same QoS flow/DRB, to accommodate the characteristics of some emerging use cases in 6G.

Proposal 6: RAN2 should study how 6G user plane protocols can be enhanced to prevent or reduce the delay of more critical QoS flows(s) caused by head of line blocking issues within a DRB.

Proposal 7: RAN2 should study how C-DRX flexibility can be enhanced to improve UE power efficiency.

# References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO.

[2] R2-2507114, Views on Directions of 6G AI/ML general framework and data transfer, Apple, RAN2 #131bis, Prague, Czech Republic, October 2025.

[3] 3GPP TS 26.522 V19.1.0, 5G Real-time Media Transport Protocol Configurations (Release 19), June 2025.

[4] https://lawsofux.com/doherty-threshold/