# 3GPP TSG RAN WG2 Meeting #131b      	                        R2-2507114
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	10.3.3

Source:	Apple

Title:	Views on Directions of 6G AI/ML general framework and data transfer

WID/SID:	FS_6G_Radio – Release 20

Document for:	Discussion and Decision

# 1 Introduction

The SID of 6G Radio (SID RP-251881) was agreed in RAN#108 [1]. The SID objective on 6G AI/ML framework can be found as follows:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

And the SID objective on 6G “data transfer” is:

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we analyze the lessons learnt from 5G AI/ML specification and share Apple’s view on study directions of 6G AI/ML general framework, 6G AI/ML data collection framework and 6G unified “data transfer”.

# 2 Discussion

## 2.1 General consideration of 6G AI/ML LCM framework

### 2.1.1: 6G direction 1: UE-initiated AI/ML management framework

In our understanding, 5G AI/ML LCM framework includes the following aspects as captured in Section 7.2.1 of TR 38.843 [2]:

- UE capability

- Data collection and model training

- AI/ML functionality management

- Model transfer/delivery

- Performance monitoring

For general consideration of 6G AI/ML LCM framework, the first question is how to understand “general” and what can be inherited from 5G AI/ML LCM framework. In Table 1, we provide a summary on general aspects of 5G LCM common to all 5 use cases studied in 5G (i.e. AI/ML based beam management, CSI prediction, AI/ML based positioning, AI mobility and AI/ML based CSI compression), our view on general 6G directions and how RAN2 can make progress.

|  | 5G LCM aspects common to all 5 studied use cases | View on general 6G directions | View on how RAN2 makes progress |
| --- | --- | --- | --- |
| UE capability | - Reuse legacy static UE capability report framework. | - Whether dynamic UE capability can be considered. - APU framework was introduced on top of CPU for CSI related use cases. Whether APU framework can be extended to general 6G AI/ML use cases. | Postpone both directions: - Depend on general 6G capability conclusion. - Need to first discuss in RAN1 based on specific use case. |
| Data collection and model training | - UE can request to start/stop data collection and preferred configuration(s) from NW provided candidate configuration list. - Only consider offline model, whose details are transparent to 3GPP. | - 5G solution as baseline and study whether UE can actively suggest configuration(s) without NW provided candidate list. - 5G assumption of offline training as baseline. FFS online model training / fine-tuning only if its necessity is justified. | Study 2-1 and postpone 2-2: - Study as UE-initiated AI/ML management enhancement. - Need to discuss based on specific 6G new AI/ML use case and spec changes, subject to valid/practical UE device processing capability. |
| AI/ML functionality management | - Initial/updated applicability reporting for each inference configuration, subject to NW-side additional condition (associated ID), UE-side additional condition and model availability. | - 5G solution as baseline and consider enhancements with more UE autonomy (e.g. the UE may select parameter value in configured range to make AI/ML configuration applicable). | Study 3) as part of UE-initiated AI/ML management enhancement. |
| Model transfer / delivery | - 3GPP transparent solution for one-sided model. - 3GPP standardized solution (open format of a known model structure at UE) for two-sided model (normative work in 5G Rel-20). | - 5G solution as baseline. FFS 3GPP standardized solution only if its necessity is justified. - First wait for 5G Rel-20 normative work before starting 6G study. | Postpone both directions: - Need to discuss based on specific 6G AI new use case. - Need to first evaluate 5G solution when available. |
| Performance monitoring | - Only NW makes decision based on UE reported measurement / specified metric (e.g. SGCS). | - 5G solution as baseline and consider enhancement with more UE autonomy (e.g. UE autonomous fallback to non-AI). | Study 5) as part of UE-initiated AI/ML management enhancement. |

Table 1 Summary of general aspects of 5G LCM framework common to all 5 use cases and Apple’s view on general consideration of 6G study directions.

From Table.1, we can have the following observations:

- 5G AI/ML LCM framework has already specified quite a lot of general aspects common to all the 5 studied use cases. Thus, it can be regarded as a general and complete AI/ML framework. In all, 5G AI/ML LCM framework is sufficient to serve as baseline of 6G AI/ML LCM framework.

- 5G AI/ML LCM framework belongs to “Network decision, network-initiated AI/ML management”, which is only one of 4 identified frameworks captured in Section 7.2.1.1 of TR 38.843 [2].

  - The other 3 UE-initiated management solutions (i.e. “Network decision, UE-initiated AI/ML management”, “UE decision, event-triggered as configured by NW” and “UE-autonomous, UE’s decision is reported to NW”) were discussed in 5G but not concluded due to limited TU in 5G.

    - For example, UE autonomous fallback to non-AI operation was discussed and is still on the table of Rel-20 AI mobility. So, this direction was already justified by existing 5G AI/ML use case.

    - Thus, the 3 UE solutions should be further studied as a general 6G direction because the UE has better local observation and device internal information which can’t be shared with the Network.

- We are open for discussion on the following directions only under specific 6G new AI/ML use case because existing 5G AI/ML use cases can’t justify its necessity:

  - Online training / model fine-tuning: RAN2 need to evaluate its necessity with specific new 6G AI/ML use case and specific spec changes, subject to valid / practical UE device processing capability.

  - 3GPP specified model transfer / delivery: RAN2 need to evaluate its necessity on 3GPP specification with specific new 6G AI/ML use case.

Observation 1: 5G LCM framework has specified quite a lot of general aspects common to all use cases (Table 1).

Observation 2: 5G LCM framework belongs to “Network decision, network-initiated AI/ML management”. 5G also studied other 3 UE-initiated management solutions captured in TR 38.843 with justification that UE has better local observation and device internal info which can’t be shared with NW. But no conclusion was made due to limited TU.

Thus, we propose:

Proposal 1: 6G LCM framework takes general aspects of 5G LCM common to all 5 use cases (summarized in Table 1) as baseline. And further study the following UE-initiated frameworks captured in TR 38.843:

- “Network decision, UE-initiated AI/ML management”

- “UE decision, event-triggered as configured by NW”

- “UE-autonomous and UE’s decision is reported to NW”.

### 2.1.2: 6G direction 2: traffic prediction for UP and power enhancement

In 5G AI/ML study, all studied use cases focus on various measurement prediction:

- L1 measurement prediction: AI/ML based beam management (e.g. predicted top-K L1 RSRP and beam index), CSI prediction (e.g. predicted PMI in future), CSI compression (e.g. compressed channel information) and AI/ML based positioning (e.g. new PRS measurements)

- L3 measurement prediction: AI mobility (e.g. predicted L3 RSRP).

We think it is RAN2 common understanding that new L1/L3 measurement prediction will be further studied in 6G AI/ML new use cases (e.g. AI/ML based RS reduction and AI based LTM/CHO). The details can be discussed later.

However, besides measurement prediction, we think that 6G can also study traffic prediction for UP performance enhancement and UE/NW energy saving enhancement. First, we think that academic study on traffic prediction is already mature. Table 2 from [3] provides a good summary of various existing approaches of traffic prediction.

Table 2 Summary of traffic prediction approaches in academia (from [3])

Then, we have done a simulation based on 5G air interface to evaluate the accuracy of traffic prediction with LSTM as an example approach. The details of simulation assumption and LSTM details are summarized as follows:

- Simulation assumption:

  - Traffic model: actual Google Stadia gaming dataset

    - It includes application specific data set which includes combination of many traffic models including human interaction (https://github.com/wn-upf/Stadia_cloud_gaming_dataset_2020).

    - An example trace of data used in simulation is provided in Figure.1.

  - OTA transmission simulation model:

    - 5G air interface with 10 UEs in one cell.

    - BSR reporting with periodicity of 10 TTI.

- LSTM implementation details:

  - 40 hidden nodes are used to predict size of next packet at the UE.

  - 60 hidden nodes are used to predict arrival time of next packet at the UE.

  - Inference for prediction runs when a new packet arrives at the buffer.

  - Complexity: 21K FLOP

Figure 1 An example trace of traffic model in simulation (RTCP: real time control protocol, STUN Session Traversal Utilities for NAT, DTLS: Datagram Transport Layer Security)

Figure 2 CDF curves of prediction errors for both packet size and arrival time of next packet at the UE

We provide simulation results in Figure 2 and Figure 3:

- Figure 2 shows the CDF curves of prediction errors for both packet size and arrival time of the next packet at the UE. It can be observed that the prediction error is quite small: MSE of packet size prediction@90% <0.01byte and MSE of packet arrival time prediction@90% <0.2ms.

- Figure 3 shows the system performance gain in term of scheduling latency calculated as the time difference between the packet arrival time in UE and the packet reception time in gNB. It can be observed that up to 21% latency gain can be achieved compared with legacy BSR without traffic prediction.

Figure 3 Scheduling latency gain of BSR with predicted traffic info over legacy BSR

Observation 3: Academic study on traffic prediction is mature. Based on simulation, AI/ML can predict future traffic with high accuracy (MSE of packet size prediction@90% <0.01byte and MSE of packet arrival time prediction@90% <0.2ms). And up to 21% performance gain over 5G can be achieved by traffic prediction.

Please note that BSR prediction is just an example to show benefit of traffic prediction. We think that traffic prediction can be used as a general tool to enhance various User Plane performances. For example:

- Throughput enhancement: traffic prediction utilized for BSR and LCP.

- Robust enhancement: traffic prediction utilized for PDCP duplication.

- UE/NW power saving enhancement: traffic prediction utilized for UE CDRX and Cell DTX/DRX.

Thus, we propose:

Proposal 2: Besides L1/L3 measurement prediction, 6G AI/ML framework study the new direction of utilizing traffic prediction for UP performance enhancement and UE/NW energy saving enhancement.

## 2.2 General consideration of 6G AI/ML data collection framework

5G studied two kinds of data collection framework: NW-side data collection and UE-side data collection. We provide a summary on their specification status in 5G in Table 3, which is the starting point of 6G study.

|  |  | Solution description | Consumer of transferred data | Agreed use cases in 3GPP | 5G specification status |
| --- | --- | --- | --- | --- | --- |
| NW-side data collection |  | - Logging L1/L3 measurements in multiple times in AS buffer and report in one RRC message - UERAN via RRC | RAN | - AI/ML based BM - AI/ML based CSI compression (Rel-20) - AI/ML mobility (Rel-20) | - Framework completed in Rel-19. - Rel-20 is expected to add new logged measurement quantities (e.g. L3 RSRP for AI mobility) |
| UE-side data collection | Solution 1a | UEUE-side server via 3GPP transparent tunnel (e.g. WiFi) | UE-side server | Expected for all use cases | No need of 3GPP specification work (already widely used in industry) |
|  | Solution 1b | UEUE-side server via 3GPP non-transparent tunnel | UE-side server | Not discussed yet | Precluded in RAN plenary |
|  | Solution 2 | UECNUE-side server via 3GPP specified tunnel. FFS CP or UP solution | UE-side server and CN | Not discussed yet | Pending on SA2. Solution details are not clear. |
|  | Solution 3 | UEOAMUE-side server via 3GPP specified tunnel. FFS CP or UP solution | UE-side server and OAM | Not discussed yet | Pending on SA5. Solution details are not clear. |

Table 3 Summary of AI/ML data collection specification work in 5G

### 2.2.1: 6G NW-side data collection

According to Table 3, we think 5G NW-side data collection solution (i.e. measurement logging) is already a general data collection framework for NW-side model because it is applicable for most studied AI/ML use cases.

Furthermore, we don’t think NW-side data collection needs to be unified with UE-side data collection due to the following reasonings:

- Their data consumers are different:

  - NW-side data collection: RAN is the only consumer of the data. Thus, it doesn’t make sense to share the data with CN.

  - UE-side data collection: UE-side server, OAM or CN can be the consumer of the data. Thus, it can be considered as part of unified 6G data transfer framework.

- Their requirements for transfer of large data volume are different:

  - NW-side data collection: As discussed in Rel-19, it doesn’t have such requirement because NW can split data collection task among multiple UEs

  - UE-side data collection: It is a major challenge because one particular UE’s complete dataset needs to transfer to NW.

Observation 4: NW-side data collection doesn’t need to be unified with UE-side data collection:

- RAN is the only consumer of the data, and therefore it doesn’t make sense to share the data with CN.

- It doesn’t need to transfer large data volume as NW can split collection task among multiple UEs.

Thus, we think 5G logging measurement framework can be baseline of 6G NW-side data collection. And 6G can further study its enhancement (e.g. support data collection across all RRC states).

Proposal 3: On 6G NW-side data collection:

- Data transfer is terminated in RAN as 5G, and no need to be unified with UE-side data collection.

- Take 5G logging measurement framework as baseline and study its enhancement (e.g. support data collection across all RRC states).

### 2.2.2: 6G UE-side data collection

On UE-side data collection, we can see that SA2 and SA5 are still working on 5G solution, which may be reused in 6G. Meanwhile, UE-side data collection may be part of 6G unified “data transfer” framework. Thus, we think 6G UE-side data collection has potential overlapping with “data transfer” framework in 6G study and 5G UE-side data collection. The duplicated efforts should be avoided. Thus, RAN2 should postpone the discussion of 6G UE-side data collection after their conclusion is clear.

Proposal 4: Due to potential overlapping with 6G “data transfer” and 5G UE-side data collection, postpone the RAN2 discussion on 6G UE-side data collection after their conclusion is clear.

Finally, we observe one lesson learnt in 5G: RAN2 discussion on visibility and controllability requirements of data collection was quite inefficient in Rel-18 and Rel-19. This discussion took quite a long time in RAN2 (almost one year), but no clear conclusion can be made. Finally, such discussion was finalized in RAN plenary. We think the root cause of the inefficient discussion is because RAN2 is not the correct WG to discuss requirements on data visibility and controllability. As MNO requirement and UE privacy are expertise of Plenary/SA1/SA3, we think they should first discuss such requirements.

Observation 5: One lesson learnt in 5G is that RAN2 discussion on visibility and controllability requirements of data collection was quite inefficient and it was RAN plenary to finally conclude because RAN2 don’t have the expertise of MNO requirement and UE privacy. Instead, such requirement discussion should be in RANP/SA1/SA3.

Thus, we propose:

Proposal 5: As one lesson learnt in 5G, RAN2 leaves the discussion on visibility and controllability requirements on 6G UE-side data collection and model transfer to RAN plenary and SA1/SA3.

## 2.3 6G “Data Transfer”

For the RAN SID [1] objective to study “Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc.)”, it is very important to analyze all “data transport” requirements in AI/ML and Sensing use cases and identify whether there is gap between the existing 4G/5G methods and those requirements.

Note that in 6G data framework discussion in SA[4], there are many data transport scenarios proposed with a variety of termination points. But for the sake of RAN2 discussion, we can focus on the data transport terminated at UE as one of the end points and generally divide the 6G “data transfer” aspects as two different termination scenarios in RAN2 study:

- “Data” transport between UE and CN

- “Data” transport between UE and RAN node

It is worth noting that in certain use cases, OTT approach (e.g., between UE and OTT server) is feasible, and it is very plausible that there is no need of 3GPP solution if non-3GPP approach is sufficient. But for the sake of 3GPP study in RAN2, we can first focus on the above two categories.

For “UE-CN” data transfer in either or both directions, there are existing CP methods (e.g., NAS, LPP) can be used as baseline or we may also consider UP-based methods (e.g., reaching the data-handling NF via UPF) in 6G. In general, compared to “UE-RAN” data transport, the solution(s) for “UE-CN” data transport are assumed to have much less RAN2 impact for this kind of data transfer and the related RAN2 work is quite manageable.

For “UE-RAN” data transfer, we understand that this is something new if the “data” here is not considered as belonging to the traditional remit of “RRC” (Radio Resource Control) and may have much bigger Uu impact. Therefore, we need to understand the justification of those ‘data transfer” scenarios.

In Table 4 below, we have summarized and discussed a few typical “potential” UE-RAN data transfer cases in AI/ML and sensing services:

| Data transfer scenarios | Potential data path/direction | Delay requirements | Comments/Questions |
| --- | --- | --- | --- |
| OTA approach for Model Transfer for two-sided models | RAN UE, then to Network Function or OTT server | Non-urgent | This is only part of the OTA approach for two-side model transfer, and non-OTA approach may already be sufficient. |
| NW-side Data collection in AI/ML | UE RAN | Depends on use case | If the use case aims to tune/control RAN node scheduling, then this probably should be part of CP/RRC. |
| Sensing-assisted communication in UE-gNB bi-static sensing | UE RAN | Urgent | whether those “sensing” should be treated as same as (RRC) “measurements”? |
| TRP-UE DL in bistatic sensing for non-communication purpose (e.g., road object detection) | UE RAN | Urgent for safety application only | Whether UE reporting to RAN node is really needed as it seems more critical for road objects to be sensed by UE for safety use case? |
| UE-TRP UL in UE-gNB bi-static sensing | RANUE | Depends on use case | This can be also done via RAN-CN-UE method with some extra delay. Whether this sensing method is better than TRP-UE-DL or UE monostatic? |

Table 4: Analysis of Potential Use Scenarios for UE-RAN data transfer

As can be seen from Table 4 above, there are some concerns about “UE-RAN” data transfer scenarios.

For the RANUE (RAN node as data producer and UE as data consumer) data transfer, it is still unclear whether any such solutions will be eventually included in 6G Rel-20 work because the justification for those solutions is not yet strong.

For the UERAN (UE as data producer and RAN node as data consumer) data transfer, we need to discuss the exact use case scenarios. The undelaying question is that whether “RAN node as data consumer” is an essential part of the architecture design or not, depending on the exact use case:

- If “RAN node as data consumer” is very essential for Uu operation (e.g., like AI-based CSI prediction or sensing-assisted communication which are usually requires low-latency delivery), then probably RRC is a better place to include those “data” as “measurements”.

- If “RAN node as data consumer” is not very essential for Uu operation, but a separate side task for RAN node by leveraging the edge location of RAN node or utilizing the RAN node computation resources, then we wonder whether 3GPP should introduce a separate “data entity” in “edge cloud” to take care of this task instead of adding this logically separate function to RAN node. Furthermore, another potential solution is to deploy UPF close to RAN, i.e. reuse 5GC Edge Computing solution specified by SA2/SA6.

Therefore, we think RAN2 need first discussion those concerns and find strong justifications to support RAN-only data transfer which are terminated at UE and 6G RAN node.

Observation 6: It is unclear whether there are essential justifications for “data transfer terminated at RAN” beyond the traditional “RRC measurement” based approach in the use scenarios for AI/ML and sensing services.

Another aspect worth discussing at the very beginning of the study is whether SON/MDT should be also included for “6G data transfer” discussion. Logically, the logged MDT data collection is quite similar tor NW-side data collection proposed for AI/ML and also for logging & reporting sensor data. So far, MDT data are just recorded as (data)traces in 3GPP and any other further potential usage to optimize RAN design or deployment is not in 3GPP scope. Hence, whether the RRC-based MDT approach is the best solution for SON/MDT use cases is also subject to further reckoning, given that 3GPP wants to introduce a new “data framework” to facilitate “data”-related operations. For example, it would be strange that SON/MDT data use legacy solution, but AI/ML and sensing data are mandated to go with a new solution. Maybe this has to be clarified by RAN to further guide the RAN2 study.

Observation 7: It is unclear whether SON/MDT in 6G should be considered as “data transfer”.

In overall, we think there is a need to support data transfer in general as more and more non-conventional services are introduced in 6G. But for RAN domain, there will be dramatically different design impacts for “UE-CN” data transfer and “UE-RAN” data transfer. For RAN-terminated transfer, the use scenarios need to be clearly examined and justified. Logically, any UE-side reports to have direct impact on RAN’s Uu operation would be still in the remit of control plane and can be still conveyed via RRC. The new data transfer solutions in RAN2 need focus on the other scenarios and if justified, the QoS requirements of those data transport scenarios. Then, we can decide whether those scenarios can be covered by a homogenous solution or not.

Proposal 6: RAN2 discuss the detailed list of all data transport tasks for services belonging to  “data transfer” in 6G RAN study before deciding whether a unified “data transfer” solution in RAN2 for different 6G services is feasible.

Finally, we think data collection and data transfer in 6G RAN for AI/ML and sensing service are subject to the control of user consent framework. While 5G, and indeed even 4G, had the notion of “user consent” (which is required for certain functions at least in some jurisdictions), it was never properly defined and the situation resulted in many lengthy and ambiguous discussions.

In addition, there is one major technical problem with 5G user consent - 3GPP specifications do not define how it is obtained. It is assumed that it stored in the UDM, but it is not at all clear how it gets there. Furthermore, there is no signalling allowing the UE (and the user) to verify their consent status and to recall/revoke the consent, something which we consider a serious issue.

Therefore, we strongly suggest to support signalling (between UE and network) to provide/recall/verify user consent.

Note: for the time being, this text is provided in RAN2 for information, as we expect the initial discussions to happen in SA. However, eventually this will have RAN2 impact.

Proposal 7: Data collection and data transfer in 6G RAN for AI/ML and sensing service are subject to the control of user consent framework. FFS detailed impacts on AS layer design.

# 3 Conclusion

In this contribution, we share our general consideration on 6G AI/ML framework and data transfer. Our observation and proposals are summarized in one page as follows:

General consideration of 6G AI/ML LCM framework

Observation 1: 5G LCM framework has specified quite a lot of general aspects common to all use cases (Table 1).

Observation 2: 5G LCM framework belongs to “Network decision, network-initiated AI/ML management”. 5G also studied other 3 UE-initiated management solutions captured in TR 38.843 with justification that UE has better local observation and device internal info which can’t be shared with NW. But no conclusion was made due to limited TU.

Proposal 1: 6G LCM framework takes general aspects of 5G LCM common to all 5 use cases (summarized in Table 1) as baseline. And further study the following UE-initiated frameworks captured in TR 38.843:

- “Network decision, UE-initiated AI/ML management”

- “UE decision, event-triggered as configured by NW”

- “UE-autonomous and UE’s decision is reported to NW”.

Observation 3: Academic study on traffic prediction is mature. Based on simulation, AI/ML can predict future traffic with high accuracy (MSE of packet size prediction@90% <0.01byte and MSE of packet arrival time prediction@90% <0.2ms). And up to 21% performance gain over 5G can be achieved by traffic prediction.

Proposal 2: Besides L1/L3 measurement prediction, 6G AI/ML framework study the new direction of utilizing traffic prediction for UP performance enhancement and UE/NW energy saving enhancement.

General consideration of 6G AI/ML data collection framework

Observation 4: NW-side data collection doesn’t need to be unified with UE-side data collection:

- RAN is the only consumer of the data, and therefore it doesn’t make sense to share the data with CN.

- It doesn’t need to transfer large data volume as NW can split collection task among multiple UEs.

Proposal 3: For 6G NW-side data collection:

- Data transfer is terminated in RAN as 5G, and no need to be unified with UE-side data collection.

- Take 5G logging measurement framework as baseline and study its enhancement (e.g. support data collection across all RRC states).

Proposal 4: Due to potential overlapping with 6G “data transfer” and 5G UE-side data collection, postpone the RAN2 discussion on 6G UE-side data collection after their conclusion is clear.

Observation 5: One lesson learnt in 5G is that RAN2 discussion on visibility and controllability requirements of data collection was quite inefficient and it was RAN plenary to finally conclude because RAN2 don’t have the expertise of MNO requirement and UE privacy. Instead, such requirement discussion should be in RANP/SA1/SA3.

Proposal 5: As one lesson learnt in 5G, RAN2 leaves the discussion on visibility and controllability requirements on 6G UE-side data collection and model transfer to RAN plenary and SA1/SA3.

General consideration of 6G AI/ML data transfer

Observation 6: It is unclear whether there are essential justifications for “data transfer terminated at RAN” beyond the traditional “RRC measurement” based approach in the use scenarios for AI/ML and sensing services.

Observation 7: It is unclear whether SON/MDT in 6G should be considered as “data transfer”.

Proposal 6: RAN2 discuss the detailed list of all data transport tasks for services belonging to “data transfer” in 6G RAN study before deciding whether a unified “data transfer” solution in RAN2 for different 6G services is feasible.

Proposal 7: Data collection and data transfer in 6G RAN for AI/ML and sensing service are subject to the control of user consent framework. FFS detailed impacts on AS layer design.

# 4 References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO.

[2] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[3] Aaron Chen, Jeffrey Law and Michal Aibin (2021). A Survey on Traffic Prediction Techniques Using Artificial Intelligence for Communication Networks. Telecom. 2. 518-535. 10.3390/telecom2040029.

[4] SP-250488 Study on Architecture for 6G System SA SID, ZTE