3GPP TSG-RAN2 #131bis	R2-2507128

# Prague, Czech Republic, October 13 – October 17, 2025

Agenda item:	10.3.3 (FS_6G_Radio)

Source:	LG Electronics

Title: 	Considerations on Common User plane and Control plane for 6G

Document for:	Discussion and decision

1.	Introduction

This document states our views on common user plane and control plane for 6G.

2.	Common User plane and Control plane for 6G

2.1	Security for L2 control signal

Since LTE Rel-8, security (i.e., ciphering and integrity protection) has been provided in the PDCP layer. PDCP SDUs are security protected, but L2 control signals such as PDCP Control PDU, RLC Control PDU, and MAC CE are not security protected.

Up to NR Rel-17, this architecture was acceptable because not much essential control information was transmitted via L2 control signals. However, with the introduction of LTM (L1/L2 Triggered Mobility) in NR Rel-18, the importance of control information carried via MAC CE has significantly increased.

The LTM enables mobility decisions to be triggered directly from L1 measurements and L2 control signaling, allowing faster and more efficient handovers. This means that MAC CE can now carry critical mobility-related control information such as L1 measurement results, and mobility commands. If such information is tampered with or intercepted by an attacker, it may lead to incorrect mobility decisions, service interruptions, or degraded network performance.

In 6G, we think that LTM is still considered a promising feature, and this would be introduced from the 6G Day-1. Therefore, applying security to L2 control signals, especially MAC CE, becomes an essential task. More specifically, it would be important to apply ciphering and integrity protection at the MAC layer, in addition to PDCP and this would allow direct protection of at least critical MAC CE for mobility-related control signals.

Figure 2.1.1. Example of MAC CE security

Study application of security to L2 control signal, at least for MAC CE.

2.2	Small Data Transmission

In 5G NR, Small Data Transmission (SDT) was introduced to allow data transmission in RRC_INACTIVE. In NR Rel-17, MO-SDT (Mobile Originated SDT) was introduced targeting UL data, and in NR Rel-18, MT-SDT (Mobile Terminated SDT) was introduced targeting DL data.

In our view, RRC_INACTIVE is more efficient than RRC_IDLE from connection setup latency and signalling overhead perspective and more efficient than RRC_CONNECTED from power saving perspective. Thus, we think RRC_INACTIVE should be main state in 6G, and SDT should be basic function for data transmission in RRC_INACTIVE. However, some pain points are observed in 5G NR SDT, which makes the SDT feature unattractive.

First, the SDT procedure is restricted to initial BWP, where initial BWP has narrow bandwidth. Due to the narrow bandwidth of initial BWP, only very small data can be transmitted in the SDT procedure, which is not suitable for practical application. In some cases, data transmission may not be completed during SDT procedure, which leads to frequent RRC resume/release. Moreover, since both SDT and RACH procedure are performed on initial BWP, the collision probability is increased in the RACH procedure. In order to resolve above problem, we think that SDT specific BWP should be introduced from 6G Day-1.

Another pain point is that the maximum duration of SDT procedure is limited to 4s. We think that this short time contributes to making SDT less efficient.

One of scenario using SDT is the chat application, e.g. whatsapp. The chat application does not require large size of data, but once a chat starts, the text may be sent continuously, and the chat may not end in 4s. However, since the SDT procedure should be terminated in 4s, RRCResumeRequest message and RRCRelease message should continue to be signalled and this leads to signalling overhead. Thus, to reduce signalling overhead, the SDT duration needs to be extended in 6G.

If the SDT duration becomes longer, it also needs to be considered how to reduce the PDCCH monitoring during SDT procedure. In 5G NR, the UE continuously monitors PDCCH during SDT procedure, which may not be acceptable if the SDT duration becomes longer, e.g. 1 hour.

Figure 2.2.1. SDT enhancement in 6G

One more thing to consider is to use shared CG resource for CG-SDT. In 5G NR, dedicated CG resource is configured for CG-SDT, which is good for one UE but bad for network overall resource utilization. Moreover, if massive IoT devices, which is considered in 6G scenarios, utilizes CG-SDT, the signaling overhead to configure dedicated CG resource to numerous IoT devices would be unbearable. Therefore, we think shared CG resource should be introduced for CG-SDT in 6G.

Introduce efficient small data transmission from 6G Day-1

- a. Introduce SDT specific BWP

- b. Support longer SDT duration: longer than 4s, e.g. up to 1 hour

- c. Reduce PDCCH monitoring in SDT procedure

- d. Introduce shared CG-SDT resource

2.3	Access control and back-off for initial access

Universal Access Control (UAC) is a comprehensive L3 framework introduced in NR for controlling access attempts from multiple UEs trying to be connected to the network. Whether a UE is allowed to initiate RACH procedure for initial access in UAC is based on access category determined based on a type of service, access identity and barring parameters broadcasted in SIB1. The NAS layer maps access attempts to these categories and identities while the RRC layers perform the access barring check.

On the other hand, RACH backoff in NR is a L2 mechanism used to control and delay PRACH transmissions during a RACH procedure. It helps to manage network congestion and reduce collision probability during the initial access phase.

When it comes to access attempt control in NR, one of the pain points is the lack of integration and coordination between L3 UAC and L2 backoff. Compared to UAC based on access category as well as access identity, RACH backoff provides relatively coarse-grained control over access attempts, limiting its effectiveness in differentiated or policy-driven access management provided by UAC. Thus, a RACH procedure triggered by an access attempt which passed the access barring check in RRC layer can still be blindly and stochastically delayed by backoff mechanism in MAC layer, regardless of its access category.

We think that 6G still needs to provide RRC-based access control and RACH backoff mechanism, but 6G can provide better integration and coordination between RRC-based access control and RACH backoff mechanism for initial access.

Study unified approach for RACH backoff and RRC access control for initial access

2.4	UE power saving

Long UE battery life is always a critical KPI in mobile networks, and in 5G NR, various power saving features (UE DRX, DCP, LP‑WUS, etc.) have been introduced in RAN2 from the initial release onward. However, since each feature was developed independently in different releases, dependencies between them have emerged, leading to increased complexity and inefficiency. For example, DCP and LP‑WUS are always configured together with DRX, and RAN2 specification unnecessarily became complicated to clearly define UE behavior in at least the following cases:

- When a DCP or LP‑WUS indication is missed before onDuration, including configurations to start the associated drx‑onDurationTimer in cases where DCP is monitored but not detected (i.e., ps‑Wakeup).

- When a DCP or LP‑WUS indication overlaps with DRX active time.

These situations become even worse when dual DRX groups are configured. In addition, analogous functionalities have been defined across multiple releases. For example, one option of LP‑WUS is almost identical to DCP. Such duplication and dependency increase specification complexity and impose unnecessary burdens on UE implementation and network optimization.

Therefore, RAN2 should learn from the experience in 5G NR and, rather than maintaining multiple features for the same purpose, define the most efficient power saving mechanism as a unified solution in 6G. In particular, unlike DCP and DRX, LP‑WUS operates based on LR (Low Power Wake‑up Radio) which can achieve greater power savings than MR (Main Radio). The longer the UE remains in LR, the greater the battery saving benefit. LP‑WUS should be defined as a baseline UE power saving scheme without any dependency on DRX.

In addition, RRC_IDLE/INACTIVE operation should also be considered from 6G Day-1. In 5G NR, MR-based PEI, i.e., via DCI, was introduced first, and later LR-based PEI, i.e., LP-WUS, was added as part of LR introduction. This sequence led to complexity and debate on how the two mechanisms should coexist, and eventually both were allowed for UEs to monitor mainly to reduce paging false alarms. For 6G, it is desirable to define a well-designed LP-WUS as a clear and standalone PEI mechanism from 6G Day-1. Furthermore, the use of LR can bring additional benefits in RRC_IDLE/INACTIVE, such as RRM measurement relaxation and measurement offloading, further extending UE battery life.

Introduce LP‑WUS as a baseline UE power saving scheme for 6G.

3. Conclusion

Based on the above discussions, we present the following proposals:

- Study application of security to L2 control signal, at least for MAC CE.

- Introduce efficient small data transmission from 6G Day-1

- a. Introduce SDT specific BWP

- b. Support longer SDT duration: longer than 4s, e.g. up to 1 hour

- c. Reduce PDCCH monitoring in SDT procedure

- d. Introduce shared CG-SDT resource

Study unified approach for RACH backoff and RRC access control for initial access.

Introduce LP‑WUS as a baseline UE power saving scheme for 6G.