3GPP TSG-RAN WG2#131-bis	R2-2507132

Prague, Czech Republic, Oct 13th – 17th 2025

Source: 	Fujitsu

Agenda item:	10.2

Title:	General considerations on 6GR

Document for:	Discussion and decision

# Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

In this contribution, we would like to provide our views on deployment models, radio architecture, and spectrum utilization as 6GR general aspects.

# Discussion

## Deployment models

In RAN#109, RAN plenary has approved to study whether any additional migration option(s) is needed [1]. This study will be started from March 2026, and RAN will make a decision in September 2026. It means RAN2 should focus on the standalone, MRSS and inter-RAT mobility between NR-6G unless RAN plenary requests the additional migration option(s).

Observation 1: RAN2 is required to start studying the following 6G migration scenarios:

- Standalone, MRSS and inter-RAT mobility between NR-6G

From RAN2 perspective, it may be useful to study and identify initial 6G deployment models. For example, it is relevant to inter-RAT mobility functions supported in Day1.

According to WRC-27 agenda item 1.7, candidate 6G bands will be mid-high frequency range (4400MHz ~ 4800MHz, 7125 ~ 8400MHz and 14.8 ~ 15.35GHz). It means that 6G cells could be typically used as a capacity layer cell at the first launch. In addition, standalone operation is required in 6GR [2]. Hence, considering a practical network, as shown in Figure 1, existing 5G NR cell(s) in a lower band would be converted to MRSS cell to ensure the same coverage as 5G NR.

Figure 1 - typical deployment model in 6G

However, it would be difficult to consolidate a single migration model as different operators may adopt different 5G/6G deployment strategies due to different operating bands. For example, because 4G LTE UEs might be significantly reduced in 6G era, then one out of lower bands currently used for 4G/5G may be refarming to 6G (but 4G band may remain for voice service unless operator decides to migrate voice to 5G). In this sense, the model where 6G dedicated band may be used for a coverage layer, and MRSS cell may be used to boost cell throughput should also be considered. Besides that, to support flexible deployment, both collocated and non-collocated deployments are also considered.

Proposal 1: RAN2 to study the following options for 6G deployment models:

- Coverage of 5G/6G MRSS cell is the same or larger than that of 6G cell.

- Coverage of 6G cell is larger than that of 5G/6G MRSS cell.

- 5G/6G MRSS cell and 6G cell are collocated or non-collocated.

On top of that, TN and NTN harmonization would also be considered in 6G deployment models. Figure 2 indicates three harmonization options, i.e., a) 6G NTN cell is overlapped with 6G TN cells, b) 6G NTN cell is not overlapped with 6G TN cells and c) 6G NTN cell is partially overlapped with 6G TN cells.

| a) 6G NTN cell is overlapped with 6G TN cells | b) 6G NTN cell is not overlapped with 6G TN cells |
| --- | --- |
| c) 6G NTN cell is partially overlapped with 6G TN cells |  |

Figure 2 - NTN cell deployment models in 6G

Option a) illustrates a deployment model where 6G NTN cell is fully overlapped with 6G TN cells. In Option a), handheld device (like smartphone) may not use NTN link, but sensing/monitoring devices may connect to the NTN cell even in the TN cell coverage. Also, NTN link can be used as a backup link when TN link is unavailable due to disaster. Option b) illustrates 6G NTN cell is isolated. This option can be used in case of non-TN coverage area, like maritime and/or island. Option c) illustrates 6G NTN/TN cells form the coverage area complementarily. In this option, NTN-TN mobility may be required to continue the communication services.

Proposal 2: RAN2 to study the following options in 6G TN-NTN harmonization:

- 6G NTN cell is overlapped with 6G TN cells.

- 6G NTN cell is not overlapped with 6G TN cells.

- 6G NTN cell is partially overlapped with 6G TN cells.

## 6GR overall architecture

Based on the updated SI description [1], RAN2 does not have to consider NR-6G DC and 6G-6G DC for 6GR architecture unless required. Assuming RAN internal functional split, including CU-DU and CP-UP separation, and definition of the interface between them will be primarily studied in RAN3. In this sense, RAN2 should first study on 6G standalone and 6G CA architecture as a starting point. For CA case, as described in section 2.1, both collocated and non-collocated deployments should be also supported.

In addition to 6G-6G CA case, we think CA between 6G dedicated cell and MRSS cell should be also supported for eMBB-like communication. Figure 3 shows an example of CA operation with/without MRSS. The left figure in option b) illustrates that 5G gNB should be upgraded to support MRSS operation, and new interfaces connecting with 6G RAN node both C-plane and U-plane. However, the legacy 5G gNB may not support new C-plane interfaces with 6G RAN. In this case, as shown in the right figure of option b), new 6G RAN node should support connection with 6G C-plane.

Observation 2:	Unless required by RAN, RAN2 does not consider NR-6G DC and 6G-6G DC for 6GR architecture.

Proposal 3:	From RAN2 perspective, spectrum aggregation between 6G dedicated carriers, and 6G and MRSS carriers should be supported in 6G Day1.

Option a) 6G CA operations without MRSS

Option b) 6G CA operations with MRSS

Figure 3 - 6G CA operations with/without MRSS

Consideration of open interface

In the past generations, such as 4G LTE, the most viable deployment was to use a single vendor. In that generation, generic products were not used in eNB side and hence multi-vendor operation was not considered widely. Now in 5G NR, vRAN concept using generic products are assumed in the network side, and hence more openness, so called Open RAN (O-RAN), drew big attention from the market. In that regard, open interface related requirement was approved in the last RAN#109 meeting: “3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability” in [2].

Given this situation, we believe the mechanism of 6G spectrum aggregation should be friendly for multi-vendor deployment, which is a difference from 4G and 5G. RAN internal architecture will be discussed in RAN3, thus RAN2 should jointly study with RAN3 to enable inter-vendor CA operability in 6GR. For example, RAN2 should study to alleviate a potential scheduling delay between inter-vendor nodes, etc.

Proposal 4:	RAN2 to strive for supporting inter-vendor CA operability in 6GR.

6G NTN architecture

In RAN#109, several companies required seamless service continuity for 6G NTN [3]. In 5G NR-NTN, as a satellite type, both NGSO and GSO have been supported, since the current satellite operators are operating different satellite types according to the different services. Therefore, it doesn’t make sense to select either one of them for a baseline of 6G NTN architecture. In this sense, 6G NTN should support both GSO and NGSO satellites.

For payload type, both transparent and regenerative payload (full on board) are already supported in 5G NR-NTN. The transparent payload does not require installing a base station on the satellite. On the other hand, the regenerative payload may reduce propagation delay between the base station and the UE compared to the transparent payload. The transparent payload may be operated for low-end devices as mentioned in the section 2.1. The regenerative payload may be adopted to support higher data rate communication for non-TN covered area. From RAN2 perspective, it is possible to support both of payload types because it is transparent from UE.

Proposal 5:	RAN2 to study 6G NTN architecture with the following assumptions:

- Support both GSO and NGSO satellites.

- Support both transparent and regenerative payload types.

## 6G spectrum utilization and aggregation

Spectrum aggregation in a single cell

One of the key issues in wireless communication is how to improve spectrum efficiency. In 5G NR, two types of multi-carrier operations are specified: Carrier Aggregation (CA) and Supplementary UL (SUL), where CA is primarily used to increase UE data rate and SUL is targeted for better UL coverage.

In 6GR, the operators would have more discrete spectrums across different bands. In TR 38.914 [2], 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR, and 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

From our perspective, one of the technologies to improve spectrum utilization and aggregation can be single cell with multiple carriers. In a single cell with multiple carriers, more than one uplink carriers and/or downlink carriers (CCs) can be included in one cell as shown in Figure 4. The CCs in the cell can be in the same band or in different bands, but it is preferred that they are in the same frequency range.

Figure 4 - A single cell with multiple CCs

In the single cell with multiple carriers, the PCI of the cell can be derived based on the synchronization signals in the DL carrier where SSB is transmitted. This technology has some benefits compared with legacy CA in 5G NR:

- Less common signal or channel which is beneficial for network energy saving and less radio resource overhead. For example, synchronization signals like SSBs, system information like SIB1/SIs and paging can be transmitted in one or part of the DL carriers. The other carriers can be used to e.g. provide capacity for eMBB traffic or higher reliability for URLLC traffic for UEs in RRC_Connected, and these carriers can be SSB-less carriers or can have on-demand SSB transmissions.

- Faster carrier switching or aggregation compared with legacy CA. In 5G NR carrier aggregation, the UE should be configured with one or more SCells first and then activate or deactivate an SCell based on the activation status in RRC reconfiguration message or the received SCell activation/deactivation MAC CE, which is slow and inefficient. In a single cell with multiple carriers, the switching or aggregation of the carriers in the cell can be performed by L1 signalling, like BWP switching in 5G NR, which is faster and can also save some signalling overhead.

- Random access distributed to carriers to select a better carrier based on UE traffic/type or reduce random access resource congestion. In a single cell with multiple carriers, the resource for random access can be configured in more than one carrier, so that the UE can select a carrier with low load or matching the UE’s traffic or type for random access.

- Flexible uplink and downlink carrier linking. In 5G NR carrier aggregation, the linkage between an uplink carrier and a downlink carrier is based on the cell configuration which is fixed. In a single cell with multiple carriers, the uplink carriers and downlink carriers can be linked flexibly based on the traffic load or coverage enhancements.

Based on the above mentioned benefits, we propose to study the technology of a single cell with multiple carriers in 6GR.

Proposal 6: RAN2 to study a single cell with multiple carriers in 6GR.

# Conclusion

In this paper we presented some initial views on RAN2 study directions for 6GR general aspects and have come up with the following observations and proposals:

Deployment models

Observation 1: RAN2 is required to start studying the following 6G migration scenarios:

- Standalone, MRSS and inter-RAT mobility between NR-6G

Proposal 1: RAN2 to study the following options for 6G deployment models:

- Coverage of 5G/6G MRSS cell is the same or larger than that of 6G cell.

- Coverage of 6G cell is larger than that of 5G/6G MRSS cell.

- 5G/6G MRSS cell and 6G cell are collocated or non-collocated.

Proposal 2: RAN2 to study the following options in 6G TN-NTN harmonization:

- 6G NTN cell is overlapped with 6G TN cells.

- 6G NTN cell is not overlapped with 6G TN cells.

- 6G NTN cell is partially overlapped with 6G TN cells.

6GR overall architecture

Observation 2:	Unless required by RAN, RAN2 does not consider NR-6G DC and 6G-6G DC for 6GR architecture.

Proposal 3:	From RAN2 perspective, spectrum aggregation between 6G dedicated carriers, and 6G and MRSS carriers should be supported in 6G Day1.

Proposal 4:	RAN2 to strive for supporting inter-vendor CA operability in 6GR.

6G spectrum utilization and aggregation

Proposal 5:	RAN2 to study 6G NTN architecture with the following assumptions:

- Support both GSO and NGSO satellites.

- Support both transparent and regenerative payload types.

Proposal 6: RAN2 to study a single cell with multiple carriers in 6GR.

# References

- RP-252912, Revised SID: Study on 6G Radio

- RP-252882, TR 38.914 v0.2, Study on 6G Scenarios and Requirements; (Release 20)

- RP-252943, Moderator's summary for RAN led 6G SI: Requirements of new and existing services