3GPP TSG-RAN WG2#131-bis	R2-2507133

Prague, Czech Republic, Oct 13th – 17th 2025

Source: 	Fujitsu

Agenda item:	10.3.3

Title:	Initial consideration on 6GR CP/UP common aspects

Document for:	Discussion and decision

Introduction

The SID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

The SID also describes the following objectives applied as a common function for 6GR CP/UP.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

In this contribution, we would like to provide initial views on 6GR CP/UP common aspects in terms of possible improvements/enhancements from NR.

Discussion

CP/UP Common function

AS security aspects

In 6G, Security and Resilience is still considered as one of the key requirements in IMT-2030 [2]. In 5G NR, security functions both encryption and integrity protection were located in the NR PDCP layer. They can protect the data provided from the PDCP layer or upper layers, but in other words, the data generated in the layers lower than the PDCP layer cannot be protected.

Observation 1:	The data generated in the layers lower than the NR PDCP layer can be neither encrypted nor integrity protected.

In NR, although RAN2 targets more secure protocol design than LTE, several security vulnerabilities were studied and reported due to unencrypted and non-integrity protected data in the lower layers. For example, RAN2/SA3 discussed a potential security risk called SLIC (Stealthy Location Identification Attack exploiting Carrier Aggregation [3]) due to non-encrypted SCell Activation/Deactivation MAC CE. This issue was finally addressed by the gNB implementation and there are no specification changes in RAN2/SA3, but there is still security risk as MAC CE is not protected in 5G NR. Given that, RAN2 should study to support even further secure protocol to block an exploitation of vulnerability and to reduce potential security risk.

In NR, the system information broadcast especially the MIB and SIB1 are not security protected. The MIB and SIB1 carry most essential system information required by the UE for camping and initial access to the cell. The rogue/false gNB problem was discussed in RAN2/SA3 during the 5G NR study item wherein UEs camping on the rogue/false gNB leads to denial of service. In 5G NR no solution for designed for the security protection of system information broadcast, with one argument that there is other simple mechanism adopted by rogue actors like jamming to enforce denial of service rather than deploying rogue/false gNB. Given that 5G NR opened the doors for new services like URLLC, private networks and now 6GR envisioning ISAC and AI/ML capabilities the rogue/false gNB problem remains valid. Therefore, RAN2 in collaboration with SA3 should revisit the rogue/false gNB issue and strive to protect the essential system information (MIB and SIB1). Given that other system information (OSI) may be provided on demand by default the UEs would then make such SI request to genuine gNB and not the rogue/false gNB.

Proposal 1: 	6GR should aim to design more secure protocol, including:

- applying encryption and/or integrity protection to the data generated in the layers lower than the PDCP layer.

- applying security protection to the essential system information broadcast. RAN2 to consult with SA3 on this aspect.

Data transfer design to support various types of data

In 5G, AI/ML has been studied and specified, but only AI for NW is specified, and the use cases are limited (e.g., Beam management, CSI feedback, positioning). However, in 6G, more use cases would be considered, and NW for AI might be introduced. All cases should be handled in one unified framework, so RAN2 should study the framework including a new data plane which is extendable to future use cases.

In 5G, the current protocol stacks have control plane (CP) and user plane (UP), and if either of these two planes is used for transferring data which is required for AI/ML, there are some issues described in [4]. For instance, if CP is used for model transfer, maximum size of RRC message would be a problem. On the other hand, if UP is used for model transfer, model management at gNB is difficult. Considering the unsuitability of CP and UP for AI/ML operation, RAN2 should study a new data plane which can connect the UE and involving entities. In addition, QoS management should be more flexible in this new data plane.

The same principle should be applied to Integrated Sensing and Communication. 6G network could provide sensing services by collecting sensing data, transmitting sensing data, processing sensing data, storing sensing data and support sensing results exposure to the third application platform. The sensing data may be produced by a UE or a network node. Similarly, the data transfer for sensing data can be achieved by the new data plane.

Proposal 2: RAN2 to study a new data plane which is suitable for AI/ML operation and/or sensing.

Energy efficiency and power savings

Energy efficiency (EE) is still global trend and therefore agreed as a TPR in ITU-R, then its requirement was also captured as KPI for sustainability in [2]. RAN1 has already started discussion about power model and EE in both 6G BS and 6G UE. In 5G NR, the NW and the UE had aim to target of power saving friendly design by reducing common signals and monitoring periods. However, in fact, the eMBB services were considered at the initial launch of 5G NR, thus UE power saving was studied after NR Rel-15 and several UE power saving functions (DRX, PEI, DCP/WUS and LP-WUS/WUR) were introduced during multiple releases. It consequently caused fragmentation of UE power saving functions.

In 5G practical deployments network energy saving is mainly achieved by switching off the capacity carrier when the network load is very low. This is typically the case when mobile operator has multiple overlapping carriers at his disposal to be deployed as coverage and capacity layer as per the operator’s carrier deployment strategy. All UEs are moved to the always-on carrier which is typically the coverage layer and the capacity carriers are switched-off. This is achieved in implementation-specific manner with the minimum standard support by moving the served UEs through handover, barring the new UEs from camping on the carrier for which switching off is triggered. In certain deployments, the carrier switching is feasible only in certain time periods (e.g. during night with less traffic) where the switch-off window is statically configured to realize the network energy saving. Without implementation-specific manner, NR Rel-18/19 NES was introduced to save the network power consumption. However, there were limited applicable scenarios, and it was less efficient power saving gain because there are legacy UEs in the cell need to be considered. For example, NES techniques would be applied only for connected mode UE or SCell case. Another example is on-demand SIB1, one NES technique introduced in Rel-19. It may reduce always-on common signalling (SIB1), but it requires a configuration for a UL wake-up signal from another neighbouring cell. It means on-demand SIB1 cannot be applied to the standalone cell. In initial 6G deployment, the operator may not have multiple carriers and when a single carrier is deployed for 6G in standalone mode then network energy saving may be limited with existing implementation-specific techniques or NES techniques similar to Rel-18/19.

Further, in NR, joint power saving mechanism may potentially increase power saving gain, but it was not clearly specified (up to NW implementation), because such power saving techniques have been specified as independent feature. One can argue that joint power saving is desirable but the motivations for network energy saving and UE battery conservation may have orthogonal objectives. For instance, network energy saving applies at cell level (i.e. applicable to all served UEs) whereas connected mode DRX is UE-specific depending on the traffic scheduled to the UE. NES techniques and UE battery conservation techniques may be independent or joint as along as the combination of the two can co-exist to serve the respective objectives.

Given the lessons learnt from NR, we propose the following target for EE and power saving.

Proposal 3:	RAN2 to study and decide fundamental and essential power saving functions in Day1 (both NW and UE side).

Proposal 4:	RAN2 to study efficient mechanism to enable independent or joint power saving between UE and NW.

- For joint power saving, Cell DTX/DRX and UE I/C-DRX alignment can be a good starting point.

NTN-TN integration

In 5G NR, both NTN and TN are considered as an independent network, thus they have been evolved without any functional harmonizations. Therefore, some solutions providing the similar effects were separately introduced (DL/UL coverage enhancements, for example). For NTN-TN harmonization, it would be better to eliminate such unnecessary functions.

Additionally, as illustrated in [5], in the deployment models that TN and NTN cells are overlapped or adjacent, NTN-TN mobility will frequently occur. Considering the moderator's summary for NTN [6], several companies have proposed seamless mobility as a requirement of 6G NTN. Because, in general, mobility between different architecture networks may cause signalling/configuration complexity and long service interruption, it will be beneficial to have NTN-TN unified protocol design in the initial phase of 6G development to avoid excessive configurations and achieve seamless mobility. Based on such assumptions, we should study how to harmonize the NTN unique characteristics with the normal TN functions, e.g., for NTN moving cell which is not considered for TN mobility.

Considering these points, RAN2 should study the unified protocol design for NTN and TN. As a starting point, RAN2 needs to study the functions to achieve the seamless mobility between NTN and TN cells.

Proposal 5: RAN2 to study the unified protocol design for both TN and NTN for the seamless mobility.

Conclusion

In this paper we presented some initial views on RAN2 study directions for 6GR CP/UP common aspects and have come up with the following observations and proposals:

Observation 1:	The data generated in the layers lower than the NR PDCP layer can be neither encrypted nor integrity protected.

Proposal 1: 	6GR should aim to design more secure protocol, including:

- applying encryption and/or integrity protection to the data generated in the layers lower than the PDCP layer.

- applying security protection to the essential system information broadcast. RAN2 to consult with SA3 on this aspect.

Proposal 2: RAN2 to study a new data plane which is suitable for AI/ML operation and/or sensing.

Proposal 3:	RAN2 to study and decide fundamental and essential power saving functions in Day1 (both NW and UE side).

Proposal 4:	RAN2 to study efficient mechanism to enable independent or joint power saving between UE and NW.

- For joint power saving, Cell DTX/DRX and UE I/C-DRX alignment can be a good starting point.

Proposal 5: RAN2 to study the unified protocol design for both TN and NTN for the seamless mobility.

References

- RP-252912, Revised SID: Study on 6G Radio

- TR 38.914, Study on 6G Scenarios and requirements

- R2-2104705, GSMA

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R2-2507131, General considerations on 6GR, Fujitsu

- RP-252943, Moderator's summary for RAN led 6G SI: Requirements of new and existing services