3GPP TSG-RAN WG2 Meeting #131bis		         R2-2507137

Prague, Czech Republic, 13-17 October 2025

Agenda Item:	9.7.2

Source: 	MediaTek Inc.

Title:  	Discussion on the Support of IMS Voice over NB-IoT NTN Connected to EPC

Document for:	Discussion and decision

# Introduction

In August meeting, SA2 sent an LS [1] on several issues about the support of IMS voice over NB-IoT NTN connected to EPC. More information is needed for SA2 to make decisions on two major topics: (1) UP vs. CP solution and (2) IP vs. non-IP solution. In this contribution, we review SA2’s questions to RAN2, and provide our views.

# Discussion

## Analysing the Questions

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

This question was raised due to potential concern about the robustness of RoHC in IP solution. If RoHC is not robust enough, SA2 may prefer non-IP solution. We provide some example numbers below (assuming k1=k2=n1=n2=1):

- When UO-0 header is used, the 4-bit SN represents values -1~14, so if SN=0 packet is delivered but the consecutive 14 packets are dropped, and then the decompressor receives the 15th packet, the decompressor will fail. → Having 14 packets lost may trigger RoHC state fallback (from SO state to FO state)

- When UOR-2 header is used, the 6-bit SN represents values -1~62, so if SN=0 packet is delivered but the consecutive 62 packets are dropped, and then the decompressor receives the 63rd packet, the decompressor will fail. → Having 62 packets lost may trigger RoHC state fallback (from FO state to IR state)

We observe that the number of consecutive packets lost needed to trigger RoHC state fallback at the compressor depends on the header type used. Up to 62 packet losses may be needed to trigger the fallback.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

Due to the long RTT for NTN (e.g., ~560ms for GEO) the SR/BSR-based dynamic scheduling (DS) may not be quick enough to schedule the real-time voice packets. Notice that we introduced semi-persistent scheduling (SPS) for NB-IOT in Release 15. SPS is tailored for voice services, as it schedules uplink transmissions periodically at regular intervals and thus avoids the signaling latency in DS.

Considering different sizes of IMS voice packets, over-scheduling may appear in when SPS is applied. Our understanding is that IP packets have more dynamic sizes (voice vs. silence packets, associated different RoHC states), while non-IP packets has relatively fixed size (voice vs. silence packets). We believe that over-scheduling should be tolerable to some extent, but the level of tolerance may affect the decision on IP vs. non-IP solution.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

We think this is feasible.

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

While it is technically possible, we do not prefer designing a new SRB for voice. Our major concern is the header size: Above RLC, CP needs 9 bytes (7 bytes for NAS and 2 bytes for RRC) for header and UP needs only 1 byte (PDCP). To make CP as efficient as UP solution, we need to introduce significant optimizations for CP (e.g., removing integrity protection for NAS message, or introducing ‘UM’ SRB). The standardization efforts

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

We prefer UP solution for voice.

Based on the above analyses, we propose the following answers to

Proposal 1: RAN2 provides to following answers to SA2’s questions to RAN2 in the LS:

- Q1: The number of consecutive packets lost needed to trigger RoHC state fallback at the compressor depends on the header type used. Up to 62 packets loss may be needed to trigger the fallback.

- Q3: Semi-persistent scheduling (SPS) can overcome the signalling delay due to long RTT in NTN. The voice packet size variation may cause over-scheduling when SPS is applied, but this should be tolerable to some extent.

- Q6: It is feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

- Q9: While it is technically possible, we do not prefer designing a new SRB for voice. The main concern is the standardization efforts required to significantly reduce CP header size.

- Q10: We prefer UP over CP solution.

## RAN2 Preferences

Regarding the two major discussion points, we believe UP solution should be adopted. We can leave the decision between IP and Non-IP solution to SA2.

Proposal 2:	RAN2 specifies that we prefer UP over CP solution and leave the decision between IP and non-IP solution to SA2.

# Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1: RAN2 provides to following answers to SA2’s questions to RAN2 in the LS:

- Q1: The number of consecutive packets lost needed to trigger RoHC state fallback at the compressor depends on the header type used. Up to 62 packets loss may be needed to trigger the fallback.

- Q3: Semi-persistent scheduling (SPS) can overcome the signalling delay due to long RTT in NTN. The voice packet size variation may cause over-scheduling when SPS is applied, but this should be tolerable to some extent.

- Q6: It is feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

- Q9: While it is technically possible, we do not prefer designing a new SRB for voice. The main concern is the standardization efforts required to significantly reduce CP header size.

- Q10: We prefer UP over CP solution.

Proposal 2:	RAN2 specifies that we prefer UP over CP solution and leave the decision between IP and non-IP solution to SA2.

# Reference

- S2-2507636, LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, August 2025