3GPP TSG-RAN WG2#131-bis	R2-2507140

Prague, Czech Republic, Oct 13th – 17th 2025

Source: 	Fujitsu

Agenda item:	8.5.3

Title:	Remaining MAC open issues on common signal/channel adaptation

Document for:	Discussion and decision

# Introduction

In RAN2#130 meeting, RAN2 discussed potential RA-RNTI collision issue in case of RACH adaptation. To resolve this issue, RAN2 agreed to re-interpret the f_id value and sent the LS to RAN1 [1]. In RAN1#122 meeting, it was discussed and the reply LS was sent back to RAN2 [2].

LS to RAN1 [1] RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. Therefore, RAN2 agreed there is no need to extend the RA-RNTI space for additional PRACH occasions and sees no RAN2 impact for RA-RNTI calculation nor other MAC impact.

Reply LS from RAN1 [2] RAN1 thanks RAN2 for providing the LS on RA-RNTI for PRACH adaptation. From RAN1 perspective, there is no proper place in RAN1 specifications to capture the impact on f_id since f_id is not defined in RAN1 specifications. The index in RAN1 specification is an index for physical signal generation and cannot be directly used for determination of f_id. RAN1 kindly requests RAN2 to take the above into account, as well as the below RAN1 agreements, with regards to the impact on f_id of valid RO in TS38.321: Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted - FFS: Clarification on configuration of legacy ROs Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

# Discussion

## RA-RNTI calculation

For the RA-RNTI calculation of the case that the additional ROs are overlapped with legacy ROs in time domain, RAN2 had made an agreement that there is no need to extend the RA-RNTI calculation method. RAN1, however, investigated that there is no proper place in RAN1 specifications to capture the impact on the parameter (i.e., f_id) as it is defined in RAN2 specification, and it is reflected into RAN1 LS reply [2]. Hence, RAN2 is required to solve this issue.

The UE having additional ROs configured by addl-RACH-Config-Adaptation can also recognize legacy ROs configured by RACH-ConfigCommon. Therefore, in order to differentiate the RA-RNTI between ROs configured by addl-RACH-Config-Adaptation and RACH-ConfigCommon, we propose the simplest method for the UE to consider both the ROs to determine the f_id. TP for TS 38.321 is provided in Annex.

Proposal 1: (MAC issue 1) RAN2 to confirm that f_id is reinterpreted for the additional PRACH resources as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. This should be added as a NOTE in TS 38.321.

# Conclusion

For the remaining MAC open issue on RACH adaptation, we made the following proposal:

Proposal 1: (MAC issue 1) RAN2 to confirm that f_id is reinterpreted for the additional PRACH resources as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. This should be added as a NOTE in TS 38.321.

# References

[1] R2-2504724

[2] R1-2506587

# Annex: Text Proposal for TS 38.321

TS 38.321 5.1.3 Random Access Preamble transmission <Unchanged part omitted> The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). NOTE X: if an RO selected for preamble transmission is configured by addl-RACH-Config-Adaptation and there are ROs in RACH-ConfigCommon that is not associated with addl-RACH-Config-Adaptation FDMed with the selected RO, f_id is reinterpreted as the index of the selected RO in the frequency domain plus the number of FDMed ROs configured by RACH-ConfigCommon that is not associated with addl-RACH-Config-Adaptation. <Unchanged part omitted>