3GPP TSG-RAN WG2 Meeting #131bis	R2-2507146

Prague, 13 – 17 October 2025

Agenda item:	10.3.2

Source:	Nokia

Title:	On 6G RRC design

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

A 6G study item for RAN was first approved in June 2025 [RP-251881], and subsequently updated in September 2025 [RP-252912].

The RAN2 work is often split to two broad categories: User plane and control plane. User plane exists to allow communication, while control plane exists to setup, maintain and release the user plane in a controlled manner primarily via the RRC protocol.

To improve on the 5G RRC, the 6G RRC design should be lean: RRC structure should be more scalable and modular (i.e. concise and easily decodable) while also enabling forward-compatibility (as RRC always must provide). To ensure successful 6G, RRC should aim to enable better power efficiency for UEs and networks, while also reduce RRC complexity to provide improved operational latency and smaller typical RRC message size.

Figure 1: Design goals of 6G RRC

In this document, we discuss the design criteria for 6G control plane, with specific focus on the RRC protocol aspects considering what was done in 5G. As the 6G study begins, it is imperative that we learn from the lessons of the past generations: What were the problems, what were the best practices, and what was perhaps not considered at all.

# 2	On 6G RRC design

## 2.1	History of RRC protocol design for 5G

5G RRC protocol design was started quite early on (the first discussions happened in RAN2#94, which was the second meeting of the study item), but was largely dominated by the EN-DC dual connectivity discussions, including whether there would be disparate RRC protocols for each link or not, and whether there would be a new specification for 5G RRC or not.

After the initial decision that a 5G RRC protocol would be created, the discussion focused on how to write an efficient RRC specification based on learnings from LTE RRC specification, e.g. RRC specific guidelines and structures from RAN2 viewpoint, see e.g. R2-1700292 and R2-1709886. However, as RAN1 input their first set of L1 parameters in RAN2#99bis (see R2-1713629 for the first NR RRC with initial L1 parameters incorporated), the specification complexity multiplied as RAN2 started delving into the structure that was determined in RAN1 discussions (and wasn’t always ideal, RAN2 having to interpret as well as possible). This led to RAN2 having to create a structure in parallel while the RAN1 inputs (including aspects related to the structure!) were still evolving, which made the task complicated.

Finally, the multiple 3GPP Release freezing points of NR (i.e. “early”, “SA” and “late” versions of Rel-15 freezing) required substantial rewriting of the RRC specification, and the rushed process led to several non-backward-compatible changes, which made the specification unstable for a lengthy period of time. This was not helped by the large size of 5G RRC specification (~600 pages in Rel-15, and ~1700 pages by Rel-18) and the ever-evolving UE features and UE capabilities definition, including some very late additions to fundamental concepts such as UE-specific carrier bandwidth (see RP-182896). However, due to the efforts put by all companies and rapporteur, the Rel-15 specification no longer had major non-backward compatible changes after RAN2#104 in December 2018.

There was no single reason for Rel-15 RRC complexity, but to name the most major contributors:

- Multiple official ASN.1 freezing points in Rel-15 created time pressure to “freeze” even when not everything was truly completed

- A multitude of UE features/capabilities were defined (especially in RAN1), many of which changed even after official freezing date(s)

- RAN1 defined an excessive amount of L1 parameters, and while RAN2 was tackling the initial inputs, the L1 parameters still evolved rapidly, leading to changes and mismatches between RAN1 and RAN2 understanding of the signalling.

- The configuration structure became dominated by the introduction of two 5G features: BWPs as a central theme for L1 configuration and introduction of MR-DC.

Many additional difficulties arose in later releases due to introduction of lower-capability devices (i.e. “RedCap” devices) and their particularities, as well as a massive upscaling in the configurability and complexity of 5G PHY. This led to very complicated configuration structure and massive configuration overheads, which has made RRC specification work ever more complex with each successive release.

Finally, while this is something that can only be decided during the WI phase, avoiding the multiple ASN.1 freezing points would benefit the RAN2 work by keeping it more focused.

Observation 1: Avoiding multiple ASN.1 freezing points for 6G would be helpful in avoiding the workload problems experienced during NR work for Rel-15 in RAN2.

## 2.2	Limitations of monolithic RRC structures

In UMTS, the RRC was dominated by a large message set whose ASN.1 structures were unrolled and refreshed (via critical extensions) in every release, leading to a long list of difficult-to-maintain sets of configurations that cause troubles if (human) errors occurred in the specification process.

In LTE, the RRC was greatly simplified and the use of critical extensions was curbed to avoid the worst complications from UMTS RRC, but the overall configuration structure was designed for single-carrier operation and started showing problems with the introduction of CA in Rel-10 (e.g. PCell and SCell structure and the role of system information configuration vs. dedicated configuration were unclear). The introduction of DC in LTE Rel-12 created additional complications for the user plane structure due to the introduction of split bearers, and as LTE work continued, it was also noticed that many issues were being created due to (improper or missing) release of configurations, leading to use of fullConfig in cases when it may not have been otherwise necessary.

In NR, RRC was cleaned and configuration split between CU- and DU-originating configurations (see Figure 2), but the overall structure was dominated by the MR-DC aspects and L1 structural complications. The PCell and SCell configurations were also improved, but the handling of the common and dedicated configurations continued to be elusive at times (e.g. if SIB1 configuration provides a conflicting configuration to that provided within dedicated configuration, which configuration does UE apply?). The introduction of the SetupRelease parameterization also rather successfully reduced the problems with the release of the configurations but didn’t truly address the issue that fullConfig reconfigurations could still be needed in many handovers due to differences in L1 configurations between updated and legacy network nodes. This has led to increased signalling overhead due to the large size of NR configurations, which can lead to link failures especially at cell edge conditions.

Figure 2: Schematic of the 5G RRC structure.

To improve further on the 5G RRC, it would be important to consider how to avoid the large message sizes. Studying how to split up the monolithic structure of RRC so that the typical configuration overhead is reduced would be necessary for 6G.

Proposal 1: Study how to enable a signalling structure for 6G RRC that reduces typical RRC configuration overhead.

## 2.3	BWP-dominated configuration structure

Large part of the 5G RRC structure was dominated by two aspects: BWP design and DC design (which also had to incorporate CU-DU split in the configuration). The former came from RAN1 wanting to make almost everything in L1 an attribute of BWPs, and the latter one came from having to incorporate both EN-DC and gNB disaggregation. This led to a very inter-connected structure within the RRC, as illustrated by Figure 3 below.

Figure 3: Illustrating deep nesting in 5G RRC as part of the (downlink) BWP configuration.

The deep nesting within the RRC specification was largely due to RAN1 decisions to make most L1 parameters a property of bandwidth parts, which in retrospect seems largely unnecessary and results in a lot of repetition in the configuration: Network will not broadcast L1 signals per bandwidth part but over the total cell bandwidth. What UEs receive will be based on those and even though some UEs may only listen to a part of the signals, they are still the same signals. BWPs became so complicated that Annex B.2 was added (by R2-1902539) to ensure it is clear how the BWP configuration works (due to e.g. having two different BWP configuration options stemming from the lack of clarity between common and dedicated configurations). Furthermore, the different BWP types (i.e. first active, initial, default, common and dedicated) seems superfluous and confusing.

Finally, L1 parameter reconfigurations are sometimes required, but BWP reconfigurations incur ~16ms user plane interruption as per TS38.133 section 8.6.3 even if the parameter in question would have nothing to do with RF retuning (e.g. release of SPS configuration has the same impact as retuning the RF bandwidth). The reason for this was the late inclusion of RAN4 in the discussions, and the fallacy of treating everything within BWPs as RF-impacting, which is obviously untrue.

Proposal 2: Study how to ensure necessary 6G L1 configuration changes do not create unnecessary interruptions to user plane operation. Study how to avoid excessive nesting of configurations in RRC structure.

## 2.4	RRC states

The major addition in 5G over LTE was RRC_INACTIVE state, which allows similar level power saving as RRC_IDLE at the UE side while allowing faster transition to RRC_CONNECTED state with earlier AS security activation compared to RRC_IDLE (due to network retaining the UE context while UE is in RRC_INACTIVE). However, the IDLE and INACTIVE states in Rel-15 are almost the same in terms of features: INACTIVE only supports RAN-paging and storing of the previous RRC configuration as additional aspects, otherwise the features are the same. This means that for networks to support the INACTIVE state, additional features (RAN paging and configuration storage) are required, which means that the first 5G deployments did not utilize INACTIVE as IDLE state could perform all the necessary functions needed for first deployments.

In terms of the functions of the RRC states, the table below illustrates the differences (and similarities):

| Requirement | IDLE | INACTIVE | CONNECTED |
| --- | --- | --- | --- |
| User plane operation | No (state transition needed) | Yes (SDT) | Yes (state purpose) |
| AS security | No (state transition needed) | Yes (SDT / resume) | Yes (requires activation) |
| Power saving | Yes (eDRX, paging DRX, UE-based mobility) |  | Yes (cDRX, bandwidth adaptation, SCell deactivation, …) |
| Paging | Yes (CN-based) | Yes (RAN-based) | Yes (SI changes etc.) |
| System information | Yes (PWS/CMAS/ETWS and cell camping) |  |  |
| RRC configuration | No (only SI changes) | Yes (SI + previous RRC config) | Yes (NW control) |
| Control plane operation (e.g. mobility) | NW-assisted (i.e. cell selection/reselection for mobility) |  | NW-controlled (i.e. handovers for mobility, RRC control of connection) |
| Stored UE context | No NAS or AS context (Initial attach) NAS (subsequent RRC connections) | NAS context + stored AS configuration | NAS + AS contexts |

Table 1. Simplified Comparison of RRC state functions

As the table illustrates, the IDLE and INACTIVE state share a lot of similarities, whereas the CONNECTED state has more differences. Given this, it seems reasonable to start from the 5G state model with the aim of

- Excellent power saving capabilities in all RRC states, while allowing latency-optimized connectivity from at least one state (e.g. INACTIVE)

- Reducing overlap between the states (i.e. define the purpose of the state clearly)

- Avoid duplicated functionalities (i.e. functionalities should be unique or differentiated in each state whenever possible)

Proposal 3: Study how to enable energy-efficient RRC states and low-latency state transitions in 6G. Baseline 6G RRC states should be based on 5G IDLE, INACTIVE and CONNECTED functionalities.

## 2.5	Small Data Transmission (SDT)

Smartphones with diverse applications and IoT devices, such as sensors, may need to send and receive small amounts of data frequently or infrequently. Networks, designed for high-bandwidth applications, are inefficient for these devices since they require RRC Connection setup/resume and release procedures for each data transmission, leading to high control signalling overhead. This burdens the network and drains devices battery life.

SDT procedures were specified in 5G to address the above challenges.  SDT procedure allows data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). ​SDT is enabled on a radio bearer basis and can be initiated either by the UE in case of MO-SDT (Mobile Originated SDT) or by the network in case of MT-SDT (Mobile Terminated SDT). MO-SDT is initiated by the UE only if less than or equal to a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available. ​MT-SDT is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for radio bearers configured for SDT; based on the indication, the UE initiates the MT-SDT only if the DL RSRP is above a configured threshold. When MT-SDT is initiated by the UE, a resume cause indicating MT-SDT is included in the RRCResumeRequest. ​Network can enable MO-SDT, MT-SDT, or both in a cell. The principles of 5G MO-SDT and MT-SDT procedures can be taken as a baseline for 6G SDT without introducing totally new small data transmission procedures.

The UE storing (part of) its previous configuration also allowed introduction of SDT in later releases to address the issue of smartphone applications sending regular small packets to network (e.g. instant messaging applications use such packets to check for latest messages), which avoids network signalling and is therefore more efficient if only few small packets are transmitted and UE is released back to INACTIVE mode from SDT (as show in Figure 4 below).

Figure 4: Example trade-off in network efficiency between SDT (“RRC INACTIVE”) and connection setup (“CONNECTED”)

With the NR stand-alone network deployments only now becoming more common and the deployment of RRC_INACTIVE having been delayed, SDT is not yet deployed in real networks, and its full operational efficiency is not fully known yet. However, theoretical calculations (at least for UEs staying in the same cell) look promising and given the prevalence of smart phones, it therefore seems useful to at least study how to enable such “fast user plane” operation such as SDT for 6G Day-1 deployments to get maximum benefits for power saving for both UEs and networks.

Proposal 4: Study how to enable energy-efficient user plane transmissions of small packets from INACTIVE state in 6G, using NR SDT design as baseline.

## 2.6	System information broadcast and paging procedures

System information (SI) plays a crucial role in cellular networks by broadcasting cell-specific information to all UEs and UEs are mandated to receive certain SIBs and maintain up-to-date information constantly. This information comprises of network configuration, available services/priorities and enables UEs to access the network. The increasing number and size of SIBs with each new release presents a challenge for system information management. While this growth is inevitable, it necessitates careful consideration of SIB extensions to maintain a manageable size and prevent excessive overhead. Furthermore, the scheduling information for all SIBs is carried within SIB1, creating a bottleneck for any additional SIBs added with each new release. The positioning feature added more SIBs and increased overhead and complexity to the SI scheduling process.

5G introduced the concept of on-demand system information to address the overhead associated with broadcasting SI, particularly in the higher frequency bands (FR2). While the deployments of this mechanism have been limited, the 3GPP work has continued with the aim to allow better network energy saving, which is crucial for success of 6G for operators, and the work in Rel-19 has focused on allowing on-demand SIB1 operation.

We would also note that the system information procedures have also been successfully used to provide public-safety information such as PWS (i.e. CMAS or ETWS) information. These should continue to be supported as they are part of regulatory requirements and therefore must be supported in 6GR (as is also discussed in R2-2507079).

Paging has also had extensive discussions after the initial 5G Rel-15: The Release-17 paging early indication (PEI), TRS availability indication and in Release-19 Low Power Wake-Up Signal (LP-WUS) have been aiming to reduce power consumption of UEs, and it is expected that RAN1 will study whether those would also be beneficial for 6G. The main difficulty in the design has been to ensure sufficient coverage, which will be crucial for 6G deployments. The Release-19 also introduced the paging adaptation mechanism to enable network energy savings, and it is crucial to integrate such energy-efficient enhancements as fundamental design. Any paging mechanisms should therefore consider the 5G work.

Proposal 5: Study how to enable energy-efficient system information transmission and paging operation to allow good power saving possibilities for both networks and UE. Baseline design should be based on 5G-A network and UE energy saving operation.

## 2.7	Simplified UE capability handling in 6G RRC

The 6GR SID outlines two distinct objectives for RAN2 related to UE capabilities: 1a) Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting and 3d) Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4].

The set of complementary objectives related to handling UE capabilities is collectively aiming to achieve an integrated solution in 6GR that supports a framework that is extensible and efficient, and one that overcomes issues faced in previous generations.

The specification of UE capabilities always evolves significantly over each release with the development of numerous and advanced features. Due to support for many frequency bands, band combinations, multiple radio access technologies (e.g., NR + LTE), and feature sets (e.g. for CA, MIMO, etc.), Control Plane messages carrying UE Capabilities can become very large. In NR capability framework, network requests filtered UE capabilities, but the capability sizes can still be large. The different types of UE capabilities (per-UE, per-band, per-band combination, per-FeatureSet, per-Carrier), also complicated NR capabilities, and the fragmentation of features into too many capabilities and granularities has led to significant overhead and made it hard to distinguish the relationships between these capabilities. 6GR should further aim to improve the capability signalling framework, considering the NR complexities discussed above.

NR has currently three RAN2 specifications defining UE Capabilities: TS38.331 (capability signalling in the RRC specification), TS38.306 (capability descriptions based on the signalling) and TR38.822 (feature group descriptions that are used at the end of release for creating the capability signalling).

The NR capability discussions have usually been separated from the “main” work item discussions, with separate CRs maintained since the discussions often diverge from the main technical discussions of the feature functionality. Therefore, in practice the capabilities are a separate topic from the rest of the RRC discussion. Maintaining all this information across different specifications has shown to be laborious and has led to numerous discrepancies between the three specifications (especially since RAN2 originally did not intend to maintain TR38.822 but later chose to start doing that). The TR is good for tracking, which Feature Groups are related to one another, but encoding of the Feature Groups in RRC makes it difficult to identify the relations. Therefore, moving the capability aspects from the main RRC specification into their own specification would both reduce the complexity of RRC specification and avoid some of the discrepancies requiring more efforts for specification maintenance. To simplify management of UE capabilities specification, it seems beneficial to re-consider the three complementary specifications approach to limit redundancy and improve efficiency, as shown in Figure 5.

Figure 5: Compacting UE capabilities to one specification

Proposal 6: Study scalable design of UE capability signalling (e.g. how to ensure reliable retrieval of decodable UE capabilities with less fragmented and complex UE capability signalling structure). Study how to capture all capability aspects in one specification to simplify specification of the 6G RRC.

# 3	Conclusions

We have discussed the 6G CP design and provided the following observations and proposals:

Observation 1: Avoiding multiple ASN.1 freezing points for 6G would be helpful in avoiding the workload problems experienced during NR work for Rel-15 in RAN2.

Proposal 1: Study how to enable a signalling structure for 6G RRC that reduces typical RRC configuration overhead.

Proposal 2: Study how to ensure necessary 6G L1 configuration changes do not create unnecessary interruptions to user plane operation. Study how to avoid excessive nesting of configurations in RRC structure.

Proposal 3: Study how to enable energy-efficient RRC states and low-latency state transitions in 6G. Baseline 6G RRC states should be based on 5G IDLE, INACTIVE and CONNECTED functionalities.

Proposal 4: Study how to enable energy-efficient user plane transmissions of small packets from INACTIVE state in 6G, using NR SDT design as baseline.

Proposal 5: Study how to enable energy-efficient system information transmission and paging operation to allow good power saving possibilities for both networks and UE. Baseline design should be based on 5G-A network and UE energy saving operation.

Proposal 6: Study scalable design of UE capability signalling (e.g. how to ensure reliable retrieval of decodable UE capabilities with less fragmented and complex UE capability signalling structure). Study how to capture all capability aspects in one specification to simplify specification of the 6G RRC.