3GPP TSG-RAN WG2 Meeting #131bis	R2-2507160

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.7.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Views on RIL050 and RIL051

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss two RIL issues identified during ASN.1 review process – RIL050 on RLC and RIL051 on XR rate control:

| RIL ID | Title | Status | Rapporteur Comments |
| --- | --- | --- | --- |
| V050 | Coexistence of remaining time based RLC retransmission and polling | ToDo | It is better to be further discussed with papers 1/ whether the two features of remaining time based polling and remaining time based RLC retransmission can be configured together 2/ if they are configured together, whether any restriction needs to be specified in the field description on the value of which field should be larger than the other one. |
| V051 | Restriction on ul-RateQueryConfigList-r19 | PropRej | It could be discussed in a paper whether such restriction is needed. Note that the issue was already proposed during the post meeting email discussion. And comment was received that this is not necessary |

# 2	Discussions

## 2.1	RIL-V050

[Description]: When both remaining time based RLC polling and remaining time based RLC retransmission are configured for the same Tx RLC entity, the threshold for enabling remaining time based RLC retransmission should be set as lower than that for enabling remaining time based RLC polling. Otherwise, the configuration for enabling remaining time based RLC polling will become useless since remaining time based RLC retransmission would have already been triggered before the UE polls the Rx RLC entity to request the STATUS report. [Proposed Change]: In the field description of remainingTimeThresholdRLC-Polling-r19, it is better to clarify this restriction, e.g. remaingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds. The network configures remaingTimeThresholdRLC-Polling to be lower than remainingTimeThresholdRLC-ReTx, if it is configured.. remainingTimeThresholdRLC-ReTx Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based retransmission as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds.. [Comments]: [Rapp] It is better to be further discussed with papers 1/ whether the two features of remaining time based polling and remaining time based RLC retransmission can be configured together 2/ if they are configured together, whether any restriction needs to be specified in the field description on the value of which field should be larger than the other one.

We have a similar understanding to allow configuring both thresholds while making sure that the polling is triggered before retransmission. However, the proposed update intends to do the opposite.

The remaining time considers the time before the discard timer expires, hence, to make sure that the polling is triggered before a retransmission, the remaingTimeThresholdRLC-Polling should be set to a value which is greater/longer than remainingTimeThresholdRLC-ReTx.

Proposal 1: [RIL-V050] We propose to allow configuring both thresholds (remaining time based RLC polling and remaining time based RLC retransmission) for the same RLC entity.

We need to make sure that in case both features are enabled/configured, the remaining timer-based polling is triggered before the remaining timer based re-transmission. To achieve this, the field description of remainingTimeThresholdRLC-Polling-r19 can be updated as below:

remainingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds. The network should configure remainingTimeThresholdRLC-Polling to be greater than remainingTimeThresholdRLC-ReTx, if it is configured.

## 2.2	RIL-V051

[Description]: When both ul-RateControlConfigList and ul-RateQueryConfigList are configured, the QoS flow configured in the rate query configuration should be the subset of QoS flow configured for the rate control. Otherwise, the QoS flow in the rate control query MAC CE is useless. Thus, it is better to provide such restriction. [Proposed Change]: In the field description of ul-RateQueryConfigList-r19, it is better to clarify this restriction, e.g. ul-RateControlConfigList Includes the list of QoS flows for which the UL rate control is supported.. ul-RateQueryConfigList Includes the list of QoS flows for which the UL rate query is supported. The QoS flow(s) configured in rate query should be the subset of QoS flow(s) configured for rate control.. [Comments]: [Rapp] It could be discussed in a paper whether such restriction is needed. Note that the issue was already proposed during the post meeting email discussion. And comment was received that this is not necessary

When RAN2 discussed whether to support query or not, it was understood that query could be disabled by simply not configuring the ul-RateQueryProhibitTimer. Based on this, RAN2 agreed to support query.

At that time, the intent seemed to be that one list of QoS flows is configured for rate control, with query enabled or disabled per QoS flow depending on whether the ul-RateQueryProhibitTimer is configured. This may imply that there was an assumption that the QoS flows that support query is subset of those that support rate control.

However, since two separate lists - ul-RateControlConfigList and ul-RateQueryConfigList (in ul-RateQuery-R19) – are introduced in RRC, and ul-RateQuery-R19 always includes ul-RateQueryProhibitTimer, disabling query for a particular QoS flow now requires releasing that QoS flow from the ul-RateQueryConfigList.

That said, the principle still holds that no prohibit timer is configured for a QoS flow for which query is not supported. Therefore, having two separate lists cannot be considered incorrect in itself. Nevertheless, the underlying assumption should remain and be clarified that the query list is a subset of rate control list. Moreover, we do not see any scenario where the gNB wants to enable query for a QoS flow while having no intention to apply the rate control to that QoS flow.

Proposal 2: [RIL-V051] Specify that ul-RateQueryConfigList is the subset of ul-RateControlConfigList.

# 3	Conclusion

RIL V050 and RIL V051 were addressed and the followings are proposed:

Proposal 1: [RIL-V050] We propose to allow configuring both thresholds (remaining time based RLC polling and remaining time based RLC retransmission) for the same RLC entity. We need to make sure that in case both features are enabled/configured, the remaining timer-based polling is triggered before the remaining timer based re-transmission. To achieve this, the field description of remainingTimeThresholdRLC-Polling-r19 can be updated as below:

remainingTimeThresholdRLC-Polling Remaining time threshold used by the PDCP entity to notify the RLC entity to trigger remaining time-based polling as specified in TS 38.323 [4]. Value for the IE RLC-AM-RemainingTimeThreshold in milliseconds. The network should configure remainingTimeThresholdRLC-Polling to be greater than remainingTimeThresholdRLC-ReTx, if it is configured.

Proposal 2: [RIL-V051] Specify that ul-RateQueryConfigList is the subset of ul-RateControlConfigList.