# 3GPP TSG-RAN2 Meeting #131bis	R2-2507172

# Prague, Czech Republic, Oct 13-17, 2025

Agenda Item:	10.3.2

Study Item:	FS_6G_Radio

Source: 	Qualcomm Incorporated

Title: 	Views on Control Plane for 6G

Document for:	Discussion/Decision

# Introduction

RAN#108 approved new SI for 6G Radio (6GR) which was further revised by RAN#109 [1]. One of the RAN2-led objectives is as follows (highlight added):

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

This document provides some initial views and proposals on Control Plane for 6G.

# Discussion

## RRC Signalling Encoding/Decoding

ASN.1 (Abstract Syntax Notation One) and PER (Packed Encoding Rules) are specified by ITU-T in the ITU-T Recommendations X.680, X.681 and X.691 (which are also same as various ISO/IEC International Standards).

So far 3GPP has been using ASN.1 UPER (Unaligned PER) for encoding of RRC messages. A pre-defined ASN.1 schema is used by both the transmitter and the receiver. The receiver can decode the ASN.1 signalling sent over the air interface using the pre-defined schema and the rules.

We looked at other potential encoding methods for 6G. However, they have their merits and drawbacks. For example, JSON, XML are big, unoptimized/verbose strings. Also, they are not precise. Protobuf is not an international standard, but a serialization format developed by Google.

ASN.1 is likely the most compact encoding method compared to the other options. Therefore, it seems reasonable to continue using ASN.1 for encoding of RRC signalling over the 6G air interface.

ASN.1 encoding/decoding is compact and has been working so far.

ASN.1 is used for encoding of RRC signaling for 6G air interface.

However, there are a few drawbacks of the ASN.1 as done so far

- Large parallel lists needing the same number of elements in the same order result in very large lists with potentially only a few elements present (while still including a very large number of elements with absent indications). Variable size parallel lists could be more optimized and preferred way of signalling in case only a few elements in a large list need to signal the extended values.

- Rules on conditional presence/absence are basically captured as comments, i.e. not directly encoded. Similarly, requirements such as elements to be equal and in the same order is captured in field description but not enforced by encoding.

- In a field with multiple possible candidate values, all candidates are treated equally (thereby taking the same number of coded bits) although in reality some values are far less likely than others to be used.

- Human readability is not great. This has improved over time with use of parameterized type macros but could be further improved.

There are some shortcomings of the current way of capturing ASN.1. Further enhancements are possible to reduce duplication, increase efficiency and improve readability.

Therefore, RAN2 should study whether further enhancements are possible to reduce duplication, increase efficiency and improve readability.

RAN2 will study possible enhancements aimed at reducing duplication, increasing efficiency and improving readability of ASN.1 for RRC signaling.

## Modular RRC

In 5G NR, following control channels are used for the transfer of control plane information

- Broadcast Control Channel (BCCH): a DL channel for broadcasting system control information è mapped to BCH (for MIB) or DL-SCH (for SIBs)

- Common Control Channel (CCCH): channel for transmitting control information between UEs (having no RRC connection) and network è mapped to DL-SCH/UL-SCH

- Dedicated Control Channel (DCCH): a point-to-point bi-directional channel that transmits dedicated control information between a UE (having RRC connection) and the network è mapped to DL-SCH/UL-SCH

- Paging Control Channel (PCCH): a DL channel that carries paging messages è mapped to PCH

Note, in addition to the above, DTCH is used to transfer user plane information, which is mapped to DL-SCH/UL-SCH.

These control channels are used to carry various RRC messages as depicted in the figure 1.

Figure 1 RRC message structure in 5G NR

Historically, new verticals were added in later releases, not from day one, e.g. eMTC, NTN, IAB, RedCap, eRedCap, XR etc. All the features (initial and later releases) use the same RRCReconfiguration message to indicate the (re)configuration parameters as depicted in Figure 1. The same main RRC message is extended over for future use cases, even though the use cases are very diverse. E.g. the same message carries the configurations for Normal MBB UE, NTN UE, ATG UE, IAB Node, BAP entity, UAV UE, SL L2 Relay UE, SL L2 Remote UE, RedCap UE, XR, etc.

The ‘extensions’ of the RRC messages in this way have various issues. First, the extension markers themselves create huge encoding overheads. In 4G/5G ASN.1, extension markers are also called ‘ellipsis’ and written as ‘…’ in the code. They add usually between 2 and 3 bytes for each use of the ellipsis, even for the cases where only a single bit flag needs to be signalled from the extension. This may not be issue for MBB-type UEs but can be a substantial problem for throughput-sensitive verticals such as IoT, NTN, or from the energy saving point of view.

Secondly, the overall RRC modelling/ design became somewhat ugly. E.g. the configuration for IAB is in the same message that is to be used by IoT UE.

Furthermore, in current RRC model, the UE would have to understand/parse the whole ASN.1 schema, even though it doesn’t support many features. Many of the fields would never be present in the dedicated signalling, however the signalling would need to encode the ‘absence’ at the transmitter, and the receiver would need to decode assuming the whole schema (and consider error case if invalid fields are included). In case of broadcast messages (e.g. SIB), the UE would need to parse everything and discard the irrelevant information. E.g. IoT UE would need to parse and discard IAB information elements.

Creating/implementing a tailored-RRC, e.g. a ‘thin’ RRC for low complexity IoT devices, using the current model is not straightforward.

One-size-fits-all RRC modelling is not efficient for diverse types of devices and use cases.

For 6G Radio, in our view to overcome the above shortcomings, RAN2 should aim to design RRC such that different type of devices can efficiently implement and support only the minimum required RRC. RAN2 should aim to model the 6G RRC as a Baseline RRC module with additional Vertical specific extensions as separate modules. Obviously, these names are used just for reference.

XR module

RedCap module

NTN module

Baseline RRC module

MBB module

…

+

6G RRC

Figure 2 Modular RRC design concept

With such goal in mind, all UEs need to support the baseline RRC module. Other module(s) are only supported by capable UEs. This means that for individual UEs, there would be no need to parse/decode the whole RRC messages and implement the whole RRC procedures. The optional modules can adapt to future use cases, while the baseline module do not need to change drastically.

The design philosophy could for example be as follows. Parameters/configurations common to all UEs would be part of the baseline module. Vertical-specific/use-case-specific parameters/configurations would be specified as part of the vertical-specific module i.e., future extensions common to all UEs/features continue to be included in the baseline module. Future extensions of same/similar features would be continued in the same module. Future features that are specific to a different use-case may be specified as a new module in a future release. Only UEs supporting the vertical(s) would implement/load/execute the vertical-specific RRC module(s).

As a design goal, RAN2 will aim for modular design of RRC for 6G, e.g. consisting of baseline module and additional vertical-specific/use-case-specific modules.

## (Re)Configuration Error Handling

RRC (re)configurations in 4G/5G can be broadly categorized as SIB-based (aka broadcast configuration) and dedicated (aka unicast configuration).

SIB-based is typically cell-specific configuration (as opposed to UE specific). This also conveys cell-specific NW capability. It is typically applicable for UEs in RRC_IDLE state (before they go to RRC_CONNECTED). Furthermore, this typically consists of a conservative default configuration regardless of UE radio conditions (e.g. cell edge vs cell centre) or traffic/vertical/use-case (while it is noted that different default of different UE types is possible e.g. Redcap vs non-RedCap).

On the other hand, dedicated (re)configuration is UE-specific. UE needs to first go to RRC_CONNECTED to get this. It can be provided in various RRC messages such as RRCReconfiguration, RRCRelease (with/without suspend), etc. Typically, it is a relatively fixed/static configuration based on NW-evaluated conditions (instead of UE local conditions such as traffic/vertical/use-case, speed, height, temperature, channel condition etc.).

As captured in the 4G and 5G specifications, RRC reconfiguration procedure is used to modify an RRC connection, e.g. to establish/modify/release RBs/BH RLC channels/Uu Relay RLC channels/PC5 Relay RLC channels, to perform reconfiguration with sync, to setup/modify/release measurements, to add/modify/release SCells and cell groups, conditional reconfiguration configuration, LTM configuration, multi-path configuration. NAS dedicated information may be transferred from the Network to the UE as part of this procedure.

| NR RRC reconfiguration, successful | NR RRC reconfiguration, failure |
| --- | --- |

Figure 3 NR RRC Reconfiguration

Although RAN2 specifications assume the NW to be very good at providing the right configurations, in the real deployments we have observed a tangible portion of reconfigurations are such that the UE cannot apply the whole configuration. The reasons for such inability can be various, e.g. hardware limitations, thermal state or UE energy/power level (XR-like high throughout and processing intensive operation may need to be stopped due to rise in temperature or depleted battery level), UE operating in MUSIM mode with active traffic on secondary subscription (resulting in reduced capability for primary subscription) etc.

Then, the problem is if a UE cannot apply some (part) of the NW-provided configuration, while a majority of the configuration is good, generally the whole configuration is useless. When problematic configurations are encountered, or when reconfiguration failure occurs, currently (in NR and LTE), the UE initiates connection re-establishment procedure (e.g. see TS 38.331 section 5.3.5.8.2). Even after the UE does connection re-establishment, there is no guarantee that the NW provided new configurations can be successfully applied. There is no efficient mechanism to indicate to the network which part of the configuration could not be applied by the UE.

In 5G, if a UE cannot apply some (part) of the NW-provided configuration, while a majority of the configuration is good, generally the whole configuration is useless. When problematic configurations are encountered, or when reconfiguration failure occurs, currently the UE initiates connection re-establishment procedure.

In 5G, there is no efficient mechanism to indicate to the network which part of the configuration could not be applied by the UE.

In 6G, the design principle should be such that UE should be allowed to keep/apply the good (part of) configuration. This is to enable the UE to avoid a re-establishment procedure (while applying the partial (good) configuration). Obviously, this cannot universally work in all scenarios and situations. So, to enable this, further discussion and design would be needed e.g. for the UE to indicate to the NW a part of configuration was invalid/couldn’t be applied while the rest of the configuration was ok.

6G design will allow the UE to keep/apply the good (part of) configuration and avoid re-establishment procedure.

## RRC States

In NR, Inactive state was introduced to reduce the state transition latency and signalling overhead. In Inactive state, UE is configured RNA and UE moves within the area configured by NG-RAN (the RNA) without notifying the NG-RAN. UE context is kept in the serving gNB and the UE-associated NG connection is kept active with the serving AMF and UPF. When DL data arrives to gNB, the gNB pages the UE within the RNA. When the UE moves out of RNA, UE needs to perform RNA update.

In the meantime, in NAS layer, UE performs registration area update procedure (e.g., upon moving outside the registration area) to maintain reachability at AMF. When DL data arrives at UPF in IDLE state, AMF can page the UE.

It can be observed that in 5G, two levels of location area management need to be implemented by the UE and network, which are AS level RNA update for Inactive state and NAS level Registration Area Update for IDLE state. In addition, two levels of paging procedures need to be implemented and tested. Also, UE monitors paging in IDLE and Inactive state using different paging parameters, e.g. (extended) DRX cycle, paging record ID. In IDLE state, UE needs to monitor CN paging using DRX parameters negotiated by UE and network (CN/RAN) and S-TMSI; in Inactive state, UE needs to monitor RAN paging using DRX parameters configured by RAN and I-RNTI.

Such different location area management procedures and paging monitoring procedures in IDLE and Inactive state require duplicate standards and implementation efforts in both the UE and the network.

In 5G, different location area management and paging monitoring procedures for Idle and Inactive require duplicate standards and implementation efforts in both the UE and the network.

Currently, even though Inactive mode is mandatory for the UE in NR, the commercial deployment of Inactive state has been very slow due to the following reasons.

- Having two separate modes where IDLE state is the default makes it challenging for an additional mode. Once the IDLE state is deployed, operators have to justify the benefit and cost of upgrading the system to support Inactive state, which brings obstacle to deploying Inactive state.

- The benefit of fast state transition introduced by Inactive state (e.g. low latency, less signalling overhead) was not critical in earlier networks, and features which are specific to Inactive, such as SDT, was not also commercialized. That does not provide enough motivation to upgrade the system to Inactive state.

So, it is obvious that having two separate states where IDLE state is the default makes it challenging for Inactive state commercialization.

It is evident from 5G that having two separate states where IDLE state is the default makes it challenging for Inactive state commercialization.

In 6G, there are potential three approaches to be considered:

  - Approach 1: Only have Idle state

  - Approach 2: Keep both IDLE and Inactive states as in NR design

  - Approach 3: Unify Inactive and Idle procedures as much as possible

Approach 1 is not preferred since the RRC state transition latency and signalling cannot satisfy various service performance requirements, e.g. small data transmission. Approach 2 has lessons learned as analysed above in Observation 6 and Observation 7. Then Approach 3 is worthy to be studied in 6G.

With Approach 3 of unifying Inactive and IDLE state procedure as much as possible, keeping the UE context always at the network should be less of a problem with the cloudification of networks; and unified paging and location area management procedures can reduce standard and UE/Network implementation efforts; the state transition with low latency and less signaling overhead could be easily deployed and features which relies on UE context during transition to Connected, such as SDT, can become more successful.

There are mainly three aspects to be considered for unifying IDLE and Inactive state as follows, and RAN2 can study the feasibility from these aspects.

- Unified location area management. Only one location area management should be supported in the state.

- Unified paging procedure. One set of paging parameters and one procedure should be used for UE to monitor paging.

- UE context storage in RAN and/or CN. To study where UE context should be stored in this state.

RAN2 studies the feasibility of unifying IDLE and Inactive state procedures from the following aspects:

- Unified location area management

- Unified paging procedure

- UE context storage in RAN and/or CN

## System Information

In NR, SIB1 is broadcast with the periodicity of 160ms to provide necessary information for UE camping and access on the cell. SIB1 mainly contains the following information:

| Main parameters in 5G SIB1 | Purpose | Necessary in SIB1? |
| --- | --- | --- |
| cellSelectionInfo | Cell selection | Yes |
| cellAccessRelatedInfo | Network discovery | Yes |
| connEstFailureControl | Connection control | No? |
| si-SchedulingInfo | SIB Schedule | Yes |
| servingCellConfigCommon DownlinkConfigCommonSIB UplinkConfigCommonSIB | Initial access | No? |
| uac-BarringInfo | Initial access | No? |
| Others (cell status, cell capabilities etc.) | Network status | Yes |

As shown in above table, currently SIB1 mainly contains the information for cell selection, network discovery, SIB scheduling, some initial access related information and network status related information. Among the information, network discovery, cell selection, SIB schedule and network status information is necessary because UE needs the information to find a suitable cell and to camp on that cell. However, in some cases, even after UE camps on a cell, UE may not perform access to the network. RAN2 can therefore study whether initial access related information is necessary to be included in SIB1 and rather be made available in other SI.

Another aspect that needs to be studied is, along with the introduction of service-based architecture, how to be compatible with service-based architecture and the mechanism to download a default set of system information from the server.

Furthermore, in current RAN sharing scenario, all the SIBs are shared by multiple PLMN(s), except UAC parameters. This does not provide differentiation in network management for different operators. RAN2 can study the feasibility of providing more SIB content differentiation for different PLMNs.

For system information modification, currently, when the network changes system information, it first notifies the UEs about this change (using Short Message transmitted with P-RNTI over DCI). Upon receiving a change notification, the UE needs to wake up to obtain SIB1 and check valueTag and area scope for other SIB(s) and acquire the updated SIB(s). Current mechanism is wasteful as it adds UE efforts to obtain SIB1 even if the UE is ultimately not interested in the updated SIB(s). RAN2 can consider enhancing the modification mechanism to reduce UE efforts of SIB1 acquisition.

As a summary, the following potential enhancements could be considered in 6G system information, and RAN2 is proposed to study the feasibility of the enhancement directions.

- Minimize the amount of mandatory broadcast information and make the delivery of other SI information more flexible.

- Support service-specific system information, and semi-static acquisition, e.g. download from server.

- Provide more SIB content differentiation for different PLMNs.

- Minimize UE wake ups due to uninterested SIB updates.

RAN2 studies System Information enhancement from the following aspects:

- Minimize the amount of mandatory broadcast information and make the delivery of other SI information more flexible.

- Support service-specific system information, and semi-static acquisition, e.g. download from server.

- Provide more SIB content differentiation for different PLMNs (e.g., in case of RAN sharing).

- Minimize UE wake ups due to uninterested SIB updates.

# Summary

Based on the discussion above, we have following observations and proposals:

Observation 1.	ASN.1 encoding/decoding is compact and has been working so far.

Observation 2.	There are some shortcomings of the current way of capturing ASN.1. Further enhancements are possible to reduce duplication, increase efficiency and improve readability.

Observation 3.	One-size-fits-all RRC modelling is not efficient for diverse types of devices and use cases.

Observation 4.	In 5G, if a UE cannot apply some (part) of the NW-provided configuration, while a majority of the configuration is good, generally the whole configuration is useless. When problematic configurations are encountered, or when reconfiguration failure occurs, currently the UE initiates connection re-establishment procedure.

Observation 5.	In 5G, there is no efficient mechanism to indicate to the network which part of the configuration could not be applied by the UE.

Observation 6.	In 5G, different location area management and paging monitoring procedures for Idle and Inactive require duplicate standards and implementation efforts in both the UE and the network.

Observation 7.	It is evident from 5G that having two separate states where IDLE state is the default makes it challenging for Inactive state commercialization.

Proposal 1:	ASN.1 is used for encoding of RRC signalling for 6G air interface.

Proposal 2:	RAN2 will study possible enhancements aimed at reducing duplication, increasing efficiency and improving readability of ASN.1 for RRC signalling.

Proposal 3:	As a design goal, RAN2 will aim for modular design of RRC for 6G, e.g. consisting of baseline module and additional vertical-specific/use-case-specific modules.

Proposal 4:	6G design will allow the UE to keep/apply the good (part of) configuration and avoid re-establishment procedure.

Proposal 5:	RAN2 studies the feasibility of unifying IDLE and Inactive state procedures from the following aspects:

-	Unified location area management

-	Unified paging procedure

-	UE context storage in RAN and/or CN

Proposal 6:	RAN2 studies System Information enhancement from the following aspects:

-	Minimize the amount of mandatory broadcast information and make the delivery of other SI information more flexible.

-	Support service-specific system information, and semi-static acquisition, e.g. download from server.

-	Provide more SIB content differentiation for different PLMNs (e.g., in case of RAN sharing).

-	Minimize UE wake ups due to uninterested SIB updates.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, RAN#109, Sep 2025