3GPP TSG-RAN WG2 Meeting #131bis	R2-2507174

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	MAC open issues

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we address several open issues identified during MAC open issue discussion based on the rapporteur’s summary:

Proposal 1: Revisit the UE operation in LP-WUS collision for Option 1-2, which should be based on critical issues in the current RAN2 agreements. (based on contribution)

Proposal 3: Confirm UE enables the LP-WUS monitoring only based on RRC configuration.

In addition, we would like to discuss another issue related to uplink resources based on the last agreement RAN2 made:

-	RAN2 confirm that the available UL occasions (e.g. SR occasion, RACH occasion, CG occasion) are MR-ready. Can further check whether any spec change is needed

# 2	Discussions

## 2.1	Collision handling and disabling of LP-WUS

RAN2 agreed in RAN2#131 that, for option 1-2, the UE does not start lpwus-PDCCH-MonitoringTimer in collision cases, while leaving room for further discussion if any critical issues arise. The assumption was that option 1-2 would typically use more frequent LP-WUS MOs to reduce latency compared to option 1-1.

During email discussion, one concern was raised that, the UE could miss 2-3 LP-WUS MOs and become unreachable when the LP-WUS MOs are sparsely configured. To address this, it was proposed to start the lpwus-PDCCH-MonitoringTimer in such case.

For normal collision cases, e.g., overlapping with MG, such fallback mechanism seems not essential, since the UE will resume LP-WUS monitoring. But when the UE cannot monitor LP-WUS at all (e.g., due to poor radio conditions), we believe careful design is needed to avoid the UE being stuck with non-functional LP-WUS monitoring. Otherwise, the UE may not receive any PDCCH scheduling and the gNB may not even be able to disable LP-WUS by releasing its configuration. In contrast, in option 1-1, since the drx-onDurationTimer is started in collision case, such a fallback behavior may not be necessary.

Menawhile, we consider the drx-InactivityTimer more appropriate than the lpwus-PDCCH-MonitoringTimer in this situation. The lpwus-PDCCH-MonitoringTimer is designed to start after the UE has received an LP-WUS, on the assumption that a PDCCH will follow, and therefore does not need to be long. By contrast, the drx-InactivityTimer is used when the presence of PDCCH is uncertain, so it can be typically configured with a longer duration, making it more suitable for fallback to PDCCH monitoring when LP-WUS does not work reliably.

Proposal 1: RAN2 discuss whether the UE should start PDCCH monitoring – by starting either drx-InactivityTimer or lpwus-PDCCH-MonitoringTimer – when the UE has not been able to monitor the LP-WUS MOs for some time.

During post email discussion, regarding enabling/disabling of LP-WUS MOs, one understanding seemed that enabling of LP-WUS is based on RRC, whereas disabling could be left to UE implementation. In detail, it was considered as one possible implementation that if the UE decides not to monitor LP-WUS for some reason, then the UE behaves as if the LP-WUS were detected at every LP-WUS MO, i.e., runs either drx-onDurationTimer or lpwus-PDCCH-MonitoringTimer.

It is not quite clear if this is RAN2’s common understanding. In our view, ‘disabling’ is disabling of the whole feature. Thus, if LP-WUS is disabled, it means that the UE goes back to normal C-DRX operation without LP-WUS feature configured (releasing LP-WUS). In addition, if the UE by implementation decides that it goes back to PDCCH monitoring, the UE cannot expect any PDCCH scheduling from the gNB since the gNB does not know whether the UE monitors the PDCCH, which complicates the overall LP-WUS and C-DRX operation at the cost of unnecessary power consumption.

As long as the collision cases or situations where LP-WUS monitoring is not possible can be handled with fallback mechanisms, we can rely only on RRC to disable LP-WUS operation.

Proposal 2: RAN2 confirm that UE disables the LP-WUS operation only based on RRC configuration.

## 2.2	UL aspect

In the last meeting, RAN2 agreed that:

RAN2 confirm that the available UL occasions (e.g. SR occasion, RACH occasion, CG occasion) are MR-ready. Can further check whether any spec change is needed.

This agreement raises the following two questions that require clarification:

-	Q1. Is the MR woken up even when there is no actual uplink transmission available on an SR or a CG occasion?

-	Q2. Are LP-WUS MOs that occur during the period between when uplink transmission becomes available and when uplink transmission is to be performed considered valid?

On Q1, given that CG and SR occasions are configured periodically, waking-up MR according to those periodicities seem inefficient especially when there is no actual data to send via CG/SR occasions. Moreover, considering the ramp-up time, such behaviour could even require complicated additional handling, e.g., determining not to go back to LP-WUS monitoring by comparing the CG/SR periodicity and ramp-up time. Therefore, it seems logical to assume that the UE wakes up MR only when SR or CG transmission is expected.

Proposal 3: RAN2 confirms that the MR does not need to wake up for an available CG or SR occasions if there is no actual uplink transmission on that CG/SR occasion.

On Q2, the question arises because the UE can still be outside of active time after triggering uplink transmission, e.g., uplink data becomes available, or SR is triggered, which could then trigger LP-WUS monitoring shortly before performing uplink transmission.

For example, after triggering an SR, there may be a LP-WUS MO (green box in Figure 1) occurring before the earliest SR occasion (red box in Figure 1). Under the current DRX mechanism, the UE only starts active time only after SR transmission, so the UE may still monitor LP-WUS on that LP-WUS MO (green box). In this case, depending on ramp-up time (could be up to 37ms for CONNECTED mode), the UE may not be able to wake up MR in time for the earliest SR occasion, and may end up sending the SR only at the next available SR occasion (yellow box).

Figure 1. An example case where LP-WUS monitoring impacts SR transmission.

Given that the UE already knows that SR is triggered, such monitoring of LP-WUS MO can be avoided easily by, e.g., specifying such LP-WUS MO as a part of collision case (TP1) or defining such duration as an additional active time (TP2).

Proposal 4: RAN2 discuss how to handle a LP-WUS MO that occurs before the uplink occasions, i.e., whether to ignore such LP-WUS MO (TP1) or to define additional active time (TP2).

In addition to CG/SR, similar issue can arise for SPS. In the current DRX mechanism, the UE can receive a MAC PDU on SPS even outside of active time. As the UE cannot predict whether it can receive a TB or not on SPS, it would need to always wake-up MR for SPS occasions. The question would be whether the UE monitors LP-WUS if there is LP-WUS MOs occurring between SPS occasions, i.e., not exactly overlapping with the SPS occasions. As the SPS case is similar to CG/SR case except that it should monitor all SPS occasions, we think the same rule can be applied to SPS.

# 3	Conclusion

We discuss some remaining MAC open issues and proposed:

Proposal 1: RAN2 discuss whether the UE should start PDCCH monitoring – by starting either drx-InactivityTimer or lpwus-PDCCH-MonitoringTimer – when the UE has not been able to monitor the LP-WUS MOs for some time.

Proposal 2: RAN2 confirm that UE disables the LP-WUS operation only based on RRC configuration.

Proposal 3: RAN2 confirms that the MR does not need to wake up for an available CG or SR occasions if there is no actual uplink transmission on that CG/SR occasion.

Proposal 4: RAN2 discuss how to handle a LP-WUS MO that occurs before the uplink occasions, i.e., whether to ignore such LP-WUS MO (TP1) or to define additional active time (TP2).

# 4	Text proposal

### TP1 to clause 5.7

… 1> if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is not configured, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or 1> if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number drx-StartOffset] modulo (drx-NonIntegerShortCycle)) = 0: 2> if the lpwus-PDCCH-MonitoringTimer is not configured: 3> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or 1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset: 2> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3: 3> if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or during a MUSIM gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. 2> else if LP-WUS monitoring is configured: 3> if lpwus-PDCCH-MonitoringTimer is not configured: 4> if LP-WUS indication associated with the current DRX cycle received from lower layer indicates to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 4> if the UE is unable to monitor all LP-WUS monitoring occasion(s), as specified in TS 38.213 [6], due to conflicts with other activities (e.g. all LP-WUS monitoring occasion(s) are associated with the current DRX cycle occurred in Active Time of any DRX group considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request triggered or uplink data becoming available for transmission on CG until [xx] ms prior to start of the first LP-WUS occasion, or during a measurement gap, or during a MUSIM gap or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4)): 5> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 2> else: 3> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if LP-WUS monitoring is configured and the lpwus-PDCCH-MonitoringTimer for this DRX group is configured: 2> if LP-WUS indication received from lower layer indicates to start lpwus-PDCCH-MonitoringTimer, as specified in TS 38.213 [6]; or 2> if the UE is unable to monitor a LP-WUS monitoring occasion(s), as specified in TS 38.213 [6], due to Scheduling Request triggered or uplink data becoming available for transmission on CG until [xx]ms prior to start of the first LP-WUS monitoring occasion(s): 3> start lpwus-PDCCH-MonitoringTimer from the beginning of the subframe as specified in TS 38.213 [6]. NOTE 2: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration. NOTE 2a: In case LP-WUS monitoring is configured and the lpwus-PDCCH-MonitoringTimer for this DRX group is configured, if the UE is unable to monitor LP-WUS monitor occasion(s), it does not start the lpwus-PDCCH-MonitoringTimer.

### TP2 to Clause 5.7

… When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer, drx-InactivityTimer or lpwus-PDCCH-MonitoringTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5), if LP-WUS is not configured. If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or - there is an ongoing RACH-less LTM cell switch; or - there is an ongoing RACH-less handover in a terrestrial network; or - a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]); or - a Scheduling Request is triggered and pending if LP-WUS monitoring occasion(s) occur before the Scheduling Request is sent.