3GPP RAN WG2 Meeting #131bis	R2-2507180

# Prague, Czech Republic,  October 13th – 17th, 2025

Agenda Item:	10.3.3

Source:	InterDigital, Inc.

Title:	Energy efficiency and AS security for 6GR

Document for:	Discussion

- Introduction

The study on 6G contains the following energy efficiency related objectives [1]:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Target scalable and forward compatible design for diverse device types.

The study item also has the following objective for AS security:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2]

This contribution outlines initial views on energy efficiency and the AS security frameworks for 6G.

- Energy efficiency

Energy efficiency aims to reduce energy consumption, emissions, and operational costs associated with cellular systems, by enabling UE and network energy savings. Energy efficiency can be achieved by adaptation of network and UE transmission and reception resources, while limiting the impact on user performance and experience.

## 2.1	Considerations and design targets

While NR provided some support for UE and NW energy saving, the following drawbacks and observations from NR should be considered for 6G:

- Fragmentation of supported techniques: NR introduced UE power saving starting in R16 and network energy saving (NES) features later in R18, never from day-1 in R15. Introduced features vary from one UE to another, as many UEs are not capable of energy and power saving techniques, which makes it difficult for the network to assume a common baseline that all UEs will assume for energy savings. This resulted in relying on cell barring for some UEs in order to support cell-wide energy savings techniques, which in turn makes some features less attractive to use if it only serves a subset of UEs.

- Backwards compatibility: Even though NR reduced always-on signals compared to LTE, some broadcast signals are still periodically broadcast at fixed periodicities, e.g. for PCells, to support legacy UEs. Among the network energy saving methods studied in the NR NES study item [2], few were considered for NR R18 and R19 specification, as others had considerable impact on legacy UEs. Many evaluated techniques with higher energy saving gains had impact on legacy UEs, e.g. for PCell deployments and due to legacy assumptions for cell common signals and channels. Most considered NES techniques were therefore limited to SCells or for UE dedicated signals and channels, without impacting common signals and channels or camped UEs in non-connected states. The energy saving gain from R18 and R19 NES techniques were therefore limited.

- Disjoint UE and network energy efficiency: NR introduced UE power saving and network energy saving features independently, and often UE-NW features were not designed jointly with interoperability in mind. This resulted in a suboptimal design for the integration of UE and network power saving, and in some cases duplicated or separate signalling to support such features independently, largely due to the fragmentation of supported techniques, as explained in point 1 above.

- Deployment issues: Some NW energy saving features in NR have seen limited field adoption due to backwards compatibility impacts, limited NES gains, or need for baring legacy and non-capable UEs in the cell deploying the NES feature. This can be alleviated if there are clear assumptions on which UE and NW power saving features are mandator, and if the 6g design target can made forward compatible to ensure minimal impact in future releases.

- Complexity: Some introduced features either were too complex to implement, or other features with high complexity resulted in limiting the amount of UE power saving.

- Scheduling and QoS impact: NR scheduling assumptions, including PDCCH monitoring, C-DRX, SR/BSR triggering, are largely based on the LTE protocol stack. These assumptions do not consider the delay budget tolerance that data has, in light of energy saving. NR introduced cell DTX/DRX based on a cell-wide periodic pattern for all UEs, but it does not consider the QoS requirements and attributes associated with the data or different UEs.

Observation 1: 	The following drawbacks from NR are observed for NW and UE energy efficiency:

- Fragmentation of supported techniques across different UE capabilities and releases

- Limited adoption of NES techniques due to backwards compatibility concerns for legacy UEs

- Disjoint UE and network energy efficiency, developed independently

- Limited deployment of NES features due to impact to legacy and non-capable UEs

- Complexity of some features and associated energy consumption impacts

- Limited consideration for impact of energy efficiency on scheduling and QoS.

To address these drawbacks, the following design targets for 6G should be considered:

- Predictability of UE supported techniques: 6G design in the first release needs to identify which techniques to be mandatory for all UEs. Considerable energy saving gains is not realizable without common support from all UEs. There needs to be an understanding of which day-1 power and energy saving features are considered mandatory (e.g. C-DRX, LP-WUS/WUR, PDCCH skipping, cell DTX/DRX, on demand common signals and channels).

- Scalability and forward compatibility: Energy efficiency features need to be possible to introduce and deploy without having to bar UEs of the first release of 6G. Therefore, the design of 6G should be forward compatible to allow initial release UEs to adapt to features deployed by the network later. A new 6G design thus provides prospects to support UE and network power savings natively without backwards compatibility constraints.

- Joint consideration of network energy and UE power saving: 6G should consider power and energy saving features jointly, to ensure alignment between the UE and NW and achieve most gains for both the UE and the NW. Therefore, 6G should ensure that UE and NW power saving features are designed with interoperability, and while consolidating features and associated signalling.

- Minimization of always on or periodic common signals: 6GR needs to be efficient from the perspective of minimizing transmissions of always-on signals, while not setting hard assumptions on periodicities of broadcast signals and channels.

- Modularity of protocol processing: The impact of scaling UP and CP protocol processing on UE and NW energy saving should be considered, regardless of which features are in use or supported by the UE and considering all UE types and capabilities.

- QoS-awareness: energy efficiency features in use should in general not result in a considerable impact on UE throughput, power consumption, or QoE. All supported verticals should be supported with limited impact while energy efficiency is applied, or the mobility framework should be adaptable to ensure QoS continuity once a serving cell cannot guarantee the UE’s QoS requirements.

- Energy consumption awareness in CP and UP protocols: as a general design target, the design of the UP and CP protocol stack should consider network and UE power consumption associated with protocol processing scalability.

Proposal 1: 	6G considers the following design targets for energy efficiency:

- Predictability of UE supported energy efficiency techniques by all UEs in cell.

- Scalability and forward compatibility, to allow adoption of later release features.

- Joint consideration of network energy and UE power saving

- Minimization of always on or periodic common signals

- QoS-awareness, including delay budgets, UE throughput, and QoE.

- Energy consumption awareness in protocols, considering UP and CP processing and scalability.

## 2.2	UE and network energy efficiency

From an L2/3 protocol perspective, energy efficiency techniques can generally be classified as follows:

- Control channel monitoring

Forward compatibility should be built in to allow adaptation of monitoring periodicities without having to resort to cell barring. This includes UE and network power saving features for LP-WUS/WUR, C-DRX, PDCCH skipping, cell DTX. These mechanisms should be mandatory for all UEs at day-1 to ensure predictability and allow network-wide assumptions.

Proposal 2: 	For energy efficiency in control channel monitoring, the following features are assumed from day-1 for all UEs: LP-WUS/WUR, C-DRX, PDCCH skipping, and cell DTX/DRX.

- Turn on/off and adaption of common and dedicated signals and channels:

These techniques address the need to minimize always-on periodic signals, and to ensure forward compatibility for deployment without negatively impacting first-release UEs. This includes adaption of common signals for one or more carriers (SSB-less cells, SSB period adaptation), turn on/off or broadcast channels (e.g. SIB-less, SIB adaptation), reception and requests for on-demand common signals and channels (e.g. on-demand SSB/RS/SIBs), time-domain adaptation of data and resources (PUSCH, PDSCH, PUCCH).

Most energy saving potential comes from increasing the time during which the network turns off, including transmissions, reception, and baseband processing. The network still consumes energy when not transmitting for other activities such as baseband digital beamforming processing. Such idle power consumption can be considerable in dense networks, e.g., during low cell load conditions. Network energy consumption can therefore be reduced when the network does not exchange transmissions with the UE.

Depending on cell load and impact on UE QoS performance requirements, it can be more beneficial to operate a cell in a cell DTX/DRX discontinuous periodic pattern, whereby the cell can be available only for a portion of slots in a radio frame. During the non-active period, it also desirable to adapt the availability of common and dedicated downlink signals and channels for both IDLE and connected mode UEs to realize most NW and UE energy savings, including SSBs, PDCCH, PDSCH, paging, and SI. Similarly for the uplink, common and dedicated uplink signals and channels (e.g. PRACH, PUSCH occasions, PRACH, SRS, UCI) can also adapt to the cell DTX/DRX pattern, while the network is not blind detecting on those UL resources. The UE’s C-DRX cycle configured in connected mode can also switched to align the network’s cycle.

Proposal 3:	For turn on/off and adaption of common and dedicated signals and channels, the following techniques are considered for NW energy saving:

- Adaption of common signals for one or more carriers (SSB-less cells, SSB period adaptation, common cell DTX/DRX cycle for connected and Idle modes)

- Turn on/off or broadcast channels (e.g. SIB-less, SIB adaptation, paging adaptation),

- Reception and requests for on-demand common signals and channels (e.g. on-demand SSB/RS/SIBs),

- Time-domain adaptation of data-associated and resources (PUSCH, PDSCH, PUCCH)

- Protocol processing:

These techniques reflect the modularity of protocol processing, where energy consumption awareness is incorporated into both UP and CP stacks, while also maintaining QoS-awareness. This includes impacts on user plane protocols from time domain adaption of user data and control resources, impacts on control plane protocols from time domain adaptation of cell common signals (SI acquisition, cell selection/reselection, paging), energy consumption associated with scaled integrity protection and QoS.

Proposal 4: 	Modular protocol processing is considered in 6G by aligning UP/CP procedures with time-domain adaptations of signals and resources, scaling of power consumption with bandwidth and data rates, and protocol stack awareness of energy consumption.

- AS security

## 3.1	Motivations for new security framework

Security in 3GPP based systems has been generally robust and improved from one generation to another. NR enhanced from LTE already by applying mandatory integrity protection (IP) for control plane messages (over SRBs) and added support for IP also for data (DRBs). However, the support of non-standalone architecture with EPC from the beginning limited some design choices for NR.

However, NR comes also with limitations in its AS security framework, especially, when some 6GR related requirements are accounted:

- Unprotected System Information & Paging: All system information (MIB & SIBs) and paging messages are sent in plaintext without integrity or authenticity verification. Consequently, rogue base stations could trick UEs into camping, enabling IMSI catcher or downgrade attacks. Furthermore, public safety alerts (e.g. ETWS/CMAS) could be spoofed making UE alerting the user about earthquake or terrorist attacks.

- MAC CE vulnerability: MAC CEs are transmitted without integrity or encryption. For example, an attacker could spoof or modify LTM cell switch command which opens path for denial-of-service or misdirection of the UE. Furthermore, C-RNTI is provided to the UE as part of Random Access procedure in plaintext enabling tracking of the UE unless C-RNTI is updated (e.g., during HO procedure).

- Optional UP integrity protection: UP IP exists in NR and is mandatory to be supported, however, its exploitation hasn’t materialized in practice due to overhead as well as computational complexity it creates for the UEs especially with high data rates. Applying UP IP is heavy and may get turned off purely to save battery/processing.

- Quantum safety: NR relies mainly on 128-bit symmetric encryption (e.g., AES-128, ZUC-128) and specified 256-bit algorithm options (NEA6/NIA6/NCA6) in Release-18 which are expected to be used more in the future to better protect from quantum attacks.

- Limited coverage of new data domains: NR AS security concept does not account for 6GR-native data flows, like AI/ML model transfer, sensing data, etc.

Observation 2:	NR AS security framework has some limitations which may be considered for 6GR: unprotected system information and paging, MAC CE vulnerability, optional UP IP, resiliency against quantum attacks, and limited coverage on new data domains (like AI/ML, sensing).

## 3.2	MAC security

MAC security has been discussed on occasions also for NR; lately the specification of LTM cell switch by the MAC CE raised the issue on the table (e.g., NCC value tampering possibly leading to HO failure). Furthermore, the C-RNTI assignment in plaintext as well as its usage as UE identifier without authenticity verification (e.g., in RA procedure) has been discussed in exposing security threat in the past. Generally speaking, MAC CEs could be used to alter UE operation in many ways that could in worst case lead to denial-of-service. In 6GR, it is envisioned that the MAC layer may be even more responsible in reconfiguring the UE in a fast manner, not necessarily limited only to the mobility scenarios. Additionally, the legacy scenarios, like identifying the UE in RA procedure and fast controlling UE lower layer behaviour through MAC control information, will continue to exist in 6GR. Hence, looking into better securing the control information transmitted over MAC layer is well motivated for 6GR.

First of all, it needs to be discussed whether the MAC security may be limited to integrity protecting, e.g., the MAC CEs to allow authenticity check of the received control information, or whether also the ciphering is required. Additionally, the study needs to conclude whether the security would be applied for all MAC control or only selective security, e.g., of specific MAC control information, is performed. Performing only selective security may alleviate the computational overhead introduced by securing the MAC control but may also increase the specification effort in deciding which MAC control needs to be secured as well as create overhead (e.g., in case each MAC control information is separately secured and an own MAC-I is appended for each separate control message). Additionally, the tight timing requirements in the MAC should not be affected by the security mechanisms. Furthermore, it should be discussed how and whether this has relation to securing data from upper layers – duplicate protection should be avoided to ensure computational overhead is not increased unnecessarily.

Consequently, the following design goals for MAC security could be listed for 6GR:

- Ensure authenticity for critical MAC control information while preserving the tight timing requirements in MAC.

- Avoid duplicating protection already provided by upper layers (e.g., PDCP) for upper-layer control.

Proposal 5:	Enable securing critical MAC control information starting from integrity protection and identify the MAC control that at least shall be secured. Identify whether ciphering beyond integrity protection needs to be supported.

## 3.3	Data security

NR AS security for both control and data is performed by PDCP. As said above, the ciphering and IP is always performed for securing control messages, however, for data, only the ciphering is mandatorily configured and IP may be optionally configured (while it is mandatory to be supported by all devices). PDCP has proven to be effective in terms of security enforcement given the data can be pre-processed by the UE and the NW before even the actual transmission opportunity is provided. Additionally, with CU/DU type of base station implementations, the PDCP is placed in the CU avoiding frequent security key updates while the UE moves from one cell to another.

As said above, for 6GR, the AS security framework should consider all the possible security layers, e.g., in case security capability is also included in MAC layer. Whether MAC layer is used for securing everything (ie., including data and control) or whether PDCP also performs parts/all of the functions (ciphering and IP) as before needs to be discussed. The PDCP security enjoys the privilege of being able to perform the security procedures “offline” while MAC security would mostly need to be performed upon transmission opportunity is indicated. Additionally, with CU/DU split, the MAC security key update will need to be performed much more frequently compared to PDCP security key. From that perspective, assuming the PDCP security for both IP and ciphering could be assumed as a solid baseline while, in case the MAC security is also enabled, the overall framework needs to be considered such that no duplication of security functionality would need to be performed.

While IP is a crucial component of secure communication and is expected to be more commonly used also for data in 6GR, it is not without challenges – it is one of the most processing hungry features in the radio layer; the computational overhead is introduced by the need to calculate and verify the MAC-I for each data packet. This can impact the overall performance, particularly in scenarios where a large volume of data is being transmitted. Additionally, there is always a trade-off between security and performance, hence, also the NGAP supports ue-max-integrity-protected-data-rate-reason capability indication allowing the network to enforce security policies based on the device's limitations. Higher security often requires more complex algorithms and longer key lengths, which can further increase computational requirements. Consequently, 6GR needs to consider both the security as well as trade-off between the computational and energy efficiency as well as means to better enable the actual usage of IP also for data. Selective application of IP for only a portion of packets or applying IP only for concatenated data units (e.g., at the PDCP or MAC layer) could facilitate the number of calculations the

transmitter/receiver needs to make and enhance the computational as well as energy efficiency.

Hence, the following design goals for 6GR AS security framework for data can be listed:

- Keep PDCP as baseline for data and upper layer confidentiality and integrity protection.

- Make UP integrity protection more deployable while mitigating battery/processing costs.

Proposal 6:	Assume PDCP security for ciphering and integrity protection as baseline for data and upper layer control.

Proposal 7:	Identify means to facilitate the computational overhead introduced by the UP integrity protection.

As discussed in our companion contribution [3], effective transfer of huge volumes of AI/ML and Sensing operational data requires support for supporting flexible routing, compression, and segmentation, bypassing reliance on conventional PDU sessions. The model of transferring such data needs to be considered in the overall security framework and should be accounted as early as possible in the design as well.

Observation 3: 	Identifying the impact of transferring huge volumes of AI/ML and Sensing operational data in the AS security framework should be done early in the study.

## 3.4	System information/paging security

In NR, all system information (MIB & SIBs) and paging messages are sent in plaintext without any protection, like integrity verification. This approach is vulnerable in many types of attacks, like rogue base station tricking UE into camping, faking access of a real UE by the S-TMSI/I-RNTI used in paging, faking public safety alerts making UE alerting user about earthquakes or terrorist attacks, etc. 6GR needs to support ever wider set of different use cases, services, and device types, like eMBB, IoT, NTN, AIML, sensing, slicing, enterprise scenarios, NES, etc. All these expose the need to consider whether more secure framework also for system information and paging is required.

Observation 4:	6GR needs to support ever wider set of different use cases, services, and device types requiring to consider whether more secure framework also for system information and paging is required.

For example, the following aspects may be considered in the 6GR design:

- Authenticity, integrity protection for system information and paging allowing UE to verify the received system information and paging.

- Selective visibility to make certain system information visible only to authorized UEs (e.g., for slicing and enterprise scenarios).

On the other hand, SA3 has conducted an extensive study previously on false base station (FSB) over multiple releases without normative conclusion [4]. Hence, rather than jumping into studying means for protecting the SI and paging by RAN2, input from SA3 would be needed if there was some new requirements in 6GR the study did not previously conclude and consequently would justify the work.

Observation 5: 	SA3 has previously conducted an extensive study on false base station (FSB) without normative conclusion. Hence, if any work were to be done to protect SI/paging messages, it should be first coordinated with SA3.

- Conclusion

This contribution outlines initial views on energy efficiency and AS security in 6G. The following observations and proposals are provided:

Observation 1: 	The following drawbacks from NR are observed for NW and UE energy efficiency:

- Fragmentation of supported techniques across different UE capabilities and releases

- Limited adoption of NES techniques due to backwards compatibility concerns for legacy UEs

- Disjoint UE and network energy efficiency, developed independently

- Limited deployment of NES features due to impact to legacy and non-capable UEs

- Complexity of some features and associated energy consumption impacts

- Limited consideration for impact of energy efficiency on scheduling and QoS.

Proposal 1: 	6G considers the following design targets for energy efficiency:

- Predictability of UE supported energy efficiency techniques by all UEs in cell.

- Scalability and forward compatibility, to allow adoption of later release features.

- Joint consideration of network energy and UE power saving

- Minimization of always on or periodic common signals

- QoS-awareness, including delay budgets, UE throughput, and QoE.

- Energy consumption awareness in protocols, considering UP and CP processing and scalability.

Proposal 2: 	For energy efficiency in control channel monitoring, the following features are assumed from day-1 for all UEs: LP-WUS/WUR, C-DRX, PDCCH skipping, and cell DTX/DRX.

Proposal 3:	For turn on/off and adaption of common and dedicated signals and channels, the following techniques are considered for NW energy saving:

- Adaption of common signals for one or more carriers (SSB-less cells, SSB period adaptation, common cell DTX/DRX cycle for connected and Idle modes)

- Turn on/off or broadcast channels (e.g. SIB-less, SIB adaptation, paging adaptation),

- Reception and requests for on-demand common signals and channels (e.g. on-demand SSB/RS/SIBs),

- Time-domain adaptation of data-associated and resources (PUSCH, PDSCH, PUCCH)

Proposal 4: 	Modular protocol processing is considered in 6G by aligning UP/CP procedures with time-domain adaptations of signals and resources, scaling of power consumption with bandwidth and data rates, and protocol stack awareness of energy consumption.

Observation 2:	NR AS security framework has some limitations which may be considered for 6GR: unprotected system information and paging, MAC CE vulnerability, optional UP IP, resiliency against quantum attacks, and limited coverage on new data domains (like AI/ML, sensing).

Proposal 5:	Enable securing critical MAC control information starting from integrity protection and identify the MAC control that at least shall be secured. Identify whether ciphering beyond integrity protection needs to be supported.

Proposal 6:	Assume PDCP security for ciphering and integrity protection as baseline for data and upper layer control.

Proposal 7:	Identify means to facilitate the computational overhead introduced by the UP integrity protection.

Observation 3: 	Identifying the impact of transferring huge volumes of AI/ML and Sensing operational data in the AS security framework should be done early in the study.

- References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).

- TR 38.864, “Study on network energy savings for NR”, Release 18

- R2-2507314, “On Framework for AI/ML and Transfer of Various Data Types”, InterDigital

- 3GPP TR 33.809 v18.1.0, “Study on 5G security enhancements against False Base Stations (FBS)”